3GPP TSG-RAN5 Meeting #101	R5-237284

# Chicago, USA, Nov 13-17, 2023

Agenda item:	5.3.13.9

Source: 	Qualcomm Inc

Title: 	Updates to DL1024QAM simulation results

Document for:	Discussion and Endorsement

## 1.	Introduction

In [1] simulation results for newly introduced fading propagation model TDLD-5 for DL1024QAM PDSCH Demodulation test cases based on the requirements defined in [5]. Those simulation needed to run for a longer period of time to let the throughput values converge. In this paper, results based on the extended simulation time is presented.

## 2.	Discussion

To recap, below is the procedure used to come up with the minimum test time for PDSCH throughput test cases.

- Run the PDSCH simulations for an extended period of time at the SNR of interest for multiple seeds.

- The minimum test time is determined by the time it took the throughput curve to settle within +/- 2% of the target throughput value across all seeds.

- If the throughput does not settle within the +/- 2% throughput limit within a reasonable amount of time for any seed, expand the threshold to +/- 3.5% of the target throughput value.

- To account for the additional 1.5% throughput variation, determine from the Throughput vs SNR curve, what is the SNR delta corresponding to this 1.5 % throughput increase.

- That SNR value is the added SNR uncertainty due to finite test time.

PDSCH simulations for all the defined DL1024QAM test cases in [4] were run.

Table 5.2.2.1.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-8 | R.PDSCH.1-17.1 FDD | 10 / 15 | 1024QAM, 0.79 | TDLD30-5 | 2x2, ULA Low | 70 | 29.5 |

Table 5.2.2.1.1_3.4-1: Test Requirements for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-8 | R.PDSCH.1-17.1 FDD | 10 / 15 | 1024QAM, 0.79 | TDLD30-5 | 2x2, ULA Low | 70 | 29.5+TT |

Table 5.2.2.2.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-12 | R.PDSCH.2-25.1 TDD | 40 / 30 | 1024QAM, 0.79 | FR1.30-1 | TDLD30-5 | 2x2, ULA Low | 70 | 29.4 |

Table 5.2.2.2.1_3.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-12 | R.PDSCH.2-25.1 TDD | 40 / 30 | 1024QAM, 0.79 | FR1.30-1 | TDLD30-5 | 2x2, ULA Low | 70 | 29.4+TT |

Figure1: PDSCH Throughput Simulations for FDD 2Rx at 70% target throughput SNR, 30 seeds

Figure2: PDSCH Throughput Simulations for TDD 2Rx at 70% target throughput SNR, 30 seeds

Figure3: PDSCH Throughput Simulations for FDD 4Rx at 70% target throughput SNR, 30 seeds

Figure3: PDSCH Throughput Simulations for TDD 4Rx at 70% target throughput SNR, 30 seeds

Below is the summary of the throughput observed across 30 seeds for the 4 scenarios at their respective convergence point.

| Scenario | Peak Tput (Mbps) | 70% of peak Tput (Mbps) | +/- 2% of 70% Tput (Mbps) | Highest Tput among seeds at convergence (Mbps) | Lowest Tput among seeds at convergence (Mbps) |
| --- | --- | --- | --- | --- | --- |
| FDD 2Rx Test 1-8 | 51.6 | 36.12 | 36.8 - 35.4 | 36.3 | 35.3 |
| FDD 4Rx Test 1-8 | 51.6 | 36.12 | 36.8 - 35.4 | 36.8 | 35.4 |
| TDD 2Rx Test 1-12 | 156 | 109.2 | 111.4 - 107.0 | 110.4 | 107 |
| TDD 4Rx Test 1-12 | 156 | 109.2 | 111.4 - 107.0 | 111.3 | 107 |

As seen from the Throughput plot and the summary, all the seeds converged within 2% of the target 70% throughput with max convergence of 70 seconds.

Hence adopt minimum test time of 70 seconds for all the 4 scenarios. Since we did not have to expand the throughput convergence limit beyond 2%, no additional MU factor SNR uncertainty due to finite test time needed for these TDLD DL1024QAM cases.

## 3.	Conclusion

Proposal 1: Adopt a minimum test time of 70 seconds for all 4 scenarios. SNR uncertainty due to finite test time for these cases = 0

## 4.		References

[1]	R5-235875, Simulation results for TDLD channels for DL1024 Demod test cases, Qualcomm

[2]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[3]	R5-194687, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	TS 38.521-4 v17.3.0

[4]	TS 38.101-4 v17.9.0