3GPP TSG-RAN5 Meeting #101	R5-237336

Chicago, USA, Nov. 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4094 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 6.4.2.3 for NR slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update the value of dl-ImplicitCarrierFreq IE for TC 6.4.2.3 Slice-based cell reselection in RRC_INACTIVE state / Reselection priorities provided by SIB16, as the value/remark of this IE is incorrect. Update the ‘FFS’ of FR2 power levels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the value of dl-ImplicitCarrierFreq IE from ARFCN to Integer type and replace the ‘FFS’ in FR2 power levels with ‘-82’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

6.4.2.3	Slice-based cell reselection in RRC_INACTIVE state / Reselection priorities provided by SIB16

6.4.2.3.1	Test Purpose (TP)

(1)

with { UE supporting slice-based cell reselection and in NR RRC_INACTIVE state }

ensure that {

when { UE detects the cell reselection criteria is met for two cells on different frequencies supporting different NAS-provided NSAG priorities  }

then { UE reselects to the cell on the frequency which supports higher priority NSAG and remains in RRC_INACTIVE state }

}

(2)

with { UE supporting slice-based cell reselection and in NR RRC_INACTIVE state }

ensure that {

when { UE detects the cell reselection criteria is met for two cells on different frequencies supporting same NAS-provided NSAG priority  }

then { UE reselects to the cell on the frequency which supports the NSAG having higher nsag-CellReselectionPriority and remains in RRC_INACTIVE state  }

}

6.4.2.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.304, clause 5.2.4.11. Unless otherwise stated these are Rel-17 requirements.

[TS 38.304, clause 5.2.4.11]

The UE derives reselection priorities for slice-based cell reselection by using:

-	NSAGs and their priorities provided by NAS,

-	sliceInfoList and or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in system information and/or dedicated signalling,

-	cellReselectionPriority per frequency provided in system information and/or dedicated signalling.

The UE considers an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency (see TS 38.331).

The UE considers a cell on an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency (see TS 38.331); and

-	the cell is either listed in the sliceAllowedCellListNR (if provided in the used slice-based cell reselection information) or the cell is not listed in the sliceExcludedCellListNR (if provided in the used slice-based cell reselection information); or

-	Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the used slice-based cell reselection information

The UE shall derive reselection priorities for slice-based cell reselection according to the following rules:

-	Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS.

-	Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency.

-	Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies).

-	Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority;

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause,

-	if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency);

-	Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

This re-derived reselection priority is used for a maximum of 300 seconds, or until new information of NSAG(s) and their priorities are received from NAS. UE shall ensure the cell reselection criteria above are fulfilled based on the newly derived priorities.

6.4.2.3.3	Test description

6.4.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 3 and NR Cell 6.

-	System information combination NR-30 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

6.4.2.3.3.2	Test procedure sequence

Table 6.4.2.3.3.2-1 for FR1 and Table 6.4.2.3.3.2-2 for FR2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.4.2.3.3.2-3.

Table 6.4.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -78 | Off | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 | -78 | The power level values are assigned to satisfy RNR Cell 3 < RNR Cell 6. |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.4.2.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Off | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -91 | -82 | The power level values are assigned to satisfy RNR Cell 3 < RNR Cell 6. |

Table 6.4.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-13 | Steps 2 to 13 of the generic procedure for NR RRC_INACTIVE specified in TS 38.508-1 subclause 4.5.3 are performed. | - | - | - | - |
| 14 | The SS transmits a REGISTRATION ACCEPT message | <-- | REGISTRATION ACCEPT | - | - |
| 15-20 | Steps 15 to 20 of the generic procedure for NR RRC_INACTIVE specified in TS 38.508-1 subclause 4.5.3 are performed. | - | - | - | - |
| 21 | The SS changes NR Cell 3 and NR Cell 6 SSS levels according to the row "T1" in table 6.4.2.3.3.2-1/2. | - | - | - | - |
| 22 | Void | - | - | - | - |
| 23 | Check: Does the UE transmit an RRCResumeRequest message on NR Cell 3? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 23A | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 23B | The UE transmit an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 24 | The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. | <-- | NR RRC: RRCRelease | - | - |
| 25 | The SS changes NR Cell 1 and NR Cell 6 SSS levels according to the row "T2" in table 6.4.2.3.3.2-1/2. | - | - | - | - |
| 26 | Void | - | - | - | - |
| 27 | Check: Does the UE transmit an RRCResumeRequest message on NR Cell 6? | --> | NR RRC: RRCResumeRequest | 2 | P |
| 27A | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 27B | The UE transmit an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 28 | The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. | <-- | NR RRC: RRCRelease |  |  |

6.4.2.3.3.3	Specific message contents

Table 6.4.2.3.3.3.3-1: REGISTRATION ACCEPT (step 14, Table 6.4.2.3.3.2-3)

| Derivation Path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NSAG information |  |  |  |  |
| NSAG information IEI |  |  |  |  |
| Length of NSAG information contents |  | 2 entities |  |  |
| NSAG 1 |  |  |  |  |
| Length of NSAG |  | 1 entity |  |  |
| NSAG identifier |  | ‘00000001’B |  |  |
| S-NSSAI list of NSAG |  |  |  |  |
| Length of S-NSSAI list of NSAG |  |  |  |  |
| S-NSSAI value 1 |  |  |  |  |
| S-NSSAI IEI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000001’B | SST value 1 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not Present |  |  |
| Mapped HPLMN SD |  | Not Present |  |  |
| S-NSSAI value 2 |  |  |  |  |
| S-NSSAI IEI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000010’B | SST value 2 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not Present |  |  |
| Mapped HPLMN SD |  | Not Present |  |  |
| NSAG priority |  | ‘00000011’B |  |  |
| TAI list |  | Not checked |  |  |
| NSAG 2 |  |  |  |  |
| Length of NSAG |  | 1 entity |  |  |
| NSAG identifier |  | ‘00000010’B |  |  |
| S-NSSAI list of NSAG |  |  |  |  |
| Length of S-NSSAI list of NSAG |  |  |  |  |
| S-NSSAI value 1 |  |  |  |  |
| S-NSSAI IEI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000011’B | SST value 3 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not Present |  |  |
| Mapped HPLMN SD |  | Not Present |  |  |
| S-NSSAI value 2 |  |  |  |  |
| S-NSSAI IEI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000001’B | SST |  |
| SST |  | ‘00000100’B | SST value 4 |  |
| SD |  | Not Present |  |  |
| Mapped HPLMN SST |  | Not Present |  |  |
| Mapped HPLMN SD |  | Not Present |  |  |
| NSAG priority |  | ‘00000010’B |  |  |
| TAI list |  | Not checked |  |  |

Table 6.4.2.3.3.3.3-2: SIB16 for NR Cell 1 (preamble and all steps, Table 6.4.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB16-r17 ::= SEQUENCE { |  |  |  |  |
| freqPriorityListSlicing-r17 SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17 { |  | 2 entities |  |  |
| FreqPrioritySlicing-r17 SEQUENCE{ |  |  | entry 1 |  |
| dl-ImplicitCarrierFreq-r17 |  | 0 | Downlink carrier frequency used for NR cell 1 |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { |  | 1 entry |  |  |
| SliceInfo-r17 SEQUENCE { |  |  | entry 1 |  |
| nsag-IdentityInfo-r17 SEQUENCE { |  |  |  |  |
| nsag-ID-r17 |  | ‘00000001’B |  |  |
| trackingAreaCode-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nsag-CellReselectionPriority-r17 |  | 5 |  |  |
| nsag-CellReselectionSubPriority-r17 |  | Not present |  |  |
| sliceCellListNR-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| FreqPrioritySlicing-r17 SEQUENCE{ |  |  | entry 2 |  |
| dl-ImplicitCarrierFreq-r17 |  | 1 | Downlink carrier frequency used for NR cell 3 |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { |  | 1 entry |  |  |
| SliceInfo-r17 SEQUENCE { |  |  | entry 1 |  |
| nsag-IdentityInfo-r17 SEQUENCE { |  |  |  |  |
| nsag-ID-r17 |  | ‘00000010’B |  |  |
| trackingAreaCode-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nsag-CellReselectionPriority-r17 |  | 4 |  |  |
| nsag-CellReselectionSubPriority-r17 |  | Not present |  |  |
| sliceCellListNR-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.4.2.3.3.3.3-3: SIB16 for NR Cell 3 (preamble and all steps, Table 6.4.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB16-r17 ::= SEQUENCE { |  |  |  |  |
| freqPriorityListSlicing-r17 SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17 { |  | 2 entities |  |  |
| FreqPrioritySlicing-r17 SEQUENCE{ |  |  | entry 1 |  |
| dl-ImplicitCarrierFreq-r17 |  | 0 | Downlink carrier frequency used for NR cell 3 |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { |  | 1 entry |  |  |
| SliceInfo-r17 SEQUENCE { |  |  | entry 1 |  |
| nsag-IdentityInfo-r17 SEQUENCE { |  |  |  |  |
| nsag-ID-r17 |  | ‘00000010’B |  |  |
| trackingAreaCode-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nsag-CellReselectionPriority-r17 |  | 4 |  |  |
| nsag-CellReselectionSubPriority-r17 |  | Not present |  |  |
| sliceCellListNR-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| FreqPrioritySlicing-r17 SEQUENCE{ |  |  | entry 2 |  |
| dl-ImplicitCarrierFreq-r17 |  | 1 | Downlink carrier frequency used for NR cell 6 |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { |  | 1 entry |  |  |
| SliceInfo-r17 SEQUENCE { |  |  | entry 1 |  |
| nsag-IdentityInfo-r17 SEQUENCE { |  |  |  |  |
| nsag-ID-r17 |  | ‘00000010’B |  |  |
| trackingAreaCode-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nsag-CellReselectionPriority-r17 |  | 6 |  |  |
| nsag-CellReselectionSubPriority-r17 |  | Not present |  |  |
| sliceCellListNR-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.4.2.3.3.3.3-4: SIB16 for NR Cell 6 (preamble and all steps, Table 6.4.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB16-r17 ::= SEQUENCE { |  |  |  |  |
| freqPriorityListSlicing-r17 SEQUENCE (SIZE (1..maxFreqPlus1)) OF FreqPrioritySlicing-r17 { |  | 1 entity |  |  |
| FreqPrioritySlicing-r17 SEQUENCE{ |  |  | entry 1 |  |
| dl-ImplicitCarrierFreq-r17 |  | 0 | Downlink carrier frequency used for NR cell 6 |  |
| sliceInfoList-r17 SEQUENCE (SIZE (1..maxSliceInfo-r17)) OF SliceInfo-r17 { |  | 1 entry |  |  |
| SliceInfo-r17 SEQUENCE { |  |  | entry 1 |  |
| nsag-IdentityInfo-r17 SEQUENCE { |  |  |  |  |
| nsag-ID-r17 |  | ‘00000010’B |  |  |
| trackingAreaCode-r17 |  | Not present |  |  |
| } |  |  |  |  |
| nsag-CellReselectionPriority-r17 |  | 6 |  |  |
| nsag-CellReselectionSubPriority-r17 |  | Not present |  |  |
| sliceCellListNR-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section>