3GPP TSG-RAN5 Meeting #101	R5-237383

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5264 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NTN NB-IoT TC 22.4.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MediaTek, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - If the UE supports re-establishment then the test case expects the UE to sends RRCConnectionReestablishmentRequest-NB at Step 27. The network in SIB2 shall indicate the support of cp_Reestablishment_r14 for the UE to trigger this. - If the UE supports pc_RRC_re_establishment_CP_CIoT, then the expectation of the current prose is to do a complete re-establishment procedure. According to 36.331 “RRCConnectionReestablishment-NB message is used to re-establish SRB1”. The current test case execution is for control plane and SRB1 is not configured. It is proposed to modify the test sequence to receive RRCConnectionReestablishmentRequest-NB and wait for T301 to expire and the UE goes into idle. The change requires an update to prose. - As the DL data sent in Step 24D may be buffered in UE memory, after step 26A a UE not supporting pc_RRC_re-establishment_CP_CIoT enters RRC Idle state and may initiate RRC connection setup procedure to send the MO non-SMS data. - It is also proposed to remove Note 1. There is no need to send the ESM Data at the end of T317 timer, its not influencing the TPs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added SIB2 contents to include cp_Reestablishment_r14 - Modfied the test cases to not respond to RRCConnectionReestablishment-NB and wait for 2.5s (T301) to expire. - The generic procedure 8.1.5A.3 was added for UE not supporting pc_RRC_re-establishment_CP_CIoT. - Removed the Note in Table 22.4.30.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is final version of R5-236582 Fixed editorial issues. |  |  |  |  |  |  |  |  |

### 22.4.30	NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry

#### 22.4.30.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType31-NB and starts timer T317 with the duration ul-SyncValidityDuration }

ensure that {

when { T317 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronization is lost and start T318 }

}

(2)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { SystemInformationBlockType31-NB is acquired successfully }

then { UE stops T318 and informs lower layers that the UL synchronization is restored }

}

(3)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { T318 expires }

then { UE considers radio link failure and initiates the connection re-establishment procedure }

}

#### 22.4.30.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.1.3, 5.2.2.4, 5.2.2.39, 5.3.18, 5.3.11.33 and 5.3.7.2; TS 36.321 clause 5.2a. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.1.3]

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

[TS 36.331, clause 5.2.2.39]

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

1>	start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

[TS 36.331, clause 5.3.18]

The UE shall:

1>	if in RRC_CONNECTED:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

3>	stop timer T318;

3>	inform lower layers that the UL synchronisation is restored;

NOTE:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

[TS 36.331, clause 5.3.11.3]

The UE shall:

…

- upon T318 expiry; or

…

2>	consider radio link failure to be detected for the MCG i.e. RLF;

…

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure either when AS security has been activated or for a NB-IoT UE supporting RRC connection re-establishment for the Control Plane CIoT EPS/5GS optimisation. The UE initiates the procedure when one of the following conditions is met:

[TS 36.321, clause 5.2a]

If upper layer informs that the UL synchronization is lost according to the clause 5.3.18 of TS 36.331 [8], the MAC entity shall:

-	flush all HARQ buffers;

-	not perform any uplink transmission.

If upper layer informs that the UL synchronization is restored for the SpCell according to the clause 5.3.18 of TS36.331 [8], uplink transmissions can be performed.

#### 22.4.30.3	Test description

##### 22.4.30.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 1-NB switched off according to TS 36.508 [18].

##### 22.4.30.3.2	Test procedure sequence

Table 22.4.30.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. | - | - | - | - |
| 3-18b1 | Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE MODE G (loopback of the User data container content of any received downlink ESM DATA TRANSPORT) are performed on Ncell 1. Note: After receiving the message in step 4, SS start a timer T1 with value = T317. | - | - | - | - |
| 18A-18B | Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. | - | - | - | - |
| 19 | SS stops broadcasting SystemInformationBlockType31-NB. | - | - | - | - |
| 19 A | SS stops periodic grant allocation | - | - | - | - |
| 20 | The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. | <-- | NAS: ESM DATA TRANSPORT | - | - |
| 21-22 | Void | - | - | - | - |
| 23 | Wait until expiry of T1. Note: UE starts timer T318. SS start a timer T2 with value = T318. | - | - | - | - |
| 24 | The SS starts the UL periodic grant transmissionallowing the UE to return the IP packet as received in step 20. | - | - | - | - |
| 24A | SS resumes broadcasting SystemInformationBlockType31-NB. Note: Timer T318 shall be stopped and T317 shall be started in UE side. | - | - | - | - |
| 24B | Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24? SS starts a timer T3 with value = T317. | --> | NAS: ESM DATA TRANSPORT | 2 | P |
| 24C | SS stops broadcasting SystemInformationBlockType31-NB. | - | - | - | - |
| 24CA | SS stops periodic grant allocation. | - | - | - | - |
| 24D | The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. | <-- | NAS: ESM DATA TRANSPORT | - | - |
| 24E | Wait until expiry of T3. Note: UE starts timer T318. SS starts a timer T4 with value = T318. | - | - | - | - |
| 24F | SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. | - | - | - | - |
| 25 | Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? | --> | NAS: ESM DATA TRANSPORT | 1 | F |
| 26 | Wait until expiry of T4. | - | - | - | - |
| 26A | SS resumes broadcasting SystemInformationBlockType31-NB. | - | - | - | - |
| - | EXCEPTION: Steps 27-28 describe behaviour that depends on whether the UE supports RRC connection re-establishment. | - | - | - | - |
| 27 | IF pc_RRC_re-establishment_CP_CIoT Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? | --> | RRCConnectionReestablishmentRequest-NB | 3 | P |
| 28 | The SS does not respond for 2.5s (T301) and the SS shall handle any re-transmitted RRCConnectionReestablishmentRequest-NB message on Ncell 1 | - |  | - | - |
| 29 | Void | - | - | - | - |
| 30 | Void | - | - | - | - |
| 31 | The SS starts timer T5 = 30 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 32a1 to 32b1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. | - | - | - | - |
| 32a1 | The UE transmits an RRCConnectionRequest-NB message on SRB0. | --> | RRCConnectionRequest-NB | - | - |
| 32a2 | Stop timer T5. | - | - | - | - |
| 32a3 | The UE initiation of Control Plane CIoT MO user data transfer non-SMS transport' as described in TS 36.508 [18], clause 8.1.5A.3 takes place. NOTE: The UE will transmit one ESM DATA TRANSPORT message containing user data matching the message sent in step 24D. | - | - | - | - |
| 32b1 | Timer T5 expires. | - | - | - | - |

##### 22.4.30.3.3	Specific message contents

Table 22.4.30.3.3-1: Void

Table 22.4.30.3.3-2: SystemInformationBlockType31-NB (steps 2, 24A and 26A, Table 22.4.30.3.2-1)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ul-SyncValidityDuration-r17 | s60 | Value of T317 |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.30.3.3-3: CLOSE UE TEST LOOP (step 18A, Table 22.4.30.3.2-1)

| Derivation path: 36.508 Table 8.1.5.2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup | TL_MODE_G |
| Operation mode and repetitions |  |  |  |
| M0 | 0 |  | return_via_EMM_SMC |
| R6..R0 | '0000001'B | 1 The received DL message in uplink shall be looped back 1 time (once) |  |
| Uplink data delay | '00000000'B | T_delay_modeGH timer = 0 sec |  |

Table 22.4.30.3.3-4: SystemInformationBlockType2-NB for Ncell 1, (preamble and all steps)

| Derivation Path: 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t301-r13 | ms2500 |  |  |
| t311-r13 | ms10000 |  |  |
| } |  |  |  |
| cp-Reestablishment-r14 | true |  |  |
| } |  |  |  |