3GPP TSG-RAN5 Meeting #101	R5-237401

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4002 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to FR2 Power level table for 8.1.5.10.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu EMEA Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Updated FR2 Power Tables |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated FR2 Power Tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tests would not be available for FR2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

##### 8.1.5.10.4	UE Assistance Information / RRM measurement relaxation / RedCap

Note: This test case is only applicable to certain specific UE implementation of RRM Relaxation.

8.1.5.10.4.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is fulfilled and the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment }

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled }

}

(2)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is not fulfilled and the last UEAssistanceInformation message indicated fulfilment of the criterion with rrm-MeasRelaxationFulfilment as true}

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled }

}

(3)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is fulfilled and the last UEAssistanceInformation message indicated the criterion is not fulfilled with rrm-MeasRelaxationFulfilment as false}

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled}

}

8.1.5.10.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.304 clause 5.2.4.9 and TS 38.331 clause 5.7.4

[TS 38.304 clause 5.2.4.9]

The relaxed measurement criterion for a stationary RedCap UE is fulfilled when:

-	(SrxlevRefStationary – Srxlev) < SSearchDeltaP-Stationary,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRefStationary = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRefStationary) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Stationary:

-	The UE shall set the value of SrxlevRefStationary to the current Srxlev value of the serving cell.

…

The relaxed measurement criterion for a stationary RedCap UE not at cell edge is fulfilled when:

-	the relaxed measurement criterion in clause 5.2.4.9.3 is fulfilled for a period of TSearchDeltaP-Stationary,

-	Srxlev > SSearchThresholdP2, and,

-	Squal > SSearchThresholdQ2, if SSearchThresholdQ2 is configured.

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

[TS 38.331 clause 5.7.4]

…

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to send indications of RRM measurement relaxation criterion fulfilment:

2>	if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected:

3>	if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or

3>	the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled;

2>	else:

3>	if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

…

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication about whether the criterion for RRM relaxation for connected mode is fulfilled or not fulfilled:

2>	if the criterion for RRM measurement relaxation for connected mode is fulfilled:

3>	set the rrm-MeasRelaxationFulfilment to true;

2>	else:

3>	set the rrm-MeasRelaxationFulfilment to false.

…

The relaxed measurement criterion for a stationary UE is met when:

-	(SS-RSRPRefStationaryConnected – SS-RSRP) < SSearchDeltaP-StationaryConnected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the PCell based on SSB (dB).

-	SS-RSRPRefStationaryConnected = reference SS-RSRP value of the PCell cell (dB), set as follows:

-	At the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or

-	If (SS-RSRP – SS-RSRPRefStationaryConnected) > 0; or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected:

-	The UE shall set the value of SS-RSRPRefStationaryConnected to the current SS-RSRP value of the serving cell.

8.1.5.10.4.3	Test description

8.1.5.10.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	UE supporting RedCap

-	UE supporting RRM measurement relaxation

-	UE supporting initiation of UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.10.4.3.2	Test procedure sequence

Table 8.1.5.10.4.3.2-1-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", “T2” and "T3", are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.10.4.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Power level is such that measurement relaxation Stationary criterion is fulfilled |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | Power level is such that measurement relaxation Stationary criterion is not fulfilled |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Power level is such that measurement relaxation Stationary criterion is fulfilled |

Table 8.1.5.10.4.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that measurement relaxation Stationary criterion is fulfilled |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Power level is such that measurement relaxation Stationary criterion is not fulfilled |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that measurement relaxation Stationary criterion is fulfilled |

Table 8.1.5.10.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to setup RRM measurement relaxation | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 4 | The SS waits for 5s (for t-SearchDeltaP-Stationary-r17 to expire). | - | - | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 7 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “false”? | --> | NR RRC: UEAssistanceInformation | 2 | P |
| 8 | The SS re-adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 9 | The SS waits for 5s (for t-SearchDeltaP-Stationary-r17 to expire). | - | - | - | - |
| 10 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 3 | P |

8.1.5.10.4.3.3	Specific message contents

Table 8.1.5.10.4.3.3-1: RRCReconfiguration (Step 1, Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| ul-GapFR2-PreferenceConfig-r17 | Not present |  |  |
| musim-GapAssistanceConfig-r17 | Not present |  |  |
| musim-LeaveAssistanceConfig-r17 | Not present |  |  |
| successHO-Config-r17 | Not present |  |  |
| maxBW-PreferenceConfigFR2-2-r17 | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB3 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| } |  |  |  |
| propDelayDiffReportConfig-r17 | Not present |  |  |
| } |  |  |  |
| sl-L2RelayUE-Config-r17 | Not present |  |  |
| sl-L2RemoteUE-Config-r17 | Not present |  |  |
| dedicatedPagingDelivery-r17 | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 | Not present |  |  |
| musim-GapConfig-r17 | Not present |  |  |
| ul-GapFR2-Config-r17 | Not present |  |  |
| scg-State-r17 | Not present |  |  |
| appLayerMeasConfig-r17 | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.4.3.3-2: UEAssistanceInformation (Steps 5 and 10,Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rrm-MeasRelaxationFulfilment-r17 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.10.4.3.3-3: UEAssistanceInformation (Step 7,Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rrm-MeasRelaxationFulfilment-r17 |  | false |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Change>