3GPP TSG-RAN5 Meeting #101	R5-237421

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2963 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to USIM configuration for 5G eCall over IMS test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition of the contents of file EFFPLMN for USIM configuration 6.4.1-26 (referenced by 38.523-1 11.5.5) states “The HPLMN (MCC+MNC) of the IMSI is set to PLMN4”. However the beginning of clause 6.4.1 states “PLMN settings are defined in TS 36.523-1 [42] Table 6.0.1-1” – this table gives a different definition of “PLMN4” than the one in the test case (38.523-1 11.5.5.3.1) so the SIM FPLMN list will not match the PLMN used in the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Give the PLMN ID 004-31 (defined as PLMN4 in test case 11.5.5) directly for FPLMN file (also correct the wording to remove the reference to IMSI). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail 38.523-1 11.5.5. test case due to incorrect SIM file contents |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s220679 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-236625-> R5-237421 |  |  |  |  |  |  |  |  |

### 6.4.1	General

The default USIM fields are specified in section 4.8.3. Specific USIM fields are set according to the USIM configuration specified in the tables below. PLMN settings are defined in TS 36.523-1 [42] Table 6.0.1-1.

Note:	Changes to any existing USIM configuration can be done only if the change WILL NOT HAVE IMPACT on any of the tests which are referring to the configuration! To establish whether this might be the case, the test case author needs to review all tests in all RAN5 test specifications, which refer to the particular USIM configuration e.g. all test cases in TS 38.523-1 [12].

Table 6.4.1-1: USIM Configuration 1

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | Default | Default |
|  | 2 | PLMN17 | All specified |
|  | 3 | PLMN16 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN15 | All specified |
|  |  | Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |

Table 6.4.1-2: USIM Configuration 2

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 | PLMN15 |  |
|  | 2 | PLMN1 |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-3: USIM Configuration 3

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |

Table 6.4.1-4: USIM configuration 4

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 |  |
|  |  | Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN3 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFUST |  | Services 20, 42 and 71 are supported. |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |

Table 6.4.1-5: USIM configuration 5

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service 71 is not supported |  |
|  |  | Service 74 is supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN |  | 0xFF..FF |  |

Table 6.4.1-6: USIM configuration 6

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN2 | NG-RAN |
|  | 2 | PLMN4 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service 71 is not supported |  |

Table 6.4.1-7: USIM configuration 7

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| E.FPLMNwAcT | 1 | PLMN13 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
| EFOPLMNwAcT | 1 | PLMN2 | All |  |
|  | 2 | PLMN14 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |  |

Table 6.4.1-8: USIM configuration 8

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN15 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
|  | 3 | PLMN17 | E-UTRAN |  |
|  | 4 | PLMN16 | NG-RAN |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |
| EFUST |  | Service n°127 is not"available" |  |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |  |

Table 6.4.1-9: USIM configuration 9

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
| EFHPPLMN |  | 1(=6 min) |  | The HPLMN Search Period on the USIM shall be set to 6 minutes. |

Table 6.4.1-10: USIM configuration 10

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |

Table 6.4.1-11: USIM configuration 11

| USIM field |  | Priority |  | Value |  | Access Technology Identifier |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EF5GS3GPPLOCI |  |  |  | PLMN15 (See preamble) |  |  |  |
| EFIMSI |  |  |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1 |  |  |  |
| EFPLMNwAcT |  | 1 |  | Default |  | Default |  |
|  |  | 2 |  | PLMN16 |  | NG-RAN |  |
|  |  |  |  | Remaining defined entries use default values |  | Default |  |
| EFOPLMNwACT |  | 1 |  | PLMN15 |  | NG-RAN |  |
|  |  |  |  | Remaining defined entries use default values |  | Default |  |
| EFHPLMNwAcT |  | 1 |  | PLMN1 |  | NG-RAN |  |
| EFUST |  |  |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list) |  |  |  |
| EFHPPLMN |  |  |  | 1 (6 minutes) |  |  |  |
| EFNASCONFIG |  |  |  | MinimumPeriodicSearchTimer set to 7 minutes |  |  |  |

Table 6.4.1-12: USIM configuration 12

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT |  | 3GPP TS 31.102, Annex E |  | The EF is empty. |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |
| EFUST |  | Service n°127 is not"available" |  |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |  |

Table 6.4.1-13: USIM configuration 13

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN2 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |

Table 6.4.1-14: Void

Table 6.4.1-15: USIM Configuration 15

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFFPLMN |  | PLMN2 |  |

Table 6.4.1-16: Void

Table 6.4.1-17: USIM Configuration 17

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 is "available". |  |
| EFEHPLMN |  | PLMN1 |  |
| EFUAC_AIC and EFACC |  | For Bits b4 and b8 in byte 1 of EFACC (defined in TS 31.102 clause 4.2.15), only single bit is set to 1. Bits b1 and b2 in byte 1 of EFUAC_AIC (defined in TS 31.102 clause 4.4.11.7), and all remaining bits of EFACC and EFUAC_AIC are set to 0. |  |

Table 6.4.1-18: USIM Configuration 18

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b1 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b2 in byte 1 is set to 0. |  |

Table 6.4.1-19: USIM Configuration 19

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
| EFHPLMNwAcT | 1 | PLMN1 | NG-RAN |
| EFUST |  | Service n°126 (for UAC Access Identities Configuration) defined in TS 31.102 clause 4.2.8 is declared "available" |  |
| EFUAC_AIC |  | Bit b2 in byte 1 defined in TS 31.102 clause 4.4.11.7 is set to 1 and bit b1 in byte 1 is set to 0. |  |

Table 6.4.1-20: USIM Configuration 20

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFECC |  | 144, 117 |  |

Table 6.4.1-21: USIM configuration 21

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 | PLMN14 | NG-RAN |
|  | 2 | PLMN13 | NG-RAN |
|  | 3 | PLMN2 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service n°127 is not "available" |  |
| EFHPPLMN |  | 1(=6 min) |  |
| EF5GS3GPPLOCI |  | FF FF…FF FE 01 (20 Bytes) |  |
| EFEPSLOCI |  | FF FF…FF FE 01 (18 Bytes) |  |
| EFPSLOCI |  | FF FF…FE FF 01 (14 Bytes) |  |
| EFLOCI |  | FF FF…FE FF 01 (11 Bytes) |  |
| Note: LOCI fields of this USIM configuration may get overwritten upon execution of test cases using a UICC loaded with this USIM configuration. The test operator shall ensure that USIM contents are as per this table before each execution of a test case that requires this USIM configuration. |  |  |  |

Table 6.4.1-22: USIM Configuration 22

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°19 and n°51 defined in TS 31.102 clause 4.2.8 is declared "service not available" |  |

Table 6.4.1-23: USIM Configuration 23

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°4 Service Dialling Numbers (SDN), Service n°99 URI support by UICC, Service n°89 eCall Data and Service n°112 eCall Data over IMS are available |  |
| EFEST |  | Services n°1 Fixed Dialling Numbers (FDN) is disabled |  |
| EFSDN |  | Two entries of SDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |  |
| EFSDNURI |  | Two entries of SDNs, eCall Test Number (tel:123456) and eCall reconfiguration number (tel:345678) |  |

Table 6.4.1-24: USIM Configuration 24

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°2 Fixed Dialling Numbers (FDN), Service n°99 URI support by UICC, Service n°89 eCall Data and Service n°112 eCall Data over IMS are available |  |
| EFEST |  | Service n°1 Fixed Dialling Numbers (FDN) is enabled |  |
| EFFDN |  | Two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |  |
| EFFDNURI |  | Two entries of FDNs, eCall Test Number (tel:123456) and eCall reconfiguration number (tel:345678) |  |

Table 6.4.1-25: USIM Configuration 25

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN15. |  |

Table 6.4.1-26: USIM Configuration 26

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°2 Fixed Dialling Numbers (FDN), Service n°99 URI support by UICC, Service n°89 eCall Data and Service n°112 eCall Data over IMS are available |  |
| EFEST |  | Service n°1 Fixed Dialling Numbers (FDN) is enabled |  |
| EFFDN |  | Two entries of FDNs, eCall Test Number (123456) and eCall reconfiguration number (345678) |  |
| EFFDNURI |  | Two entries of FDNs, eCall Test Number (tel:123456) and eCall reconfiguration number (tel:345678) |  |
| EFFPLMN |  | 004-31 |  |

Table 6.4.1-27: USIM Configuration 27

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-01. |  |
| EFFPLMN |  | 001-02 |  |

Table 6.4.1-28: USIM Configuration 28

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to 001-02. |  |
| EFFPLMN |  | 001-01 |  |