3GPP TSG-RAN5 Meeting #101	R5-237422

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2942 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default PCI of NR cells for signalling test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In slots where SIB1 or SI are scheduled and Offset Carrier CORESET#0 is different than 0, there is not enough RBs available to transmit 2 CCEs for UL/DL-SCH on UE-specific search space. It is proposed to update the PCI so that CCE used for CORESET#0 for SIB1/SI are not in RBs 0 to 11, so that there is room for CCE for DL/UL DCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Physical Cell Identity for SIG testing in Table 4.4.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to schedule data in the same slot as SIB1 and SI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is related to RAN5 SIG AP#99.02. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-236359. |  |  |  |  |  |  |  |  |

### 4.4.2	Simulated cells

NOTE 1: For NAS test cases see clause 6.3.2.

NOTE 2: Test frequency and range defined in table 4.4.2-1 do not apply to TS 38.521-1, TS 38.521-2 and TS 38.521-3 test cases.

Test frequencies and simulated NR cells are defined in table 4.4.2-1. Test frequencies and simulated E-UTRA and UTRA cells are defined in TS 36.508 [2] table 4.4.2-1.

For NR cells, NRf1 is the default test frequency. For UTRA cells, f8 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency. For E-UTRA cells, f1 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency.

Default parameters for simulated NR cells are specified in table 4.4.2-2 and table 4.4.2-3.

Default parameters for simulated E-UTRA cells are specified in TS 36.508 [2] table 4.4.2-1A and table 4.4.2-2.

Default parameters for simulated UTRA cells are specified in TS 36.508 [2] clause 4.4.4.

Common parameters for NR simulated cells are specified in clauses 4.4.3.

Other cell specific parameters are specified in TS 36.508 [2] clause 4.4.3 to 4.4.7.

Table 4.4.2-1: Definition of test frequencies and simulated NR cells

| Test frequency | RAT | Operating band | Range | Simulated NR cells |
| --- | --- | --- | --- | --- |
| NRf1 | NR | Operating band under test | Mid (Note 1, Note 3 | NR Cell 1, NR Cell 2, NR Cell 4, NR Cell 11, NR Cell 489 |
| NRf2 | NR | Operating band under test | High (Note 1, Note 3) | NR Cell 3, NR Cell 12, NR Cell 23 |
| NRf3 | NR | Operating band under test | Low (Note 1, Note 3) | NR Cell 6, NR Cell 13 |
| NRf4 | NR | Operating band under test | (Note 1) | NR Cell 14 |
| NRf5 | NR | Operating band for inter-band cells | Mid (Note 1) | NR Cell 10, NR Cell 30, NR Cell 31 |
| NRf6 | NR | Operating band for inter-band cells | High (Note 1) | NR Cell 28, NR Cell 29 |
| NRf7 | NR | Operating band for inter-band cells | Low (Note 1) |  |
| NRf8 | NR | Operating band for SDL cell | Mid (Note 1) | NR Cell 32 |
| NRf9 | NR | Operating band for SUL cell | Mid (Note 1) | NR Cell 33 |
| NRf10 | NR | Operating band for inter-band cells | Mid (Note bb) | NR Cell 500 |
| Note 1: For signalling test, see clause 6.2.3. Note 2: Void. Note 3: For RRM test with NR intra-band non-contiguous CA, the test frequencies for the set of non-contiguous component carriers are specified in clauses 4.3.1.1.4 for FR1 and in clause 4.3.1.2.4 for FR2 without any regard to range. Thus “Low”, “Mid” and “High” information in this table does not apply. Unless otherwise stated, test point with maximum Wgap is chosen. Note 4: For signalling test with NR inter-band CA three bands only, the test band is different from the Operating band for NRf1 and the Operating band for NRf5. |  |  |  |  |

Table 4.4.2-2: Default NR parameters for simulated NR cells

| cell ID |  | NR Cell Identifier |  |  | Physical layer cell identity |  | Physical layer cell identity for SIG |  | PRACH-rootSequenceIndex FDD |  | PRACH-rootSequenceIndex TDD |  | SSB-Index2,3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | gNB Identifier |  | Cell Identity |  |  |  |  | LRA=139 Note 1 |  | LRA=139 Note 1 |  |  |  |
| NR Cell 1 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0000'B | 0 |  | 42 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 2 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0010'B | 2 |  | 50 |  | 32 |  | 32 |  |  | 0 |
| NR Cell 3 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 0011'B | 3 |  | 58 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 4 |  | '00 0000 0000 0000 0000 0000 0011'B |  | '00 0000 0100'B | 4 |  | 46 |  | 64 |  | 64 |  |  | 1 |
| NR Cell 6 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 0110'B | 6 |  | 18 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 10 |  | '00 0000 0000 0000 0000 0000 0101'B |  | '00 0000 1010'B | 10 |  | 10 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 11 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0000 1011'B | 11 |  | 54 |  | 96 |  | 96 |  |  | 0 |
| NR Cell 12 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 1100'B | 12 |  | 62 |  | 32 |  | 32 |  |  | 0 |
| NR Cell 13 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 1101'B | 13 |  | 22 |  | 32 |  | 32 |  |  | 0 |
| NR Cell 14 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0000 1110'B | 14 |  | 14 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 23 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 0111'B | 23 |  | 66 |  | 64 |  | 64 |  |  | 1 |
| NR Cell 28 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0001 1100'B | 28 |  | 46 |  | 0 |  | 0 |  |  | 1 |
| NR Cell 29 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0001 1101'B | 29 |  | 34 |  | 32 |  | 32 |  |  | 0 |
| NR Cell 30 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0001 1110'B | 30 |  | 26 |  | 32 |  | 32 |  |  | 0 |
| NR Cell 31 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 1111'B | 31 |  | 38 |  | 64 |  | 64 |  |  | 1 |
| NR Cell 32 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0000'B | 32 |  | N/A |  | - |  | - |  |  | 1 |
| NR Cell 33 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0001'B | 33 |  | N/A |  | - |  | - |  |  | - |
| NR Cell 489 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '01 1110 1001'B | 489 |  | N/A |  | 128 |  | 128 |  |  | 0 |
| NR Cell 500 |  | '00 0000 0000 0000 0000 0000 1000'B |  | '00 0000 0111'B | 500 |  | N/A |  | 0 |  | 0 |  |  | 1 |
| Note 1: To avoid collision of the preambles between intra-frequency cells, with the default zeroCorrelationZoneConfig value set to 15, the PRACH-rootSequenceIndex values have been separated by 32 root sequences per intra-frequency cell. Note 2: This SSB-Index does not apply for RRM test cases in TS 38.533 [18]. RRM test cases shall use the SSB index defined in A.3 of TS 38.533 [18]. Note 3: For signalling test, simultaneous co-existence of active NR cells defined on the same frequency and same SSB-Index is not recommended due to the same timing used for intra-frequency NR cells in TS 38.523-3 [23]. |  |  |  |  |  |  |  |

Table 4.4.2-3: Default NAS parameters for simulated NR cells

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC | MNC |  |  | AMF region ID | AMF Set ID | AMF Pointer |  |
| NR Cell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 [26] clause 2.10 |
| NR Cell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 10 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 12, NR Cell 28 | TAI-3 | 002 | 11 | 1 | TAI-3 | 253 | 1 | 1 |  |
| NR Cell 13, NR Cell 29 | TAI-4 | 003 | 21 | 1 | TAI-4 | 252 | 1 | 1 |  |
| NR Cell 14, NR Cell 30 | TAI-5 | 004 | 31 | 1 | TAI-5 | 251 | 1 | 1 |  |
| NR Cell 31 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 489 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 500 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates 5G-GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |  |

Table 4.4.2-4: Default parameters for simulated SNPN cells

| cell ID | Network Identifier (NID) |  |
| --- | --- | --- |
|  | Assignment mode (Note 1) | NID value |
| NR Cell 1 | 1 | 1 |
| NR Cell 2 | 1 | 2 |
| NR Cell 3 | 1 | 3 |
| NR Cell 4 | 1 | 4 |
| NR Cell 6 | 1 | 6 |
| NR Cell 10 | 1 | 10 |
| NR Cell 11 | 1 | 11 |
| NR Cell 12 | 1 | 12 |
| NR Cell 13 | 1 | 13 |
| NR Cell 14 | 1 | 14 |
| NR Cell 23 | 1 | 23 |
| NR Cell 28 | 1 | 28 |
| NR Cell 29 | 1 | 29 |
| NR Cell 30 | 1 | 30 |
| NR Cell 31 | 1 | 32 |
| NR Cell 32 | 1 | 32 |
| NR Cell 33 | 1 | 33 |
| NR Cell 489 | 1 | 489 |
| NR Cell 500 | 1 | 500 |
| Note 1: As per TS 23.003 [26] clause 12.7.1, Network Identifier comprises of Assignment mode (one hexadecimal digit) and NID value (10 hexadecimal digits). Self-assignment NIDs are chosen by setting the assignment mode to value 1. |  |  |