3GPP TSG-RAN5 Meeting #101	R5-237449

# Chicago USA, 13th - 17th, Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0412 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for eNPN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add applicability for Rel-17 eNPN TC 8.1.7.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability for Rel-17 eNPN TC 8.1.7.2.1 is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test case applicability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version revised from R5-236264 |  |  |  |  |  |  |  |  |

## <Start of modifed section>

4.1	Protocol conformance test cases applicability

Table 4.1-1a: Applicability of Protocol conformance Idle mode test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- |
| 6 | Idle mode operations |  |  |  |
| 6.1 | NR idle mode operations |  |  |  |
| 6.1.1 | NG-RAN Only PLMN Selection |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.3 | Cell reselection of ePLMN in manual mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.4 | PLMN selection in shared network environment / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.5 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection | Rel-15 | C36 | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |
| 6.1.1.6 | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-15 | C34 | UEs supporting 5G Core and MinimumPeriodicSearchTimer |
| 6.1.1.7 | PLMN selection of RPLMN or (E)HPLMN; Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.8 | PLMN selection of RPLMN or (E)HPLMN; Manual mode | Rel-15 | C91 | UEs supporting 5G Core and ManualModeNetworkSelectionException |
| 6.1.2 | NG-RAN Only Cell Selection |  |  |  |
| 6.1.2.1 | Cell selection / Qrxlevmin & Cell reselection (Intra NR) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.2 | Cell selection / Qqualmin / Intra NR / Serving cell becomes non-suitable (Srxlev > 0, Squal < 0) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.3 | Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.3a | Intra frequency reselection not allowed | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.4 | Cell reselection for interband operation | Rel-15 | C37 | UEs supporting 5G Core and more than 1 FDD or TDD NR band |
| 6.1.2.5 | Cell reselection for interband operation using Pcompensation / Between FDD and TDD | Rel-15 | C38 | UEs supporting 5G Core and NR FDD and NR TDD |
| 6.1.2.7 | Cell reselection / Equivalent PLMN | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.8 | Cell reselection / Equivalent PLMN / Single Frequency operation | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.9 | Cell reselection using Qhyst, Qoffset and Treselection | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.11 | Area Specific SIBs using systemInformationAreaID | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.12 | Cell reselection using cell status and cell reservations / cellReservedForOtherUse | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.13 | Cell reselection using cell status and cell reservations / Access Identity 0, 1, 2 and 12 to 14 - cellReservedForOperatorUse | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.14 | Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.15 | Cell reselection in shared network environment | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.15a | Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.16 | Inter-frequency cell reselection (equal priority) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.17 | Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.18 | Cell reselection, Sintrasearch, Snonintrasearch | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.19 | Speed dependent cell reselection | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.20 | Inter-frequency cell reselection according to cell reselection priority provided by SIBs | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.21 | Cell reselection, SIntraSearchQ and SnonIntraSearchQ | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.22 | Inter-frequency cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.23 | Cell reselection / MFBI | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.24 | Slice-based cell reselection / Re-seletion priorities provided by SIB16 | Rel-17 | C240 | UEs supporting 5G Core and slice based cell reselection |
| 6.1.2.25 | Slice-based cell reselection / Re-derive reselection priority for frequency | Rel-17 | C240 | UEs supporting 5G Core and slice based cell reselection |
| 6.1.2.26 | Cell Selection / RedCap | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 6.1.2.27 | Cell reselection / inter-frequency / RedCap | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 6.2 | Multi-mode environment |  |  |  |
| 6.2.1 | Inter-RAT PLMN selection |  |  |  |
| 6.2.1.1 | Inter-RAT PLMN Selection / Selection of correct RAT for OPLMN / Automatic mode | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.1.2 | Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.1.3 | Inter-RAT PLMN Selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.1.4 | Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.1.5 | Inter-RAT Background HPLMN Search / Search for correct RAT for HPLMN / Automatic mode | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.2 | Inter-RAT Cell Selection |  |  |  |
| 6.2.2.1 | Inter-RAT cell selection / From NR RRC_IDLE to EUTRA_Idle / Serving cell becomes non-suitable | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.2.2 | Inter-RAT cell selection / From E-UTRA_Idle to NR RRC_IDLE / Serving cell becomes non-suitable | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3 | Inter-RAT Cell Reselection |  |  |  |
| 6.2.3.1 | Inter-RAT cell reselection / From E-UTRA_IDLE to NR RRC_IDLE (lower priority & higher priority, Srxlev based) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.2 | Inter-RAT cell reselection / From E-UTRA_IDLE to NR RRC_IDLE (lower priority & higher priority, Squal based) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.3 | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA_IDLE (lower priority & higher priority, Srxlev based) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.4 | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA_IDLE (lower priority & higher priority, Squal based) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.5 | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA_IDLE according to RAT priority provided by dedicated signalling (RRCRelease) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.6 | Inter-RAT cell reselection / From E-UTRA_IDLE to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.7 | Inter-RAT cell reselection / From NR RRC_IDLE to E-UTRA RRC_IDLE, Snonintrasearch | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.8 | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to NR RRC_Idle, Snonintrasearch | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.9 | Void |  |  |  |
| 6.2.3.10 | Inter-RAT cell reselection / From E-UTRA_IDLE to NR RRC_IDLE / schedulingInfoList-v12j0 | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.2.3.11 | Inter-RAT cell reselection / From E-UTRA_IDLE to NR RRC_IDLE / schedulingInfoListExt-r12 | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 6.3 | 5GS Steering of Roaming |  |  |  |
| 6.3.1 | Steering of Roaming |  |  |  |
| 6.3.1.1 | Steering of UE in roaming during registration/security check successful using List Type 1 | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.2 | Steering of UE in roaming during registration/security check successful but SOR Transparent container indicates ACK has been NOT been requested | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.3 | Steering of UE in roaming during registration/security check unsuccessful/Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.4 | Steering of UE in roaming during registration/security check unsuccessful/Manual mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.5 | Steering of UE in roaming during registration/UE configured to receive Steering of Roaming information but does not receive Steering of Roaming from Network | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.7 | Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 6.3.1.8 | Steering of UE in roaming after registration/Automatic PLMN selection mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.9 | Steering of UE in roaming after registration/Manual PLMN selection mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.1.10 | Steering of UE in roaming during mobility update registration | Rel-15 | C21 | UEs supporting 5G Core |
| 6.3.2 | Steering of Roaming with using SOR-CMCI |  |  |  |
| 6.3.2.1 | Steering of UE in roaming after registration / SOR-CMCI rule / DNN of the PDU session / DL NAS transport | Rel-17 | C21 | UEs supporting 5G Core |
| 6.3.2.2 | Steering of UE in roaming after registration / SOR-CMCI rule / MMTEL voice call / DL NAS transport | Rel-17 | C234 | NR and IMS voice over NR and MTSI Speech and preconditions and NG.114 v1.0 |
| 6.3.2.3 | Steering of UE in roaming after registration / SOR-CMCI rule / match all / DL NAS transport | Rel-17 | C21 | UEs supporting 5G Core |
| 6.3.2.4 | Steering of UE in roaming after registration / SOR-CMCI rule / DNN of the PDU session / update Tsor-cm Timer / DL NAS transport | Rel-17 | C21 | UEs supporting 5G Core |
| 6.3.2.5 | Steering of UE in roaming after registration / SOR-CMCI rule / DNN of the PDU session / store SOR-CMCI in ME / DL NAS transport | Rel-17 | C21 | UEs supporting 5G Core |
| 6.3.2.6 | Steering of UE in roaming after registration / SOR-CMCI rule / match all / Emergency call / DL NAS transport | Rel-17 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 6.4 | UE Procedures in RRC_INACTIVE state |  |  |  |
| 6.4.1 | NG-RAN Only PLMN Selection in RRC_INACTIVE state |  |  |  |
| 6.4.1.1 | PLMN Selection / Higher priority/HPLMN in Automatic PLMN Selection mode | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 6.4.1.2 | Cell reselection of ePLMN in manual mode | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 6.4.2 | Cell Selection / Qrxlevmin & Cell Reselection (Intra NR in RRC_INACTIVE state |  |  |  |
| 6.4.2.1 | Cell Selection / Qrxlevmin & Cell Reselection (Intra NR in RRC_INACTIVE state) | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 6.4.2.2 | Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 6.4.2.3 | Slice-based cell reselection in RRC_INACTIVE state / Re-selection priorities provided by SIB16 | Rel-17 | C241 | UEs supporting 5G Core and RRC_INACTIVE and slice based cell reselection |
| 6.4.3 | Inter-RAT Cell Reselection |  |  |  |
| 6.4.3.1 | Inter-RAT cell reselection From NR RRC_INACTIVE to E-UTRA RRC_IDLE (lower priority & higher priority, Srxlev based) | Rel-15 | C110 | UEs supporting 5G Core and E-UTRA and RRC_INACTIVE |
| 6.5 | SNPN and CAG Selection |  |  |  |
| 6.5.1 | SNPN Only Selection |  |  |  |
| 6.5.1.1 | SNPN Selection in Manual Mode | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 6.5.1.2 | SNPN Selection in Automatic Mode | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 6.5.1.3 | SNPN / User Reselection in Automatic Mode | Rel-16 | C167 | UEs supporting 5G Core and SNPN and user initiated SNPN reselection in automatic mode on NR |
| 6.5.2 | CAG (Closed Acccess Group) |  |  |  |
| 6.5.2.1 | CAG Selection in Manual Mode | Rel-16 | C132 | UEs supporting 5G Core and CAG |
| 6.5.2.2 | CAG Selection in Automatic Mode | Rel-16 | C132 | UEs supporting 5G Core and CAG |
| 6.5.2.3 | CAG / Limited Service / No Suitable cell | Rel-16 | C132 | UEs supporting 5G Core and CAG |
| 6.5.2.4 | CAG / cell reselection / Within allowed CAG/ non-CAG cell to CAG cell | Rel-16 | C168 | UEs supporting 5G Core and CAG and Autonomous search function on NR |
| 6.5.2.5 | Void |  |  |  |
| 6.5.2.6 | CAG / Cell Reservation | Rel-16 | C132 | UEs supporting 5G Core and CAG |
| 6.5.3 | SNPN Selection |  |  |  |
| 6.5.3.1 | SNPN Selection in Manual Mode / Using credentials from a credentials holder | Rel-17 | C304 | UEs supporting 5G Core and access using credentials assigned by a Credentials Holder separate from the SNPN |
| 6.5.3.2 | SNPN Selection in Manual Mode / Onboarding services in SNPN | Rel-17 | C305 | UEs supporting 5G Core and onboarding services in SNPN |
| 6.5.3.3 | SNPN Selection in Manual Mode / Switch to Automatic Mode | Rel-17 | C306 | UEs supporting 5G Core and emergency services in SNPN |
| 6.5.3.4 | SNPN Selection in Automatic Mode / Onboarding services in SNPN | Rel-17 | C305 | UEs supporting 5G Core and onboarding services in SNPN |
| 6.5.3.5 | SNPN Selection in Automatic Mode / Using credentials from a credentials holder | Rel-17 | C304 | UEs supporting 5G Core and accessing SNPN using credentials assigned by a Credentials Holder separate from the SNPN |
| 6.5.3.6 | SNPN / Limited service / No valid subscriber data | Rel-17 | C305 | UEs supporting 5G Core and Onboarding SNPN (hence supports Default UE Credentials) |
| 6.5.3.7 | SNPN / User Reselection in Automatic Mode / Using credentials from a credentials holder | Rel-17 | C307 | UEs supporting 5G Core and accessing SNPN using credentials from a Credentials Holder and user initiated SNPN reselection in automatic mode on NR |
| 6.5.3.8 | SNPN / cell reselection for IMS emergency services | Rel-17 | C306 | UEs supporting 5G Core and emergency services in NR connected to 5GCN in SNPN Access mode |
| 6.5.3.9 | SNPN / cell reselection / SNPN to PLMN | Rel-17 | C308 | UEs supporting 5G Core and PLMN access in SNPN Access mode and emergency services in NR connected to 5GCN in SNPN Access mode And IMS voice over NR |
| 6.6 | NR Shared Spectrum idle mode operations |  |  |  |
| 6.6.1 | NR Shared Spectrum cell selection |  |  |  |
| 6.6.1.1 | Cell selection / next strongest cell / Intra frequency reselection not allowed | Rel-16 | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |
| 6.6.1.2 | Cell selection / next strongest cell / Intra frequency reselection not allowed / RRC Inactive | Rel-16 | C247 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |
| 6.6.2 | NR Shared Spectrum cell reselection |  |  |  |
| 6.6.2.1 | Cell reselection / next best cell / intra frequency | Rel-16 | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |
| 6.6.2.2 | Cell reselection / next best cell not suitable / inter frequency | Rel-16 | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |
| 6.6.2.3 | Cell reselection / next best cell / intra frequency / RRC Inactive | Rel-16 | C247 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |
| 6.6.2.4 | Cell reselection / next best cell not suitable / inter frequency / RRC Inactive | Rel-16 | C247 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |
| 6.7 | NTN Idle mode operations |  |  |  |
| 6.7.1 | NTN cell Seleciton |  |  |  |
| 6.7.1.1 | Cell Selection / GNSS location / NTN | Rel-17 | C309 | UEs supporting 5G Core and NR NTN access |
| 6.7.1.2 | Cell Selection / MultiTAC / NTN / trackingAreaList-r17 | Rel-17 | C309 | UEs supporting 5G Core and NR NTN access |

Table 4.1-1b: Additional Information of Applicability of Protocol conformance Idle mode test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 6 |  |  |  |  |
| 6.1 |  |  |  |  |
| 6.1.1.4a |  |  | If test case 6.1.1.4 has been executed, then test case 6.1.1.4a need not to be executed (Note 1) |  |
| 6.1.2.8 |  |  | If test case 6.1.2.7 has been executed then test case 6.1.2.8 needs not to be executed |  |
| 6.1.2.15a |  |  | If test case 6.1.2.15 has been executed, then test case 6.1.2.15a need not to be executed (Note 1) |  |
| 6.1.2.23 |  | px_NR_OverlappingNotSupportedBand_MFBI |  |  |
| 6.2 |  |  |  |  |
| 6.2.1 |  |  |  |  |
| 6.2.1.1 |  |  |  | Rel-15 E-UTRA |
| 6.2.1.2 |  |  |  | Rel-15 E-UTRA |
| 6.2.1.3 |  |  |  | Rel-15 E-UTRA |
| 6.2.1.4 | [10] pc_Available_PLMNs_AcT_Ind |  |  | Rel-15 E-UTRA |
| 6.2.1.5 |  |  |  | Rel-15 E-UTRA |
| 6.2.2 |  |  |  |  |
| 6.2.3 |  |  |  |  |
| 6.2.3.1 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.2 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.3 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.4 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.5 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.6 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.7 |  |  |  | Rel-15 E-UTRA |
| 6.2.3.8 |  |  |  | Rel-15 E-UTRA |
| 6.3 |  |  |  |  |
| 6.3.1 |  |  |  |  |
| 6.3.1.2 | pc_SOR_ACKNotReqLocalRel |  |  |  |
| 6.4 |  |  |  |  |
| 6.4.1 |  |  |  |  |
| 6.4.2 |  |  |  |  |
| 6.4.3 |  |  |  |  |
| 6.4.3.1 |  |  |  | Rel-15 E-UTRA |
| Note 1: The two TCs verify the same core spec requirement(s) however in a different cell configuration to address different network deployments i.e., Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities |  |  |  |  |

Table 4.1-2a: Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- |
| 7 | Layer 2 |  |  |  |
| 7.1 | NR Layer 2 |  |  |  |
| 7.1.1 | MAC |  |  |  |
| 7.1.1.1 | Random Access Procedures |  |  |  |
| 7.1.1.1.1 | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.1a | Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.2 | Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.3 | Random access procedure / Successful / SI request | Rel-15 | C21 | UEs supporting 5G Core |
| 7.1.1.1.4 | Random access procedure / Successful / Beam Failure / Preamble selected by MAC itself / non-Contention Free RACH procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.5 | Random access procedure / Successful / Supplementary Uplink | Rel-15 | C28 | UEs supporting 5GS and supplemental uplink with dynamic switch |
| 7.1.1.1.6 | Random access procedure / Successful / Temporary C-RNTI Based / Preamble selected by MAC itself | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.1.7 | Random access procedure / 2-step RACH / RA_TYPE selection | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.8 | Correct selection of RACH parameters / 2-step RACH/MSGA and PRACH resource explicitly signalled to the UE by RRC / contention free random access procedure | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9 | Random access procedure / Successful / 2-step RACH/C-RNTI Based / Preamble selected by MAC itself | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.9a | Random access procedure / 2-step RACH / Successful / RRC_IDLE | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.10 | Random access procedure / 2-step RACH/not complete/ RA_TYPE to 4-stepRA | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.10a | Random access procedure / 2-step RACH/ Fallback for CBRA | Rel-16 | C135 | UEs supporting 2-Step RACH |
| 7.1.1.1.11 | Random access procedure / Successful / Slice specific RACH configuration | Rel-17 | C262 | UEs supporting slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.12 | Random access procedure / Successful / ra-PrioritizationForSlicing | Rel-17 | C263 | UEs supporting slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.13 | Random access procedure / Successful / Slice specific RACH configuration / 2-step RACH | Rel-17 | C264 | UEs supporting 2-Step RACH, slice-based RACH partitioning and slice-based RACH prioritisation |
| 7.1.1.1.14 | Random access procedure / Successful / ra-PrioritizationForSlicingTwoStep / 2-step RACH | Rel-17 | C265 | UEs supporting 2-Step RACH, slice-based RACH partitioning, slice-based RACH prioritisation and RACH prioritisation for Access Identity 1 |
| 7.1.1.1.15 | Random access procedure / RedCap UE / SI request | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.16 | Random access procedure / RedCap UE identification / Msg3-based / CCCH1 | Rel-17 | C212a | UEs supporting 5G Core and RedCap and RRC_INACTIVE |
| 7.1.1.1.17 | Random access procedure / RedCap UE identification | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.1.18 | Random access procedure / Msg3 repetition indication / Random access resources selection | Rel-17 | C211 | UEs supporting repetition of Message 3 PUSCH |
| 7.1.1.2 | Downlink Data Transfer |  |  |  |
| 7.1.1.2.1 | Correct Handling of DL MAC PDU / Assignment / HARQ process | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.2 | Correct Handling of DL HARQ process PDSCH Aggregation | Rel-15 | C20 | UEs supporting 5GS and PDSCH aggregation |
| 7.1.1.2.3 | Correct HARQ process handling / CCCH | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.4 | Correct HARQ process handling / BCCH | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.2.5 | Correct HARQ process handling / DL grant prioritization | Rel-16 | C179 | UEs supporting DCI DL Priority Indicator |
| 7.1.1.2.6 | Correct HARQ process handling / dynamic PUCCH repetition indication | Rel-17 | C287 | UEs supporting dynamic indication of PUCCH repetition |
| 7.1.1.3 | Uplink Data Transfer |  |  |  |
| 7.1.1.3.1 | Correct Handling of UL MAC PDU / Assignment / HARQ process | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.2 | Logical channel prioritization handling | Rel-15 | C02 | UEs supporting 5GS and RLC UM Mode |
| 7.1.1.3.2b | Logical channel prioritization handling with Mapping restrictions | Rel-15 | C175 | UEs supporting 5GS and selection of logical channels for each UL grant based on RRC configured restriction |
| 7.1.1.3.3 | Correct handling of MAC control information / Scheduling requests | Rel-15 | C53 | UEs supporting 5GS and Logical Channel SR-Delay Timer |
| 7.1.1.3.4 | Correct handling of MAC control information / Buffer status / UL data arrive in the UE Tx buffer / Regular BSR | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.5 | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Padding BSR | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.6 | Correct handling of MAC control information / Buffer status / Periodic BSR timer expires | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.7 | UE power headroom reporting / Periodic reporting / DL pathloss change reporting | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.3.8 | UE power headroom reporting / SCell activation / DL pathloss change reporting |  |  |  |
| 7.1.1.3.8.1 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band Contiguous CA | Rel-15 | C81 | UEs supporting 5GCore and intra-band contiguous CA and UL NR CA with 2 carriers |
|  |  |  | C81A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.8.2 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Inter-band CA | Rel-15 | C82 | UEs supporting 5GCore and inter-band CA and UL NR CA with 2 carriers |
|  |  |  | C82A | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.8.3 | UE power headroom reporting / SCell activation / DL pathloss change reporting / Intra-band non Contiguous CA | Rel-15 | C83 | UEs supporting 5GCore and intra-band non-contiguous CA and UL NR CA with 2 carriers |
|  |  |  | C83A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR UL carriers |
| 7.1.1.3.9 | Correct Handling of UL HARQ process / PUSCH Repetition Type A / PUSCH Aggregation | Rel-15 | C51 | UEs supporting 5GS and PUSCH aggregation |
| 7.1.1.3.10 | Correct Handling of HARQ process / Multiple CORESETPoolIndex | Rel-16 | C107 | UEs supporting 5GS and multi-DCI based Multi-TRP |
| 7.1.1.3.11 | Correct handling of UL grant prioritization | Rel-16 | C114 | UEs supporting 5GS and LCH-based UL grant prioritization |
| 7.1.1.3.12 | Correct Handling of UL HARQ process / PUSCH Repetition Type B | Rel-16 | C134 | UEs supporting PUSCH repetition type B |
| 7.1.1.3.13 | Logical channel prioritization handling with Mapping restrictions / physical layer priority | Rel-16 | C180 | UEs supporting DCI UL Priority Indicator and LCH grant prioritisation |
| 7.1.1.3.14 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement |  |  |  |
| 7.1.1.3.14.1 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / Increased maximum repetition number / dynamic grant | Rel-17 | C288 | UEs supporting increased maximum number of PUSCH Type A repetitions and dynamic indication of the number of repetitions for PUSCH |
| 7.1.1.3.14.2 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / Increased maximum repetition number / configured grant | Rel-17 | C289 | UEs supporting increased maximum number of PUSCH Type A repetitions and PUSCH transmissions with configured grant |
| 7.1.1.3.14.3 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / dynamic grant | Rel-17 | C290 | UEs supporting PUSCH repetitions based on available slots and dynamic indication of the number of repetitions for PUSCH |
| 7.1.1.3.14.4 | Correct Handling of UL HARQ process / PUSCH Repetition Type A enhancement / repetition based on available slots / configured grant | Rel-17 | C291 | UEs supporting PUSCH repetitions based on available slots and PUSCH transmissions with configured grant |
| 7.1.1.3.15 | Correct Handling of UL HARQ process / TBoMS procedure |  |  |  |
| 7.1.1.3.15.1 | Correct Handling of UL HARQ process / TBoMS procedure / DG and CG based transmission | Rel-17 | C292 | UEs supporting TB processing over multi-slot PUSCH |
| 7.1.1.3.15.2 | Correct Handling of UL HARQ process / TBoMS procedure / Repetition of TBoMS | Rel-17 | C293 | UEs supporting repetition of TB processing over multi-slot PUSCH |
| 7.1.1.3.16 | Correct Handling of UL grant / DRB configured with survival time |  |  |  |
| 7.1.1.3.16.1 | Correct Handling of UL grant / DRB configured with survival time / Split DRB | Rel-17 | C256 | UEs supporting services with survival time and NR-DC and PDCP-duplication over split DRB |
| 7.1.1.3.16.2 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band contiguous CA | Rel-17 | C257 | UEs supporting services with survival time and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.3 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Intra-band non-contiguous CA | Rel-17 | C258 | UEs supporting services with survival time and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.3.16.4 | Correct Handling of UL grant / DRB configured with survival time / MCG or SCG DRB / Inter-band CA | Rel-17 | C259 | UEs supporting services with survival time and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB |
| 7.1.1.4 | Transport Size Selection |  |  |  |
| 7.1.1.4.1 | DL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.1.1 | DL-SCH Transport Block Size selection / DCI format 1_0 | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.1.2 | Void |  |  |  |
| 7.1.1.4.1.3 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled | Rel-15 | C64 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| 7.1.1.4.1.4 | DL-SCH transport block size selection / DCI format 1_1 / RA type 0/RA Type 1 / 2 Codewords enabled / 256QAM | Rel-15 | C65 | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier and 256QAM for PUSCH |
| 7.1.1.4.1.5 | DL-SCH transport block size selection / DCI format 1_2 | Rel-16 | C146 | Ues supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| 7.1.1.4.2 | UL-SCH Transport Block Size Selection |  |  |  |
| 7.1.1.4.2.1 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.2 | Void |  |  |  |
| 7.1.1.4.2.3 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / Transform precoding disabled | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.4 | UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled | Rel-15 | C11 | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |
| 7.1.1.4.2.5 | UL-SCH Transport Block Size selection / DCI format 0_0 / Transform precoding and 64QAM | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.4.2.6 | UL-SCH Transport Block Size selection / DCI format 0_2 | Rel-16 | C146 | UEs supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| 7.1.1.4.2.7 | UL-SCH Transport Block Size selection / TBoMS procedure | Rel-17 | C292 | UEs supporting TB processing over multi-slot PUSCH |
| 7.1.1.5 | Discontinuous reception |  |  |  |
| 7.1.1.5.1 | DRX operation / Short cycle not configured / Parameters configured by RRC | Rel-15 | C03 | UEs supporting 5GS and long DRX cycle |
| 7.1.1.5.2 | DRX operation / Short cycle not configured / Long DRX command MAC control element reception | Rel-15 | C03 | UEs supporting 5GS and long DRX cycle |
| 7.1.1.5.3 | DRX operation / Short cycle configured / Parameters configured by RRC | Rel-15 | C04 | UEs supporting 5GS and short DRX cycle |
| 7.1.1.5.4 | DRX operation / Short cycle configured / DRX command MAC control element reception | Rel-15 | C04 | UEs supporting 5GS and short DRX cycle |
| 7.1.1.5.5 | DRX operation / Short cycle configured / Long DRX command MAC control element reception | Rel-15 | C70 | UEs supporting 5GS and long DRX cycle and short DRX cycle |
| 7.1.1.6 | Semi-Persistent Scheduling |  |  |  |
| 7.1.1.6.1 | Correct handling of DL assignment / Semi-persistent case | Rel-15 | C17 | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |
| 7.1.1.6.2 | Correct handling of UL grant / configured grant Type 1 | Rel-15 | C18 | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |
| 7.1.1.6.3 | Correct handling of UL grant / configured grant Type 2 | Rel-15 | C19 | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |
| 7.1.1.6.4 | Correct handling of DL assignment / Multi Semi-persistent configuration | Rel-16 | C113 | UEs supporting 5G Core and PDSCH reception based on semi-persistent scheduling and up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group |
| 7.1.1.6.5 | Correct handling of UL grant / Multi configured uplink grants | Rel-16 | C142 | UEs supporting 5G Core and PUSCH transmissions on multiple configured uplink grants |
| 7.1.1.7 | Activation/Deactivation of SCells |  |  |  |
| 7.1.1.7.1 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer |  |  |  |
| 7.1.1.7.1.1 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA | Rel-15 | C44 | UEs supporting 5GS and intra-band contiguous CA |
| 7.1.1.7.1.2 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-band CA | Rel-15 | C45 | UEs supporting 5GS and inter-band CA |
| 7.1.1.7.1.3 | Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA | Rel-15 | C46 | UEs supporting 5GS and intra-band non-contiguous CA |
| 7.1.1.8 | Bandwidth Part (BWP) operation |  |  |  |
| 7.1.1.8.1 | Bandwidth Part (BWP) operation UL/DL | Rel-15 | C66 | UEs supporting 5GS and (DCI and timer based active BWP switching delay type1 or type2) and ((BWP adaptation upto2 NR FR1 FDD or NR FR1 TDD or NR FR2) or (BWP adaptation up to 4 NR FR1 FDD or NR FR1 TDD or NR FR2)) |
| 7.1.1.8.3 | Separate BWP / IDLE / RedCap | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.8.4 | Separate BWP / RedCap-specific initial DL BWP without CORESET#0 / NCD-SSB | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 7.1.1.9 | MAC Reconfiguration and Reset |  |  |  |
| 7.1.1.9.1 | MAC Reset | Rel-15 | R | UEs supporting 5GS |
| 7.1.1.10 | Other Procedures |  |  |  |
| 7.1.1.10.1 | DataInactivityTimer expiry | Rel-15 | C21 | UEs supporting 5G Core |
| 7.1.1.10.2 | Recommended Bit Rate | Rel-15 | C100 | UEs supporting 5G Core and MTSI speech and bit rate recommendation query message |
| 7.1.1.10.3 | NR CA / LBT failure on Scell / MAC CE indication | Rel-16 | C300 | UEs supporting 5G Core and NR CA with NR shared spectrum channel access and UL NR CA with 2 carriers |
| 7.1.1.11 | NR Dual Connectivity |  |  |  |
| 7.1.1.11.1 | DC power headroom reporting / PSCell activation and DL pathloss change reporting | Rel-15 | C80 | UEs supporting NR-DC |
| 7.1.1.12 | UE Power Saving |  |  |  |
| 7.1.1.12.1 | Void |  |  |  |
| 7.1.1.12.3 | DRX adaptation / UE wakeup indication | Rel-16 | C103 | UEs supporting 5GS and Long DRX Cycle and DRX adaptation |
| 7.1.1.12.4.1 | DRX adaptation / SCell dormancy indication / Intra-band Contiguous CA | Rel-16 | C118 | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band contiguous CA |
| 7.1.1.12.4.2 | DRX adaptation / SCell dormancy indication / Intra-band non Contiguous CA | Rel-16 | C119 | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band non-contiguous CA |
| 7.1.1.12.4.3 | DRX adaptation / SCell dormancy indication / Inter-band CA | Rel-16 | C120 | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and inter-band CA |
| 7.1.1.13 | SDT |  |  |  |
| 7.1.1.13.1 | RA Based SDT / 2-step RACH / Successful | Rel-17 | C232 | UEs Supporting 2-Step RACH and Random access SDT |
| 7.1.1.13.2 | RA Based SDT / 4-step RACH / Successful | Rel-17 | C233 | UEs supporting Random Access SDT |
| 7.1.1.13.3 | RA Based SDT / 2-step RACH / not complete / RA_TYPE to 4-stepRA | Rel-17 | C232 | UEs Supporting 2-Step RACH and Random access SDT |
| 7.1.1.13.4 | RA Based SDT / 4-step RA based SDT / Time Alignment Timer expiry | Rel-17 | C233 | UEs supporting Random Access SDT |
| 7.1.1.13.5 | RA Based SDT/ CG Based SDT/ cg-SDT-TimeAlignmentTimer | Rel-17 | C269 | UEs supporting 5GC Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state. |
| 7.1.2 | RLC |  |  |  |
| 7.1.2.2 | RLC Unacknowledged Mode |  |  |  |
| 7.1.2.2.1 | UM RLC / Segmentation and reassembly / 6-bit SN / Segmentation Info (SI) field | Rel-15 | C05 | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |
| 7.1.2.2.2 | UM RLC / Segmentation and reassembly / 12-bit SN / Segmentation Info (SI) field | Rel-15 | C06 | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |
| 7.1.2.2.3 | UM RLC / 6-bit SN / Correct use of sequence numbering | Rel-15 | C05 | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |
| 7.1.2.2.4 | UM RLC / 12-bit SN / Correct use of sequence numbering | Rel-15 | C06 | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |
| 7.1.2.2.5 | UM RLC / Receive Window operation and t-Reassembly expiry | Rel-15 | C02 | UEs supporting 5GS and RLC UM Mode |
| 7.1.2.2.6 | UM RLC / RLC re-establishment procedure | Rel-15 | C02 | UEs supporting 5GS and RLC UM Mode |
| 7.1.2.3 | RLC Acknowledged Mode |  |  |  |
| 7.1.2.3.1 | AM RLC / 12-bit SN / Segmentation and reassembly / Segmentation Info (SI) field | Rel-15 | C07 | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |
| 7.1.2.3.2 | AM RLC / 18-bit SN / Segmentation and reassembly / Segmentation Info (SI) field | Rel-15 | C07A | UEs supporting 5GS and RLC AM with 18-bit length of RLC sequence number |
| 7.1.2.3.3 | AM RLC / 12-bit SN / Correct use of sequence numbering | Rel-15 | C07 | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |
| 7.1.2.3.4 | AM RLC / 18-bit SN / Correct use of sequence numbering | Rel-15 | C07A | UEs supporting 5GS and RLCand RLC AM with 18-bit length of RLC sequence number |
| 7.1.2.3.5 | AM RLC / 12-bit SN / Control of transmit window / Control of receive window | Rel-15 | C07 | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |
| 7.1.2.3.5a | AM RLC / 18-bit SN / Control of transmit window / Control of receive window | Rel-15 | C07A | UEs supporting 5GS and RLC AM with 18-bit length of RLC sequence number |
| 7.1.2.3.6 | AM RLC / Polling for status | Rel-15 | R | UEs supporting 5GS |
| 7.1.2.3.7 | AM RLC / Receiver status triggers | Rel-15 | R | UEs supporting 5GS |
| 7.1.2.3.8 | AM RLC / Reconfiguration of RLC parameters by upper layers | Rel-15 | R | UEs supporting 5GS |
| 7.1.2.3.9 | AM RLC / Reassembling of AMD PDUs | Rel-15 | R | UEs supporting 5GS |
| 7.1.2.3.10 | AM RLC / Re-transmission of RLC PDU with and without re-segmentation | Rel-15 | R | UEs supporting 5GS |
| 7.1.2.3.11 | AM RLC / RLC re-establishment procedure | Rel-15 | R | UEs supporting 5GS |
| 7.1.3 | PDCP |  |  |  |
| 7.1.3.1 | Maintenance of PDCP sequence numbers for radio bearers |  |  |  |
| 7.1.3.1.1 | Maintenance of PDCP sequence numbers / User plane / 12-bit SN | Rel-15 | C08 | UEs supporting 5GS and 12-bit length of PDCP sequence number |
| 7.1.3.1.2 | Maintenance of PDCP sequence numbers / User plane / 18-bit SN | Rel-15 | C08A | UEs supporting 5GS and 18-bit length of PDCP sequence number |
| 7.1.3.2 | PDCP Integrity protection |  |  |  |
| 7.1.3.2.1 | Integrity protection / Correct functionality of integrity algorithm SNOW3G / SRB / DRB | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.2.2 | Integrity protection / Correct functionality of integrity algorithm AES / SRB / DRB | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.2.3 | Integrity protection / Correct functionality of integrity algorithm ZUC / SRB / DRB | Rel-15 | C09 | UEs supporting 5GS and ZUC algorithm |
| 7.1.3.2.6 | Integrity protection / Correct functionality of UP integrity protection / multiple DRBs | Rel-17 | C286 | UEs supporting EN-DC and user plane integrity protection with EPS |
| 7.1.3.3 | PDCP Ciphering and deciphering |  |  |  |
| 7.1.3.3.1 | Ciphering and deciphering / Correct functionality of encryption algorithm SNOW3G / SRB / DRB | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.3.2 | Ciphering and deciphering / Correct functionality of encryption algorithm AES / SRB / DRB | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.3.3 | Ciphering and deciphering / Correct functionality of encryption algorithm ZUC / SRB / DRB | Rel-15 | C09 | UEs supporting 5GS and ZUC algorithm |
| 7.1.3.4 | PDCP Handover |  |  |  |
| 7.1.3.4.1 | PDCP handover / Lossless handover / PDCP sequence number maintenance / PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover / In-order delivery and duplicate elimination in the downlink | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.4.2 | PDCP handover / Non-lossless handover / PDCP sequence number maintenance | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.4.3 | PDCP handover / DAPS handover / Status reporting / Intra-frequency | Rel-16 | C101 | UEs supporting 5G Core and intra-frequency DAPS handover |
| 7.1.3.4.4 | PDCP handover / DAPS handover / Status reporting / Inter-frequency | Rel-16 | C130 | UEs supporting 5G Core and inter-frequency DAPS handover |
| 7.1.3.5 | PDCP other |  |  |  |
| 7.1.3.5.1 | PDCP Discard | Rel-15 | C02 | UEs supporting 5GS and RLC UM Mode |
| 7.1.3.5.2 | PDCP Uplink Routing / Split DRB | Rel-15 | C10 | UEs supporting EN-DC and UL transmission via both MCG path and SCG path for the split DRB |
|  |  |  | C97 | UEs supporting NR-DC and UL transmission via both MCG path and SCG path for the split DRB |
|  |  |  | C194 | UEs supporting NE-DC and UL transmission via both MCG path and SCG path for the split DRB |
| 7.1.3.5.3 | PDCP Data Recovery | Rel-15 | C01 | UEs supporting EN-DC |
|  |  |  | C80 | UEs supporting NR-DC |
| 7.1.3.5.4 | PDCP reordering / Maximum re-ordering delay below t-Reordering / t-Reordering timer operations | Rel-15 | R | UEs supporting 5GS |
| 7.1.3.5.5 | PDCP Duplication | Rel-15 | C62 | UEs supporting EN-DC and PDCP duplication over split DRB |
|  |  |  | C98 | UEs supporting NR-DC and PDCP duplication over split DRB |
| 7.1.3.5.6.1 | PDCP Duplication / 3 RLC entities / Intra-band Contiguous CA | Rel-16 | C104 | UEs supporting 5GC and Intra-band contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| 7.1.3.5.6.2 | PDCP Duplication / 3 RLC entities / Intra-band non-Contiguous CA | Rel-16 | C181 | UEs supporting 5GC and Intra-band non-contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| 7.1.3.5.7 | Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression | Rel-16 | C105 | UEs supporting 5GS and RLC UM Mode and PDCP ethernet header compression |
| 7.1.3.6 | PDCP UDC |  |  |  |
| 7.1.3.6.1 | PDCP UDC / No dictionary | Rel-17 | C235 | UEs supporting 5GS and uplink data compression operation |
| 7.1.3.6.2 | PDCP UDC / Pre-defined dictionary | Rel-17 | C236 | UEs supporting 5GS and uplink data compression operation and UL data compression with SIP static dictionary |
| 7.1.3.6.3 | PDCP UDC / checksum error / Reset | Rel-17 | C235 | UEs supporting 5GS and uplink data compression operation |
| 7.1.3.6.4 | PDCP UDC/ Handover/ Intra-frequency | Rel-17 | C271 | UEs supporting 5GS and uplink data compression operation and continuation of uplink data compression protocol operation |
| 7.1.3.6.5 | PDCP UDC/ Handover/ Inter-frequency | Rel-17 | C271 | UEs supporting 5GS and uplink data compression operation and continuation of uplink data compression protocol operation |
| 7.1.3.6.6 | PDCP UDC/ RRC resume | Rel-17 | C271 | UEs supporting 5GS and uplink data compression operation and continuation of uplink data compression protocol operation |
| 7.1.3.6.7 | PDCP UDC/ RRC reestablishment | Rel-17 | C271 | UEs supporting 5GS and uplink data compression operation and continuation of uplink data compression protocol operation |
| 7.1.3.6.8 | PDCP UDC/ PSCell addition / SCG DRB with UDC configuration/ NR-DC | Rel-17 | C272 | UEs supporting NR-DC and uplink data compression operation |
| 7.1.3.6.9 | PDCP UDC/ PSCell addition / SCG DRB with UDC configuration/ NE-DC | Rel-17 | C273 | UEs supporting NE-DC and uplink data compression operation |
| 7.1.4 | SDAP |  |  |  |
| 7.1.4.1 | SDAP Data Transfer and PDU Header Handling UL/DL | Rel-15 | C21A | UEs supporting 5G Core and reflective QoS |
| 7.1.4.2 | SDAP Data Transfer handling without Header UL/DL | Rel-15 | C21 | UEs supporting 5G Core |

Table 4.1-2b: Additional Information of Applicability of Protocol conformance Layer 2 test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 7 |  |  |  |  |
| 7.1 |  |  |  |  |
| 7.1.1 |  |  |  |  |
| 7.1.1.1 |  |  |  |  |
| 7.1.1.1.4 | pc_csi_RS_CFRA_ForHO |  |  |  |
| 7.1.1.3 |  |  |  |  |
| 7.1.1.3.2b | pc_configuredUL_GrantType1 |  |  |  |
| 7.1.1.4 |  |  |  |  |
| 7.1.1.4.1 |  |  |  |  |
| 7.1.1.4.1.3 | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  |
| 7.1.1.4.1.4 | pc_dynamicSwitchRA_Type0_1_PDSCH |  |  |  |
| 7.1.1.4.2 |  |  |  |  |
| 7.1.1.4.2.3 | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  |
| 7.1.1.4.2.4 | pc_dynamicSwitchRA_Type0_1_PUSCH |  |  |  |
| 7.1.1.6 |  |  |  |  |
| 7.1.1.6.4 | pc_um_WithShortSN |  |  |  |
| 7.1.1.7 |  |  |  |  |
| 7.1.1.7.1 |  |  |  |  |
| 7.1.1.7.1.1 | pc_UL_NR_CA_2CC or pc_EN_DC_NR_UL_2CC |  |  |  |
| 7.1.1.7.1.2 | pc_UL_NR_CA_2CC or pc_EN_DC_NR_UL_2CC |  |  |  |
| 7.1.1.7.1.3 | pc_UL_intra_non_contiguous_CA_NR_FR1_Class_(2A) or pc_UL_intra_non_contiguous_CA_NR_FR2_Class_(2A) |  |  |  |
| 7.1.1.12 |  |  |  |  |
| 7.1.1.12.3 |  |  |  | Rel-15 E-UTRA |
| 7.1.1.13 |  |  |  |  |
| 7.1.1.13.1 | pc_logicalChannelSR_DelayTimer |  |  |  |
| 7.1.1.13.2 | pc_logicalChannelSR_DelayTimer |  |  |  |
| 7.1.1.13.5 | pc_ra_SDT_r17 |  |  |  |
| 7.1.2 |  |  |  |  |
| 7.1.2.2 |  |  |  |  |
| 7.1.2.2.5 | pc_um_WithShortSN |  |  |  |
| 7.1.2.2.6 | pc_um_WithShortSN |  |  |  |
| 7.1.3 |  |  |  |  |
| 7.1.3.2.1 | pc_srb3 |  |  |  |

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release |  | Applicability Condition | Applicability Comment |
| --- | --- | --- | --- | --- | --- |
| 8 | RRC |  |  |  |  |
| 8.1 | NR RRC |  |  |  |  |
| 8.1.1 | RRC connection management procedures |  |  |  |  |
| 8.1.1.1 | Paging |  |  |  |  |
| 8.1.1.1.1 | RRC / Paging for connection / Multiple paging records | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.1.2 | RRC / Paging for connection / Shared network environment | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.1a | Paging Early Indication and Subgrouping |  |  |  |  |
| 8.1.1.1a.1 | Paging Early Indication with Subgrouping / RRC_IDLE / lastUsedCellOnly not configured / Subgroup ID selection | Rel-17 |  | C224 | UEs supporting 5G Core and PEI |
| 8.1.1.1a.2 | Paging Early Indication with Subgrouping / RRC_INACTIVE / lastUsedCellOnly configured | Rel-17 |  | C239 | UEs supporting 5G Core and RRC_INACTIVE and PEI |
| 8.1.1.1a.3 | Paging Early Indication without Subgrouping / RRC_IDLE | Rel-17 |  | C224 | UEs supporting 5G Core and PEI |
| 8.1.1.2 | RRC connection establishment |  |  |  |  |
| 8.1.1.2.1 | RRC connection establishment / Return to idle state after T300 expiry | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.2.2 | Void |  |  |  |  |
| 8.1.1.2.3 | RRC connection establishment / RRC Reject with wait time | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.2.4 | RRC connection establishment / Extended and spare fields in SI | Rel-15 and Rel-16 only |  | C21 | UEs supporting 5G Core |
| 8.1.1.3 | RRC release |  |  |  |  |
| 8.1.1.3.1 | RRC connection release / Redirection to another NR frequency | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.3.2 | RRC connection release / Redirection from NR to E-UTRA | Rel-15 |  | C32 | UEs supporting 5G Core and E-UTRA |
| 8.1.1.3.3 | RRC connection release / Success / With priority information | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.1.3.4 | RRC connection release / Success / With priority information / E-UTRA | Rel-15 |  | C26 | UEs supporting 5GS and E-UTRA |
| 8.1.1.3.5 | Void |  |  |  |  |
| 8.1.1.3.6 | Void |  |  |  |  |
| 8.1.1.3.7 | RRC connection release / Success / Deprioritisation / Frequency / T325 expiry | Rel-15 |  | C133 | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| 8.1.1.3.7a | RRC connection release / Success / Deprioritisation / NR / T325 expiry | Rel-15 |  | C148 | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |
| 8.1.1.3.7b | RRC connection release / Success / Deprioritisation / Deletion of Stored deprioritisation request | Rel-15 |  | C161 | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |
| 8.1.1.3.8 | RRC connection release / Redirection to another NR frequency / MPS Priority Indication | Rel-16 |  | C274 | UEs supporting 5G Core and RRC Connection release with MPS priority indication |
| 8.1.1.3.9 | RRC connection release / Success / With slice specific cell reselection information | Rel-17 |  | C240 | UEs supporting 5G Core and slice based cell reselection |
| 8.1.1.4 | RRC resume |  |  |  |  |
| 8.1.1.4.1 | RRC resume / Suspend-Resume / RNA update / Success | Rel-15 |  | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.2 | RRC resume / Suspend-Resume / RRC setup / T319 expiry | Rel-15 |  | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.3 | Void |  |  |  |  |
| 8.1.1.4.4 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Intra-band Contiguous CA | Rel-16 |  | C154 | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.5 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Intra-band non-Contiguous CA | Rel-16 |  | C155 | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.6 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Inter-band CA | Rel-16 |  | C156 | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.7 | RRC resume / Suspend-Resume / RRC reconfiguration / Active SCG SCell addition / Intra-band Contiguous CA | Rel-16 |  | C221 | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |
| 8.1.1.4.8 | RRC resume / Suspend-Resume / RRC reconfiguration / Active SCG SCell addition / Intra-band non-Contiguous CA | Rel-16 |  | C222 | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |
| 8.1.1.4.9 | RRC resume / Suspend-Resume / RRC reconfiguration / Active SCG SCell addition / Inter-band CA | Rel-16 |  | C223 | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR SCG SCell activation and NR-DC |
| 8.1.2 | RRC reconfiguration |  |  |  |  |
| 8.1.2.1 | Radio bearer establishment / reconfiguration / release |  |  |  |  |
| 8.1.2.1.1 | RRC reconfiguration / DRB / SRB / Establishment / Modification / Release / Success | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.2.1.2 | RRC reconfiguration / RRC bearer establishment / uplinkTxDirectCurrentList | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.2.1.3 | Void |  |  |  |  |
| 8.1.2.1.4 | RRC reconfiguration / Dedicated RLF timer | Rel-15 |  | C21 | UEs supporting 5GCore |
| 8.1.2.1.5 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success |  |  |  |  |
| 8.1.2.1.5.1 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.2.1.5.2 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.2.1.5.3 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Intra-band non-contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.2.1.5.4 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Active MCG SCell addition / Intra-band Contiguous CA | Rel-16 |  | C226 | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band contiguous CA |
| 8.1.2.1.5.5 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Active MCG SCell addition / Intra-band non-contiguous CA | Rel-16 |  | C227 | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band non-contiguous CA |
| 8.1.2.1.5.6 | NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Active MCG SCell addition / Inter-band CA | Rel-16 |  | C228 | UEs supporting 5G Core and direct NR MCG SCell activation and inter-band CA |
| 8.1.2.1.6 | RRC reconfiguration/ MUSIM / MUSIM- gap / Addition / Modification / Release | Rel-17 |  | C246 | UEs supporting 5G Core and MUSIM gap feature |
| 8.1.3 | Measurement configuration control and reporting |  |  |  |  |
| 8.1.3.1 | Intra NR measurements |  |  |  |  |
| 8.1.3.1.1 | Measurement configuration control and reporting / Intra NR measurements / Event A1 / Event A2 | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.2 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Intra-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.3 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.4 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-band measurements | Rel-15 |  | C94 | UEs supporting 5G Core and multiple NR bands |
| 8.1.3.1.5 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Intra-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.6 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.7 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-band measurements | Rel-15 |  | C94 | UEs supporting 5G Core and multiple NR bands |
| 8.1.3.1.8 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.9 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-frequency measurements | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.10 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-band measurements | Rel-15 |  | C94 | UEs supporting 5G Core and multiple NR bands |
| 8.1.3.1.11 | Measurement configuration control and reporting / Intra NR measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements | Rel-15 |  | C21 | UEs supporting 5GCore |
| 8.1.3.1.12 | Measurement configuration control and reporting / Intra NR measurements / Two simultaneous events A5 (intra and inter-frequency measurements) / SINR based measurements | Rel-15 |  | C40 | UEs supporting 5G Core and SS-SINR measurements |
| 8.1.3.1.13 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR cell | Rel-15 |  | C52 | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQmeasurement |
| 8.1.3.1.14 | Void |  |  |  |  |
| 8.1.3.1.14A | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based inter-frequency measurements / Measurement of Neighbour NR cell | Rel-15 |  | C52 | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQmeasurement |
| 8.1.3.1.15 | Void |  |  |  |  |
| 8.1.3.1.15A | Measurement configuration control and reporting / Intra NR measurements / Exclude-listed cells | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.16 | Measurement configuration control and reporting / Intra NR measurements / Allow-listed cells | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.1.17 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 |  |  |  |  |
| 8.1.3.1.17.1 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.3.1.17.2 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.3.1.17.3 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Event A6 / Intra-band non-Contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.3.1.18 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting |  |  |  |  |
| 8.1.3.1.18.1 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.3.1.18.2 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.3.1.18.3 | NR CA / Measurement configuration control and reporting / Intra NR measurements / Additional measurement reporting / Intra-band non-Contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.3.1.19 | Measurement configuration control and reporting / Inter-frequency measurements/ SFTD | Rel-15 |  | C150 | UEs supporting 5G Core and SFTD measurements between NR PCell and NR neighbour cell |
| 8.1.3.1.20 | Measurement configuration control and reporting / Measurement Gaps / gapFR1 | Rel-15 |  | C49 | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |
| 8.1.3.1.21 | Measurement configuration control and reporting / Measurement Gaps / gapFR2 | Rel-15 |  | C49 | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |
| 8.1.3.1.23 | Measurement configuration control and reporting / Intra NR measurements / Periodic reporting / Continuation of the measurements after RRC Resume | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.3.2 | Inter-RAT measurements |  |  |  |  |
| 8.1.3.2.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of E-UTRA cells | Rel-15 |  | C31 | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| 8.1.3.2.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells | Rel-15 |  | C31 | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| 8.1.3.2.3 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / RSRQ based measurements | Rel-15 |  | C31 | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| 8.1.3.2.4 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / SINR based measurements | Rel-15 |  | C50 | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting and E-UTRA RS-SINR measurements |
| 8.1.3.2.5 | Void |  |  |  |  |
| 8.1.3.2.6 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / NR to UTRA | Rel-16 |  | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 8.1.3.2.7 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / NR to UTRA | Rel-16 |  | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 8.1.3.2.8 | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / NR to UTRA | Rel-16 |  | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 8.1.3.3 | Measurement for self-optimized networks |  |  |  |  |
| 8.1.3.3.1 | Measurement configuration control and reporting / CGI reporting of NR cell | Rel-15 |  | C59 | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |
| 8.1.3.3.2 | Measurement configuration control and reporting / CGI reporting of E-UTRA cell | Rel-15 |  | C60 | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |
| 8.1.3.4 | Measurement relaxation |  |  |  |  |
| 8.1.3.4.1 | Void |  |  |  |  |
| 8.1.4 | Handover |  |  |  |  |
| 8.1.4.1 | Intra NR handover |  |  |  |  |
| 8.1.4.1.1 | Void |  |  |  |  |
| 8.1.4.1.2 | Intra NR handover / Success / Inter-frequency | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.4.1.3 | Void |  |  |  |  |
| 8.1.4.1.4 | Void |  |  |  |  |
| 8.1.4.1.5 | Intra NR handover / Failure / Re-establishment successful | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.4.1.6 | Intra NR handover / Failure / Re-establishment failure | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.4.1.7 | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release |  |  |  |  |
| 8.1.4.1.7.1 | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.4.1.7.2 | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.4.1.7.3 | NR CA / Intra NR handover / Success / PCell Change and SCell addition / SCell release / Intra-band non-contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.4.1.8 | NR CA / Intra NR handover / Success / PCell Change / SCell no Change |  |  |  |  |
| 8.1.4.1.8.1 | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.4.1.8.2 | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.4.1.8.3 | NR CA / Intra NR handover / Success / PCell Change / SCell no Change / Intra-band non-contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.4.1.9 | NR CA / Intra NR handover / Failure / Re-establishment successful |  |  |  |  |
| 8.1.4.1.9.1 | NR CA / Intra NR handover / Failure / Re-establishment successful / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.4.1.9.2 | NR CA / Intra NR handover / Failure / Re-establishment successful / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.4.1.9.3 | NR CA / Intra NR handover / Failure / Re-establishment successful / Intra-band non-contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.4.1.10 | eCall Only mode / Intra NR handover / Success / Inter-frequency | Rel-16 |  | C184 | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 8.1.4.2 | Inter-RAT handover |  |  |  |  |
| 8.1.4.2.1 | Inter-RAT handover from NR |  |  |  |  |
| 8.1.4.2.1.1 | Inter-RAT handover / From NR to E-UTRA / Success | Rel-15 |  | C32 | UEs supporting 5G Core and E-UTRA |
| 8.1.4.2.1.2 | Inter-RAT handover / From NR to EN-DC / Success | Rel-16 |  | C96 | UEs supporting 5G Core and EN-DC and inter-RAT Handover from NR to EN-DC |
| 8.1.4.2.2 | Inter-RAT handover to NR |  |  |  |  |
| 8.1.4.2.2.1 | Inter-RAT handover / From E-UTRA to NR / Success | Rel-15 |  | C99 | UEs supporting 5GS and E-UTRA and (inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC or inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC or inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC) |
| 8.1.4.4 |  |  |  |  |  |
| 8.1.4.4.4 | pc_condHandoverFailure-r16 |  |  |  |  |
| 8.1.4.3 | DAPS handover |  |  |  |  |
| 8.1.4.3.1 | DAPS handover with key change / Success / Intra-frequency | Rel-16 |  | C101 | UEs supporting 5G Core and intra-frequency DAPS handover |
| 8.1.4.3.2 | DAPS handover / HO Failure and source link available / HO Success and RLF in source / Intra-frequency | Rel-16 |  | C101 | UEs supporting 5G Core and intra-frequency DAPS handover |
| 8.1.4.3.4 | DAPS handover with key change / Success / Inter-frequency | Rel-16 |  | C130 | UEs supporting 5G Core and inter-frequency DAPS handover |
| 8.1.4.3.5 | DAPS handover / HO Failure and source link available / HO Success and RLF in source / Inter-frequency | Rel-16 |  | C130 | UEs supporting 5G Core and inter-frequency DAPS handover |
| 8.1.4.4 | Conditional handover |  |  |  |  |
| 8.1.4.4.1 | Conditional handover / Success / A3 / A5 / A3+A5 | Rel-16 |  | C116 | UEs supporting 5G Core and conditional handover and supporting 2 trigger events for same execution condition |
| 8.1.4.4.2 | Conditional handover / modify conditional handover configuration | Rel-16 |  | C115 | UEs supporting 5G Core and conditional handover |
| 8.1.4.4.3 | Conditional handover / Failure | Rel-16 |  | C117 | UEs supporting 5G Core and conditional handover and conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |
| 8.1.4.4.4 | Conditional handover / legacy Handover / legacy Handover Failure | Rel-16 |  | C115 | UEs supporting 5G Core and conditional handover |
| 8.1.5 | RRC others |  |  |  |  |
| 8.1.5.1 | UE capability transfer |  |  |  |  |
| 8.1.5.1.1 | UE Capability transfer / Success | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.2 | SI change / On-demand SIB |  |  |  |  |
| 8.1.5.2.1 | Void |  |  |  |  |
| 8.1.5.2.2 | SI change / Notification of BCCH modification / Short message for SI update in NR RRC_CONNECTED state | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.2.3 | eDRX / IDLE / Paging for notification of BCCH modification | Rel-17 |  | C210 | UEs supporting 5G Core and eDRX |
| 8.1.5.3 | PWS notification |  |  |  |  |
| 8.1.5.3.1 | PWS notification / PWS reception in NR RRC_IDLE state | Rel-15 |  | C35 | UEs supporting 5G Core and (ETWS reception or CMAS reception) |
| 8.1.5.3.2 | PWS notification / PWS reception in NR RRC_INACTIVE state | Rel-15 |  | C111 | UEs supporting 5G Core and (ETWS reception or CMAS reception) and RRC_INACTIVE |
| 8.1.5.3.3 | PWS notification / PWS reception in NR RRC_CONNECTED state | Rel-15 |  | C35 | UEs supporting 5G Core and (ETWS reception or CMAS reception) |
| 8.1.5.3.4 | PWS notification / PWS reception using dedicatedSystemInformationDelivery | Rel-15 |  | C35 | UEs supporting 5G Core and (ETWS reception or CMAS reception) |
| 8.1.5.4 | Counter check |  |  |  |  |
| 8.1.5.4.1 | Counter check / Reception of CounterCheck message by the UE | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.5 | Redirection to NR |  |  |  |  |
| 8.1.5.5.1 | Redirection to NR / From E-UTRA / Success | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.6 | Radio link failure |  |  |  |  |
| 8.1.5.6.1 | Radio link failure / RRC connection re-establishment success | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.6.2 | Void |  |  |  |  |
| 8.1.5.6.3 | Radio link failure / T311 expiry | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.6.4 | Void |  |  |  |  |
| 8.1.5.6.5 | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on Pcell |  |  |  |  |
| 8.1.5.6.5.1 | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.5.6.5.2 | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.5.6.5.3 | NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band non-Contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.5.6.6 | Radio link failure / Shared spectrum / LBT Failure |  |  |  |  |
| 8.1.5.6.6.1 | Radio link failure / LBT Failure | Rel-16 |  | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |
| 8.1.5.7 | Failure information |  |  |  |  |
| 8.1.5.7.1 | Failure information / RLC failure / MCG |  |  |  |  |
| 8.1.5.7.1.1 | Failure information / RLC failure / MCG / Intra-band Contiguous CA | Rel-15 |  | C72 | UEs supporting 5G Core and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.1.5.7.1.2 | Failure information / RLC failure / MCG / Inter-band CA | Rel-15 |  | C73 | UEs supporting 5G Core and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.1.5.7.1.3 | Failure information / RLC failure / MCG / Intra-band non Contiguous CA | Rel-15 |  | C74 | UEs supporting 5G Core and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.1.5.8 | Processing delay |  |  |  |  |
| 8.1.5.8.1 | Processing delay / RRC_Idle to RRC_Connected / RRC_Inactive to RRC_Connected / Success / Latency check | Rel-15 |  | C21 | UEs supporting 5G Core |
| 8.1.5.8.2 | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition |  |  |  |  |
| 8.1.5.8.2.1 | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band Contiguous CA | Rel-15 |  | C41 | UEs supporting 5G Core and intra-band contiguous CA |
| 8.1.5.8.2.2 | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Inter-band CA | Rel-15 |  | C42 | UEs supporting 5G Core and inter-band CA |
| 8.1.5.8.2.3 | Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band non-Contiguous CA | Rel-15 |  | C43 | UEs supporting 5G Core and intra-band non-contiguous CA |
| 8.1.5.9 | Message Segment transfer |  |  |  |  |
| 8.1.5.9.1 | RACS / UL Message Segment transfer / UECapabilityInformation | Rel-16 |  | C129 | UEs supporting 5G Core and RRC message Segmentation in the UL |
| 8.1.5.9.2 | RRC reconfiguration / DL segment transfer | Rel-16 |  | C207 | UEs supporting 5G core and reception of segmented DL RRC messages. |
| 8.1.5.9.3 | RRC resume / DL segment transfer | Rel-16 |  | C207 | UEs supporting 5G core and reception of segmented DL RRC messages. |
| 8.1.5.10 | UE Assistance Information |  |  |  |  |
| 8.1.5.10.1 | UE Assistance Information/ Release Preference | Rel-16 |  | C145 | UEs supporting 5G Core and release preference assistance information |
| 8.1.5.10.2 | UE Assistance Information / MUSIM | Rel-17 |  | C294 | UEs supporting 5G Core and Multi-SIM features and MUSIM related assistance information |
| 8.1.5.10.3 | UE Assistance Information / MUSIM / Leaving RRC_CONNECTED / T346g expires | Rel-17 |  | C245 | UEs supporting 5G Core and Multi-SIM features and release preference assistance information |
| 8.1.5.10.4 | UE Assistance Information / RRM measurement relaxation / RedCap | Rel-17 |  | C209 | UEs supporting 5G Core and RedCap and relaxed RRM measurements in RRC_CONNECTED and initiating UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode. |
| 8.1.5.11 | Idle/Inactive Measurements |  |  |  |  |
| 8.1.5.11.1 | Idle/Inactive Measurements / Idle mode / SIB11 configuration / Measurement of NR cells | Rel-16 |  | C190 | UEs supporting 5G Core and Idle/Inactive Measurements |
| 8.1.5.11.2 | Idle/Inactive Measurements / Idle mode / RRCRelease configuration / Measurement of NR cells | Rel-16 |  | C190 | UEs supporting 5G Core and Idle/Inactive Measurements |
| 8.1.5.11.3 | Idle/Inactive measurements / Inactive mode / SIB11 configuration / Measurement of NR cells | Rel-16 |  | C192 | UEs supporting 5GC Core and RRC_INACTIVE and Idle/Inactive Measurements |
| 8.1.5.11.4 | Idle/Inactive measurements / Inactive mode / RRCRelease configuration / Measurement of NR cells | Rel-16 |  | C192 | UEs supporting 5GC Core and RRC_INACTIVE and Idle/Inactive Measurements |
| 8.1.5.13 | Small Data Transmission |  |  |  |  |
| 8.1.5.13.1 | RRC SDT/CG based SDT/Success | Rel-17 |  | C269 | UEs supporting 5GC Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state |
| 8.1.5.13.2 | RRC SDT / CG based SDT ongoing / Data on non-SDT Radio Bearers | Rel-17 |  | C269 | UEs supporting 5G Core and SDT via Configured Grant Type 1 in RRC_INACTIVE state |
| 8.1.5.13.3 | RRC SDT / CG based SDT / SDT-SRB2-Indication | Rel-17 |  | C270 | UEs supporting 5G Core and SRB SDT and SDT via Configured Grant Type 1 in RRC_INACTIVE state |
| 8.1.6 | SON and MDT support for NR |  |  |  |  |
| 8.1.6.1 | Intra NR MDT |  |  |  |  |
| 8.1.6.1.1 | Immediate MDT |  |  |  |  |
| 8.1.6.1.1.1 | Immediate MDT / Measurement reporting / Location information | Rel-16 |  | C126 | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information |
| 8.1.6.1.1.2 | Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB | Rel-16 |  | C122 | UEs supporting 5G Core and UL PDCP Packet Delay per DRB |
| 8.1.6.1.2 | Logged MDT |  |  |  |  |
| 8.1.6.1.2.1 | Logged MDT / RRC_IDLE / Logging and reporting / Intra-frequency measurement | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.2 | Logged MDT / RRC_INACTIVE / Logging and reporting / Inter-frequency measurement | Rel-16 |  | C125 | UEs supporting 5G core and RRC_INACTIVE and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.3 | Logged MDT / RRC_IDLE / Logging and reporting / Limiting area scope | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.4 | logged MDT/ RRC_IDLE / Logging and reporting / periodic measurement trigger | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.5 | logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.6 | logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger / out-of-coverage | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.7 | Logged MDT / RRC_IDLE / Logging and reporting / Reporting at NR re-establishment | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.8 | Logged MDT / Logging and reporting / Reporting at RRC reconfiguration | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.9 | Logged MDT / Location information | Rel-16 |  | C124 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and equipped with a GNSS receiver to provide detailed location information. |
| 8.1.6.1.2.10 | Logged MDT / Maintaining logged measurement configuration / UE mobility | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.11 | Logged MDT / Maintaining logged measurement configuration / UE state transitions | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.12 | Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.13 | Logged MDT / Release of logged MDT measurement configuration / Reception of new logged measurement configuration | Rel-16 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.2.14 | Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism | Rel-17 |  | C266 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and IDC mechanism |
| 8.1.6.1.2.15 | Logged MDT / RRC_IDLE / early measurements | Rel-17 |  | C267 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and early measurements |
| 8.1.6.1.2.16 | Logged MDT / RRC_IDLE / sig-based logged MDT | Rel-17 |  | C123 | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.1.3 | Radio Link Failure report |  |  |  |  |
| 8.1.6.1.3.1 | Radio Link Failure / Reporting of Intra-frequency measurements | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.2 | Radio Link Failure / Reporting of Inter-frequency measurements | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.3 | Radio Link Failure / Reporting at RRC connection establishment and reestablishment | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.4 | Radio Link Failure / Reporting at NR handover | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.5 | Radio Link Failure / Location information | Rel-16 |  | C126 | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information |
| 8.1.6.1.3.6 | Radio Link Failure / RACH failure report | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.7 | Radio Link Failure / Logging and reporting / Reporting at intra NR handover / PLMN list | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.3.8 | Radio Link Failure / Logging and reporting / Event A3 / CHO | Rel-17 |  | C301 | UEs supporting 5G Core and RLF-Report for conditional handover |
| 8.1.6.1.3.9 | Radio Link Failure / Logging and reporting / Event A5 / CHO | Rel-17 |  | C301 | UEs supporting 5G Core and RLF-Report for conditional handover |
| 8.1.6.1.3.10 | Radio Link Failure / Logging and reporting / DAPS HO | Rel-17 |  | C302 | UEs supporting 5G Core and RLF-Report for DAPS handover. |
| 8.1.6.1.3.11 | Radio Link Failure / Logging and reporting / Successful Handovers Reports | Rel-17 |  | C303 | UEs supporting 5G Core and the storage and delivery of Successful Handover Report. |
| 8.1.6.1.4 | Connection Establishment Failure |  |  |  |  |
| 8.1.6.1.4.1 | Connection Establishment Failure / Logging and reporting / T300 expiry | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.4.2 | Connection Establishment Failure / Logging and reporting / RRC Resume | Rel-16 |  | C109 | UEs supporting 5G Core and RRC_INACTIVE. |
| 8.1.6.1.4.3 | Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.4.4 | Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.4.5 | Connection Establishment Failure / Logging and reporting / Location Information | Rel-16 |  | C126 | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information. |
| 8.1.6.1.4.6 | Connection Establishment Failure / Logging and reporting / Reporting of Intra-frequency measurements | Rel-16 |  | C21 | UEs supporting 5G Core. |
| 8.1.6.1.4.7 | Connection Establishment Failure / Logging and reporting / Reporting of Inter-frequency measurements | Rel-16 |  | C21 | UEs supporting 5G Core |
| 8.1.6.1.4.8 | Connection Establishment Failure / Logging and reporting / RACH failure report | Rel-16 |  | C136 | UEs supporting 5G Core and delivery of rachReport upon request from the network |
| 8.1.6.1.4.9 | Connection Establishment Failure / Logging and reporting / T300 expiry / Multiple CEF reports | Rel-17 |  | C250 | UEs supporting 5G Core and storage and delivery of multiple CEF report upon request from the network |
| 8.1.6.2 | Inter-RAT MDT |  |  |  |  |
| 8.1.6.2.1 | Inter-RAT MDT / Immediate MDT / Periodic reporting of E-UTRAN/ Location information | Rel-16 |  | C143 | UEs supporting 5G Core and E-UTRA and standalone GNSS receiver to provide detailed location information |
| 8.1.6.2.2 | Inter-RAT MDT / Logged MDT / E-UTRA Inter-RAT measurement, logging and reporting | Rel-16 |  | C144 | UEs supporting 5G Core and E-UTRA and logged measurements in RRC_IDLE and RRC_INACTIVE |
| 8.1.6.2.3 | Inter-RAT MDT / Radio Link Failure / Reporting at E-UTRA Inter-RAT handover | Rel-16 |  | C32a | UEs supporting 5G Core and E-UTRA and logged MDTa |
| 8.1.6.2.4 | Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA measurement | Rel-16 |  | C32a | UEs supporting 5G Core and E-UTRA and logged MDT |
| 8.1.6.3 | Inter-System MDT |  |  |  |  |
| 8.1.6.3.1 | Inter-System MDT / Immediate MDT |  |  |  |  |
| 8.1.6.3.1.1 | Inter-System MDT / Immediate MDT / Measurement reporting / Bluetooth measurement collection |  | Rel-16 | C140 | UEs supporting 5G core and Bluetooth Measurement Collection in Immediate MDT |
| 8.1.6.3.1.2 | Inter-System MDT / Immediate MDT / Measurement reporting / WLAN measurement collection |  | Rel-16 | C141 | UEs supporting 5G core and WLAN Measurement Collection in Immediate MDT |
| 8.1.6.3.1.3 | Inter-System MDT / Immediate MDT / Measurement reporting / Sensor measurement collection |  | Rel-16 | C139 | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. |
| 8.1.6.3.2 | Inter-System MDT / Logged MDT |  |  |  |  |
| 8.1.6.3.2.1 | Inter-System MDT / Logged MDT / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.2.2 | Inter-System MDT / Logged MDT / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and WLAN measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.2.3 | Inter-System MDT / Logged MDT / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139 Editor's note: shall be C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. Editor's note: shall add "and logged MDT." |
| 8.1.6.3.3 | Inter-System MDT / Radio Link Failure |  |  |  |  |
| 8.1.6.3.3.1 | Inter-System MDT / Radio Link Failure / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.3.2 | Inter-System MDT / Radio Link Failure / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and WLAN measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.3.3 | Inter-System MDT / Radio Link Failure / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139 Editor's note: shall be C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. Editor's note: shall add "and logged MDT." |
| 8.1.6.3.4 | Inter-System MDT / Connection Establishment Failure |  |  |  |  |
| 8.1.6.3.4.1 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.4.2 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and WLAN measurements in RRC_IDLE and RRC_INACTIVE state Editor's note: shall add "and logged MDT." |
| 8.1.6.3.4.3 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139 Editor's note: shall be C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. Editor's note: shall add "and logged MDT." |
| 8.1.7 | Non-public networks |  |  |  |  |
| 8.1.7.1 | Measurement for self-optimized networks |  |  |  |  |
| 8.1.7.1.1 | Measurement configuration control and reporting / CGI reporting of NR NPN cell |  | Rel-16 | C169 | UEs supporting 5G Core and CAG and acquisition of CGI information from neighbour NR NPN cell |
| 8.1.7.2 | RRC connection establishment |  |  |  |  |
| 8.1.7.2.1 | RRC connection establishment / RRC setup complete with onboarding request |  | Rel-17 | C305 | UEs supporting 5G Core and onboarding services in SNPN |
| 8.1.6.4 | SON / RACH Optimisation |  |  |  |  |
| 8.1.6.4.1 | SON / RACH logging and reporting |  | Rel-16 | C136 | UEs supporting 5G Core and delivery of rachReport upon request from the network. |
| 8.1.6.4.2 | SON / RACH logging and reporting / logging of on-demand SI |  | Rel-17 | C278 | UEs supporting 5G Core and delivery of on-Demand SI information upon request from the network. |
| 8.1.6.4.3 | SON / RACH logging and reporting / 2-step RACH report |  | Rel-17 | C279 | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network. |
| 8.1.6.4.4 | SON / RACH logging and reporting / fallback to 4-step RA |  | Rel-17 | C279 | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network. |
| 8.1.8 | Shared spectrum access |  |  |  |  |
| 8.1.8.1 | Measurement configuration control and reporting for Shared spectrum |  |  |  |  |
| 8.1.8.1.1 | Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / intra-frequency |  | Rel-16 | C218 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |
| 8.1.8.1.2 | Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / inter-frequency |  | Rel-16 | C218 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |
| 8.1.8.2 | Paging monitoring |  |  |  |  |
| 8.1.8.2.1 | Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring |  | Rel-16 | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |
| 8.1.8.2.2 | Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring / RRC inactive |  | Rel-16 | C247 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |
| 8.1.6.3 | Inter-System MDT |  |  |  |  |
| 8.1.6.3.1 | Inter-System MDT / Immediate MDT |  |  |  |  |
| 8.1.6.3.1.1 | Inter-System MDT / Immediate MDT / Measurement reporting / Bluetooth measurement collection |  | Rel-16 | C140 | UEs supporting 5G core and Bluetooth Measurement Collection in Immediate MDT |
| 8.1.6.3.1.2 | Inter-System MDT / Immediate MDT / Measurement reporting / WLAN measurement collection |  | Rel-16 | C141 | UEs supporting 5G core and WLAN Measurement Collection in Immediate MDT |
| 8.1.6.3.1.3 | Inter-System MDT / Immediate MDT / Measurement reporting / Sensor measurement collection |  | Rel-16 | C139 | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. |
| 8.1.6.3.2 | Inter-System MDT / Logged MDT |  |  |  |  |
| 8.1.6.3.2.1 | Inter-System MDT / Logged MDT / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and logged MDT and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.2.2 | Inter-System MDT / Logged MDT / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and logged MDT and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.2.3 | Inter-System MDT / Logged MDT / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 and logged MDT. |
| 8.1.6.3.3 | Inter-System MDT / Radio Link Failure |  |  |  |  |
| 8.1.6.3.3.1 | Inter-System MDT / Radio Link Failure / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and logged MDT and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.3.2 | Inter-System MDT / Radio Link Failure / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and logged MDT and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| 8.1.6.3.3.3 | Inter-System MDT / Radio Link Failure / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 and logged MDT. |
| 8.1.6.3.4 | Inter-System MDT / Connection Establishment Failure |  |  |  |  |  |
| 8.1.6.3.4.1 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / Bluetooth measurement collection |  | Rel-16 | C137 | UEs supporting 5G Core and logged MDT and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  |
| 8.1.6.3.4.2 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / WLAN measurement collection |  | Rel-16 | C138 | UEs supporting 5G Core and logged MDT and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  |
| 8.1.6.3.4.3 | Inter-System MDT / Connection Establishment Failure / Logging and reporting / Sensor measurement collection |  | Rel-16 | C139a | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 and logged MDT. |  |
| 8.1.7 | Non-public networks |  |  |  |  |  |
| 8.1.7.1 | Measurement for self-optimized networks |  |  |  |  |  |
| 8.1.7.1.1 | Measurement configuration control and reporting / CGI reporting of NR NPN cell |  | Rel-16 | C169 | UEs supporting 5G Core and CAG and acquisition of CGI information from neighbour NR NPN cell |  |
| 8.1.6.4 | SON / RACH Optimisation |  |  |  |  |  |
| 8.1.6.4.1 | SON / RACH logging and reporting |  | Rel-16 | C136 | UEs supporting 5G Core and delivery of rachReport upon request from the network. |  |
| 8.1.6.4.2 | SON / RACH logging and reporting / logging of on-demand SI |  | Rel-17 | C278 | UEs supporting 5G Core and delivery of on-Demand SI information upon request from the network. |  |
| 8.1.6.4.3 | SON / RACH logging and reporting / 2-step RACH report |  | Rel-17 | C279 | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network. |  |
| 8.1.6.4.4 | SON / RACH logging and reporting / fallback to 4-step RA |  | Rel-17 | C279 | UEs supporting 5G Core and delivery of delivery of 2-step RACH related information upon request from the network. |  |
| 8.1.8 | Shared spectrum access |  |  |  |  |  |
| 8.1.8.1 | Measurement configuration control and reporting for Shared spectrum |  |  |  |  |  |
| 8.1.8.1.1 | Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / intra-frequency |  | Rel-16 | C218 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |  |
| 8.1.8.1.2 | Measurement configuration control and reporting for Shared spectrum / RMTC / RSSI measurements / Channel Occupancy reporting / inter-frequency |  | Rel-16 | C218 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |  |
| 8.1.8.2 | Paging monitoring |  |  |  |  |  |
| 8.1.8.2.1 | Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring |  | Rel-16 | C217 | UEs supporting 5G Core and NR standalone shared spectrum channel access |  |
| 8.1.8.2.2 | Paging monitoring / multiple PDCCH monitoring occasions / Short message indication / stopPagingMonitoring / RRC inactive |  | Rel-16 | C247 | UEs supporting 5G Core and NR standalone shared spectrum channel access and RRC_INACTIVE |  |
| 8.2 | MR-DC RRC |  |  |  |  |
| 8.2.1 | UE Capability |  |  |  |  |
| 8.2.1.1 | UE capability transfer / Success |  |  |  |  |
| 8.2.1.1.1 | UE capability transfer / Success / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.1.1.2 | UE capability transfer / Success / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.1.2 | Void |  |  |  |  |
| 8.2.2 | Radio Bearer Addition, Modification and Release |  |  |  |  |
| 8.2.2.1 | Radio Bearer Addition, Modification and Release / SRB |  |  |  |  |
| 8.2.2.1.1 | SRB3 Establishment, Reconfiguration and Release / NR addition, modification and release / EN-DC | Rel-15 |  | C22 | UEs supporting EN-DC and SRB3 |
| 8.2.2.1.2 | SRB3 Establishment, Reconfiguration and Release / NR addition, modification and release / NR-DC | Rel-15 |  | C86 | UEs supporting NR-DC and SRB3 |
| 8.2.2.2 | Split SRB Establishment and Release |  |  |  |  |
| 8.2.2.2.1 | Split SRB Establishment and Release / EN-DC | Rel-15 |  | C61 | UEs supporting EN-DC and PDCP duplication over split SRB1/2 |
| 8.2.2.2.2 | Split SRB Establishment and Release / NR-DC | Rel-15 |  | C195 | UEs supporting NR-DC and PDCP duplication over split SRB1/2 |
| 8.2.2.2.3 | Split SRB Establishment and Release / NE-DC | Rel-15 |  | C196 | UEs supporting NE-DC and PDCP duplication over split SRB1/2 |
| 8.2.2.3 | Simultaneous SRB3 and Split SRB / Sequential message flow on SRB3 and Split SRB |  |  |  |  |
| 8.2.2.3.1 | Simultaneous SRB3 and Split SRB / Sequential message flow on SRB3 and Split SRB with one UL path / EN-DC | Rel-15 |  | C23 | UEs supporting EN-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| 8.2.2.3.2 | Simultaneous SRB3 and Split SRB / Sequential message flow on SRB3 and Split SRB with one UL path / NR-DC | Rel-15 |  | C157 | UEs supporting NR-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| 8.2.2.4 | PSCell Addition, Modification and Release / SCG DRB |  |  |  |  |
| 8.2.2.4.1 | PSCell addition, modification and release / SCG DRB / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.4.2 | PSCell addition, modification and release / SCG DRB / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.2.4.3 | PSCell addition, modification and release / SCG DRB / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.2.5 | PSCell Addition, Modification and Release / Split DRB |  |  |  |  |
| 8.2.2.5.1 | PSCell addition, modification and release / Split DRB / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.5.2 | PSCell addition, modification and release / Split DRB / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.2.5.3 | PSCell addition, modification and release / Split DRB / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.2.6 | Bearer Modification / MCG DRB |  |  |  |  |
| 8.2.2.6.1 | Bearer Modification / MCG DRB / SRB / PDCP version change / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.7 | Bearer Modification / Handling for bearer type change without security key change |  |  |  |  |
| 8.2.2.7.1 | Bearer Modification / Handling for bearer type change without security key change / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.7.2 | Bearer Modification / Handling for bearer type change without security key change / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.2.7.3 | Bearer Modification / Handling for bearer type change without security key change / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.2.8 | Bearer Modification / Handling for bearer type change with security key change |  |  |  |  |
| 8.2.2.8.1 | Bearer Modification / Handling for bearer type change with security key change / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.8.2 | Bearer Modification / Handling for bearer type change with security key change / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.2.8.3 | Bearer Modification / Handling for bearer type change with security key change / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.2.9 | Bearer Modification / Uplink data path / Split DRB Reconfiguration |  |  |  |  |
| 8.2.2.9.1 | Bearer Modification / Uplink data path / Split DRB Reconfiguration / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.2.9.2 | Bearer Modification / Uplink data path / Split DRB Reconfiguration / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.2.9.3 | Bearer Modification / Uplink data path / Split DRB Reconfiguration / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.3 | Measurement Configuration Control and Reporting / Handovers |  |  |  |  |
| 8.2.3.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells |  |  |  |  |
| 8.2.3.1.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.1.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / NE-DC |  | Rel-15 | C160 | UEs supporting NE-DC |
| 8.2.3.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements |  |  |  |  |
| 8.2.3.2.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.2.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B1 / Measurement of NR cells / RSRQ based measurements / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.3.3 | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of NR cells |  |  |  |  |
| 8.2.3.3.1 | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of NR cells / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.4 | Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell |  |  |  |  |
| 8.2.3.4.1 | Measurement configuration control and reporting / Event A1 / Measurement of NR PSCell / EN-DC | Rel-15 |  | C13 | UEs supporting EN-DC and NR measurements and Event A triggered reporting |
| 8.2.3.4.2 | Measurement configuration control and reporting / Event A1 / Measurement of E-UTRA PSCell / NE-DC |  | Rel-15 | C160 | UEs supporting NE-DC. |
| 8.2.3.5 | Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell |  |  |  |  |
| 8.2.3.5.1 | Measurement configuration control and reporting / Event A2 / Measurement of NR PSCell / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |
| 8.2.3.5.2 | Measurement configuration control and reporting / Event A2 / Measurement of E-UTRA PSCell / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC. |
| 8.2.3.6 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cells |  |  |  |  |
| 8.2.3.6.1 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cells / Intra-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.6.1a | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.6.1b | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 |  | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.6.2 | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cells / Intra-frequency measurements / NE-DC | Rel-15 |  | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| 8.2.3.6.2a | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cell / Inter-frequency measurements / NE-DC | Rel-15 |  | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.6.2b | Measurement configuration control and reporting / Event A3 / Measurement of Neighbour E-UTRA and NR cell / Inter-band measurements / NE-DC | Rel-15 |  | C183 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.7 | Measurement configuration control and reporting / Event A4 (intra-frequency, inter-frequency and inter-band measurements) / Measurement of Neighbour NR cell |  |  |  |  |
| 8.2.3.7.1 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.7.1a | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.7.1b | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 |  | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.7.2 | Measurement configuration control and reporting / Event A4 / Measurement of Neighbour E-UTRA and NR cells / Intra-frequency measurements / NE-DC | Rel-15 |  | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| 8.2.3.7.2a | Measurement configuration control and reporting / Event A4 / Measurement of Neighbor E-UTRA and NR cells / Inter-frequency measurements / NE-DC | Rel-15 |  | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.7.2b | Measurement configuration control and reporting / Event A4 / Measurement of Neighbor E-UTRA and NR cells / Inter-band measurements / NE-DC | Rel-15 |  | C183 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.8 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell |  |  |  |  |
| 8.2.3.8.1 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Intra-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.8.1a | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-frequency measurements / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.8.1b | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour NR cell / Inter-band measurements / EN-DC | Rel-15 |  | C93 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.8.2 | Measurement configuration control and reporting / Event A5 / Measurement of Neighbour E-UTRA and NR cells / Intra-frequency measurements / NE-DC |  | Rel-15 | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| 8.2.3.8.2a | Measurement configuration control and reporting / Event A5 / Measurement of Neighbor E-UTRA and NR cells / Inter-frequency measurements / NE-DC |  | Rel-15 | C182 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) |
| 8.2.3.8.2b | Measurement configuration control and reporting / Event A5 / Measurement of Neighbor E-UTRA and NR cells / Inter-band measurements / NE-DC |  | Rel-15 | C183 | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| 8.2.3.9 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR cell |  |  |  |  |
| 8.2.3.9.1 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based intra-frequency measurements / Measurement of Neighbour NR Cell / EN-DC | Rel-15 |  | C15 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| 8.2.3.10 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based inter-frequency measurements / Measurement of Neighbour NR cell |  |  |  |  |
| 8.2.3.10.1 | Measurement configuration control and reporting / SS/PBCH block based / CSI-RS based inter-frequency measurements / Measurement of Neighbour NR Cell / EN-DC | Rel-15 |  | C15 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements) and CSI-RSRP and CSI-RSRQ measurement |
| 8.2.3.11 | Measurement configuration control and reporting / Measurement Gaps |  |  |  |  |
| 8.2.3.11.1 | Measurement configuration control and reporting / Measurement Gaps / NR FR1 / EN-DC | Rel-15 |  | C24 | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |
| 8.2.3.11.2 | Measurement configuration control and reporting / Measurement Gaps / NR FR2 / EN-DC | Rel-15 |  | C25 | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |
| 8.2.3.11.3 | Measurement configuration control and reporting / Measurement Gaps / NR-DC | Rel-15 |  | C149 | UEs supporting NR-DC and two independent measurement gap configurations for FR1 and FR2 |
| 8.2.3.12 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of NR cells |  |  |  |  |
| 8.2.3.12.1 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of NR cells / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.12.2 | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of E-UTRA cells / NE-DC | Rel-15 |  | C206 | UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| 8.2.3.13 | PCell Handover with SCG change / Reconfiguration with sync / SCG DRB |  |  |  |  |
| 8.2.3.13.1 | PCell Handover with SCG change / Reconfiguration with sync / SCG DRB / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.13.2 | PCell Handover with SCG change on same PSCell / mobilityControlInfoSCG / SCG DRB / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.3.14 | SCG change / Reconfiguration with sync / Split DRB |  |  |  |  |
| 8.2.3.14.1 | SCG change / Reconfiguration with sync / Split DRB / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.3.14.2 | SCG change / Reconfiguration with sync / Split DRB / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.3.14.3 | SCG change with HO /mobilityControlInfoSCG / Split DRB / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.3.15 | Measurement configuration control and reporting / Two simultaneous events A2 and A3 (intra-frequency measurements) / Measurement of Neighbour NR cells |  |  |  |  |
| 8.2.3.15.1 | Measurement configuration control and reporting / Two simultaneous events A2 and A3 (intra-frequency measurements) / Measurement of Neighbour NR cells / EN-DC | Rel-15 |  | C14 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |
| 8.2.3.16 | Measurement configuration control and reporting / SRB3 |  |  |  |  |
| 8.2.3.16.1 | Measurement configuration control and reporting / SRB3 / Intra NR measurements / EN-DC | Rel-15 |  | C71 | UEs supporting EN-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| 8.2.3.16.2 | Measurement configuration control and reporting / SRB3 / Intra NR measurements / NR-DC | Rel-15 |  | C87 | UEs supporting NR-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| 8.2.3.17 | Measurement configuration control and reporting / SFTD |  |  |  |  |
| 8.2.3.17.1 | Measurement configuration control and reporting / SFTD / EN-DC | Rel-15 |  | C151 | UEs supporting EN-DC and SFTD measurement between E-UTRA PCell and an NR neighbour cell, and SFTD measurement between E-UTRA PCell and NR PSCell |
| 8.2.3.17.2 | Measurement configuration control and reporting / SFTD / NR-DC | Rel-15 |  | C152 | UEs supporting NR-DC and SFTD measurement between NR PCell and an NR neighbour cell, and SFTD measurement between NR PCell and NR PSCell |
| 8.2.3.17.3 | Measurement configuration control and reporting / SFTD / NE-DC | Rel-15 |  | C268 | UEs supporting NE-DC and SFTD measurement between NR PCell and E-UTRA PSCell |
| 8.2.3.18 | Conditional PSCell change |  |  |  |  |
| 8.2.3.18.1 | Conditional PSCell change / Success / EN-DC | Rel-16 |  | C153 | UEs supporting EN-DC and Conditional PSCell change |
| 8.2.3.18.2 | Conditional PSCell change / Failure / EN-DC | Rel-16 |  | C153 | UEs supporting EN-DC and Conditional PSCell change |
| 8.2.3.18.3 | Conditional PSCell change / PCell change / PSCell change / EN-DC | Rel-16 |  | C153 | UEs supporting EN-DC and Conditional PSCell change |
| 8.2.4 | Carrier Aggregation |  |  |  |  |
| 8.2.4.1 | NR CA / NR SCell addition / modification / release / Success |  |  |  |  |
| 8.2.4.1.1 | NR CA / NR SCell addition / modification / release / Success / EN-DC |  |  |  |  |
| 8.2.4.1.1.1 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band Contiguous CA | Rel-15 |  | C67 | UEs supporting EN-DC and Intra-Band Contiguous CA |
| 8.2.4.1.1.2 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Intra-band non-Contiguous CA | Rel-15 |  | C68 | UEs supporting EN-DC and Intra-Band Non-Contiguous CA |
| 8.2.4.1.1.3 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Inter-band CA | Rel-15 |  | C69 | UEs supporting EN-DC and Inter-Band CA |
| 8.2.4.1.1.4 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Intra-band Contiguous CA | Rel-16 |  | C199 | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Contiguous CA |
| 8.2.4.1.1.5 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Intra-band non-Contiguous CA | Rel-16 |  | C200 | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Non-Contiguous CA |
| 8.2.4.1.1.6 | NR CA / NR SCell addition / modification / release / Success / EN-DC / Active SCG SCell addition / Inter-band CA | Rel-16 |  | C201 | UEs supporting EN-DC, direct NR SCG SCell activation and Inter-Band CA |
| 8.2.4.1.2 | NR CA / NR SCell addition / modification / release / Success / NR-DC / Active SCG SCell addition |  |  |  |  |
| 8.2.4.1.2.1 | NR CA / NR SCell addition / modification / release / Success / NR-DC / Active SCG SCell addition / Intra-band Contiguous CA | Rel-16 |  | C202 | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band contiguous CA |
| 8.2.4.1.2.2 | NR CA / NR SCell addition / modification / release / Success / NR-DC / Active SCG SCell addition / Intra-band non-contiguous CA | Rel-16 |  | C203 | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band non-contiguous CA |
| 8.2.4.1.2.3 | NR CA / NR SCell addition / modification / release / Success / NR-DC / Active SCG SCell addition / Inter-band CA | Rel-16 |  | C204 | UEs supporting NR-DC, direct NR SCG SCell activation and inter-band CA |
| 8.2.4.2 | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release |  |  |  |  |
| 8.2.4.2.1 | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC |  |  |  |  |
| 8.2.4.2.1.1 | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Intra-band Contiguous CA | Rel-15 |  | C67 | UEs supporting EN-DC and Intra-Band Contiguous CA |
| 8.2.4.2.1.2 | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Intra-band non-Contiguous CA | Rel-15 |  | C68 | UEs supporting EN-DC and Intra-Band Non-Contiguous CA |
| 8.2.4.2.1.3 | NR CA / Simultaneous PSCell and SCell addition / PSCell and SCell change / CA Release / EN-DC / Inter-band CA | Rel-15 |  | C69 | UEs supporting EN-DC and Inter-Band CA |
| 8.2.4.3 | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 |  |  |  |  |
| 8.2.4.3.1 | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC |  |  |  |  |
| 8.2.4.3.1.1 | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band Contiguous CA | Rel-15 |  | C55 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band contiguous CA and SRB3 |
| 8.2.4.3.1.2 | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band non-Contiguous CA | Rel-15 |  | C57 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band non-contiguous CA and SRB3 |
| 8.2.4.3.1.3 | NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Inter-band CA | Rel-15 |  | C56 | UEs supporting EN-DC and NR measurements and Event A triggered reporting and inter-band CA and SRB3 |
| 8.2.5 | Reconfiguration Failure / Radio link failure |  |  |  |  |
| 8.2.5.1 | Radio link failure / PSCell addition failure |  |  |  |  |
| 8.2.5.1.1 | Radio link failure / Random access problem / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.5.1.2 | Radio link failure / Random access problem / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.5.2 | Radio link failure / PSCell out of sync indication |  |  |  |  |
| 8.2.5.2.1 | Radio link failure / PSCell out of sync indication / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.5.2.2 | Radio link failure / PSCell out of sync indication / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.5.3 | Radio link failure / rlc-MaxNumRetx failure |  |  |  |  |
| 8.2.5.3.1 | Radio link failure / rlc-MaxNumRetx failure / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.5.3.2 | Radio link failure / rlc-MaxNumRetx failure / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.5.3.3 | Radio link failure / rlc-MaxNumRetx failure / NE-DC | Rel-15 |  | C160 | UEs supporting NE-DC |
| 8.2.5.4 | Reconfiguration failure / SCG change failure |  |  |  |  |
| 8.2.5.4.1 | Reconfiguration failure / SCG change failure / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.5.4.2 | Reconfiguration failure / SCG change failure / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.5.5 | Reconfiguration failure / SCG Reconfiguration failure / SRB3 |  |  |  |  |
| 8.2.5.5.1 | Void |  |  |  |  |
| 8.2.5.6 | Reconfiguration failure / SCG Reconfiguration failure / SRB1 |  |  |  |  |
| 8.2.5.6.1 | Void |  |  |  |  |
| 8.2.5.7 | Radio link failure / Shared spectrum / LBT Failure |  |  |  |  |
| 8.2.5.7.1 | Radio link failure / LBT Failure / EN-DC | Rel-16 |  | C243 | UEs supporting 5G Core and EN-DC with NR shared spectrum channel access |
| 8.2.5.7.2 | Radio link failure / LBT Failure / NR-DC | Rel-16 |  | C244 | UEs supporting 5G Core and NR-DC with NR shared spectrum channel access |
| 8.2.6 | MR-DC RRC others |  |  |  |  |
| 8.2.6.1 | Failure information / RLC failure / SCG |  |  |  |  |
| 8.2.6.1.1 | Failure information / RLC failure / SCG / EN-DC |  |  |  |  |
| 8.2.6.1.1.1 | Failure information / RLC failure / SCG / EN-DC / Intra-band Contiguous CA | Rel-15 |  | C75 | UEs supporting EN-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| 8.2.6.1.1.2 | Failure information / RLC failure / SCG / EN-DC / Inter-band CA | Rel-15 |  | C76 | UEs supporting EN-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| 8.2.6.1.1.3 | Failure information / RLC failure / SCG / EN-DC / Intra-band non Contiguous CA | Rel-15 |  | C77 | UEs supporting EN-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |
| 8.2.6.1.2 | Failure information / RLC failure / SCG / NR-DC |  |  |  |  |
| 8.2.6.1.2.1 | Failure information / RLC failure / SCG / NR-DC / Intra-band Contiguous CA | Rel-15 |  | C88 | UEs supporting NR-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.2.6.1.2.2 | Failure information / RLC failure / SCG / NR-DC / Inter-band CA | Rel-15 |  | C89 | UEs supporting NR-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.2.6.1.2.3 | Failure information / RLC failure / SCG / NR-DC / Intra-band non Contiguous CA | Rel-15 |  | C90 | UEs supporting NR-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| 8.2.6.2 | Processing delay |  |  |  |  |
| 8.2.6.2.1 | Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC | Rel-15 |  | C01 | UEs supporting EN-DC |
| 8.2.6.2.2 | Processing delay / Latency check / NR-DC | Rel-15 |  | C80 | UEs supporting NR-DC |
| 8.2.6.2.4 | Processing delay / RRC_INACTIVE / Latency check / NR-DC | Rel-16 |  | C229 | UEs supporting 5G Core and NR-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |
| 8.2.6.3 | Idle/Inactive measurements |  |  |  |  |
| 8.2.6.3.1 | Idle/Inactive measurements / Idle mode / EN-DC / SIB5 & SIB24 configuration | Rel-16 |  | C225 | UEs supporting EN-DC and Idle/Inactive Measurements |
| 8.2.6.3.2 | Idle/Inactive measurements / Idle mode / EN-DC / RRCConnectionRelease configuration | Rel-16 |  | C225 | UEs supporting EN-DC and Idle/Inactive Measurements |
| 8.2.6.3.3 | Idle/Inactive measurements / Inactive mode / NE-DC / SIB11 configuration | Rel-16 |  | C193 | UEs supporting 5GC Core, E-UTRA, RRC_INACTIVE and Idle/Inactive Measurements |
| 8.2.6.3.4 | Idle/Inactive measurements / Inactive mode / NE-DC / RRCRelease configuration | Rel-16 |  | C193 | UEs supporting 5GC Core, E-UTRA, RRC_INACTIVE and Idle/Inactive Measurements |
| 8.2.6.3.5 | Idle/Inactive Measurements / Idle mode / NE-DC / SIB11 configuration | Rel-16 |  | C191 | UEs supporting 5G Core, E-UTRA and Idle/Inactive Measurements |
| 8.2.6.3.6 | Idle/Inactive Measurements / Idle mode / NE-DC / RRCRelease configuration | Rel-16 |  | C191 | UEs supporting 5GC Core, E-UTRA and Idle/Inactive Measurements |
| 8.2.6.4 | UPIP / RRC connection |  |  |  |  |
| 8.2.6.4.1 | UPIP / RRC connection establishment / Success / Reception of SecurityModeCommand by the UE |  | Rel-17 | C286 | UEs supporting EN-DC and user plane integrity protection with EPS |
| 8.2.6.4.2 | UPIP / RRC connection re-establishment / Reception of the RRCConnection Reestablishment by UE |  | Rel-17 | C286 | UEs supporting EN-DC and user plane integrity protection with EPS |
| 8.2.6.4.3 | UPIP / RRC connection reconfiguration / Handover / Success / Reception of RRCConnctionReconfiguration including mobilityControlInfo by UE | Rel-17 |  | C286 | UEs supporting EN-DC and user plane integrity protection with EPS |
| 8.2.6.4.4 | UPIP / Inter-RAT mobility - Handover to E-UTRA - Reception of RRCConnection Reconfiguration by UE | Rel-17 |  | C286 | UEs supporting EN-DC and user plane integrity protection with EPS |
| 8.2.7 | RRC resume |  |  |  |  |
| 8.2.7.1 | RRC resume / EN-DC |  |  |  |  |
| 8.2.7.2 | RRC resume / NR-DC |  |  |  |  |
| 8.2.7.2.1 | RRC Resume / Suspend-Resume / RRC reconfiguration / NR-DC / Resume with SCG | Rel-16 |  | C229 | UEs supporting 5G Core and NR-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |
| 8.2.7.3.1 | RRC Resume / Suspend-Resume / RRC reconfiguration / NE-DC / Resume with SCG | Rel-16 |  | C255 | UEs supporting 5G Core and NE-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |

Table 4.1-3b: Additional Information of Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 8.1.1 |  |  |  |  |
| 8.1.1.1 |  |  |  |  |
| 8.1.1.1.1 | pc_inactiveState |  |  |  |
| 8.1.1.1.2 | pc_inactiveState |  |  |  |
| 8.1.1.3 |  |  |  |  |
| 8.1.1.3.2 |  |  |  | Rel-15 E-UTRA |
| 8.1.1.3.4 |  |  |  | Rel-15 E-UTRA |
| 8.1.1.3.7a |  |  |  | Rel-15 E-UTRA |
| 8.1.3 |  |  |  |  |
| 8.1.3.1 |  |  |  |  |
| 8.1.3.1.2 |  |  |  |  |
| 8.1.3.1.3 |  |  | If 8.1.3.1.2 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.4 |  |  | If 8.1.3.1.2 or 8.1.3.1.3 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.5 |  |  | If 8.1.3.1.6 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.6 |  |  |  |  |
| 8.1.3.1.7 |  |  | If 8.1.3.1.5 or 8.1.3.1.6 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.8 |  |  | If 8.1.3.1.9 or 8.1.3.1.10 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.9 |  |  | If 8.1.3.1.10 is executed this test case is optional (Note 2) |  |
| 8.1.3.1.10 |  |  |  |  |
| 8.1.3.1.23 | pc_inactiveState |  |  |  |
| 8.1.3.2 |  |  |  |  |
| 8.1.3.2.6 |  |  |  | Rel-16 UTRA |
| 8.1.3.2.7 |  |  |  | Rel-16 UTRA |
| 8.1.4 |  |  |  |  |
| 8.1.4.1 |  |  |  |  |
| 8.1.4.1.2 |  | px_NAS_5GC_CipheringAlgorithm px_NAS_5GC_IntegrityAlgorithm |  |  |
| 8.1.4.1.10 |  |  | Note 4 |  |
| 8.1.4.2 |  |  |  |  |
| 8.1.4.2.1 |  |  |  |  |
| 8.1.4.2.1.1 |  |  |  | Rel-15 E-UTRA |
| 8.1.4.2.1.2 |  |  |  | Rel-16 EN-DC |
| 8.1.4.2.2 |  |  |  |  |
| 8.1.4.2.2.1 |  |  |  | Rel-15 E-UTRA |
| 8.1.5 |  |  |  |  |
| 8.1.5.1 |  |  |  |  |
| 8.1.5.1.1 |  |  | If 8.2.1.1.2 is executed this test case is optional |  |
| 8.1.5.7 |  |  |  |  |
| 8.1.5.7.1 |  |  |  |  |
| 8.1.5.7.1.1 |  |  | If 8.1.5.7.1.2 or 8.1.5.7.1.3 is executed this test case is optional |  |
| 8.1.5.7.1.2 |  |  | If 8.1.5.7.1.1 or 8.1.5.7.1.3 is executed this test case is optional |  |
| 8.1.5.7.1.3 |  |  | If 8.1.5.7.1.1 or 8.1.5.7.1.2 is executed this test case is optional |  |
| 8.1.5.8 |  |  |  |  |
| 8.1.5.8.1 | pc_inactiveState |  |  |  |
| 8.1.5.8.2 |  |  |  |  |
| 8.1.5.8.2.1 | pc_inactiveState |  | If 8.1.5.8.2.2 or 8.1.5.8.2.3 is executed this test case is optional |  |
| 8.1.5.8.2.2 | pc_inactiveState |  | If 8.1.5.8.2.1 or 8.1.5.8.2.3 is executed this test case is optional |  |
| 8.1.5.8.2.3 | pc_inactiveState |  | If 8.1.5.8.2.1 or 8.1.5.8.2.2 is executed this test case is optional |  |
| 8.1.5.9 |  |  |  |  |
| 8.1.5.9.1 | pc_Set_UE_Cap_Info_NR |  |  |  |
| 8.1.6 |  |  |  |  |
| 8.1.6.1 |  |  |  |  |
| 8.1.6.1.3 |  |  |  |  |
| 8.1.6.1.3.1 |  |  | If 8.1.6.1.3.5 is executed this test case is optional. |  |
| 8.1.6.2 |  |  |  |  |
| 8.1.6.2.1 |  |  |  | Rel-15 E-UTRA |
| 8.1.6.2.2 |  |  |  | Rel-15 E-UTRA |
| 8.1.6.2.3 |  |  |  | Rel-15 E-UTRA |
| 8.1.6.2.4 |  |  |  | Rel-15 E-UTRA |
| 8.2.1 |  |  |  |  |
| 8.2.2 |  |  |  |  |
| 8.2.2.1 |  |  |  |  |
| 8.2.2.1.1 |  |  | Only executed if test case 8.2.2.3.1 is not applicable (Note 1) |  |
| 8.2.2.1.2 |  |  | Only executed if test case 8.2.2.3.2 is not applicable (Note 1) |  |
| 8.2.3 |  |  |  |  |
| 8.2.3.6 |  |  |  |  |
| 8.2.3.6.1 |  |  |  |  |
| 8.2.3.6.1a |  |  | If 8.2.3.6.1 is executed this test case is optional (Note 3) |  |
| 8.2.3.6.1b |  |  | If 8.2.3.6.1 or 8.2.3.6.1a is executed this test case is optional (Note 3) |  |
| 8.2.3.7 |  |  |  |  |
| 8.2.3.7.1 |  |  |  |  |
| 8.2.3.7.1a |  |  | If 8.2.3.7.1 is executed this test case is optional (Note 3) |  |
| 8.2.3.7.1b |  |  | If 8.2.3.7.1 or 8.2.3.7.1a is executed this test case is optional (Note 3) |  |
| 8.2.3.8 |  |  |  |  |
| 8.2.3.8.1 |  |  |  |  |
| 8.2.3.8.1a |  |  | If 8.2.3.8.1 is executed this test case is optional (Note 3) |  |
| 8.2.3.8.1b |  |  | If 8.2.3.8.1 or 8.2.3.8.1a is executed this test case is optional (Note 3) |  |
| 8.2.4 |  |  |  |  |
| 8.2.4.1 |  |  |  |  |
| 8.2.4.1.1 |  |  |  |  |
| 8.2.4.1.1.4 |  |  |  | Rel-15 E-UTRA |
| 8.2.4.1.1.5 |  |  |  | Rel-15 E-UTRA |
| 8.2.4.1.1.6 |  |  |  | Rel-15 E-UTRA |
| 8.2.6 |  |  |  |  |
| 8.2.6.1 |  |  |  |  |
| 8.2.6.1.1 |  |  |  |  |
| 8.2.6.1.1.1 |  |  | If 8.2.6.1.1.2 or 8.2.6.1.1.3 is executed this test case is optional |  |
| 8.2.6.1.1.2 |  |  | If 8.2.6.1.1.1 or 8.2.6.1.1.3 is executed this test case is optional |  |
| 8.2.6.1.1.3 |  |  | If 8.2.6.1.1.1 or 8.2.6.1.1.2 is executed this test case is optional |  |
| 8.2.6.1.2 |  |  |  |  |
| 8.2.6.1.2.1 |  |  | If 8.2.6.1.2.2 or 8.2.6.1.2.3 is executed this test case is optional |  |
| 8.2.6.1.2.2 |  |  | If 8.2.6.1.2.1 or 8.2.6.1.2.3 is executed this test case is optional |  |
| 8.2.6.1.2.3 |  |  | If 8.2.6.1.2.1 or 8.2.6.1.2.2 is executed this test case is optional |  |
| 8.2.6.2 |  |  |  |  |
| 8.2.6.2.4 | pc_reducedCP_Latency |  |  |  |
| Note 1: Test cases 8.2.2.3.1 also verifies the core requirements covered by test case 8.2.2.1.1 but it is not applicable to all UE. Test case 8.2.2.3.2 and 8.2.2.1.2 are also in the same situation. Note 2: Only one among the three intra-frequency, inter-frequency and inter-band variants is required to be executed making sure all three variants are tested at least once across measurement events A3/A4/A5. Note 3: Only intra frequency among the three (intra-frequency, inter-frequency and inter-band) variants is required to be executed for measurement events A3/A4/A5 based on initial market requirements. May change in future similar to Note 2. Note 4: This test case can optionally be executed from Release 15 onwards. |  |  |  |  |

Table 4.1-4a: Applicability of Protocol conformance Mobility and Session management test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 9 | Mobility management |  |  |  |
| 9.1 | 5GS mobility management |  |  |  |
| 9.1.1 | Primary authentication and key agreement |  |  |  |
| 9.1.1.1 | EAP based primary authentication and key agreement / EAP-AKA' related procedures | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.1.2 | EAP based primary authentication and key agreement / Reject | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.1.3 | EAP based primary authentication and key agreement / EAP message transport / Abnormal | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.1.4 | 5G AKA based primary authentication and key agreement / 5G-AKA related procedures | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.1.5 | 5G AKA based primary authentication and key agreement / Reject | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.1.6 | 5G AKA based primary authentication and key agreement / Abnormal | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2 | Security mode control |  |  |  |
| 9.1.2.1 | NAS security mode command | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.2 | Protection of initial NAS signalling messages | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.3 | Integrity protection / Correct functionality of 5G NAS integrity algorithm / SNOW3G | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.4 | Integrity protection / Correct functionality of 5G NAS integrity algorithm / AES | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.5 | Integrity protection / Correct functionality of 5G NAS integrity algorithm / ZUC | Rel-15 | C84 | UEs supporting 5G Core and ZUC algorithm |
| 9.1.2.6 | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / SNOW3G | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.7 | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / AES | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.2.8 | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / ZUC | Rel-15 | C84 | UEs supporting 5G Core and ZUC algorithm |
| 9.1.3 | Identification |  |  |  |
| 9.1.3.1 | Identification procedure | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.4 | Generic UE configuration update |  |  |  |
| 9.1.4.1 | Generic UE configuration update / New 5G-GUTI, NITZ, registration requested, network slicing indication, new allowed NSSAI / Acknowledgement from the UE | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.4.2 | UAS / Generic UE configuration update / Revocation | Rel-17 | C310 | UEs supporting 5G Core and UAS |
| 9.1.5 | Registration |  |  |  |
| 9.1.5.1 | Initial registration |  |  |  |
| 9.1.5.1.1 | Initial registration / Success / 5G-GUTI reallocation, last visited TAI | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.2 | Initial registration / 5GS services / Equivalent PLMN list handling | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.3 | Initial registration / 5GS services / NSSAI handling | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.3a | Initial registration / 5GS services / NSSAI handling / NSSAI storage | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.4 | Initial registration / 5GS services / MICO mode / TAI list handling | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.5 | Initial registration / Abnormal / Failure after 5 attempts | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.6 | Initial registration / Rejected / Illegal UE | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.7 | Void |  |  |  |
| 9.1.5.1.8 | Initial registration / Rejected / Serving network not authorized | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.9 | Initial registration / Abnormal / Change of cell into a new tracking area | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.10 | Initial registration / Rejected / PLMN not allowed | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.11 | Initial registration / Rejected / Tracking area not allowed | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.12 | Initial registration / Rejected / Roaming not allowed in this tracking area | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.13 | Initial registration / Rejected / No suitable cells in tracking area | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.14 | Initial registration / Rejected / Congestion / Abnormal / T3346 | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.1.15 | Initial registration / Success / Extended and spare fields in CAG information list | Rel-15 only | C21 | UEs supporting 5G Core |
| 9.1.5.1.16 | Initial Registration / Success / MUSIM | Rel-17 | C219 | UEs supporting 5G Core and Multi-SIM features |
| 9.1.5.1.17 | Initial registration / Success / UAS | Rel-17 | C310 | UEs supporting 5G Core and UAS |
| 9.1.5.2 | Mobility and periodic registration update |  |  |  |
| 9.1.5.2.1 | Mobility registration update / TAI list handling | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.2.2 | Periodic registration update / Accepted | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.2.4 | Mobility registration update / The lower layer requests NAS signalling connection recovery | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.2.5 | Void |  |  |  |
| 9.1.5.2.7 | Mobility and periodic registration update / Rejected / UE identity cannot be derived by the network | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.2.8 | Mobility and periodic registration update / Rejected / Implicitly de-registered | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.5.2.9 | Void |  |  |  |
| 9.1.5.2.10 | Mobility registration update / MUSIM / NAS signalling connection release | Rel-17 | C242 | UEs supporting 5G Core and Multi-SIM N1 NAS signalling connection release |
| 9.1.6 | De-registration |  |  |  |
| 9.1.6.1 | UE-initiated de-registration |  |  |  |
| 9.1.6.1.1 | UE-initiated de-registration / Switch off / Abnormal / De-registration and 5GMM common procedure collision | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.6.1.2 | UE-initiated de-registration / Normal de-registration / Abnormal / Transmission failure without TAI change from lower layers, de-registration and 5GMM common procedure collision, T3521 timeout | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.6.1.3 | UE-initiated de-registration / Abnormal / Change of cell into a new tracking area | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.6.1.4 | Void |  |  |  |
| 9.1.6.2 | Network-initiated de-registration |  |  |  |
| 9.1.6.2.1 | Network-initiated de-registration / De-registration for 3GPP access / Re-registration required | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.6.2.2 | Network-initiated de-registration / De-registration for 3GPP access / Re-registration not required | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.6.2.3 | UAS / De-registration / UE-initiated / Network-initiated | Rel-17 | C310 | UEs supporting 5G Core and UAS |
| 9.1.7 | Service request |  |  |  |
| 9.1.7.1 | Service request / Idle mode uplink user data transport / Rejected / Restricted service area, abnormal / T3517, T3525 | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.7.2 | Service request / Connected mode user data transport / Abnormal / T3517 | Rel-15 | C21 | UEs supporting 5G Core |
| 9.1.7.3 | Service request / MUSIM / NAS signalling connection release | Rel-17 | C242 | UEs supporting 5G Core and Multi-SIM N1 NAS signalling connection release |
| 9.1.7.4 | Service request / MUSIM / Rejection of paging | Rel-17 | C220 | UEs supporting 5G Core and Multi-SIM Reject paging request |
| 9.1.8 | SMS over NAS |  |  |  |
| 9.1.8.1 | SMS over NAS / MO and MT SMS over NAS / Idle mode | Rel-15 | C33 | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |
| 9.1.8.2 | SMS over NAS / Multiple MO and MT SMS over NAS / Connected mode | Rel-15 | C33 | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |
| 9.1.9 | RACS |  |  |  |
| 9.1.9.1 | RACS / Network assigned UE radio capability ID | Rel-16 | C108 | UEs supporting 5G Core and RACS |
| 9.1.9.2 | RACS / UE configuration update / UE radio capability ID | Rel-16 | C108 | UEs supporting 5G Core and RACS |
| 9.1.9.3 | RACS / PLMN change within registration area / From NW assigned to Manufacturer assigned UE Radio Capability ID | Rel-16 | C177 | UEs supporting 5G Core and RACS and Manufacturer assigned Radio Capability ID |
| 9.1.9.4 | RACS / USIM change / Handling of URCID | Rel-16 | C108 | UEs supporting 5G Core and RACS |
| 9.1.9.5 | RACS / Handling of delete indication for NW assigned UE radio capability ID | Rel-16 | C108 | UEs supporting 5G Core and RACS |
| 9.1.9.6 | RACS / Change in radio capability / NW assigned URCID | Rel-16 | C108 | UEs supporting 5G Core and RACS |
| 9.1.9.7 | RACS / Inter-system mobility registration update / Handling of UE radio capability ID | Rel-16 | C178 | UEs supporting 5G Core and E-UTRA and RACS |
| 9.1.10 | Network slice-specific authentication and authorization |  |  |  |
| 9.1.10.1 | NSSAA / EAP message transport / Success | Rel-16 | C147 | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| 9.1.10.2 | NSSAA / EAP message transport / Abnormal | Rel-16 | C147 | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| 9.1.10.3 | NSSAA / Initial registration / Rejected NSSAI, pending NSSAI | Rel-16 | C147 | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| 9.1.10.4 | NSSAA / Initial registration / Reject | Rel-16 | C147 | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| 9.1.10.6 | NSSAA / UE configuration update / Rejected NSSAI | Rel-16 | C147 | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| 9.1.11 | SNPN / Mobility management aspects |  |  |  |
| 9.1.11.1 | SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 9.1.11.2 | SNPN / Initial registration / Rejected / Permanently not authorized for this SNPN | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 9.1.11.3 | SNPN / EAP based primary authentication and key agreement / EAP-AKA' related procedures | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 9.1.12 | NSAC / Mobility management aspects |  |  |  |
| 9.1.12.1 | NSAC / Initial registration / Back-off timer | Rel-17 | C21 | UEs supporting 5G Core |
| 9.1.12.2 | NSAC / Initial registration / Back-off timer is not provided or zero | Rel-17 | C21 | UEs supporting 5G Core |
| 9.1.12.3 | NSAC / Initial registration / Rejected / equivalent PLMNs | Rel-17 | C21 | UEs supporting 5G Core |
| 9.1.12.4 | NSAC / Generic UE configuration update / Rejected NSSAI | Rel-17 | C21 | UEs supporting 5G Core |
| 9.1.12.5 | NSAC / De-registration / 5GMM cause value #62 and rejected NSSAI | Rel-17 | C21 | UEs supporting 5G Core |
| 9.1.13 | NSSRG / Mobility management aspects |  |  |  |
| 9.1.13.1 | NSSRG / Initial registration | Rel-17 | C230 | UEs supporting 5G Core and NSSRG |
| 9.1.13.2 | NSSRG / Generic UE configuration update | Rel-17 | C230 | UEs supporting 5G Core and NSSRG |
| 9.1.14 | Paging Early Indication with Paging Subgrouping Assistance |  |  |  |
| 9.1.14.1 | Paging Early Indication with Subgrouping / RRC_IDLE / lastUsedCellOnly not configured / Subgroup ID selection | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 9.2 | 5GS Non-3GPP Access Mobility Management |  |  |  |
| 9.2.1 | Primary authentication and key agreement procedure |  |  |  |
| 9.2.1.1 | EAP based primary authentication and key agreement | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.1.2 | 5G AKA based primary authentication and key agreement | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.2 | Security Mode Control |  |  |  |
| 9.2.2.1 | NAS security mode command | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.2.2 | Protection of initial NAS signalling messages | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.3 | Void |  |  |  |
| 9.2.4 | Generic UE configuration |  |  |  |
| 9.2.4.1 | Generic UE configuration update | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.5 | Registration |  |  |  |
| 9.2.5.1 | Initial Registration |  |  |  |
| 9.2.5.1.1 | Initial registration / Success / 5G-GUTI reallocation, Last visited TAI | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.5.1.2 | Initial registration / 5GS services / NSSAI handling | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.5.1.3 | Void |  |  |  |
| 9.2.5.1.4 | Initial registration / Rejected / Congestion / Abnormal cases / T3346 | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.5.2 | Mobility Registration |  |  |  |
| 9.2.5.2.1 | Void |  |  |  |
| 9.2.5.2.2 | Mobility registration update/Change of SMS over NAS capability | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.6 | De-registration |  |  |  |
| 9.2.6.1 | UE-initiated de-registration |  |  |  |
| 9.2.6.1.1 | UE-initiated de-registration / switch off | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.6.2 | Network-initiated de-registration |  |  |  |
| 9.2.6.2.1 | Network-initiated de-registration / De-registration for Non-3GPP access / Re-registration required | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.6.2.2 | Network-initiated de-registration / De-registration for Non 3GPP access / Re-registration not required | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.7 | Service request |  |  |  |
| 9.2.7.1 | Service request / IDLE mode uplink user data transport / Rejected / Restricted service area, Abnormal / T3517 | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 9.2.7.2 | Service request / CMM CONNECTED mode/uplink user data transport / Abnormal / T3517 | Rel-15 | C58 | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |
| 9.2.8 | SMS over NAS |  |  |  |
| 9.2.8.1 | SMS over NAS / MO SMS over NAS - 5GMM-Idle mode | Rel-15 | C30 | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |
| 9.3 | Inter-system mobility |  |  |  |
| 9.3.1 | 5GS-EPC Inter-system mobility |  |  |  |
| 9.3.1.1 | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / 5GC to EPC | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 9.3.1.2 | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 9.3.1.3 | Inter-system mobility and periodic registration update / Rejected / Single-registration mode with N26 / Handling of EPC relevant parameters | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 9.3.1.4 | NSAC / interworking with EPC | Rel-17 | C260 | UEs supporting 5GS and E-UTRA and NSSRG |
| 10 | Session management |  |  |  |
| 10.1 | 5GS session management |  |  |  |
| 10.1.1 | PDU session authentication and authorization |  |  |  |
| 10.1.1.1 | PDU session authentication and authorization / During the UE-requested PDU session procedure | Rel-15 | C39A | UEs supporting 5G Core and additional UE-requested PDU establishment and support of EAP-AKA’ as EAP method for PDU session authentication and authorization |
| 10.1.1.2 | PDU session authentication and authorization / After the UE-requested PDU session procedure | Rel-15 | C39A | UEs supporting 5G Core and additional UE-requested PDU establishment and support of EAP-AKA’ as EAP method for PDU session authentication and authorization |
| 10.1.2 | Network-requested PDU session modification |  |  |  |
| 10.1.2.1 | Network-requested PDU session modification / Accepted | Rel-15 | C21 | UEs supporting 5G Core |
| 10.1.2.2 | Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE | Rel-15 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.3 | Network-requested PDU session release |  |  |  |
| 10.1.3.1 | Void |  |  |  |
| 10.1.3.2 | Network-requested PDU session release / Insufficient resources, insufficient resources for specific slice and DNN, abnormal / Invalid PDU session identity | Rel-15 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.4 | UE-requested PDU session establishment |  |  |  |
| 10.1.4.1 | UE-requested PDU session establishment / Abnormal / T3580 | Rel-15 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.4.3 | UAS / UE requested PDU session establishment / UUAA / C2 authorisation / Modification / Release | Rel-17 | C310 | UEs supporting 5G Core and UAS |
| 10.1.5 | UE-requested PDU session modification |  |  |  |
| 10.1.5.1 | UE-requested PDU session modification | Rel-15 | C63 | UEs supporting 5G Core and UE requested PDU session modification procedure |
| 10.1.6 | UE-requested PDU session release |  |  |  |
| 10.1.6.1 | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session modification procedure | Rel-15 | C21 | UEs supporting 5G Core |
| 10.1.6.2 | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session release procedure | Rel-15 | C21 | UEs supporting 5G Core |
| 10.1.7 | Network-requested PDU session release |  |  |  |
| 10.1.7.1 | Void |  |  |  |
| 10.1.8 | NSAC / Session management aspects |  |  |  |
| 10.1.8.1 | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is neither zero nor deactivated | Rel-17 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.8.2 | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is deactivated | Rel-17 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.8.3 | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is zero or not included | Rel-17 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.8.4 | NSAC / 5GSM message not forwarded / Back-off timer | Rel-17 | C39 | UEs supporting 5G Core and additional UE-requested PDU establishment |
| 10.1.8.5 | NSAC / Maximum number of PDU sessions reached / Emergency service | Rel-17 | C261 | UEs supporting 5G Core and additional UE-requested PDU establishment and emergency services in NR connected to 5GCN |
| 10.2 | EN-DC session management |  |  |  |
| 10.2.1 | Network initiated procedures |  |  |  |
| 10.2.1.1 | Default EPS bearer context activation | Rel-15 | C01 | UEs supporting EN-DC |
| 10.2.1.2 | Dedicated EPS bearer context activation | Rel-15 | C01 | UEs supporting EN-DC |
| 10.2.2 | UE initiated procedures |  |  |  |
| 10.2.2.1 | EPS bearer resource allocation / modification | Rel-15 | C16 | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |
| 10.3 | 5GS Non-3GPP Access Session Management |  |  |  |
| 10.3.1 | PDU session authentication and authorization |  |  |  |
| 10.3.1.1 | PDU session authentication and authorization / during the UE-requested PDU session procedure | Rel-15 | C159 | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment |
| 10.3.2 | Network-requested PDU session modification |  |  |  |
| 10.3.2.1 | Network-requested PDU session modification /Accepted/Rejected | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 10.3.3 | Network-requested PDU session Release |  |  |  |
| 10.3.3.1 | Network-requested PDU session release / accepted/ with and without reactivation | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 10.3.4 | UE-requested PDU session establishment |  |  |  |
| 10.3.4.1 | UE-requested PDU session establishment / Abnormal / T3580 | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 10.3.5 | UE-requested PDU session modification |  |  |  |
| 10.3.5.1 | UE-requested PDU session modification/Success | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 10.3.6 | UE-requested PDU session release |  |  |  |
| 10.3.6.1 | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session modification procedure | Rel-15 | C29 | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| 10.4 | ATSSS session management |  |  |  |
| 10.4.1 | UE-requested MA PDU session management |  |  |  |
| 10.4.1.1 | UE-requested MA PDU session establishment / ATSSS / Registered to same PLMNs over 3GPP and non-3GPP accesses simultaneously / Success | Rel-16 | C251 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.1.2 | UE-requested MA PDU session establishment / ATSSS / Registered to same PLMNs over 3GPP and non-3GPP accesses asimultaneously / Success | Rel-16 | C251 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.1.3 | UE-requested MA PDU session establishment / ATSSS / Registered to different PLMNs over 3GPP and non-3GPP accesses simultaneously/ Success | Rel-16 | C251 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.1.4 | UE-requested MA PDU session establishment / ATSSS / Registered to different PLMNs over 3GPP and non-3GPP accesses asynchronously / Success | Rel-16 | C251 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.2 | Network-requested MA PDU session management |  |  |  |
| 10.4.2.2 | Network-requested MA PDU session release / ATSSS / Accepted | Rel-16 | C251 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.1.5 | UE-requested MA PDU session modification / ATSSS / Success | Rel-17 | C275 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |
| 10.4.1.6 | UE-requested MA PDU session modification / ATSSS / Abnormal / MA PDU session is not allowed | Rel-17 | C275 | UEs supporting 5G Core and 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment and ATSSS |

Table 4.1-4b: Additional Information of Applicability of Protocol conformance Mobility and Session Management test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 9 |  |  |  |  |
| 9.1 |  |  |  |  |
| 9.1.6 |  |  |  |  |
| 9.1.6.1 |  |  |  |  |
| 9.1.6.1.1 | [10] pc_USIM_Removal |  |  |  |
| 9.2 |  |  |  |  |
| 9.2.6 |  |  |  |  |
| 9.2.6.1 |  |  |  |  |
| 9.2.6.1.1 | [10] pc_USIM_Removal |  |  |  |
| 9.2.7 |  |  |  |  |
| 9.2.7.2 | [10] pc_IPv4 [10] pc_IPv6 |  |  |  |
| 9.3 |  |  |  |  |
| 9.3.1 |  |  |  |  |
| 9.3.1.1 |  |  |  | Rel-15 E-UTRA |
| 9.3.1.2 |  |  |  | Rel-15 E-UTRA |
| 9.3.1.3 |  |  |  | Rel-15 E-UTRA |
| 10 |  |  |  |  |
| 10.1 |  |  |  |  |

Table 4.1-5a: Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 11 | Multi-layer and Services |  |  |  |
| 11.1 | 5GS / EPS Fallback |  |  |  |
| 11.1.1 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.1a | MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.2 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3a | MO MMTEL enhanced voice service call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.4 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell selection using cell status barred / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.5 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell selection using cell status reservation / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.6 | MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback |
| 11.1.7 | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success | Rel-15 | C47 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| 11.1.8 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.1.9 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.2 | 5G-SRVCC |  |  |  |
| 11.2.1 | 5G-SRVCC from NG-RAN to 3GPP UTRAN | Rel-16 | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 11.3 | Unified Access Control (UAC) |  |  |  |
| 11.3.1 | UAC / Access Identity 0 / 0% access probability / MTSI MO speech call / SMSoIP | Rel-15 | C78 | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |
| 11.3.1a | UAC / Access Identity 0 / 0% access probability / Uplink user data transfer / RRC_INACTIVE | Rel-15 | C109A | UEs supporting 5G Core and RRC_INACTIVE and UE's usage setting as data centric |
| 11.3.2 | UAC / Access Identity 0 / 0% access probability / Paging for MT access/Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.3 | UAC / Access Identity 0 / AC8 / RRC_INACTIVE / RNA update / RRC resume | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 11.3.4 | UAC / Access Identity 0 / Registration procedure for mobility and periodic registration update / Barring per PLMN / Implicit AC barring list | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.5 | UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MPS indicator / HPLMN/0%/100% accessibility AC5 / MMTEL-Video call | Rel-15 | C79 | UEs supporting 5G Core and Initiating session and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 and NG114 v1.0 |
| 11.3.6 | UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.6a | UAC / Access Identity 2 / MCS indicator / SNPN / 0% / 100% accessibility AC7 / RRC_INACTIVE | Rel-16 | C231 | UEs supporting 5G Core and SNPN and configuration of access identities in the list of subscriber data |
| 11.3.7 | UAC / Access Identity 11..15 / High priority access / HPLMN/0% accessibility AC2 / Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.8 | UAC / Access Identity 0 / NR RRC_IDLE / Cell re-selection while T390 is running | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9 | UAC / Access Identity 0 / ODAC / PLMN / RPLMN / not EPLMN | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9a | UAC / Access Identity 0 / ODAC / SNPN / RSNPN / new SNPN | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 11.3.10 | UAC / Access Identity 0 / AC9 / 0% access probability / SIP Re-registration | Rel-16 | C198 | UEs supporting 5G Core and IMS security |
| 11.3.11 | UAC / Access Identity 1 / 0% access probability / release with redirect with mpsPriorityIndication / RRC_INACTIVE | Rel-16 | C274A | UEs supporting 5G Core and RRC Connection release with MPS priority indication AND RRC_INACTIVE |
| 11.3.12 | UAC / Access Identity 0 / AC7 / 0% access probability / Uplink user data transfer | Rel-15 | C21 | UEs supporting 5G Core |
| 11.4 | Emergency Services |  |  |  |
| 11.4.1 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.1a | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / PEIPS assistance information | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 11.4.2 | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.3 | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services | Rel-15 | C238 | UEs supporting 5G Core and emergency services in NR connected to 5GCN and test execution with No USIM |
| 11.4.4 | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.5 | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.6 | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.7 | Handling of Local and Extended emergency numbers / Mobility | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.8 | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.9 | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.10 | Void |  |  |  |
| 11.4.10a | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / N1 mode to S1 mode transfer of an existing emergency PDU session | Rel-15 | C85B | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |
| 11.4.11 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection | Rel-15 | C85A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| 11.4.12 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success | Rel-15 | C176 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| 11.4.13 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / obtaining new IP address different than the IP address | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.14 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.5 | eCall over IMS |  |  |  |
| 11.5.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS / 5GS to EPS | Rel-16 | C170 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / 5GS to EPS | Rel-16 | C171 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.3 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.4 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.5 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS | Rel-16 | C174 | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.6 | eCall capable / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA | Rel-16 | C185 | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| 11.5.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success / 5GS | Rel-16 | C186 | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| 11.5.8 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported / RACH failure in NR cell / eCall using the CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.9 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C187 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.10 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.11 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.12 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.13 | eCall over IMS / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN / 5GS | Rel-16 | C189 | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.14 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall using CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.6 | 3GPP PS Data Off |  |  |  |
| 11.6.1 | Data Off / MO Voice Call | Rel-15 | C162 | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| 11.6.2 | Data Off / MO Video Call | Rel-15 | C172 | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video and MTSI video H.265 MP MT Level 3.1 and MTSI video H.264 CHP Level 3.1 and H.264 CBP Level 3.1 |
| 11.6.3 | Data Off / SMSoIP | Rel-15 | C162A | UEs supporting 5G Core and NG.114 v2.0 and 3GPP PS data off and Initiating session and SMS over IP |
| 11.7 | eDRX |  |  |  |
| 11.7.1 | eDRX / IDLE | Rel-17 | C210 | UEs supporting 5G Core and eDRX |
| 11.7.2 | eDRX / Inactive / RAN-initiated paging | Rel-17 | C210A | UEs supporting 5G Core and eDRX and RRC_INACTIVE |
| 11.7.3 | eDRX / Inactive / CN-initiated paging / eDRX Allowed / Not Allowed | Rel-17 | C210A | UEs supporting 5G Core and eDRX and RRC_INACTIVE |
| 11.8 | Inter-system mobility between untrusted Non-3GPP and 3GPP system |  |  |  |
| 11.8.1 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from NR to N3IWF/5GC | Rel-15 | C276 | UEs supporting 5G Core and handover from 5G Core Network to 5G Core over non-3GPP Access Network and WLAN |
| 11.8.2 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to NR / UE in 5GMM-DEREGISTERED states | Rel-15 | C248 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to 5G Core Network and WLAN |
| 11.8.3 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from E-UTRAN/EPC to N3IWF/5GC | Rel-15 | C277 | UEs supporting 5G Core and handover from EPC Network to 5G Core over non-3GPP Access Network and WLAN |
| 11.8.4 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to E-UTRAN/EPC | Rel-15 | C249 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to EPC Network and WLAN |
| 11.8.5 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from 5GS to EPC/ePDG | Rel-15 | C208 | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |
| 11.8.6 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from EPC/ePDG to 5GS/ UE in 5GMM-DEREGISTERED and EMM-DEREGISTERED states | Rel-15 | C237 | UEs supporting WLAN and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and handover from EPC over non-3GPP Access Network to 5G Core and IMS and 5G Core. |

Table 4.1-5b: Additional Information of Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 11 |  |  |  |  |
| 11.1 |  |  |  |  |
| 11.1.1 |  |  |  | Rel-15 E-UTRA |
| 11.1.2 |  |  |  | Rel-15 E-UTRA |
| 11.1.3 |  |  |  | Rel-15 E-UTRA |
| 11.1.4 |  |  |  | Rel-15 E-UTRA |
| 11.1.5 |  |  |  | Rel-15 E-UTRA |
| 11.1.6 |  |  |  | Rel-15 E-UTRA |
| 11.1.7 |  |  |  | Rel-15 E-UTRA |
| 11.1.8 |  |  |  | Rel-16 E-UTRA |
| 11.1.9 |  |  |  | Rel-16 E-UTRA |
| 11.2 |  |  |  |  |
| 11.2.1 |  |  |  | Rel-16 UTRA |
| 11.3 |  |  |  |  |
| 11.3.1 | pc_inactiveState |  |  |  |
| 11.3.6 | pc_inactiveState |  |  |  |
| 11.3.6a | pc_inactiveState |  |  |  |
| 11.4 |  |  |  |  |
| 11.4.10a |  |  |  | Rel-15 E-UTRA |
| 11.4.11 |  |  |  | Rel-15 E-UTRA |
| 11.5 |  |  |  |  |
| 11.5.1 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.2 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.3 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.4 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.5 |  |  | Note 1 |  |
| 11.5.6 |  |  | Note 1 |  |
| 11.5.7 |  |  |  | Rel-16 UTRA |
| 11.5.8 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| 11.5.9 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| 11.5.10 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| 11.5.11 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| 11.5.12 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.13 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| 11.5.14 |  | px_NR_RATComb_Tested | Note 1 | Rel-9 UTRA |
| Note 1: This test case can optionally be executed from Release 15 onwards. |  |  |  |  |

Table 4.1-6a: Applicability of Protocol conformance NR sidelink test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 12 | NR sidelink |  |  |  |
| 12.1 | PC5-only operation |  |  |  |
| 12.1.1 | PC5-only operation / Sidelink communication |  |  |  |
| 12.1.1.2 | PC5-only operation / Sidelink communication / Reception | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.2 | PC5-only operation / Sidelink synchronization related procedure |  |  |  |
| 12.1.2.1 | PC5-only operation / Sidelink synchronization related procedure / Synchonization reference source (re-)selection | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.2.2 | PC5-only operation / Sidelink synchronization related procedure / SL-SSB transmission Initiation and Cease | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.3 | PC5-only operation / Measurement configuration and reporting via PC5 RRC |  |  |  |
| 12.1.3.1 | PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement configuration | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.3.2 | PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement reporting / Event S1 and S2 | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.3.3 | PC5-only operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement reporting / Periodical reporting | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.4 | PC5-only operation / Sidelink Reconfiguration via PC5 RRC |  |  |  |
| 12.1.4.1 | PC5-only operation / Sidelink Reconfiguration via PC5 RRC / SL DRB management / initiating UE side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.4.2 | PC5-only operation / Sidelink Reconfiguration via PC5 RRC / SL DRB management / Peer UE side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.5 | PC5-only operation / Sidelink CSI reporting |  |  |  |
| 12.1.5.1 | PC5-only operation / Sidelink CSI reporting / Configuration | Rel-16 | C163 | UE supporting 5G core and NR sidelink and Sidelink CSI report |
| 12.1.5.2 | PC5-only operation / Sidelink CSI reporting / Reporting | Rel-16 | C163 | UE supporting 5G core and NR sidelink and Sidelink CSI report |
| 12.1.6 | PC5-only operation / Sidelink failure |  |  |  |
| 12.1.6.1 | PC5-only operation / Sidelink failure / PC5 RRC reconfiguration failure / Initiating UE side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.6.2 | PC5-only operation / Sidelink failure / PC5 RRC reconfiguration failure / Peer UE side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.6.3 | PC5-only operation / Sidelink failure / Sidelink radio link failure / Transmission side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.6.4 | PC5-only operation / Sidelink failure / Sidelink radio link failure / Reception side | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.1.7 | PC5-only operation / Sidelink UE capability transfer via PC5 RRC |  |  |  |
| 12.1.7.1 | PC5-only operation / Sidelink UE capability transfer via PC5 RRC / One-way and two-way transfer | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 12.2 | Inter-carrier concurrent operation |  |  |  |
| 12.2.1 | Inter-carrier concurrent operation / Sidelink communication |  |  |  |
| 12.2.1.2 | Inter-carrier concurrent operation / Sidelink communication / RRC_IDLE / Reception | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.1.3 | Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Transmission / Network scheduling | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.1.5 | Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Transmission / Exceptional pool | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.1.6 | Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Reception | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.2 | Inter-carrier concurrent operation / Sidelink synchronization related procedure |  |  |  |
| 12.2.2.1 | Inter-carrier concurrent operation / Sidelink synchronization related procedure / Synchonization reference source (re-)selection | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.2.2 | Inter-carrier concurrent operation / Sidelink synchronization related procedure / SL-SSB transmission Initiation and Cease | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.3 | Inter-carrier concurrent operation / Measurement configuration and reporting via Uu RRC |  |  |  |
| 12.2.3.1 | Inter-carrier concurrent operation / Measurement configuration and reporting via Uu RRC / CBR measurement reporting / Event C1 and C2 | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.3.2 | Inter-carrier concurrent operation / Measurement configuration and reporting via Uu RRC / CBR measurement reporting / Periodical reporting | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.4 | Inter-carrier concurrent operation / Sidelink Reconfiguration via Uu RRC |  |  |  |
| 12.2.4.1 | Inter-carrier concurrent operation / Sidelink Reconfiguration via Uu RRC / SL DRB management / transmission side | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.5 | Inter-carrier concurrent operation / Measurement configuration and reporting via PC5 RRC |  |  |  |
| 12.2.5.1 | Inter-carrier concurrent operation / Measurement configuration and reporting via PC5 RRC / SL-RSRP measurement configuration | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.5.2 | Inter-carrier concurrent operation / Measurement configuration and reporting via PC5 RRC / SL-RSRP measurement reporting / Event S1 and S2 | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.5.3 | Inter-carrier concurrent operation / Measurement configuration and reporting via PC5 RRC / PSBCH-RSRP measurement reporting / Periodical reporting | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.6 | Inter-carrier concurrent operation / Sidelink Reconfiguration via PC5 RRC |  |  |  |
| 12.2.6.1 | Inter-carrier concurrent operation / Sidelink Reconfiguration via PC5 RRC / SL DRB management / Initiating UE side | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.7 | Inter-carrier concurrent operation / Sidelink CSI reporting |  |  |  |
| 12.2.7.1 | Inter-carrier concurrent operation / Sidelink CSI reporting / Configuration | Rel-16 | C164 | UE supporting 5G core and NR sidelink mode 1 transmission and Sidelink CSI report |
| 12.2.7.2 | Inter-carrier concurrent operation / Measurement configuration and reporting via PC5 RRC / SL-RSRP measurement reporting / Event S1 and S2 | Rel-16 | C164 | UE supporting 5G core and NR sidelink mode 1 transmission and Sidelink CSI report |
| 12.2.8 | Inter-carrier concurrent operation / Sidelink failure |  |  |  |
| 12.2.8.1 | Inter-carrier concurrent operation / Sidelink CSI reporting / Reporting | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.8.2 | Inter-carrier concurrent operation / Sidelink failure / PC5 RRC Reconfiguration Failure / Peer UE side | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |
| 12.2.8.3 | Inter-carrier concurrent operation / Sidelink failure / Sidelink radio link failure / transmission side | Rel-16 | C106 | UE supporting 5G core and NR sidelink mode 1 transmission |

Table 4.1-6b: Additional Information of Applicability of Protocol conformance NR sidelink test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| TBD |  |  |  |  |

Table 4.1-7a: Applicability of Protocol conformance NR V2X NAS layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 13 | V2X NAS layer |  |  |  |
| 13.1 | V2X policy provisioning |  |  |  |
| 13.1.1 | V2X policy provisioning / Precedence / Validity timer expires / geographical area changes | Rel-16 | C166 | UE supporting 5G Core and V2X communication over NR-PC5 |
| 13.2 | PC5 unicast |  |  |  |
| 13.2.1 | PC5 unicast / link establishment / Reject / Conflict Layer 2 ID | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 13.2.2 | PC5 unicast / link Security Mode | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 13.2.3 | PC5 unicast / link modification | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 13.2.4 | PC5 unicast / link Release / Reestablish PC5 unicast link to same UE | Rel-16 | C128 | UE supporting 5G core and NR sidelink |
| 13.2.5 | PC5 unicast / link identifier update | Rel-16 | C128 | UE supporting 5G core and NR sidelink transmission mode 2 |
| 13.2.6 | PC5 unicast / link keep alive | Rel-16 | C128 | UE supporting 5G core and NR sidelink |

Table 4.1-7b: Additional Information of Applicability of Protocol conformance NR V2X NAS layer test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| TBD |  |  |  |  |

Table 4.1-8a: Applicability of Protocol conformance NR MBS test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 14 | MBS |  |  |  |
| 14.1 | MBS Broadcast |  |  |  |
| 14.1.1 | MBS Broadcast/ MCCH Information Acquisition |  |  |  |
| 14.1.1.1 | MBS Broadcast/ MCCH Information Acquisition/ entering the cell providing SIB20 | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.1.2 | MBS Broadcast/ MCCH Information Acquisition/ becoming interested to receive MBS broadcast services | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.1.3 | MBS Broadcast/ MCCH Information Acquisition/ MCCH Information change notification | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.1.4 | MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling |  |  |  |
| 14.1.1.4.1 | MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling / Intra-band Contiguous CA | Rel-17 | C280 | UE supporting 5G Core and broadcast reception on SCell and Intra-band Contiguous CA |
| 14.1.1.4.2 | MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling / Inter-band CA | Rel-17 | C281 | UE supporting 5G Core and broadcast reception on SCell and Inter-band CA |
| 14.1.1.4.3 | MBS Broadcast/ MCCH Information Acquisition/ receiving SIB20 of an SCell via dedicated signalling / Intra-band non Contiguous CA | Rel-17 | C282 | UE supporting 5G Core and broadcast reception on SCell and Intra-band non-Contiguous CA |
| 14.1.2 | MBS Broadcast/ Service Continuity |  |  |  |
| 14.1.2.1 | MBS Broadcast/ Service Continuity/ Cell reselection/ frequency prioritization | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.2.2 | MBS Broadcast/ Service Continuity/ Handover/ MBS Interest Indication/ inter-frequency | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.2.3 | MBS Broadcast/ Service Continuity/ Handover/ MBS Interest Indication/ intra-frequency | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.3 | MBS Broadcast/ MAC |  |  |  |
| 14.1.3.1 | MBS Broadcast/ MAC/ Correct HARQ process handling | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.1.3.2 | MBS Broadcast/ MAC/ DRX operation | Rel-17 | C213 | UE supporting 5G Core and broadcast reception. |
| 14.2 | MBS Multicast |  |  |  |
| 14.2.1 | MBS Multicast/ MAC |  |  |  |
| 14.2.1.1 | MBS Multicast/ MAC / DL Data Transfer |  |  |  |
| 14.2.1.1.1 | MBS Multicast / MAC / DL Data Transfer / PTM transmission / PTP transmission / DCI format 4_1 | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.1.1.2 | MBS Multicast / MAC / DL Data Transfer/ PTM transmission/ DCI format 4_2 | Rel-17 | C283 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4_2 |
| 14.2.1.1.3 | MBS Multicast / MAC / DL Data Transfer/ PTM transmission / PTP transmission / Multiple G-RNTIs | Rel-17 | C295 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and Multiple G-RNTIs. |
| 14.2.1.1.4 | MBS Multicast/ MAC / DL Data Transfer/ PTM retransmission for multicast/ RRC-based enabling-disabling HARQ feedback for Multicast / ACK-NACK | Rel-17 | C215 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast |
| 14.2.1.1.5 | MBS Multicast/ MAC / DL Data Transfer/ PTP retransmission for multicast/ RRC-based enabling-disabling HARQ feedback for Multicast/ ACK-NACK | Rel-17 | C216 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and PTP retransmission for multicast on the same cell as multicast initial transmission |
| 14.2.1.1.6 | MBS Multicast/ MAC / DL Data Transfer/ PTM retransmission for multicast/ DCI-based enabling-disabling HARQ feedback for Multicast/ ACK-NACK | Rel-17 | C284 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4-2 and DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 |
| 14.2.1.1.7 | MBS Multicast/ MAC / DL Data Transfer/ RRC-based enabling-disabling HARQ feedback for Multicast / NACK-only | Rel-17 | C252 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming |
| 14.2.1.1.8 | MBS Multicast/ MAC / DL Data Transfer/ Multiplex multicast HARQ-ACK information with unicast HARQ-ACK information | Rel-17 | C253 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and multiplexing HARQ-ACK for unicast and for multicast with the same priority and different HARQ-ACK codebook types in a PUCCH or in a PUSCH |
| 14.2.1.1.9 | MBS Multicast/ MAC / DL Data Transfer/ DCI-based enabling-disabling HARQ feedback for Multicast/ NACK-only | Rel-17 | C285 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and DCI formate 4-2 and DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 |
| 14.2.1.2 | MBS Multicast/ MAC/ DRX operation |  |  |  |
| 14.2.1.2.1 | MBS Multicast/ MAC/ DRX operation/ PTM transmission / PTP transmission | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.1.2.2 | MBS Multicast/ MAC/ DRX operation/ PTM retransmission for multicast | Rel-17 | C215 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast |
| 14.2.1.2.3 | MBS Multicast/ MAC/ DRX operation/ PTP retransmission for multicast | Rel-17 | C216 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and PTP retransmission for multicast on the same cell as multicast initial transmission |
| 14.2.1.3 | MBS Multicast/ MAC/ SPS |  |  |  |
| 14.2.1.3.1 | MBS Multicast/ MAC/ SPS/ PTM transmission | Rel-17 | C296 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group common PDSCH for multicast on PCell. |
| 14.2.1.3.2 | MBS Multicast/ MAC/ SPS/ PTM retransmission for multicast | Rel-17 | C297 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast. |
| 14.2.1.3.3 | MBS Multicast/ MAC/ SPS/ PTP retransmission for multicast | Rel-17 | C298 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast and PTP retransmission associated with CS-RNTI for SPS multicast on the cell same as multicast initial transmission. |
| 14.2.1.3.4 | MBS Multicast/ MAC/ SPS/ CS-RNTI release | Rel-17 | C299 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and SPS group-common PDSCH for multicast on PCell and unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH. |
| 14.2.2 | MBS Multicast/ RLC |  |  |  |
| 14.2.2.1 | MBS Multicast/ UM RLC / 6bit SN /Correct set initial value for UM receive state variable/ PTM | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.2.2 | MBS Multicast/ UM RLC / 12bit SN /Correct set initial value for UM receive state variable/ PTM | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.3 | MBS Multicast / PDCP |  |  |  |
| 14.2.3.1 | MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 12 bit SN | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.3.2 | MBS Multicast / PDCP/ PDCP HFN and SN maintenance / Non-Lossless handover / 18 bit SN | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.3.3 | MBS Multicast / PDCP/ PDCP HFN and SN maintenance /Lossless handover/ PDCP status report / 12 bit SN | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.3.4 | MBS Multicast / PDCP/ PDCP HFN and SN maintenance /Lossless handover/ PDCP status report / 18 bit SN | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.4 | MBS Multicast / RRC |  |  |  |
| 14.2.4.1 | MBS Multicast / RRC / Paging |  |  |  |
| 14.2.4.1.1 | MBS Multicast / RRC / Paging for group notification / RRC_IDLE | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.4.1.2 | MBS Multicast / RRC / Paging for group notification / RRC_INACTIVE | Rel-17 | C254 | UE supporting 5G Core and dynamic scheduling for multicast for PCell and RRC_INACTIVE |
| 14.2.4.2 | MBS Multicast / RRC / MRB Reconfiguration |  |  |  |
| 14.2.4.2.1 | MBS Multicast / RRC / MRB Reconfiguration / Establishment / Modification / Release / Success | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.4.3 | MBS Multicast/ RRC/ Handover |  |  |  |
| 14.2.4.3.1 | MBS Multicast/ RRC/ Handover between multicast supporting cell / Success | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.4.3.2 | MBS Multicast / RRC / Handover between multicast supporting cell / Failure/ Re-establishment successful | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.4.3.3 | MBS Multicast/ RRC/ Handover between Multicast-supporting cell and Multicast non-supporting cell / Success | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5 | MBS Multicast/ Session management |  |  |  |
| 14.2.5.1 | MBS Multicast/ Session management / Network-requested PDU session modification |  |  |  |
| 14.2.5.1.1 | MBS Multicast/ Session management / Network-requested PDU session modification / Remove UE from multicast MBS session | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5.1.2 | MBS Multicast/ Session management / Network-requested PDU session modification / MBS service area update | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5.2 | MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification |  |  |  |
| 14.2.5.2.1 | MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Accepted | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5.2.2 | MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Rejected / User is outside of local MBS service area | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5.2.3 | MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Rejected / MBS session has not started or will not start soon | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |
| 14.2.5.2.4 | MBS Multicast/ Session management / UE-requested PDU session modification / Leave MBS multicast session / Accepted | Rel-17 | C214 | UE supporting 5G Core and dynamic scheduling for multicast for PCell |

Table 4.1-8b: Additional Information of Applicability of Protocol conformance NR MBS test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 14 |  |  |  |  |
| 14.1 |  |  |  |  |
| 14.1.1 |  |  |  |  |
| 14.1.1.1 | pc_inactiveState |  |  |  |
| 14.1.2 |  |  |  |  |
| 14.1.2.1 | pc_inactiveState |  |  |  |
| 14.1.3 |  |  |  |  |
| 14.1.3.2 | pc_inactiveState |  |  |  |
| 14.2.1 |  |  |  |  |
| 14.2.1.1 |  |  |  |  |
| 14.2.1.1.7 | pc_mux_HARQ_ACK_UnicastMulticast_r17 |  |  |  |

<End of modified section>