3GPP TSG-RAN5 Meeting # 101	R5-237471

# Chicago, USA,  13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1451 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | k-Offset update for IoT NTN RF/demod/RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - K-Offset value for RF and demod testing needs to be updated in order to allow testing of delays up to 6.67ms for NGSO (highest for LEO orbit 1200 km) and up to 128.79ms for GSO satellites according to R4-2321062 and R4-2321186. - SIB31-NB configuration for RRM test cases is undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - k-Offset corrected to allow testing of delays up to 6.67ms for NGSO (highest for LEO orbit 1200 km) and up to 128.79ms for GSO satellites according to R4-2321062 and R4-2321186. - Define SIB31-NB configuration for RRM test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Maximum delays could not be tested, NB-IoT NTN RRM test cases could not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.3, 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on RAN4 CR R4-2321062 and R4-2321186. This CR is dependent on RAN5 CR R5-237936. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-237957 and the outcome of 1 revision. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

### 8.2.2	NB-IoT RF Reference system configurations

#### 8.2.2.1	NB-IoT Common parameters for simulated E-UTRA cells

##### 8.2.2.1.1		NB-IoT Combinations of system information blocks

The NB-IoT combination of system information blocks required by a test case depends on the test case scenario. In clause 8.1.4 of this document, the NB-IoT combinations of system information blocks are defined.

Combination 1, defined in clause 8.1.4, is used by default in test cases defined in TS 36.521-1 [21].

##### 8.2.2.1.2	NB-IoT Scheduling of system information blocks

Same NB-IoT scheduling of system information blocks as defined in clause 8.1.4.3.1.2.

##### 8.2.2.1.3	NB-IoT Common contents of system information messages

MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1.

SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

SystemInformationBlockType31-NB

Table 8.2.2.1.3-1: SystemInformationBlockType31-NB

| Derivation Path: TS 36.508 clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | See cl. 8.2.6.2 |  |  |
| positionY-r17 | See cl. 8.2.6.2 |  |  |
| positionZ-r17 | See cl. 8.2.6.2 |  |  |
| velocityVX-r17 | See cl. 8.2.6.2 |  |  |
| velocityVY-r17 | See cl. 8.2.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.2.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

##### 8.4.2.1.3	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1 with the following exceptions.

Table 8.4.2.1.3-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

- SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

- SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

- SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

- SystemInformationBlockType31-NB

Table 8.4.2.1.3-2: SystemInformationBlockType31-NB

| Derivation Path: TS 36.508 clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | [TC_EF_SV] |
| positionX-r17 | See cl. 8.4.6.2 |  |  |
| positionY-r17 | See cl. 8.4.6.2 |  |  |
| positionZ-r17 | See cl. 8.4.6.2 |  |  |
| velocityVX-r17 | See cl. 8.4.6.2 |  |  |
| velocityVY-r17 | See cl. 8.4.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.4.6.2 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | [TC_EF_OP] |
| semiMajorAxis-r17 | See cl. 8.4.6.2 |  |  |
| eccentricity-r17 | See cl. 8.4.6.2 |  |  |
| periapsis-r17 | See cl. 8.4.6.2 |  |  |
| longitude-r17 | See cl. 8.4.6.2 |  |  |
| inclination-r17 | See cl. 8.4.6.2 |  |  |
| anomaly-r17 | See cl. 8.4.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  |  |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s900 |  | GSO |
|  | S5 |  | NGSO |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TC_EF_SV | Test cases using ephemeris with State Vector format according to section 8.4.6.2 |
| TC_EF_OP | Test cases using ephemeris with State Vector format according to section 8.4.6.2 |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 8.4.2A	NB-IoT Generic RRM procedures

## <<< END OF CHANGES 2>>>