3GPP TSG-RAN5 Meeting #101	R5-237612

# Chicago, United States, Nov 13 - 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0549 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC1.5 CA_n41C ICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The ICS for PC1.5 CA_n41C needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new table for intra-band contiguous CA PC1.5 UE RF ICS. - The ICS for PC1.5 CA_n41C has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1.5 CA_n41C UE conformance test can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

A.4.3.2A.2	NR Intra-band contiguous CA

A.4.3.2A.2.1	NR Intra-band contiguous CA within FR1

Table A.4.3.2A.2.1-1: Downlink Bandwidth Class capabilities for NR Intra-band contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.2.1-3)

| Item |  | DL NR FR1 Intra-band contiguous CA Bandwidth Class |  | Ref. |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | DL NR FR1 Intra-band contiguous CA BW Class A |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_A |  |  |  |
| 2 |  | DL NR FR1 Intra-band contiguous CA BW Class B |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_B |  |  |  |
| 3 |  | DL NR FR1 Intra-band contiguous CA BW Class C |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_C |  |  |  |
| 4 |  | DL NR FR1 Intra-band contiguous CA BW Class D |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_D |  |  |  |
| 5 |  | DL NR FR1 Intra-band contiguous CA BW Class E |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_E |  |  |  |
| 6 |  | void |  | void |  | void |  |  |  |
| 7 |  | DL NR FR1 Intra-band contiguous CA BW Class G |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_G |  |  |  |
| 8 |  | DL NR FR1 Intra-band contiguous CA BW Class H |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_H |  |  |  |
| 9 |  | DL NR FR1 Intra-band contiguous CA BW Class I |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_NR_FR1_CA_Class_I |  |  |  |
| 10 |  | DL NR FR1 Intra-band contiguous CA BW Class J |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_J |  |  |  |
| 11 |  | DL NR FR1 Intra-band contiguous CA BW Class K |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_K |  |  |  |
| 12 |  | DL NR FR1 Intra-band contiguous CA BW Class L |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_L |  |  |  |
| 13 |  | DL NR FR1 Intra-band contiguous CA BW Class M |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_M |  |  |  |
| 14 |  | DL NR FR1 Intra-band contiguous CA BW Class N |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_N |  |  |  |
| 15 |  | DL NR FR1 Intra-band contiguous CA BW Class O |  | 38.101-1, 5.3A.5 |  | pc_DL_intra_contiguous_CA_NR_FR1_Class_O |  |  |  |

Table A.4.3.2A.2.1-2: Uplink Bandwidth Class capabilities for NR Intra-band contiguous CA configurations within FR1 (for one or more of the supported configurations in Table A.4.3.2A.2.1-3)

| Item | UL NR FR1 Intra-band contiguous CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR FR1 Intra-band contiguous CA BW Class A | 38.101-1, 5.3A.5 | pc_UL_intra_contiguous_CA_NR_FR1_Class_A |  |
| 2 | UL NR FR1 Intra-band contiguous CA BW Class B | 38.101-1, 5.3A.5 | pc_UL_intra_contiguous_CA_NR_FR1_Class_B |  |
| 3 | UL NR FR1 Intra-band contiguous CA BW Class C | 38.101-1, 5.3A.5 | pc_UL_intra_contiguous_CA_NR_FR1_Class_C |  |
| 4-12 | Void |  |  |  |

Table A.4.3.2A.2.1-3: Supported configurations for NR Intra-band contiguous CA within FR1

| NR FR1 Intra-band contiguous CA configuration / Item (Note 1, 7) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) |
| --- | --- | --- | --- | --- |
| CA_n1B | Rel-16 |  |  |  |
| CA_n41C | Rel-15 |  |  |  |
| CA_n48B | Rel-16 |  |  |  |
| CA_n48C | Rel-16 |  |  |  |
| CA_n66B (Note 6) | Rel-16 |  |  |  |
| CA_n77C | Rel-15 |  |  |  |
| CA_n78B | Rel-16 |  |  |  |
| CA_n78C | Rel-15 |  |  |  |
| CA_n79C | Rel-15 |  |  |  |
| Note 1: Notation used for intra-band contiguous CA Bands is according to TS 38.101-1 [23] Table 5.5A.1-1, e.g. ‘CA_n77C’ indicates CA operation on NR band n77 with DL CA Bandwidth Class C. Note 2: The UL CA capabilities as per Table A.4.3.2A.2.1-2 can be supported on a single band. The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.1-1. For this release of specification valid choices are ’N’, ‘nXB’ and ‘nXC’, where nX is the NR band. For example, for CA_n1B, ’N’ would mean only DL CA, ‘n1B’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.1-1. Note 4: Void. Note 5: See UL(table_index) in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |

Table A.4.3.2A.2.1-4: Intra-band contiguous CA PC2 UE RF Baseline Implementation Capabilities

| Item | Intra-band contiguous CA PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2A.1 | Rel-17 | pc_nrBand41_C_PC2_Supp | CA_n41C |

Table A.4.3.2A.2.1-5: Intra-band contiguous CA PC1.5 UE RF Baseline Implementation Capabilities

| Item | Intra-band contiguous CA PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 2496-2690 MHz | 38.101-1, 6.2A.1 | Rel-17 | pc_nrBand41_C_PC1.5_Supp | CA_n41C |

A.4.3.2A.2.2	NR Intra-band contiguous CA within FR2

<End of Changes>