3GPP TSG-RAN5 Meeting #101	R5-237634

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2502 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.2F.3 UE additional maximum output power reduction for shared spectrum access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson GmbH, Eurolab |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the work plan, test case 6.2F.3 Test Configuration for NS_28 and NS_29 added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Test configuration for NS_28 and NS_29 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U work item cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TR 38.905 CR 0835 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in R5-237633, This CR is pending aproval of R5-236496 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-237634 is the final version of R5-236525 with below changes. Revision 1: - Revised: “Source to TSG” of this cover sheet. |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### 6.2F.3	UE additional maximum output power reduction for shared spectrum access

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- Test points are TBD except for NS_28 and NS_29

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

- RMC in Annex A.

- Test coverage for UL-MIMO

- Message exceptions

- Test state and generic procedure are TBD in 38.508-1

6.2F.3.1	Test purpose

Same test purpose as in 6.2.3.1

6.2F.3.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

6.2F.3.3	Minimum conformance requirements

6.2F.3.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2F.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max (MPR, A-MPR) where MPR is defined in clause 6.2F.2.

Table 6.2F.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.3.1-1A.

Table 6.2F.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.2 |
| NS_29 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.3 |
| NS_30 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.4 |
| NS_31 |  | n46 | 20, 40, 60, 80 |  | 6.2F.3.3.5 |
| NS_53 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.3.6 |
| NS_54 |  | n96 | 20, 40, 60, 80 |  | 6.2F.3.3.7 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2F.3.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  |  |
| n96 | NS_01 | NS_53 | NS_54 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

6.2F.3.3.2	A-MPR for NS_28

When "NS_28" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.2-1.

Table 6.2F.3.3.2-1: A-MPR for NS_28 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK4 | ≤ 4.0 | ≤ 6.0 | See Table 6.2F.2-1 |
|  | QPSK | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 4.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 5.5 | ≤ 6.5 |  |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 |  |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

6.2F.3.3.3	A-MPR for NS_29

When "NS_29" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.3-1.

Table 6.2F.3.3.3-1: A-MPR for NS_29 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz | 40 MHz |  | 60 MHz, 80 MHz |  |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | See Table 6.2F.2-1 | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | QPSK |  | ≤ 2.0 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 2.5 | ≤ 4.0 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 3: Larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

6.2F.3.3.4	A-MPR for NS_30

When "NS_30" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.4-1.

Table 6.2F.3.3.4-1: A-MPR for NS_30 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 15.0 | ≤ 2.5 | ≤ 5.0 | See Table 6.2F.2.3-1 |
|  | 16 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 3.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 9.0 | ≤ 15.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 9.0 | ≤ 16.0 | ≤ 5.5 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 14.0 | ≤ 4.0 | ≤ 6.0 |  |
|  | 16 QAM | ≤ 9.5 | ≤ 14.5 | ≤ 4.0 | ≤ 6.0 |  |
|  | 64 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 5.5 | ≤ 6.5 |  |
|  | 256 QAM | ≤ 9.5 | ≤ 15.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centred at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centred at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centred at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centred at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centred at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centred at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

6.2F.3.3.5	A-MPR for NS_31

When "NS_31" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.5-1.

Table 6.2F.3.3.5-1: A-MPR for NS_31 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 4.0 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 64 QAM |  | ≤ 4.0 | ≤ 6.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.5 |
| CP-OFDM | QPSK |  | ≤ 5.5 | ≤ 6.5 |
|  | 16 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 7.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centred at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 2. |  |  |  |  |

6.2F.3.3.6	A-MPR for NS_53

When "NS_53" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.6-1.

Table 6.2F.3.3.6-1: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |

6.2F.3.3.7	A-MPR for NS_54

When "NS_54" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.3.7-1.

Table 6.2F.3.3.7-1: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-ODFM | QPSK | See Table 6.2F.2-1 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 6.0 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 6.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 6.0 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 6.0 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centred at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centred at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centred at the nearest NR-ARFCN corresponding to [5985 MHz]. |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2F.3.

6.2F.3.4	Test description

6.2F.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1 that are restricted to shared channel access. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2F.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2F.3.4.1-1: Test Configuration Table for NS_28 and NS_29

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 20,40,60 and 80 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_28, NS_29 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1, Note 3) |
|  |  |  |  | N/A for A-MPR test cases | DFT-s OFDM |  |  |  |
| 1 | Default | 20 | Default |  |  | PI/2 BPSK |  | Full |
| 2 | Default | 20 | Default |  |  | QPSK |  | Full |
| 3 | Default | 20 | Default |  |  | 16 QAM |  | Full |
| 4 | Default | 20 | Default |  |  | 64 QAM |  | Full |
| 5 | Default | 20 | Default |  |  | 256 QAM |  | Full |
| 6 | Default | 40 | Default |  |  | PI/2 BPSK |  | Full |
| 7 | Default | 40 | Default |  |  | QPSK |  | Full |
| 8 | Default | 40 | Default |  |  | 16 QAM |  | Full |
| 9 | Default | 40 | Default |  |  | 64 QAM |  | Full |
| 10 | Default | 40 | Default |  |  | 256 QAM |  | Full |
| 11 | Default | 60 | Default |  |  | PI/2 BPSK |  | Full |
| 12 | Default | 60 | Default |  |  | QPSK |  | Full |
| 13 | Default | 60 | Default |  |  | 16 QAM |  | Full |
| 14 | Default |  | 60 |  | Default |  |  |  |  |  | 64 QAM |  |  | Full |  |
| 15 | Default |  | 60 |  | Default |  |  |  |  |  | 256 QAM |  |  | Full |  |
| 16 | Default |  | 80 |  | Default |  |  |  |  |  | PI/2 BPSK |  |  | Full |  |
| 17 |  | Default |  | 80 |  | Default |  |  |  |  |  | QPSK |  |  | Full |
| 18 |  | Default |  | 80 |  | Default |  |  |  |  |  | 16 QAM |  |  | Full |
| 19 |  | Default |  | 80 |  | Default |  |  |  |  |  | 64 QAM |  |  | Full |
| 20 |  | Default |  | 80 |  | Default |  |  |  |  |  | 256 QAM |  |  | Full |
| 21 | Low | 20 | Default |  | CP-s OFDM | QPSK |  | Partial_Left |
| 22 | High | 20 | Default |  |  | QPSK |  | Partial_Right |
| 23 | Default | 20 | Default |  |  | QPSK |  | Full |
| 24 | Low | 20 | Default |  |  | 16 QAM |  | Partial_Left |
| 25 | High | 20 | Default |  |  | 16 QAM |  | Partial_Right |
| 26 | Default | 20 | Default |  |  | 16 QAM |  | Full |
| 27 | Low | 20 | Default |  |  | 64 QAM |  | Partial_Left |
| 28 | High | 20 | Default |  |  | 64 QAM |  | Partial_Right |
| 29 | Default | 20 | Default |  |  | 64 QAM |  | Full |
| 30 | Low | 20 | Default |  |  | 256 QAM |  | Partial_Left |
| 31 | High | 20 | Default |  |  | 256 QAM |  | Partial_Right |
| 32 | Default | 20 | Default |  |  | 256 QAM |  | Full |
| 33 | High | 40 | Default |  |  | QPSK |  | Partial_Left |
| 34 | Default | 40 | Default |  |  | QPSK |  | Partial_Right |
| 35 | Low | 40 | Default |  |  | 16 QAM |  | Full |
| 36 | High | 40 | Default |  |  | 16 QAM |  | Partial_Left |
| 37 | Default | 40 | Default |  |  | 16 QAM |  | Partial_Right |
| 38 | Low | 40 | Default |  |  | 64 QAM |  | Full |
| 39 | High | 40 | Default |  |  | 64 QAM |  | Partial_Left |
| 40 | Default | 40 | Default |  |  | 64 QAM |  | Partial_Right |
| 41 | Low | 40 | Default |  |  | 256 QAM |  | Full |
| 42 | High | 40 | Default |  |  | 256 QAM |  | Partial_Left |
| 43 | Default | 40 | Default |  |  | 256 QAM |  | Partial_Right |
| 44 | High | 60 | Default |  |  | QPSK |  | Full |
| 45 | Default | 60 | Default |  |  | QPSK |  | Partial_Left |
| 46 | Low | 60 | Default |  |  | 16 QAM |  | Partial_Right |
| 47 | High | 60 | Default |  |  | 16 QAM |  | Full |
| 48 | Default | 60 | Default |  |  | 16 QAM |  | Partial_Left |
| 49 | Low | 60 | Default |  |  | 64 QAM |  | Partial_Right |
| 50 | High | 60 | Default |  |  | 64 QAM |  | Full |
| 51 | Default | 60 | Default |  |  | 64 QAM |  | Partial_Left |
| 52 | Low | 60 | Default |  |  | 256 QAM |  | Partial_Right |
| 53 | High | 60 | Default |  |  | 256 QAM |  | Full |
| 54 | Default | 60 | Default |  |  | 256 QAM |  | Partial_Left |
| 55 | High | 80 | Default |  |  | QPSK |  | Partial_Right |
| 56 | Default | 80 | Default |  |  | QPSK |  | Full |
| 57 | Low | 80 | Default |  |  | 16 QAM |  | Partial_Left |
| 58 | High | 80 | Default |  |  | 16 QAM |  | Partial_Right |
| 59 | Default | 80 | Default |  |  | 16 QAM |  | Full |
| 60 | Low | 80 | Default |  |  | 64 QAM |  | Partial_Left |
| 61 | High | 80 | Default |  |  | 64 QAM |  | Partial_Right |
| 62 | Default | 80 | Default |  |  | 64 QAM |  | Full |
| 63 | Low | 80 | Default |  |  | 256 QAM |  | Partial_Left |
| 64 | High | 80 | Default |  |  | 256 QAM |  | Partial_Right |
| 65 | Default | 80 | Default |  |  | 256 QAM |  | Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1F-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Partial RB allocation consists of non-contiguous RB allocation in one or several interlaces and therefore DFT-s-OFDM is not applicable. |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2F.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2F.3.4.3.

6.2F.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2F.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. Symbols with transient periods are not under test.

6.2F.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Information element additionalSpectrumEmission is set in the SIB1 as part of the cell broadcast message.

Table 6.2.3.4.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.508-1 [10] clause 4.6.3, Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_28), 2 (NS_29), 3 (NS_30), 4(NS_31) | for band n46 |  |
| AdditionalSpectrumEmission | 1 (NS_53), 2 (NS_54) | for band n96 |  |

FFS

6.2F.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2F.3.5-1.

Table 6.2F.3.5-1: Test requirement for Power Class 5

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2F.3.5-2. |  |  |  |  |  |  |  |  |  |

Table 6.2F.3.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 5.925GHz | 5.925GHz < f ≤ 7.125GHz |
| --- | --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.