3GPP TSG-RAN5 Meeting #101	R5-237642

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2490 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General updates of TS 38.521-1 clause 5 for R16 CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CU Digital Technology, Nokia, KDDI CORPORATION, ZTE, China Telecom,Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some R16 NR CA combos which are already 100% completion in RAN4 and assigned to interested companies need to be added into operating bands and channel bandwidth of TS38.521-1 clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of new two bands combination CA_n1A-n28A, CA_n3A-n28A, CA_n25A-66A, CA_n25A-n77A, CA_n25A-n78A and CA_n66A-n78A in clause 5. - Addition of new three bands combination CA_n1A-n3A-n28A, CA_n1A-n3A-n78A, CA_n3A-n28A-n77A, CA_n3A-n28A-n78A and CA_n25A-n66A-n78A in clause 5. - Addition of new four bands combination CA_n1A-n3A-n28A-n78A, CA_n2A-n5A-n48A-n66A, CA_n2A-n5A-n48A-n77A, CA_n2A-n5A-n66A-n77A, CA_n2A-n48A-n66A-n77A, CA_n5A-n48A-n66A-n77A in Table 5.2A.2.3-1 and Table 5.5A.3.3-1. - The CA_n77(2A) uplink config of CA_n25A-n77(2A) is belong to R18 CADC WI in Table 5.5A.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activated NR CA configurations in clause 5 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.1 5.2A.2.2 5.2A.2.3 5.5A.3.1 5.5A.3.2 5.5A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an outcome of R5-236403. |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n771 | n1, n77 | No |
| CA_n1-n781 | n1, n78 | No |
| CA_n1-n791 | n1, n79 | No |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n14 | n2, n14 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n3-n5 | n3, n5 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n411 | n3, n41 | No |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n5-n7 | n5, n7 |  |
| CA_n5-n48 | n5, n48 |  |
| CA_n5-n66 | n5, n66 |  |
| CA_n5-n771 | n5, n77 |  |
| CA_n5-n781 | n5, n78 | No |
| CA_n7-n781 | n7, n78 |  |
| CA n8-n781 | n8, n78 | No |
| CA_n14-n30 | n14, n30 |  |
| CA_n14-n66 | n14, n66 |  |
| CA_n14-n77 | n14, n77 |  |
| CA_n20-n78 | n20, n78 |  |
| CA_n24-n41 | n24, n41 |  |
| CA_n24-n48 | n24, n48 |  |
| CA_n24-n77 | n24, n77 |  |
| CA_n25-n66 | n25, n66 |  |
| CA_n25-n77 | n25, n77 |  |
| CA_n25-n78 | n25, n78 |  |
| CA_n25-n466 | n25, n46 |  |
| CA_n26-n66 | n26, n66 |  |
| CA_n26-n70 | n26, n70 |  |
| CA_n28-n411 | n28, n41 |  |
| CA_n28-n771 | n28, n77 | No |
| CA_n28_n781 | n28, n78 | No |
| CA_n28-n791 | n28, n79 |  |
| CA_n29-n66 | n29, n66 |  |
| CA_n29-n70 | n29, n70 |  |
| CA_n29-n71 | n29, n71 |  |
| CA_n39-n41 | n39, n41 | No |
| CA_n41-n66 | n41, n66 |  |
| CA_n41-n711 | n41, n71 |  |
| CA_n41-n791,3 | n41, n79 | No |
| CA_n46-n481,6 | n46, n48 |  |
| CA_n46-n666 | n46, n66 |  |
| CA_n48-n66 | n48, n66 |  |
| CA_n48-n70 | n48, n70 |  |
| CA_n48-n71 | n48, n71 |  |
| CA_n48-n7713,14,18 | n48, n77 |  |
| CA_n66-n70 | n66, n70 |  |
| CA_n66-n71 | n66, n71 |  |
| CA_n66-n77 | n66, n77 |  |
| CA_n66-n78 | n66, n78 |  |
| CA_n70-n71 | n70, n71 |  |
| CA_n71-n77 | n70, n71 |  |
| CA_n77-n795,7 | n77, n79 |  |
| CA_n78-n795 | n78, n79 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: FFS. NOTE 3: The frequency range below 2506MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: FFS NOTE 10 FFS NOTE 11: FFS NOTE 12: FFS NOTE 13: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n48 with an n77 implementation. NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: FFS NOTE 16: FFS NOTE 17: FFS NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. |  |  |

5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n28 | n1, n3, n28 |
| CA_n1-n3-n783 | n1, n3, n78 |
| CA_n1-n78-n79 | n1, n78, n79 |
| CA_n2-n5-n48 | n2, n5, n48 |
| CA_n2-n5-n77 | n2, n5, n77 |
| CA_n2-n48-n66 | n2, n48, n66 |
| CA_n2-n48-n77 | n2, n48, n77 |
| CA_n2-n66-n77 | n2, n66, n77 |
| CA_n3-n28-n773 | n3, n28, n77 |
| CA_n3-n28-n783 | n3, n28, n78 |
| CA_n5-n48-n66 | n5, n48, n66 |
| CA_n5-n48-n77 | n5, n48, n77 |
| CA_n5-n66-n77 | n5, n66, n77 |
| CA_n26-n66-n70 | n26, n66, n70 |
| CA_n28-n41-n793 | n28, n41, n79 |
| CA_n29-n66-n70 | n29, n66, n70 |
| CA_n41-n66-n71 | n41, n66, n71 |
| CA_n48-n66-n70 | n48, n66, n70 |
| CA_n48-n66-n71 | n48, n66, n71 |
| CA_n48-n66-n77 | n48, n66, n77 |
| CA_n48-n70-n71 | n48, n70, n71 |
| CA_n66-n70-n71 | n66, n70, n71 |
| NOTE 1: FFS NOTE 2: FFS NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

5.2A.2.3	Inter-band CA (four bands)

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n28-n781 | n1, n3, n28, n78 |
| CA_n2-n5-n48-n66 | n2, n5, n48, n66 |
| CA_n2-n5-n48-n77 | n2, n5, n48, n77 |
| CA_n2-n5-n66-n77 | n2, n5, n66, n77 |
| CA_n2-n48-n66-n77 | n2, n48, n66, n77 |
| CA_n5-n48-n66-n77 | n5, n48, n66, n77 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

## <<Unchanged sections skipped>>

5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| CA_n48(2A) |  | 10, 15, 20, 40, 50, 60 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1402 | 1 |
| CA_n66(2A) | - | 5 (note), 10, 15, 20 | 5 (note), 10, 15, 20, 40 |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 2 |
| CA_n66(3A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 0 |
| CA_n71(2A) | - | 5,10, 15, 20 | 5,10,15, 20 |  | 30 | 0 |
| CA_n77(2A) | n77 CA_n77(2A) | 20, 40, 80, 100 | 20, 40, 80, 100 |  | 200 | 0 |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 1 |
| CA_n78(2A) |  | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 |  | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 200 | 2 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. |  |  |  |  |  |  |

5.5A.3	Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier5 |  | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  | CA_n1A-n3A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3A |  | - |  | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n5A |  | - |  | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n8A |  | CA_n1A-n8A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n8A |  | - |  | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n28A |  | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n77A |  | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n78A |  | n784 CA_n1A-n78A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n78(2A) |  | CA_n1A-n78A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  |  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n1A-n78C |  | CA_n1A-n78A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | CA_n78C_BCS0 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  |  |  | n78 | CA_n78C_BCS0 |  |
|  |  |  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  |  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1(2A)-n78A |  | - |  | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n79A |  | CA_n1A-n79A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n2A-n5A |  | CA_n2A-n5A |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n14A |  | CA_n2A-n14A |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n14 | 5, 10 |  |
| CA_n2A-n48A |  | CA_n2A-n48A |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n2A-n66A |  | - |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | CA_n2A-n66A |  | n2 | 5, 10, 15, 20 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n66A |  | - |  | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n77A |  | n774 CA_n2A-n77A4 |  | n2 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n5A |  | CA_n3A-n5A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n5A |  | - |  | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  |  |  | n5 | 5, 10, 15, |  |
| CA_n3A-n8A |  | CA_n3A-n8A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n8A |  | - |  | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  |  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3A-n28A |  | CA_n3A-n28A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n41A |  | n414 CA_n3A-n41A4 |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77A |  | CA_n3A-n77A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77(2A) |  | CA_n77(2A) CA_n3A-n77A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n78A |  | n784 CA_n3A-n78A4 |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n78(2A) |  | CA_n3A-n78A CA_n78(2A) |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n3(2A)-n78A |  | - |  | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n79A |  | CA_n3A-n79A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n5A-n7A |  | - |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n5A-n48A |  | CA_n5A-n48A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n5A-n66A |  | CA_n5A-n66A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n77A |  | n774 CA_n5A-n77A4 |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n78A |  | CA_n5A-n78A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n78(2A) |  | CA_n5A-n78A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n7A-n78A |  | CA_n7A-n78A |  | n7 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n78A |  | CA_n8A-n78A |  | n8 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n8A-n78(2A) |  | CA_n8A-n78A |  | n8 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |
| CA_n14A-n30A |  | CA_n14A-n30A |  | n14 | 5, 10 | 0 |
|  |  |  |  | n30 | 5, 10 |  |
| CA_n14A-n66A |  | CA_n14A-n66A |  | n14 | 5, 10 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n77A |  | n77 CA_n14A-n77A |  | n14 | 5, 10 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n78A |  | CA_n20A-n78A |  | n20 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41A |  | CA_n24A-n41A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n41(2A) |  | CA_n24A-n41A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n41 | CA_n41(2A)_BCS1 |  |
| CA_n24A-n48A |  | CA_n24A-n48A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n48B |  | CA_n24A-n48A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n48 | CA_n48B_BCS1 |  |
| CA_n24A-n48(2A) |  | CA_n24A-n48A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n24A-n77A |  | CA_n24A-n77A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n77C |  | CA_n24A-n77A |  | n24 | 5, 10 | 0 |
|  |  |  |  | n77 | CA_n77C_BCS1 |  |
| CA_n25A-n46A |  | - |  | n25 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n46 | 20, 40, 60, 80 |  |
| CA_n25A-n66A |  | CA_n25A-n66A |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77A |  | CA_n25A-n77A4 |  | n25 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A) |  | CA_n25A-n77A4 CA_n77(2A) |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  |  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n78A |  | CA_n25A-n78A |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n78(2A) |  | CA_n25A-n78A |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  |  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n26A-n66A |  | CA_n26A-n66A |  | n26 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n66(2A) |  | CA_n26A-n66A |  | n26 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n70A |  | CA_n26A-n70A |  | n26 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n41A |  | CA_n28A-n41A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n28 | 5, 10, 15, 20,30 | 1 |
|  |  |  |  | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77A |  | CA_n28A-n77A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77(2A) |  | CA_n77(2A) CA_n28A-n77A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n78A |  | CA_n28A-n78A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n78(2A) |  | CA_n28A-n78A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n28A-n79A |  | CA_n28A-n79A |  | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n66A |  | - |  | n29 | 5, 10 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n29A-n66B |  | - |  | n29 | 5, 10 | 0 |
|  |  |  |  | n66 | CA_n66B_BCS0 |  |
| CA_n29A-n66(2A) |  | - |  | n29 | 5, 10 | 0 |
|  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n29A-n70A |  | - |  | n29 | 5, 10 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201,, 251 |  |
| CA_n29A-n71A |  | - |  | n29 | 5, 10 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n39A-n41A |  | CA_n39A-n41A |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n41A-n66A |  | - |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n41A-n71A |  | - |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n41A-n79A |  | CA_n41A-n79A |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41C-n79A |  | CA_n41A-n79A CA_n41C |  | n41 | CA_n41C_BCS0 | 0 |
|  |  |  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n46A-n48A |  | CA_n46A-n48A |  | n46 | 20, 40, 60, 80 | 0 |
|  |  |  |  | n48 | 20 |  |
| CA_n46B-n48A |  | CA_n46A-n48A |  | n46 | CA_n46B_BCS0 | 0 |
|  |  |  |  | n48 | 20 |  |
| CA_n46C-n48A |  | CA_n46A-n48A |  | n46 | CA_n46C_BCS0 | 0 |
|  |  |  |  | n48 | 20 |  |
| CA_n46D-n48A |  | CA_n46A-n48A |  | n46 | CA_n46D_BCS0 | 0 |
|  |  |  |  | n48 | 20 |  |
| CA_n46A-n66A |  | - |  | n46 | 20, 40, 60, 80 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48A-n66A |  | CA_n48A-n66A |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 2 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
| CA_n48A-n66(2A) |  | CA_n48A-n66A |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48B-n66A |  | CA_n48A-n66A |  | n48 | CA_n48B_BCS0 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  |  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66A |  | CA_n48A-n66A |  | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66(2A) |  | CA_n48A-n66A |  | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  |  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48A-n70A6 |  | CA_n48A-n70A |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n70A |  | CA_n48A-n70A |  | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n70A |  | CA_n48A-n70A |  | n48 | CA_n48B_BCS2 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n71A |  | CA_n48A-n71A |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701, 801, 901, 1001 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n71(2A) |  | CA_n48A-n71A |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(2A)-n71A |  | CA_n48A-n71A |  | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n71(2A) |  | CA_n48A-n71A |  | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48B-n71A |  | CA_n48A-n71A |  | n48 | CA_n48B_BCS2 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n77A |  | n774 |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n70A |  | - |  | n66 | 5, 10, 15, 20, 25 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66B-n70A |  | - |  | n66 | CA_n66B_BCS0 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(2A)-n70A |  | - |  | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  |  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66A-n71A |  | CA_n66A-n71A |  | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n71(2A) |  | - |  | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | CA_n66A-n71A |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66(2A)-n71A |  | CA_n66A-n71A |  | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n71(2A) |  | CA_n66A-n71A |  | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  |  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n71A |  | CA_n66A-n71A |  | n66 | CA_n66B_BCS0 | 0 |
|  |  |  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n77A |  | n774 CA_n66A-n77A4 |  | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n77(2A) |  | CA_n66A-n77A4 CA_n77(2A) |  | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  |  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66A-n78A |  | CA_n66A-n78A |  | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66A-n78(2A) |  | CA_n66A-n78A |  | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n70A-n71A | CA_n70A-n71A |  | n70 |  | 5, 10, 15, 201, 251 | 0 |
|  |  |  | n71 |  | 5, 10, 15, 20 |  |
| CA_n70A-n71(2A) | CA_n70A-n71A |  | n70 |  | 5, 10, 15, 201 | 0 |
|  |  |  | n71 |  | CA_n71(2A)_BCS0 |  |
| CA_n71A-n77A | n774 CA_n71A-n77A4 |  | n71 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n71 |  | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n77 |  | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n79A | - |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |
| CA_n78A-n79A | - |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 |  | 40, 50, 60, 80, 100 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE 2: The minimum requirements for intra-band contiguous or non-contiguous CA apply. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed. NOTE 6: The same configuration applies to corresponding NR-DC configuration in Table 5.5B.1-1. If UE supporting NR-DC configuration do not support the corresponding CA configuration, NR-DC configuration is used in CA test cases |  |  |  |  |  |  |

5.5A.3.2	Configurations for inter-band CA (three bands)

Table 5.5A.3.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n78A-n79A4 | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90 |  |
|  |  | n79 | 40, 50, 60, 80 |  |
| CA_n2A-n5A-n48A | CA_n2A-n5A CA_n2A-n48A CA_n5A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n77A | n777, 9 CA_n2A-n5A CA_n2A-n77A7 CA_n5A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A | CA_n2A-n48A CA_n2A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n48A-n77A | n77 CA_n2A-n48A CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n66A-n77A | n777, 9 CA_n2A-n66A CA_n66A-n77A7 CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n41B_BCS0 |  |
| CA_n3A-n28A-n78A | - | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n5A-n48A-n66A | CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n48A-n77A | n77 CA_n5A-n48A CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n66A-n77A | n777, 9 CA_n5A-n66A CA_n66A-n77A7 CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0. |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66(2A)-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n66A-n71A | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n66A-n71(2A) | CA_n48A-n71A CA_n66A-n71A CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48A-n66A-n77A | n77 CA_n48A-n66A CA_n66A-n77A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n70A-n71A | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n70A-n71(2A) | CA_n48A-n71A CA_n70A-n71A CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A CA_n70A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A CA_n70A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation. NOTE 5: FFS NOTE 6: FFS NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 8: FFS NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination. |  |  |  |  |

5.5A.3.3	Configurations for inter-band CA (four bands)

Table 5.5A.3.3-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (four bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
| CA_n2A-n5A-n48A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n66A CA_n5A-n48A CA_n5A-n66A CA_n48A-n66A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n5A-n48A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n5A CA_n2A-n48A CA_n2A-n77A CA_n5A-n48A CA_n5A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n5A-n66A-n77A | CA_n2A-n5A CA_n2A-n66A CA_n2A-n77A5 CA_n5A-n66A CA_n5A-n77A5 CA_n66A-n77A5 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A-n66A-n77A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n2A-n48A CA_n2A-n66A CA_n2A-n77A CA_n48A-n66A CA_n66A-n77A | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A-n66A-n77A | - | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  | CA_n5A-n48A CA_n5A-n66A CA_n5A-n77A CA_n48A-n66A CA_n66A-n77A | n5 | 5, 10, 15, 20, 25 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. NOTE 3: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 4: Only single uplink carriers with power class other than PC3 are listed. NOTE 5: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### <<End of changes>>