3GPP TSG-RAN5 Meeting #101	R5-237669

# Chicago, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1710 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to Table 7.3B.2.3.4.2.1-6 of 3GPP TS 38.521-3, the uplink and downlink frequency of n41 for executing test ID 1 of DC_8A_n41A shall be set at 2655 MHz(ARFCN 531000). However, 2655 MHz is far as much as 35 MHz from the upper edge of n41. That means when we set n41 be configured as 100 MHz channel bandwidth, the upper lobe of the testing channel bandwidth exceeds 15 MHz from the boundary of n41. The frequency range of n41 is 2496 MHz – 2690 MHz. 100 MHz CH BW at 2655 MHz frequency is not used in actual network because it exceeds the upper limit of n41’s frequency range. CH BW 60 MHz can be used instead of 100 MHz to test the strictest conditions as much as possible. In addition, CBW of Test ID 2 should be updated accordingly because the purpose of Test ID 2 is avoiding UL harmonic of Test ID1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the UL / DL CH BW of n41 band for test ID 1 and 2 of DC_8A_n41A from 100 MHz to 60 MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test ID1 for DC_8A_n41A Configuration cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.4.2.1-6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-237265r1 |  |  |  |  |  |  |  |  |

### << start of changes >>

### << Unchanged sections omitted >>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Band | Range |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  | Range |  |  |  | NRB |  |  |  |  |
|  |  |  | UL MOD | DL MOD |  |  |  |  |  |  | CH BW |  |  |  |  |  |  |  |  | DLalloc / UL alloc |  |  |  |  | UL MOD |  |  |  |  | DL MOD |  |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 |  |  | 1 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| 24,10 |  |  | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 36 |  |  | 1 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 1 | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 713.3 / DL 768.3 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |
| 28 |  |  | 1 | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 708 / DL 763 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |
| Test Settings for a DC_1A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | 1 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 2 |  |  | 1 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3900 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 |  |  | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3870 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 |  |  | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4090.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 |  |  | 1 | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3709.995 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 1 | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3709.995 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 2 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 2 | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | DL 2175 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 |  |  | 2 | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | DL 2150 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 2 | DL 1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL 665.5 MHz |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 23 |  |  | 2 | DL1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL 673 MHz |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 35 |  |  | 2 | UL 1881 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 2 | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 |  |  | 2 | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3690.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 3 8 |  |  | 2 | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3850.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / 0 |  |  |  |  |
| 45 |  |  | 2 | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3920.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 4a 9 |  |  | 2 | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 54 |  |  | 2 | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 64 |  |  | 2 | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 74 |  |  | 2 | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3810 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 2 | UL 1870 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3740.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 |  |  | 2 | UL 1885 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3739.995 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 |  |  | 2 | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 |  |  | 2 | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3690 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 |  |  | 3 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 26 |  |  | 3 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34 |  |  | 3 | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 3 | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 |  |  | 3 | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 3 | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  | UL 2535 / DL 2655 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 3 | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 900 / DL 945 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 |  |  | 3 | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 3 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 26 |  |  | 3 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34, 10 |  |  | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL/DL 2657.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 3 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3495 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 |  |  | 3 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |
| 35 |  |  | 3 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3685.005 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 39 |  |  | 3 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs/0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 44 |  |  | 3 | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3574.995 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 54 |  |  | 3 | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3435 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 5 | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 5 | UL 837.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3350.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 |  |  | 5 | UL 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4145.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 38 |  |  | 5 | UL Mid (836.5 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 44, 13 |  |  | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 54, 13 |  |  | 5 | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4177.5 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 5 | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3360 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 5 | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3560.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 |  |  | 5 | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 7 | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1825 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 7 | DL 2667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | DL 879 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 7 | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | DL 2130 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 7 | DL 2672 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL/ 668 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |
| 28 |  |  | 7 | DL 2630 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  | UL 668 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |
| Test Settings for a DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 7 | DL 2660 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N77 |  |  |  |  | DL 3870 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 7 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 8 | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | DL 2155 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 8 | DL 945 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1850 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 |  |  | 8 | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | N/A |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | N/A |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 8 | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 |  |  | 8 | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL / DL 2655 MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 60 MHz | All RBs / 0 |  |  |  |
| 28 |  |  | 8 | UL 885 / DL 942.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL / DL 2546 MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 60 MHz | All RBs / 0 |  |  |  |
| 34 |  |  | 8 | UL 882.5 / DL 927.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL / DL 2685MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 8 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3590.01 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 2,8 |  |  | 8 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3520.005 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 |  |  | 8 | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3634.995 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 11 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4457.7 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 |  |  | 11 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4512.69 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 12 | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1730 /DL 2130 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 12 | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1770 /DL 2170 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_12A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 12 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3537.495 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 18 |  |  | 12 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3351 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 25 |  |  | 12 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3687.495 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 |  |  | 12 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3750 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 |  |  | 12 | UL 702/DL 732 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_12A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3535 MHz |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 12 | UL 707/DL 737 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3750 MHz |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34,10 |  |  | 12 | UL 710/DL 740 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3580 MHz |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 13 | UL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 13 | UL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 35 |  |  | 13 | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3755 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 49 |  |  | 13 | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3850 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 54 |  |  | 13 | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3891.495 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_14A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 14 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3964.995 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 18 |  |  | 14 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3350.1 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 25 |  |  | 14 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3814.995 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 |  |  | 14 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3350.01 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 |  |  | 14 | UL 795.5/DL 765.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3947.5 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_18A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 18 | UL 820 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4100 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 18 | UL 820 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4000 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_19A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 19 | UL 835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4175 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| 28 |  |  | 19 | UL 835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 4100 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 19 | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 |  |  | 19 | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4445.025 |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 20 | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 |  |  | 20 | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 20 | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 20 | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 |  |  | 20 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 |  |  | 20 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3387.99 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 |  |  | 20 | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3358.995 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_21A_n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 713 / DL 780.5 |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4510.2 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 |  |  | 21 | DL 1503.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4800 |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 |  |  | 21 | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4420.5 |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_25A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 25 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |
| 26 |  |  | 25 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |
| 36 |  |  | 25 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL/DL 2524.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz |  |  |  |  |  | All RBs / 0 |
| 2,8 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL/DL 2572 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 50 MHz |  |  |  |  |  | All RBs / 0 |
| 34 |  |  | 26 | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL/DL 2562 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |
| 45 |  |  | 26 | DL 886.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | UL/DL 2660 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | UL/DL 3366 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |
| 2,8 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | UL/DL 3446.01 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |
| 34 |  |  | 26 | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | UL/DL 3391.005 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4432.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 |  |  | 26 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | UL/DL 4470.51 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 15 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 28 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 28 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 28 |  |  | 28 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 34 |  |  | 28 | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3582.495 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| 45 |  |  | 28 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  | UL/DL 3815.01 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 30 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| Test Settings for a DC_30A_n77A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 30 | UL 2310 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | UL/DL 3487.5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_38A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 38 | DL High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL Low |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 40 | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 |  |  | 40 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  | 41 | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | UL/DL 3750 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |
| 26 |  |  | 41 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |
| 36 |  |  | 41 | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK | QPSK |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 48 |  |  |  |  |  |  | UL/DL 3555 |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1760 MHZ |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 23 | 48 |  |  |  |  |  |  | UL/DL 3565 |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1770 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 38 | 48 |  |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL1760 |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 44 | 48 |  |  |  |  |  |  | UL/DL 3630 |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 | 66 |  |  |  |  |  |  | UL1775/ DL2175 |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 24, 10 | 66 |  |  |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  |  |  |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 24 | 66 |  |  |  |  |  |  | UL 1712.5/ DL 2112.5 |  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  |  | UL 1912.5/ DL 1992.5 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 34 | 66 |  |  |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  | n25 |  |  |  |  |  |  | UL 1883.3/ DL 1963.3 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n41A Configuration (PC2 and PC3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 66 |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 66 |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 66 |  |  |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3440.01 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  | 66 |  |  |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3410 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 38 |  | 66 |  |  |  |  | UL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL Mid |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 44 |  | 66 |  |  |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3949.995 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 54 |  | 66 |  |  |  |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 66 |  |  |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3510 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  | 66 |  |  |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3480 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 34 |  | 66 |  |  |  |  | UL 1730/ DL2130 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3660 |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for DC_71A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 71 |  | UL 665.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | DL 1987.5 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |  |  | 5MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |
| 23 |  |  | 71 |  | UL 673 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | DL1980 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |  |  | 20 MHz |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |
| 35 |  |  | 71 |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1881 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| 49 |  |  | 71 |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1881 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |  |  | 20 MHz |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for DC_71A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  | 71 |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. NOTE 15: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2B.2.3.1-2 of TS 38.508-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

### << End of changes >>