3GPP TSG-RAN5 Meeting #101	R5-237696

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0781 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to DL1024QAM PDSCH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Arriver Software AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL1024QAM_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Minimum test time is not finalized for DL1024QAM PDSCH test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refer to discussion paper R5-237284 for minimum test time agreed for DL1024QAM PDSCH test cases. Updated Editor’s note for Clauses 5.2.2.1.1_3, 5.2.2.2.1_3, 5.2.3.1.1_5, 5.2.3.2.1_5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DL1024QAM PDSCH test cases will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.1_3, 5.2.2.2.1_3, 5.2.3.1.1_5, 5.2.3.2.1_5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-237284 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

###### 5.2.2.1.1_3	2Rx FDD FR1 PDSCH mapping Type A performance – 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA

Editor’s Note: Following aspects needs further investigation

-	DL EVM of <= 2.5% for f > 4.2 GHz pending further analysis by the TE vendors.

5.2.2.1.1_3.1	Test purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with DL1024QAM for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput for Rank 1 scenario.

<< Sections skipped >>

###### 5.2.2.2.1_3	2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA

Editor's Note: Following aspects needs further investigation

-	DL EVM of <= 2.5% for f > 4.2 GHz pending further analysis by the TE vendors

5.2.2.2.1_3.1	Test Purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with DL1024QAM for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput for Rank 1 scenario.

<< Sections skipped >>

###### 5.2.3.1.1_5	4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA

Editor's Note: Following aspects needs further investigation

-	DL EVM of <= 2.5% for f > 4.2 GHz pending further analysis by the TE vendors

###### 5.2.3.1.1_5.1	Test purpose

Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions with DL1024QAM for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput for Rank 1 scenario.

<< Sections skipped >>

###### 5.2.3.2.1_5	4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA

Editor's Note: Following aspects needs further investigation.

-	DL EVM of <= 2.5% for f > 4.2 GHz pending further analysis by the TE vendors

###### 5.2.3.2.1_5.1	Test purpose

Verify the PDSCH mapping Type A normal performance under 4 receive antenna conditions with DL1024QAM for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput for Rank 1 scenario.

<< Sections skipped >>

# G.1	Statistical testing of Performance Requirements with throughput

<< Sections skipped >>

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  |  | Reference Channel |  |  | Propagation condition |  |  | Demodulation scenario (doppler speed) |  |  | Minimum number of active subframes (MNAS) |  |  | MNAS to MNS Scaling factor (Note 3) |  |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | HST-750 |  |  | 750 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.2 FDD |  |  | HST-972 |  |  | 972 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | TDLC300-600 |  |  | 600 Hz |  |  | 8000 (Note 1) |  |  | 1.0526 |  |  | 9000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.0526 |  |  | 11000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 HD-FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.3333 |  |  | 14000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, R.PDSCH.5-1.1 FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.0526 |  |  | 22000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 HD-FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.3333 |  |  | 27000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-4.2 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD, R.PDSCH.5-1.2 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 HD-FDD, R.PDSCH.1-1.4 HD-FDD, R.PDSCH.1-1.5 HD-FDD, R.PDSCH.1-2.1 HD-FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.3333 |  |  | 100000 |  |  |
| NA |  |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.25 |  |  | 94000 |  |  |
| NA |  |  | R.PDSCH.1-17.1 FDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.0526 |  |  | 74000 |  |  |
| NA |  |  | R.PDSCH.1-8.3 FDD, R.PDSCH.1-13.1 FDD, R.PDSCH.1-13.2 FDD, R.PDSCH.1-13.3 FDD, R.PDSCH.1-13.4 FDD, R.PDSCH.1-13.5 FDD, R.PDSCH.1-14.1 FDD, R.PDSCH.1-14.2 FDD, R.PDSCH.1-14.3 FDD, R.PDSCH.1-14.4 FDD |  |  | HST-SFN |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-8.5 FDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 870 Hz |  |  | [30000 (Note 1)] |  |  | 1.0526 |  |  | [32000] |  |  |
| NA |  |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-15.1 FDD, R.PDSCH.1-15.2 FDD, R.PDSCH.1-15.3 FDD, R.PDSCH.1-15.4 FDD, R.PDSCH.1-15.5 FDD, R.PDSCH.1-16.1 FDD, R.PDSCH.1-16.2 FDD, R.PDSCH.1-16.3 FDD, R.PDSCH.1-16.4 FDD |  |  | HST-DPS |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-9.1 FDD, R.PDSCH.1-9.2 FDD, R.PDSCH.1-9.3 FDD, R.PDSCH.1-9.4 FDD, R.PDSCH.1-9.5 FDD, R.PDSCH.1-10.1 FDD, R.PDSCH.1-10.2 FDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 2.8571 |  |  | 215000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-2.1 TDD, R.PDSCH.1-2.2 TDD, R.PDSCH.1-2.3 TDD, R.PDSCH.1-2.4 TDD, R.PDSCH.1-2.5 TDD, R.PDSCH.1-3.1 TDD, R.PDSCH.1-3.2 TDD, R.PDSCH.1-3.3 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 2.8571 |  |  | 215000 |  |  |
| FR1.30-1A |  |  | R.PDSCH.2-1.1 TDD, R.PDSCH.2-1.5 TDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD, R.PDSCH.2-26.1 TDD, R.PDSCH.7-1.1 TDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.2903 |  |  | 26000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-3.5 TDD, R.PDSCH.2-4.1 TDD, R.PDSCH.2-4.3 TDD, R.PDSCH.7-1.2 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.3 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-25.1 TDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.2903 |  |  | 91000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-5.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-17.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 5 |  |  | 375000 |  |  |
| FR1.30-3 |  |  | R.PDSCH.2-6.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-4 |  |  | R.PDSCH.2-9.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-5 |  |  | R.PDSCH.2-11.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-6 |  |  | R.PDSCH.2-12.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD |  |  | HST-1000 |  |  | 1000 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD |  |  | HST-1667 |  |  | 1667 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.4 TDD, R.PDSCH.2-19.1 TDD, R.PDSCH.2-19.2 TDD, R.PDSCH.2-19.3 TDD, R.PDSCH.2-19.4 TDD, R.PDSCH.2-19.5 TDD, R.PDSCH.2-20.1 TDD, R.PDSCH.2-20.2 TDD, R.PDSCH.2-20.3 TDD, R.PDSCH.2-20.4 TDD, R.PDSCH.2-20.5 TDD, R.PDSCH.2-21.1 TDD |  |  | HST-SFN |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.5 TDD, R.PDSCH.2-22.1 TDD, R.PDSCH.2-22.2 TDD, R.PDSCH.2-22.3 TDD, R.PDSCH.2-22.4 TDD, R.PDSCH.2-22.5 TDD, R.PDSCH.2-23.1 TDD, R.PDSCH.2-23.2 TDD, R.PDSCH.2-23.3 TDD, R.PDSCH.2-23.4 TDD, R.PDSCH.2-23.5 TDD, R.PDSCH.2-24.1 TDD |  |  | HST-DPS |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-30.1 TDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 1667 Hz |  |  | [30000 (Note 1)] |  |  | 1.4815 |  |  | [45000] |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-13.1 TDD, R.PDSCH.2-13.2 TDD, R.PDSCH.2-13.3 TDD, R.PDSCH.2-13.4 TDD, R.PDSCH.2-13.5 TDD, R.PDSCH.2-14.1 TDD, R.PDSCH.2-14.2 TDD, R.PDSCH.2-14.3 TDD, R.PDSCH.2-14.4 TDD, R.PDSCH.2-14.5 TDD, R.PDSCH.2-15.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-7 |  |  | R.PDSCH.2-18.1 TDD, R.PDSCH.2-18.2 TDD, R.PDSCH.2-18.3 TDD, R.PDSCH.2-18.4 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | [75000] (Note 1) |  |  | 1.2903 |  |  | [97000] |  |  |
| FR2.60-1 |  |  | R.PDSCH.4-1.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1A |  |  | R.PDSCH.5-1.1 TDD |  |  | TDLC60-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  |  | TDLA30-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-1.2 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.25 |  |  | 25000 |  |  |
| FR2.120-2 |  |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-10.1 TDD |  |  | TDLD30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.26 |  |  | 26000 |  |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table G.1.5-1a: Minimum Test time for PDSCH demodulation with 1% BLER

| TDD UL-DL pattern | Reference Channel | Propagation condition | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-11.1 FDD R.PDSCH.1-11.2 FDD | TDLA30-10 | 10 Hz | 70000 (Note 1) | 1.1111 | 78000 |
| FR1.30-1 | R.PDSCH.2-16.1 TDD R.PDSCH.2-16.2 TDD | TDLA30-10 | 10 Hz | 70000 (Note 1) | 1.6667 | 117000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed). Note 3: MNS/MNAS ratio decided by scheduling pattern (how much time is required to collect required number of active DL SFs). |  |  |  |  |  |  |

<< End of Changes >>