3GPP TSG-RAN5 Meeting #101	R5-237755

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2737 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addtion of RedCap RRM TC 17.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add minimum requirements for NR RedCap timing test cases. - To add NR Redcap RRM TC – 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements for NR RedCap timing test cases are added. - NR Redcap RRM TC - 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.4.1.0 (new), 17.4.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0347 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-236710r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

17.4	Timing for RedCap

17.4.1	UE transmit timing for RedCap

17.4.1.0	Minimum Conformance Requirements

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy

The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 17.4.1.0.1-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that:

-	the SSB is within the UE’s active BWP, or

-	the SSB is not within the UE’s active BWP, and the measurement gap is configured.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at UE antenna. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in TS 38.133 [6] clause 7.3A was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table Table 17.4.1.0.1-2.

Table 17.4.1.0.1-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 2 | 240 | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 17.4.1.0.1-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR2 | 13792 |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in TS 38.133 [6] clause 7.3A is applied.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 17.4.1.0.1-3.

Table 17.4.1.0.1-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The normative reference for this requirement is TS.38.133 [6] clause 7.1A.1 and 7.1A.2.

17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is still missing

17.4.1.1.1	Test purpose

To verify that the UE can follow frame timing change of the connected gNB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 7.1A.2.

17.4.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onward and supporting FR2.

17.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 17.4.1.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.17.4.1.1

17.4.1.1.4	Test Description

17.4.1.1.4.1	Initial Conditions

Same as the initial conditions given in clause 7.4.1.1.4.1 with following exceptions:

-	Table 7.4.1.1.4.1-1 is replaced by Table 17.4.1.1.4.1-1;

-	Table 7.4.1.1.4.1-2 is replaced by Table 17.4.1.1.4.1-2;

Table 17.4.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 17.4.1.1-1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

Table 17.4.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.15-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 17.4.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

17.4.1.1.4.2	Test procedure

Same as the test procedure given in 7.4.1.1.4.2.

17.4.1.1.4.3	Message contents

Same as the message contents given in 7.4.1.1.4.3.

17.4.1.1.5	Test Requirements

Same as the test requirements given in 7.4.1.1.5 with following exceptions:

-	Table 7.4.1.1.5-1 is replaced by Table 17.4.1.1.5-1;

-	Table 7.4.1.1.5-2 is replaced by Table 17.4.1.1.5-2;

-	Table 7.4.1.1.5-3 is replaced by Table 17.4.1.1.5-3;

-	Table 7.4.1.1.5-4 is replaced by Table 17.4.1.1.5-4;

-	Table 7.4.1.1.5-5 is replaced by Table 17.4.1.1.5-5;

Table 17.4.1.1.5-1: Cell specific test parameters

| Parameter | Unit | Test1 |  | Test2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | Freq1 |  | Freq1 |  |  |
| Duplex Mode |  | TDD |  |  |  |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  |  |
| Data RBs allocated |  | 66 |  |  |  |  |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| TRS Configuration |  | TRS.2.1 TDD |  |  |  |  |
| PDSCH/PDCCH TCI state |  | TCI.State.2 |  |  |  |  |
| DRx Cycle | ms | N/A |  |  | DRX.8Note2 |  |
| PDSCH Reference measurement channel |  | SR.3.3 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | CR.3.2 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | CCR.3.7 TDD |  |  |  |  |
| OCNG Patterns |  | OP.1 |  |  |  |  |
| SSB Configuration |  | SSB.1 FR2 |  |  |  |  |
| SMTC Configuration |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition |  | AWGN |  |  |  |  |
| SRS Config |  | SRSConf.1Note3 | SRSConf.2Note3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: DRX related parameters are given in Table A.5-1 Note 3: SRS configs are given in Table A.17.4.1.1.5-2 |  |  |  |  |  |  |

Table 17.4.1.1.5-2: SRS Configurations

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1,0 | sl2560,4 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table 17.4.1.1.5-3: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in A.9.1 |  |
| Assumption for UE beamsNote 5 |  | Fine |  |
| Note1 | dBm/15kHzNote3 | -112+TT |  |
| Note1 | dBm/SCS | -100+TT |  |
|  | dB | 4+TT |  |
| SS-RSRPNote2 | dBm/SCS Note3 | -96+TT |  |
|  | dB | 4+TT |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5+TT |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone NOTE 4: As observed with 0dBi gain antenna at the centre of the quiet zone NOTE 5: Information about types of UE beam is given in TS 38.133 [6] section B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table 17.4.1.1.5-4: Te Timing Error Limit

| Frequency Range | SCS of SSB signals ( kHz) | SCS of uplink signals ( kHz) | Te |
| --- | --- | --- | --- |
| 2 | 240 | 120 | 3.75*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 17.4.1.1.5-5: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | The Maximum timing change in one adjustment Tq | The Minimum aggregate adjustment rate Tp | The Maximum aggregate adjustment rate Tq |
| --- | --- | --- | --- | --- |
| 2 | 120 | 3.125*64*Tc | -1.225*64*Tc | +3.725*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

<End of modified section 2>