3GPP TSG-RAN WG5 Meeting #101                                           R5-237831

# Chicago, USA, 13 – 17 November 2023

Agenda Item:	5.4.17

Source: 	CAICT

Title: 	Discussion on test of test cases with no exception requirements

Document for:	Discussion and Endorsement

Introduction

At RAN5#100 meeting, there were different understandings on TS 38.521-3 and TS 38.522 about how to perform the test cases with non-exception requirements.

This contribution is trying to reach an agreement on the understanding and giving a way forward on how to correct TS 38.521-3 and TS 38.522.

Discussion

The following is from TS 38.521-3：

4.5	Applicability and test coverage rules

4.5.0	General

(1)	The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following:

a)	For each NR band in a device; test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the EN-DC or NE-DC FR2 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-2 [9] for each NR band. Test only one EN-DC or NE-DC combination per FR2 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of TS 38.101-3 [4] using LTE anchor agnostic approach.

c)	Test all the EN-DC or NE-DC FR1 non-exception test requirements in this specification with test procedures which refer appropriately back to TS 38.521-1 [8] for each NR band. Test only one EN-DC or NE-DC combination per FR1 band for each EN-DC or NE-DC configuration as defined in clause 5.5B of 38.101-3 [4] using LTE anchor agnostic approach.

(2)	The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)	For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)	Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. If Standalone FR2 cannot be tested (due to test case not being complete), then test in EN-DC or NE-DC mode following (1) b) above.

c)	Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested. If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), then test in EN-DC or NE-DC mode following (1)c) above.

The different understandings are for the above green highlighted part.

Understanding 1: Test of non-exception requirements with LTE anchor agnostic approach is only for EN-DC only capable UEs and UEs not supporting NR/5GC mode in the tested band.

Understanding 2: In RAN5 specs, test of non-exception requirements with LTE anchor agnostic approach is technically the same as the standalone test case, which means for the same UE, if one is tested, the other can be skipped.

The following is from the draft spec of TS 38.522 after RAN5 #100 meeting (z_38522-i00) for Standalone FR1 and Standalone FR2 test cases, which clearly shows that “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are with revision marks which means they were approved at RAN5 #100 meeting when the different understandings were brought out. These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. (Not sure if there is still any company submitting TS 38.522 CRs with “Understanding 2” at this RAN5 #101 meeting).

Standalone FR1 test cases approved by RAN5 with “Understanding 2” are as bellow:

Standalone FR2 test cases approved by RAN5 with “Understanding 2” are as bellow:

Observation: “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are approved at RAN5 #100 meeting when the two different understandings were brought out.

Conclusion: These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. Which means “Understanding 2” is quite a common understanding in RAN5 since the beginning.

Proposal: Revise section 4.5.0 of 38.521-3, test procedure of 6.5B.3.3.1/6.5B.3.3.2 of 38.521-3, and Additional Information of SA FR1 test cases 6.5.3.1/6.5.3.2 in TS 38.522 as per “Understanding 2” at RAN5 #102 meeting.

Example to the proposal is as below:

Section 4.5.0 of TS 38.521-3：

(2) The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)  For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)  Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. Alternately, test in EN-DC or NE-DC mode following (1) b) above.

c)  Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested.  Alternately, test in EN-DC or NE-DC mode following (1)c) above.

Test procedure of 6.5B.3.3.1/6.5B.3.3.2 of TS 38.521-3:

6.5B.3.3.1.4.2	Test procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.3.1.5-1(for EN-DC configurations listed in Table 6.5B.3.3.1.5-1) or Table 6.5.3.1.5-1 in TS 38.521-1 [8] (for EN-DC configurations not listed in Table 6.5B.3.3.1.5-1). The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

5.	For UE operating on EN-DC configuration with Band n41, redo the test for frequency range 1 GHz  f < 12.75 GHz with the message content in step 7 of initial conditions with exceptions defined in Table 6.5B.3.3.1.4.3-4.

In addition to test configuration and test procedure above, UEs not tested under NR/5GC mode in the tested band need to be tested according to LTE anchor agnostic below for EN-DC configurations listed in Table 6.5B.3.3.1.5-1.

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

6.5B.3.3.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.3.1.2.4.2 with the following exceptions:

Instead of Table 6.5B.3.1.2.3-1 --> use Table 6.5B.3.3.2.5-1.

In addition to test configurations above, UEs not tested under NR/5GC mode in the tested band needs to be tested according to LTE anchor agnostic approach below.

Same test description as in clause 6.5.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

Additional Information of SA FR1 test cases 6.5.3.1/6.5.3.2 in TS 38.522：

Conclusions

Understanding 1: Test of non-exception requirements with LTE anchor agnostic approach is only for EN-DC only capable UEs and UEs not supporting NR/5GC mode in the tested band.

Understanding 2: In RAN5 specs, test of non-exception requirements with LTE anchor agnostic approach is technically the same as the standalone test case, which means for the same UE, if one is tested, the other can be skipped.

Observation: “Understanding 2” has been approved by RAN5 for 99 test cases, and even 9 of them are approved at RAN5 #100 meeting when the two different understandings were brought out.

Conclusion: These 99 test cases are supposed to be owned by many companies with time period from the very beginning of the drafting of 5G NR specs to RAN5 #100 meeting. Which means “Understanding 2” is quite a common understanding in RAN5 since the beginning.

Proposal: Revise section 4.5.0 of 38.521-3, test procedure of 6.5B.3.3.1/6.5B.3.3.2 of 38.521-3, and Additional Information of SA FR1 test cases 6.5.3.1/6.5.3.2 in TS 38.522 as per “Understanding 2” at RAN5 #102 meeting.

Example to the proposal is as below:

Section 4.5.0 of TS 38.521-3：

(2) The applicability and test coverage rules for Standalone (SA) and NSA capable devices shall include the following:

a)  For each NR band in a device, test all the EN-DC or NE-DC exception test requirements as per test procedures in this specification.

b)  Test all the Standalone FR2 test requirements as per test procedures in TS 38.521-2 [9] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR2 test requirements for that NR band and need not be retested. Alternately, test in EN-DC or NE-DC mode following (1) b) above.

c)  Test all the Standalone FR1 test requirements as per test procedures in TS 38.521-1 [8] for each NR band. This also fulfils coverage for all non-exception EN-DC or NE-DC FR1 test requirements for that NR band and need not be retested.  Alternately, test in EN-DC or NE-DC mode following (1)c) above.

Test procedure of 6.5B.3.3.1/6.5B.3.3.2 of TS 38.521-3:

6.5B.3.3.1.4.2	Test procedure

1.	E-UTRA SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.3.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands to the UE for both NR and E-UTRA carriers until the UE transmits at its PUMAX level; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5B.3.3.1.5-1(for EN-DC configurations listed in Table 6.5B.3.3.1.5-1) or Table 6.5.3.1.5-1 in TS 38.521-1 [8] (for EN-DC configurations not listed in Table 6.5B.3.3.1.5-1). The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS.

5.	For UE operating on EN-DC configuration with Band n41, redo the test for frequency range 1 GHz  f < 12.75 GHz with the message content in step 7 of initial conditions with exceptions defined in Table 6.5B.3.3.1.4.3-4.

In addition to test configuration and test procedure above, UEs not tested under NR/5GC mode in the tested band need to be tested according to LTE anchor agnostic below for EN-DC configurations listed in Table 6.5B.3.3.1.5-1.

Same test description as in clause 6.5.3.1.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

6.5B.3.3.2.4.2	Test Procedure

Same test procedure as described in clause 6.5B.3.1.2.4.2 with the following exceptions:

Instead of Table 6.5B.3.1.2.3-1 --> use Table 6.5B.3.3.2.5-1.

In addition to test configurations above, UEs not tested under NR/5GC mode in the tested band needs to be tested according to LTE anchor agnostic approach below.

Same test description as in clause 6.5.3.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA consist of test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1.

Additional Information of SA FR1 test cases 6.5.3.1/6.5.3.2 in TS 38.522：

# References

- draft spec of TS 38.522 after RAN5 #100 meeting (z_38522-i00): NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases

- TS 38.521-3 v18.0.0: NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios