3GPP TSG-RAN WG5 Meeting #101	R5-237882

Chicago, USA, 13th- 17th November 2023

Title:	IoT NTN discussion on SAN type coverage in testing

Source:	Keysight Technologies UK Ltd

Agenda item:	5.3.29.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN4 has been working on the definition of core and performance requirements for IoT NTN User Equipment (UE) supporting satellite access operation as part of the work item defined in [1] during Rel-18 with outcome applicable from Rel-17 ([2]):

The outcome of this work has been included in 3GPP TS 36.102 ([3]) for RF and demodulation requirements while RRM requirements have been added to 3GPP TS 36.133 ([4]).

These specifications cover both Geosynchronous Orbit (GSO) satellites as well as Non-Geosynchronous (NGSO) satellites although not explicitly stated in the work item definition ([1])

This document analyses the applicability of the RAN4 requirements defined for IoT NTN in terms of the type of satellite and proposes a way forward to progress on test specifications implications related to this topic. It is slightly modified resubmission of [5].

# 2. 	Discussion

## 2.1 Types of satellites

Both Geosynchronous and Non-Geosynchronous orbit satellites are under the scope of IoT NTN Work Item.

These satellites have different characteristics:

- NGSO (Non – Geosynchronous Orbit)

The main characteristics of NGSO satellites are:

- NGSO satellites includes Low-Earth Orbit (LEO) and Medium-Earth Orbit (MEO) satellites:

  - LEO satellites are in orbits in an altitude range between 300 km and 1500 km [6].

  - MEO satellites are in orbits in an altitude range between 7000 km and 25000 km [6].

- High relative satellite velocity with respect to the UE.

-GSO (Geosynchronous Orbit)

The main characteristics of GSO satellites are:

- GSO satellites are located in an Earth-centred orbit at a constant altitude of 35,786 km with an orbital period that matches Earth's rotation on its axis (23 hours, 56 minutes, and 4 seconds).

  - For an observer on Earth's surface, a satellite in a geosynchronous orbit returns to exactly the same position in the sky after a period of one sidereal day.

- Over the course of a day, the object's position in the sky may remain still or trace out a path, typically in a figure-8 form, whose precise characteristics depend on the orbit's inclination and eccentricity.

- A special case of geosynchronous orbit satellites is the geostationary orbit satellite (GEO), which is a circular geosynchronous orbit in Earth's equatorial plane with both inclination and eccentricity equal to 0.

  - A satellite in a geostationary orbit remains in the same position in the sky to observers on the surface.

- Low relative satellite velocity with respect to the UE (compared to NGSO).

## 2.2 UE capability indicating the type of satellite supported

According to 3GPP TS 36.306 ([7]), in IoT Non-Terrestrial Networks communications, the device indicates that supports IoT NTN access through the UE capability ntn-Connectivity-EPC-r17:

UE can indicate the type of satellite supported through UE capability ntn-ScenarioSupport-r17:

Observation 1: There could be devices supporting only GSO satellites.

Observation 2: There could be devices supporting only NGSO satellites.

Observation 3: There could be devices supporting both GSO and NGSO satellites.

## 2.2 Configuration of the type of satellite

According to 3GPP TS 36.331 ([8]), the type of satellite will be configured through the satellite ephemerisInfo-r17 IE embedded in servingSatelliteInfo-r17 IE sent in SIB31:

Ephemeris can be configured through position and velocity or through orbital parameters.

Observation 4: The type of satellite is configured through the ephemerisInfo-r17 IE.

## 2.3 Type of satellites in RF core requirements

There is no reference to the type of satellites in RF core requirements ([3]). Additionally, RAN4 confirmed in [9] that all core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

Observation 5: All core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

When defining UE transmitter and receiver test cases for IoT NTN, considering observations 1-5, different message contents should be defined for either GSO and NGSO. These messages contents could be defined in 36.508 if applicable to many test cases or in each test case if they can not be leveraged in other test cases.

Proposal 1: Define message contents for GSO and NGSO satellites for NTN UE transmitter and receiver test cases ([10]).

## 2.4 Type of satellites in demod requirements

Regarding demod test cases, R4-2220278 ([11]) included the following agreements:

Consequently, section 8.2.1.2 is to be added to the 3GPP TS 36.102 specification ([2]) according to R4-2305961 ([12]):

Observation 6: The only new demod requirement defined for IoT NTN PDSCH in 3GPP TS 36.102 ([3]) is only applicable to NGSO satellite.

However, the following note was added to Table 8.2.1.2-1 in [3]:

Being this note applicable to both GSO and NGSO satellites as RAN4 clarified in [9] for NR NTN.

Proposal 2: Define message contents for GSO and NGSO satellites for NTN demod test cases ([10]).

## 2.5 Type of satellites in RRM requirements

Regarding RRM test cases, according to R4-2306386 ([13]), each performance requirement defines the applicability of each type of satellite as follows:

Observation 7: In RRM requirements, it is explicitly stated the applicability of the requirements according to the type of satellites supported.

As both types of satellites will be required depending on the UE capabilities (although only one type of satellite will be tested for each UE) RRM test cases should consider the definition of message contents for both types of satellites.

Proposal 3: Define message contents for GSO and NGSO satellites for IoT NTN RRM test cases ([14]).

## 2.6 Type of satellites to be tested if device supports both GSO and NGSO

In case the device supports both GSO and NGSO satellites:

- RRM requirements already specify that only NGSO should be tested.

- RAN4 does not provide any guidance on which type of satellite should be tested.

Observation 8: RAN4 does not provide any guidance on which type of satellite should be tested in TRx and demod test cases if device supports both GSO and NGSO satellites.

Hence, it should be discussed whether TRx and demod test cases should be executed for each type of satellite or whether testing one of them could ensure compliance with the core requirements.

According to [9] and [15], except for frequency error, all TRx and demod test cases should be executed under zero Doppler and constant delay conditions.

Three options could be considered for TRx and Demod test cases:

- Option 1: Only NGSO testing is needed (similar as in RRM test cases)

- Option 2: Only GSO testing is needed.

- Option 3: Both types of satellites are needed.

For zero Doppler conditions case, GSO satellite covers the highest delays. Hence, option 2 should be the one chosen for all TRx and demod test cases under zero Doppler conditions.

Probably frequency error test under non-zero Doppler conditions should be tested separately for each type of satellite supported.

Proposal 4: For UE supporting both GSO and NGSO satellites:

- Demod test cases shall be tested only using NGSO (LEO-600) for new DEMOD requirements in 36.102 and 38.101-5.

- Demod test cases shall be tested only using GSO for legacy DEMOD requirements in 36.101 and 38.101-4

- RF test cases shall be tested using GSO when zero doppler conditions apply.

# 3. 	Conclusion

This document analyses the applicability of the RAN4 requirements defined for IoT NTN in terms of the type of satellite and proposes a way forward to progress on test specifications implications related to this topic.

The following observations and proposals are made:

Observation 1: There could be devices supporting only GSO satellites.

Observation 2: There could be devices supporting only NGSO satellites.

Observation 3: There could be devices supporting both GSO and NGSO satellites.

Observation 4: The type of satellite is configured through the ephemerisInfo-r17 IE.

Observation 5: All core requirements for UE transmitter and receiver apply to both GSO and NGSO satellites.

Proposal 1: Define message contents for GSO and NGSO satellites for NTN UE transmitter and receiver test cases ([10]).

Observation 6: The only new demod requirement defined for IoT NTN PDSCH in 3GPP TS 36.102 ([3]) is only applicable to NGSO satellite.

Proposal 2: Define message contents for GSO and NGSO satellites for NTN demod test cases ([10]).

Observation 7: In RRM requirements, it is explicitly stated the applicability of the requirements according to the type of satellites supported.

Proposal 3: Define message contents for GSO and NGSO satellites for IoT NTN RRM test cases ([14]).

Observation 8: RAN4 does not provide any guidance on which type of satellite should be tested in TRx and demod test cases if device supports both GSO and NGSO satellites.

Proposal 4: For UE supporting both GSO and NGSO satellites:

- Demod test cases shall be tested only using NGSO (LEO-600) for new DEMOD requirements in 36.102 and 38.101-5.

- Demod test cases shall be tested only using GSO for legacy DEMOD requirements in 36.101 and 38.101-4

- RF test cases shall be tested using GSO when zero doppler conditions apply.

# 4. 	References

- RP-223437, “WID on NB-IoT/eMTC core & performance requirements for NTN”, MediaTek Inc., RAN#98-e

- 3GPP TS 36.307, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) Supporting a release-independent frequency band”

- 3GPP TS 36.102, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”

- 3GPP TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”

- R5-233232, “IoT NTN discussion on satellite type coverage in testing”, Keysight Technologies UK Ltd, RAN5#99

- “3GPP TR 38.821, " Solutions for NR to support non-terrestrial networks (NTN)"

- 3GPP TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”

- 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA; Radio Resource Control (RRC) protocol specification”

- R4-2314001, “Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, Thales, RAN4#108

- 3GPP TS 36.521-4, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing”

- R4-2220278, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek, RAN4#105

- R4-2305961, “Big CR to 36.102: Demodulation requirements for IoT-NTN”, MediaTek, RAN4#106bis-e

- R4-2306386, “Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN “, MediaTek Inc., RAN4#106bis-e

- 3GPP TS 36.521-3, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing

- R4-2317006, “Further Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, RAN4#108bis