3GPP TSG-RAN5 Meeting #101	R5-237903

# Chicago , United States, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2561 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating minimum output power for UL non-contiguous CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case Minimum output power for CA is lack of test configurations for intra-band non-contiguous CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test configuration table for intra-band non-contiguous UL CA in test case 6.3A.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum output power can’t be tested for intra-band non-contiguous CA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0840 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-237052 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-237051r1. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3A.1	Minimum output power for CA

6.3A.1.0	Minimum conformance requirements

For inter-band carrier aggregation with uplink assigned to two NR bands, the minimum output power is defined per carrier and the requirement is specified in subclause 6.3.1.

For intra-band non-contiguous carrier aggregation, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

For intra-band contiguous carrier aggregation, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3A.1.

6.3A.1.1	Minimum output power for CA (2UL CA)

6.3A.1.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power for 2UL CA below the value specified in the test requirement when the power is set to a minimum value.

6.3A.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.3A.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.1.0.

6.3A.1.1.4	Test description

6.3A.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.1.1.4.1-1 or 6.3A.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.1.1.4.1-1: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.3A.1.1.4.1-2: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.3A.1.1.4.1-3: Test Configuration Table for intra-band non-contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1-1 NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.3A.1.1.4.1-1, 6.3A.1.1.4.1-2 and 6.3A.1.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.1.1.4.3.

6.3A.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause5.5.1. Message contents are defined in clause 6.3A.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.1.1.4.1-1 or 6.3A.1.1.4.1-2 or 6.3A.1.1.4.1-3 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "down" commands for both carriers in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power.

6.	Measure the mean power of the UE for each component carrier in the associated measurement channel bandwidth specified in Table 6.3A.1.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of 1ms in all active uplink slots and in the uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

6.3A.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.3A.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.3A.1.1.5	Test requirement

The minimum output power of each component carrier, derived in step 6 shall not exceed the values specified in Table 6.3A.1.1.5-1.

Table 6.3A.1.1.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 40 | -37+TT | 38.895 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3A.1.1.5-2 |  |  |

Table 6.3A.1.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 | 1.3 |
| 40MHz < BW ≤ 100MHz | 1.3 | 1.3 |

<End of Changes>