3GPP TSG-RAN5 Meeting #101	R5-237911

# Chicago , United States, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2570 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating AMPR for CA test case for CA_n28A-n41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_CA_R17_2BDL_2BUL-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In test case 6.2A.3, the AMPR test points and test requirements are absent for CA_28A-n41A with NS_17+NS_47. They need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating test case AMPR for CA for NS_17+NS_47 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMPR test case can’t be tested for CA_n28A-n41A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0847 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-237070 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-237069r1. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

FFS is left in Test applicability since there are no requirements for 6.5A.2.3 Additional Spectrum Emission mask for CA and 6.5A.3.3 Additional Spurious Emissions for CA in In Release-15.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2A.3.1.2	Test applicability

The requirements of this test apply in test case 6.5A.2.3 Additional Spectrum Emission mask for CA for network signalling values FFS to all types of NR UE release 15 and forward.

The requirements of this test apply in test case 6.5A.3.3 Additional Spurious Emissions for CA for network signalling values FFS to all types of NR UE release 15 and forward.

6.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2A.3.0.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2A.3.1.4.1-1: Test Configuration Table for inter-band CA (network signalling value NS_47/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] clause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest,Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL | UL configuration |  |  |  |  |  |  |
|  | configuration | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 | N/A for A-MPR | Default | 30 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | testing | Default | 30 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table for inter-band CA (network signalling value NS_100/NS_01)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest NRB_agg for PCC and SCC Highest NRB_agg for PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest,Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | CP-OFDM 256QAM |  | Outer Full | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Outer Full |

Table 6.2A.3.1.4.1-3: Test Configuration Table for inter-band CA (network signalling value NS_43/NS_01, NS_43U/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 910 | 10MHz | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-4: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_04)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-5: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | Low SCS | High SCS |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |

Table 6.2A.3.1.4.1-6: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_55)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation |  | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left |  | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left |  | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-7: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Refering to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full |
| 6 | Low range |  | DFT-s-OFDM Pi/2 QPSK | Inner_Full |
| 7 | Low range |  | DFT-s-OFDM Pi/2 16QAM | Inner_Full |
| 8 | Low range |  | DFT-s-OFDM Pi/2 64QAM | Inner_Full |
| 9 | Low range |  | DFT-s-OFDM Pi/2 256QAM | Inner_Full |
| 10 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 11 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 12 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 13 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

Table 6.2A.3.1.4.1-8: Test Configuration Table for inter-band CA (network signalling value NS_17/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 745.5 | 5 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-9: Test Configuration Table for PC3 inter-band CA or PC2 inter-band CA with both bands supporting PC3 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM QPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Left |

Table 6.2A.3.1.4.1-9a: Test Configuration Table for PC2 inter-band CA with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM PI/2 BPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Left |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.3.1.4.3.

6.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in 6.2A.3.1.4.1 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test. For power class 2 intra-band contiguous carrier aggregation, the additional spectrum emission mask is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in 6.2A.3.1.4.1 send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

6.2A.3.1.4.3.1	Message contents exceptions (network signalling value " NS_100" on PCC)

Table 6.2A.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2A.3.1.4.3.2	Message contents exceptions (network signalling value " NS_43" on PCC)

Table 6.2A.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_43) |  |  |

6.2A.3.1.4.3.3	Message contents exceptions (network signalling value " NS_43U" on PCC)

Table 6.2A.3.1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2A.3.1.4.3.4	Message contents exceptions (network signalling value "NS_47" on PCC)

Table 6.2A.3.1.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.4.3.5	Message contents exceptions (network signalling value "NS_56" on PCC)

Table 6.2A.3.1.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_56) |  |  |

6.2A.3.1.4.3.6	Message contents exceptions (network signalling value "CA_NS_04" on PCC and SCC)

Table 6.2A.3.1.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_04" on PCC and SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_04) |  |  |

Table 6.2A.3.1.4.3.6-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_NS_04

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Test IDs 5 |
|  | 15 |  | Power class 3 and Test IDs 6, 7 |
|  | 14 |  | Power class 3 and Test IDs 2, 4, 8 |
|  | 12 |  | Power class 3 and Test IDs 10, 11, 12 |
|  | 11 |  | Power class 3 and Test IDs 9 |
|  | 10 |  | Power class 3 and Test IDs 1, 3, 13 |
|  | 19 |  | Power class 2 and Test IDs 5 |
|  | 19 |  | Power class 2 and Test IDs 2, 4 |
|  | 17 |  | Power class 2 and Test IDs 6, 7 |
|  | 16 |  | Power class 2 and Test IDs 8 |
|  | 14 |  | Power class 2 and Test IDs 10, 11, 12 |
|  | 14 |  | Power class 2 and Test IDs 1, 3, 9 |
|  | 11 |  | Power class 2 and Test IDs 13 |

6.2A.3.1.4.3.7	Message contents exceptions (network signalling value "NS_17" on PCC)

Table 6.2A.3.1.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_17" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_17) |  |  |

6.2A.3.1.4.3.8	Message contents exceptions (network signalling value "NS_47" on SCC)

Table 6.2A.3.1.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1a: UE Power Class 2 with both bands supporting PC3 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23.9 (22.72) | 3 (52) | 28+TT | 20.9-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: UE Power Class 2 with PCC supporting PC3 and SCC supporting PC2 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 (52) | 28+TT | 21.6TT (20.6-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 with supporting higherPowerLimit-r17 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8TT (23.3-TT2) |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 29.8+TT | 21.6-TT (20.6+-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 7 | 0 | 0 | 0 | 3 | 9.8 | 7 | 25+TT | 2.8-TT |
| 3 | 23 | 6.5 | 0 | 7 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14-TT |

Table 6.2A.3.1.5-4: UE Power Class 3 test requirement (network signalling value NS_56/NS_04) for CA_n24-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 8 | 0 | 0 | 3 | 16.0 | 5 | 25+TT | 11-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 8 | 0 | 0 | 3 | 18.0 | 4 | 25+TT | 14-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 8 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 8 | 0 | 0 | 3 | 18.0 | 4 | 25+TT | 14-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-5: UE Power Class 3 test requirement (network signalling value NS_56/NS_27) for CA_n24-n48

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 6 | 0 | 0 | 3 | 17.2 | 5 | 25+TT | 12.2-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 6 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 11.5 | 0 | 0 | 3 | 14.3 | 5 | 25+TT | 9.3-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 11.5 | 0 | 0 | 3 | 16.6 | 5 | 25+TT | 11.6-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-6: UE Power Class 3 test requirement (network signalling value NS_56/NS_55) for CA_n24-n77

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 0 | 0 | 0 | 3 | 17.2 | 5 | 25+TT | 12.2-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 0 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 0 | 0 | 0 | 3 | 17.6 | 5 | 25+TT | 12.6-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 0 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-7: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 3.5 | - | 3.5 | 1.5 | 2 | 18 | 4 | 25+TT | 14-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 3.5 | - | 3.5 | 1.5 | 2 | 18 | 4 | 25+TT | 14-TT |
| 5 | 23 | 2.5 | 3.5 | 3.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 6 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 7 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 8 | 23 | 5 | 4.5 | 5 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 9 | 23 | 7 | 6 | 7 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |
| 10 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 11 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 12 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 13 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2A.3.1.5-8: UE Power Class 2 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8 | - | 8 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 2 | 26 | 3.5 | - | 3.5 | 1.5 | 2 | 21 | 2 | 25+TT | 19-TT |
| 3 | 26 | 8 | - | 8 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 4 | 26 | 3.5 | - | 3.5 | 1.5 | 2 | 21 | 2 | 25+TT | 19-TT |
| 5 | 26 | 2.5 | 5.5 | 5.5 | 0 | 2 | 21.5 | 2 | 25+TT | 19.5-TT |
| 6 | 26 | 2.5 | 6 | 6 | 0 | 2 | 20 | 2.5 | 25+TT | 17.5-TT |
| 7 | 26 | 2.5 | 6 | 6 | 0 | 2 | 20 | 2.5 | 25+TT | 17.5-TT |
| 8 | 26 | 5 | 6.5 | 6.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 9 | 26 | 7 | 8 | 8 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 10 | 26 | 3.5 | 7.5 | 7.5 | 0 | 2 | 18.5 | 4 | 25+TT | 14.5-TT |
| 11 | 26 | 3.5 | 7.5 | 7.5 | 0 | 2 | 18.5 | 4 | 25+TT | 14.5-TT |
| 12 | 26 | 5 | 7.5 | 7.5 | 0 | 2 | 18.5 | 4 | 25+TT | 14.5-TT |
| 13 | 26 | 7 | 10 | 10 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |

Table 6.2A.3.1.5-9: UE Power Class 3 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 3 | 23 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14-TT |

Table 6.2A.3.1.5-10: UE Power Class 2 test requirement (network signalling value NS_17/NS_01) for CA_n28-n79

| Test ID | PPowerclass | PPowerclass,c |  | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW (PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 23 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 23 | 26 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 21.2 | 5 | 28+TT | 16.2-TT |
| 3 | 26 | 23 | 26 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |

Table 6.2A.3.1.5-11: UE Power Class 3 test requirement (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-11a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

Table 6.2A.3.1.5-11b: UE Power Class 2 test requirement with with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 3.5 | 0 | 5 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 3.5 | 0 | 10 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 3.5 | 0 | 10 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<End of Changes>