3GPP TSG-RAN5 Meeting #101	R5-237939

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2939 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defined frequencies for n7 and certain Release 16 CBWs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test frequencies for n7 and CBW=30MHz , 40MHz are not defined in 38.508-1, section 4.3.1 despite possible based on PRD21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined test frequencies for 30MHz and 40MHz CBW for n7 for all SCS. In the same tables, unified content of “SS block SCS [kHz]” column. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Added ROHDE & SCHWARZ as co-source company. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.7	Reference test frequencies for NR operating band n7

Table 4.3.1.1.1.7-1: Test frequencies for NR operating band n7 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2622.5 | 524500 | 2620.25 | 524050 | 0 | 15 | 6554 | 524410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2634.39 | 526878 | 102 |  | 6636 | 530910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2687.5 | 537500 | 2594.53 | 518906 | 504 |  | 6718 | 537410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2502.5 | 500500 | 2500.25 | 500050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2442.03 | 488406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2567.5 | 513500 | 2564.17 | 512834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2625 | 525000 | 2620.32 | 524064 | 0 | 15 | 6555 | 524430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2631.96 | 526392 | 102 |  | 6630 | 530430 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.32 | 500064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2439.6 | 487920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2559.24 | 511848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2627.5 | 525500 | 2620.39 | 524078 | 0 | 15 | 6556 | 524450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2629.53 | 525906 | 102 |  | 6624 | 529950 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 2682.5 | 536500 | 2584.67 | 516934 | 504 |  | 6692 | 535450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.39 | 500078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2437.17 | 487434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2554.31 | 510862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2630 | 526000 | 2620.46 | 524092 | 0 | 15 | 6557 | 524650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2627.1 | 525420 | 102 |  | 6618 | 529470 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 2680 | 536000 | 2579.74 | 515948 | 504 |  | 6682 | 534530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.46 | 500092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2434.74 | 486948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2549.38 | 509876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2632.5 | 526500 | 2620.53 | 524106 | 0 | 15 | 6558 | 524670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2624.67 | 524934 | 102 |  | 6612 | 528990 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 2677.5 | 535500 | 2574.81 | 514962 | 504 |  | 6669 | 533550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.53 | 500106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2432.31 | 486462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2544.45 | 508890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2635 | 527000 | 2620.6 | 524120 | 0 | 15 | 6559 | 524690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2622.24 | 524448 | 102 |  | 6606 | 528510 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 2675 | 535000 | 2569.88 | 513976 | 504 |  | 6656 | 532570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.6 | 500120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2429.88 | 485976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2539.52 | 507904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2640 | 528000 | 2620.56 | 524112 | 0 | 15 | 6558 | 524670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2617.2 | 523440 | 102 |  | 6594 | 527550 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.56 | 500112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2424.84 | 484968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2529.48 | 505896 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2645 | 529000 | 2620.7 | 524140 | 0 | 15 | 6557 | 524650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2655 | 531000 | 2612.34 | 522468 | 102 |  | 6582 | 526590 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2665 | 533000 | 2549.98 | 509996 | 504 |  | 6607 | 528530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2525 | 505000 | 2500.7 | 500140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2419.98 | 483996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2519.62 | 503924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-2: Test frequencies for NR operating band n7 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2625 | 525000 | 2620.68 | 524136 | 0 | 15 | 6561 | 524910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2613.96 | 522792 | 102 |  | 6636 | 530910 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.68 | 500136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2349.24 | 469848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2558.52 | 511704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2627.5 | 525500 | 2620.66 | 524132 | 0 | 15 | 6562 | 524930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2655 | 531000 | 2611.44 | 522288 | 102 |  | 6630 | 530430 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2682.5 | 536500 | 2494.22 | 498844 | 504 |  | 6698 | 535930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.66 | 500132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2346.72 | 469344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2553.5 | 510700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2630 | 526000 | 2620.82 | 524164 | 0 | 15 | 6560 | 524890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2609.1 | 521820 | 102 |  | 6624 | 529950 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 2680 | 536000 | 2489.38 | 497876 | 504 |  | 6688 | 535010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.82 | 500164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2344.38 | 468876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2548.66 | 509732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2632.5 | 526500 | 2620.8 | 524160 | 0 | 15 | 6561 | 524910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2606.58 | 521316 | 102 |  | 6618 | 529470 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 2677.5 | 535500 | 2484.36 | 496872 | 504 |  | 6675 | 534030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2500.8 | 500160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2341.86 | 468372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2557.5 | 511500 | 2543.64 | 508728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2635 | 527000 | 2620.96 | 524192 | 0 | 15 | 6562 | 524930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2604.24 | 520848 | 102 |  | 6612 | 528990 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2675 | 535000 | 2479.52 | 495904 | 504 |  | 6662 | 533050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2515 | 503000 | 2500.96 | 500192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2339.52 | 467904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2555 | 511000 | 2538.8 | 507760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2640 | 528000 | 2620.92 | 524184 | 0 | 15 | 6561 | 524910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2599.2 | 519840 | 102 |  | 6600 | 528030 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2520 | 504000 | 2500.92 | 500184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2334.48 | 466896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2550 | 510000 | 2528.76 | 505752 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2645 | 529000 | 2621.06 | 524212 | 0 | 15 | 6563 | 525130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2655 | 531000 | 2594.34 | 518868 | 102 |  | 6588 | 527070 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2665 | 533000 | 2459.62 | 491924 | 504 |  | 6613 | 529010 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.06 | 500212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2329.62 | 465924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2545 | 509000 | 2518.9 | 503780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-3: Test frequencies for NR operating band n7 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2625 | 525000 | 2621.04 | 524208 | 0 | 15 | - | 524568 |
|  |  |  | Mid | 2655 | 531000 | 2577.6 | 515520 | 102 |  | - | 530568 |
|  |  |  | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
|  |  | Uplink | Low | 2505 | 501000 | 2501.04 | 500208 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2168.16 | 433632 | 504 |  | - | - |
|  |  |  | High | 2565 | 513000 | 2556.72 | 511344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2627.5 | 525500 | 2621.02 | 524204 | 0 | 15 | - | 524564 |
|  |  |  | Mid | 2655 | 531000 | 2575.08 | 515016 | 102 |  | - | 530064 |
|  |  |  | High | 2682.5 | 536500 | 2313.14 | 462628 | 504 |  | - | 535564 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2501.02 | 500204 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2165.64 | 433128 | 504 |  | - | - |
|  |  |  | High | 2562.5 | 512500 | 2551.7 | 510340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2630 | 526000 | 2621.36 | 524272 | 0 | 15 | - | 524632 |
|  |  |  | Mid | 2655 | 531000 | 2572.92 | 514584 | 102 |  | - | 529632 |
|  |  |  | High | 2680 | 536000 | 2308.48 | 461696 | 504 |  | - | 534632 |
|  |  | Uplink | Low | 2510 | 502000 | 2501.36 | 500272 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2163.48 | 432696 | 504 |  | - | - |
|  |  |  | High | 2560 | 512000 | 2547.04 | 509408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2632.5 | 526500 | 2621.34 | 524268 | 0 | 15 | - | 524628 |
|  |  |  | Mid | 2655 | 531000 | 2570.4 | 514080 | 102 |  | - | 529128 |
|  |  |  | High | 2677.5 | 535500 | 2303.46 | 460692 | 504 |  | - | 533628 |
|  |  | Uplink | Low | 2512.5 | 502500 | 2501.34 | 500268 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2160.96 | 432192 | 504 |  | - | - |
|  |  |  | High | 2557.5 | 511500 | 2542.02 | 508404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2635 | 527000 | 2621.32 | 524264 | 0 | 15 | - | 524624 |
|  |  |  | Mid | 2655 | 531000 | 2567.88 | 513576 | 102 |  | - | 528624 |
|  |  |  | High | 2675 | 535000 | 2298.44 | 459688 | 504 |  | - | 532624 |
|  |  | Uplink | Low | 2515 | 503000 | 2501.32 | 500264 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2158.44 | 431688 | 504 |  | - | - |
|  |  |  | High | 2555 | 511000 | 2537 | 507400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2640 | 528000 | 2621.64 | 524328 | 0 | 15 | - | 524688 |
|  |  |  | Mid | 2655 | 531000 | 2563.2 | 512640 | 102 |  | - | 527688 |
|  |  |  | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
|  |  | Uplink | Low | 2520 | 504000 | 2501.64 | 500328 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2153.76 | 430752 | 504 |  | - | - |
|  |  |  | High | 2550 | 510000 | 2527.32 | 505464 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2645 | 529000 | 2621.6 | 524320 | 0 | 15 | - | 524680 |
|  |  |  | Mid | 2655 | 531000 | 2558.16 | 511632 | 102 |  | - | 526680 |
|  |  |  | High | 2665 | 533000 | 2278.72 | 455744 | 504 |  | - | 528680 |
|  |  | Uplink | Low | 2525 | 505000 | 2501.6 | 500320 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2148.72 | 429744 | 504 |  | - | - |
|  |  |  | High | 2545 | 509000 | 2517.28 | 503456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>