3GPP TSG-RAN5 Meeting #101	R5-237943

# Chicago, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2996 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Note 1 and Note 1a of Table 4.3.1.0A-1: Mid Test Channel bandwidths |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.521-1 Table 5.3.5-1 the channel bandwidths are specified per operating band. Several Notes in this Table are associated with these bandwidths state that depending upon the release they are optionally supported. For e.g. Table 5.3.5-1: Channel Bandwidths for each NR band NOTE 6: This UE channel bandwidth is optional in R15. NOTE 12: This UE channel bandwidth is optional in R17. According to 38.508-1 mid-channel bandwidth is to be implemented as per the table Table 4.3.1.0A-1; however the notes associated with the mid channel bandwidth are open to interpretation. Most TEVs have updated their software to the latest 3GPP Release, assumes that all devices are Rel-17 and does not take into account if you have a Rel-15 or Rel-16 device. Furthermore test platform vendors have implemented the mid-channel in different ways meaning very difficult to test the right mid-channel sometimes for earlier releases Rel-15 or Rel-16 across different validated test platforms. In cases where some of the optional Mid channel bandwidths are not supported by the device or the test system. The interpretation is then based on an Arithmetic mean of either the specification 38.508-1 or the UE supported channel bandwidths with the added interpretation of what the test system can support. This produces differing results depending upon the system in use and the calculation method adopted which causes an issue for cross checking results. For example: TEV A may implement 60 MHz as mid-channel from the spec in band n41, TEV B may implement 50 MHz based on arithmetic average for the same NR FR1 band. Testing could be being performed with a different mid-channel bandwidth on the same device! It makes sense that the certification testing is being performed in a standardised way so labs may not question if the test vendor implements these standards in different ways. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised following text for Note 1a of Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1. If the UE does not support all listed channel bandwidths specified in Table 5.3.5-1 of TS 38.521-1 [14], the mid channel BW is the closest channel BW to the arithmetic average of all the channel BWs supported by the UE per band. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notes in Table 4.3.1.0A-1 will be ambiguous and will cause different TEV implementations of mid test channel bandwith for testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-237266r2 |  |  |  |  |  |  |  |  |

### << start of changes >>

### << Unchanged sections omitted >>

#### 4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n18 | 10 |  |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 20 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 404, 305 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n47 | 30 | 20 |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103 | 152, 103 |
| n71 | 15 | 15 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n86 | 203 | [15] |
| n91 | 52, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closer bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: If the UE does not support all listed channel bandwidths specified in Table 5.3.5-1 of TS 38.521-1 [14], the mid channel BW is the closest channel BW to the arithmetic average of all the channel BWs supported by the UE per band. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n259 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-2 [15] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closer bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NR band, NTN

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n255 | 15 |
| n256 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.1 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

### << End of changes >>