3GPP TSG-RAN5 Meeting #101	R5-237956

# Chicago, USA, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0626 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test tolerance analysis for 7.5.3.14 Fast SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case of 7.5.3.14 Fast SCell activation has been introduced into TS 38.533, so the corresponding TT analysis needs to be finished. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TT analysis into 38.903. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TT derivation can’t be documented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 2780 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2321466 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-236871 |  |  |  |  |  |  |  |  |

<Start of Changes>

8	Grouping of test cases defined in TS 38.533

<Unchanged Skipped>

Table 8-2: Grouping of FR2 test cases defined in Clauses 5, 7, 8 and 17 of TS 38.533

| Group | Test Case Numbers |  | .zip file name | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Transmit_Timing_01 | 5.4.1.1 7.4.1.1 |  | “38.533 5.4.1.1+7.4.1.1 TT.zip” | 1 NR FR2 cell, no fading |  |
| Timing_Advance_01 | 5.4.3.1 7.4.3.1 |  | “38.533 5.4.3.1+7.4.3.1 TT.zip“ | 1 NR FR2 cell, no fading |  |
| iRAT_meas_LTE_Serving | 8.4.2.5 18.3.1.5 |  | “38.533 8.4.2.5+18.3.1.5 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 time periods, No fading” |  |
| iRAT_meas_LTE_Serving_01 | 8.4.2.6 8.4.2.7 8.4.2.8 18.3.1.6 18.3.1.7 18.3.1.8 |  | “38.533 8.4.2.6+8.4.2.7+8.4.2.8+18.3.1.6 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 time periods, No fading” |  |
| iRAT_SS-RSRP_01 | 8.5.2.1.2 |  | “38.533 8.5.2.1.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |  |
| iRAT_SS-RSRQ_01 | 8.5.2.2.2 |  | “38.533 8.5.2.2.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |  |
| iRAT_SS-SINR_01 | 8.5.2.3.2 |  | “38.533 8.5.2.3.2 TT.zip” | “1 NR FR2 cell, 1 E-UTRA serving cell, 2 sub-tests, No fading” |  |
| Interruption_Transition_01 | 5.5.2.1 5.5.2.2 |  | “38.533 5.5.2.1+5.5.2.2 TT.zip” | ”1 E-UTRAN Cell, 1 NR FR2 Cell, 1 time period, No fading” |  |
| Interruption_meas_NR_SCC_01 | 5.5.2.3 5.5.2.4 7.5.2.1 |  | “38.533 5.5.2.3+5.5.2.4+7.5.2.1 TT.zip” | 1 E-UTRAN Cell, 2 NR Cells (2 NR Cells for SA case), 1 time period, No fading |  |
| SCell_Activation_01 | 5.5.3.1 5.5.3.7 |  | “38.533 5.5.3.1 TT.zip” | “2 Inter Frequency NR FR2 Cells, 3 time periods, Various number of sub-tests, No fading” |  |
| SCell_Activation_02 | 7.5.3.3 |  | “38.533 7.5.3.3 TT.zip” | “2 Inter Frequency NR FR2 Cells, 3 time periods, Various number of sub-tests, No fading” |  |
| SCell_Activation_02 | 7.5.3.4 |  | “38.533 7.5.3.4 TT.zip” | “2 Inter Frequency NR FR2 Cells, 3 time periods, Various number of sub-tests, No fading” direct Scell activation configurations |  |
| SCell_Activation_03 | 7.5.3.5 |  | “38.533 7.5.3.5 TT.zip” | “3 Cells with 2 Intra frequency and 3rd cell with Inter Frequency NR FR2 Cells, 3 time periods, Various number of sub-tests, No fading” direct Scell activation with handover test case |  |
| FastSCell_Activation | 7.5.3.14 |  | “38.533 7.5.3.14 TT.zip” | “2 Inter Frequency NR FR2 Cells, 2 time periods, No fading” |  |
| Intra_Freq_Meas_01 | 5.6.1.1 5.6.1.3 7.6.1.1 7.6.1.3 17.6.1.1 17.6.1.3 |  | "38.533 5.6.1.1+5.6.1.3+7.6.1.1+5.6.1.3 TT_v3.zip " | 2 NR FR2 Cells (1 E-UTRA Cell for NSA case), 2 SSBs, 2 time periods, no fading, 2 AoAs, both are in EIS spherical coverage and rough beams |  |
| Intra_Freq_Meas_02 | 5.6.1.2 5.6.1.4 7.6.1.2 7.6.1.4 17.6.1.2 17.6.1.4 |  | "38.533 5.6.1.2+5.6.1.4+7.6.1.2+5.6.1.4 TT.zip " | 2 NR FR2 Cells (1 E-UTRA Cell for NSA case), 2 SSBs, 2 time periods, no fading, 1 AoA in Rx peak and rough beam |  |
| Inter_Freq_Meas_01 | 5.6.2.1 5.6.2.3 7.6.2.1 7.6.2.3 |  | “38.533 5.6.2.1+5.6.2.3+7.6.2.1+7.6.2.3 TT v2.zip” | “2 Inter Frequency NR FR2 Cells, 2 time periods, Various number of sub-tests, No fading” |  |
| Inter_Freq_Meas_02 | 5.6.2.5 5.6.2.7 7.6.2.5 7.6.2.7 |  | “38.533 5.6.2.5+5.6.2.7+7.6.2.5+7.6.2.7 TT.zip” | “2 Inter Frequency NR Cells (Cell 1 on FR1 and Cell 2 on FR2), 2 time periods, Various number of sub-tests, No fading” |  |
| Inter_Freq_Meas_03 | 5.6.2.6 5.6.2.8 7.6.2.6 7.6.2.8 |  | “38.533 5.6.2.6+5.6.2.8+7.6.2.6+7.6.2.8 TT.zip” | “2 Inter Frequency NR Cells (Cell 1 on FR1 and Cell 2 on FR2), 2 time periods, Various number of sub-tests, No fading” |  |
| Inter_Freq_Meas_04 | 5.6.2.2 5.6.2.4 7.6.2.2 7.6.2.4 |  | “38.533 5.6.2.2+5.6.2.4+7.6.2.2+7.6.2.4 TT.zip” | “2 Inter Frequency NR Cells (both on FR2), 2 time periods, Various number of sub-tests, No fading” |  |
| SSB_Based_L1-RSRP-Meas | 5.6.3.1 5.6.3.2 7.6.3.1 7.6.3.2 17.6.3.1 17.6.3.2 |  | “38.533 5.6.3.1+5.6.3.2+7.6.3.1+7.6.3.2 TT.zip” | “1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 2 time periods, No fading” |  |
| SS-RSRP_01 | 5.7.1.1 7.7.1.1 |  | “38.533 5.7.1.1+7.7.1.1 TT v2.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |  |
| SS-RSRP_02 | 5.7.1.2 7.7.1.2 |  | “38.533 5.7.1.2+7.7.1.2 TT v2.zip” | “2 Inter-Frequency NR FR2 Cells, 2 sub-tests, No fading” |  |
| SS-RSRP_03 | 5.7.1.3 7.7.1.3 |  | “38.533 5.7.1.3+7.7.1.3 TT.zip” | “1 NR FR1 Cell, 1 NR FR2 Cell, 2 sub-tests, No fading” |  |
| SS-RSRQ_01 | 5.7.2.1 7.7.2.1 |  | “38.533 5.7.2.1+7.7.2.1 TT.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |  |
| SS-RSRQ_02 | 5.7.2.2 7.7.2.2 |  | “38.533 5.7.2.2+7.7.2.2 TT.zip” | “2 Inter-Frequency NR FR2 Cells, 2 sub-tests, No fading” |  |
| SS-SINR_01 | 5.7.3.1 7.7.3.1 |  | “38.533 5.7.3.1+7.7.3.1 TT.zip” | “2 Intra-Frequency NR FR2 Cells, 2 sub-tests, No fading” |  |
| SS-SINR_02 | 5.7.3.2 7.7.3.2 |  | “38.533 5.7.3.2+7.7.3.2 TT v2.zip” | “2 Inter-Frequency NR FR2 Cells, 3 sub-tests, No fading” |  |
| CSI-RS_Based_L1-RSRP-Meas | 5.6.3.3 5.6.3.4 7.6.3.3 7.6.3.4 17.6.3.3 17.6.3.4 |  | “38.533 5.6.3.3+6.6.3.4+7.6.3.3+7.6.3.4 TT.zip” | “1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 1 time period, No fading” |  |
| SSB_Based_BFD | 5.5.5.1 5.5.5.2 5.5.5.5 7.5.5.1 7.5.5.2 7.5.5.5 17.5.2.5 |  | "38.533 5.5.5.1+5.5.5.2+7.5.5.1+7.5.5.2 TT.zip " | "1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 2 SSBs, 5 time periods, fading, 1AoA Rx peak, Rough beam" |  |
| CSI-RS Based BFD and BFR | 5.5.5.3 5.5.5.4 7.5.5.3 7.5.5.4 17.5.2.3 17.5.2.4 |  | “38.533 5.5.5.3+5.5.5.4+7.5.5.3+7.5.5.4 TT v2.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |  |
| CSI-RS Based SCell BFD and BFR | 5.5.5.6 5.5.5.7 7.5.5.6 7.5.5.7 |  | “38.533 5.5.5.3+5.5.5.4+7.5.5.3+7.5.5.4 TT v2.zip” | “2 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |  |
| TRP specific CSI-RS Based BFD and BFR | 5.5.5.8 7.5.5.9 |  | “38.533 5.5.5.8+7.5.5.9 TT.zip” | “2 NR Cell (1 E-UTRA Cell for NSA case), 5 time periods, Fading” |  |
| SSB_Based_BFD_Relaxed_Meas | 5.5.5.9 |  | "38.533 5.5.5.9 TT.zip " | "1 NR FR2 Cell (1 E-UTRA Cell for NSA case), 2 SSBs, 5 time periods, fading, 1AoA Rx peak, Rough beam" |  |
| TRP specific SSB_Based_BFD | 7.5.5.10 |  | “38.533 7.5.5.10 TT.zip” | "2 NR FR2 Cell, 2 SSBs, 5 time periods, fading" |  |
| CSI-RS_WithoutIMR_L1-SINR-Meas | 5.6.6.1 7.6.6.1 |  | “38.533 5.6.6.1+7.6.6.1 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |  |
| CSI-RS_WithCSI-IM_L1-SINR-Meas | 5.6.6.3 |  | “38.533 5.6.6.3 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, one time period, No fading” |  |
| Inter_Reselection_low_mobility | 7.1.1.5 |  | “38.533 7.1.1.5 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 2 time periods, 1 AoA in Rx peak and rough beam” |  |
| CSI-RS_RLM_Out_of_Sync_02 | 5.5.1.7 |  | “38.533 5.5.1.7 TT v2.zip” | 1 NR FR2 Cell (1 E-UTRA cell), 2 CSI-RSs, 3 time periods, 1AoA beam peak directions and rough beam, fading. |  |
| CSI-RS_RLM_InSync_02 | 5.5.1.8 |  | “38.533 5.5.1.8 TT v2.zip” | 1 NR FR2 Cell (1 E-UTRA cell), 2 CSI-RSs, 5 time periods, 1AoA in beam peak directions and rough beam, fading. |  |
| SSB_WithNZP-IMR_L1-SINR-Meas | 5.6.6.2 |  | “38.533 5.6.6.2 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 2 time periods, No fading” |  |
| Inter_band_DAPS_HO | 7.3.1.4 7.3.1.5 |  | "38.533 7.3.1.4+7.3.1.5 TT.zip " | "1 NR FR2 Cell, 1 SSB, 5 time periods, 1 AoA in Rx peak and rough beam" |  |
| Intra_Freq_RRC_re-establishment_01 | 7.3.2.1.1 17.3.2.1.1 |  | “38.533 7.3.2.1.1+17.3.2.1.1 TT” | “2 Intra Frequency NR Cells, 3 time periods, No fading” |  |
| Inter_Freq_RRC_re-establishment_01 | 7.3.2.1.2 17.3.2.1.2 |  | “38.533 7.3.2.1.2+17.3.2.1.2 TT” | “2 Inter Frequency NR Cells, 3 time periods, No fading” |  |
| Intra_freq_HO | 7.3.1.2 |  | “38.533 7.3.1.2 TT.zip” | 2 NR FR2 intra-frequency Cells, 2 time periods, no fading |  |
| Inter_freq_HO | 7.3.1.3 |  | “38.533 7.3.1.3+7.3.2.3.1 TT.zip” | 2 NR FR2 inter-frequency Cells, 2 time periods, no fading |  |
| Inter_freq_redirection | 7.3.2.3.1 |  | “38.533 7.3.1.3+7.3.2.3.1 TT.zip” | 2 NR FR2 inter-frequency Cells, 2 time periods, no fading |  |
| Intra_freq_CHO | 7.3.3.1 |  | "38.533 7.3.3.1 TT.zip " | 2 NR FR2 Cells, 2 SSBs, 2 time periods, 1 AoA in Rx peak and rough beam |  |
| CSI-RS_RLM_Out_of_Sync_01 | 5.5.1.5 |  | “38.533 5.5.1.5 TT v2.zip” | 1 NR FR2 Cell (1 E-UTRA cell), 2 CSI-RSs, 3 time periods, 2AoA spherical coverage directions and rough beam, fading. |  |
| CSI-RS_RLM_InSync_01 | 5.5.1.6 |  | “38.533 5.5.1.6 TT_v2.zip” | 1 NR FR2 Cell (1 E-UTRA cell), 2 CSI-RSs, 5 time periods, 2AoA in spherical coverage directions and rough beam, fading. |  |
| SSB_RLM_Out_of_Sync_02 | 5.5.1.10 |  | “38.533 5.5.1.10 TT.zip” | 1 NR FR2 Cell (1 E-UTRA cell), 3 time periods, 1AoA in Rx peak and rough beam, fading. |  |
| CSI-RS_WithNZP_L1-SINR-Meas | 7.6.6.3 |  | “38.533 7.6.6.3 TT.zip” | “1 NR Cell, one time period, No fading” |  |
| L1-RSRP_Accuracy_1 | 5.7.4.1 7.7.4.1 |  | “38.533 5.7.4.1+7.7.4.1 TT.zip” | 1 NR FR2 Cell, 2 SSBs, 2 subtests, 1 AoA in Rx peak and rough beam |  |
| L1-RSRP_Accuracy_2 | 5.7.4.2 7.7.4.2 |  | “38.533 5.7.4.2+7.7.4.2 TT.zip” | 1 NR FR2 Cell, 2 CSI-RS, 2 subtests, 1 AoA in Rx peak and rough beam |  |
| SSB_WithCSI-IM_L1-SINR-Meas | 7.6.6.2 |  | “38.533 7.6.6.2 TT.zip” | “1 NR Cell, 2 time periods, No fading” |  |
| L1-SINR_Accuracy_1 | 5.7.6.1 7.7.6.1 |  | “38.533 5.7.6.1+7.7.6.1 TT.zip” | 1 NR FR2 Cell, 2 CSI-RSs, 1 subtest, 1 AoA in Rx peak and rough beam |  |
| L1-SINR_Accuracy_2 | 5.7.6.2 |  | “38.533 5.7.6.2 TT.zip” | 1 NR FR2 Cell, 2 SSB and 2 CSI-RS, 1 subtests, 1 AoA in Rx peak and rough beam |  |
| L1-SINR_Accuracy_3 | 5.7.6.3 |  | “38.533 5.7.6.3 TT.zip” | 1 NR FR2 Cell, 2 CSI-RS and 2 CSI-IM, 1 subtests, 1 AoA in Rx peak and rough beam |  |
| Inter_Reselection_not_at_cell_edge | 7.1.1.6 |  | “38.533 7.1.1.6 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 2 time periods, 1 AoA in Rx peak and rough beam” |  |
| SSB_WithCSI-IM_L1-SINR-Meas | 4.7.7.2 6.7.9.2 |  | “38.533 4.7.7.2+6.7.9.2 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |  |
| Intra_Reselection | 7.1.1.1 |  | “38.533 7.1.1.1 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 3 time periods, 1 AoA in Rx peak and rough beam” |  |
| Inter_Reselection | 7.1.1.2 |  | “38.533 7.1.1.2 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 3 time periods, 1 AoA in Rx peak and rough beam” |  |
| CSI-RS_Based_L1-SINR-Meas | 4.7.7.1.2 6.7.7.9.2 |  | “38.533 4.7.7.1.2+6.7.9.1.2 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |  |
| RRC_Based_BWP_Switch | 5.5.6.2.1 7.5.6.2.1 |  | “38.533 5.5.6.2.1+7.5.6.2.1 TT.zip” | “1 NR Cell (1 E-UTRA Cell for NSA case), one time period, No fading” |  |
| TCI_State_Switch | 7.5.8.1.1 7.5.8.2.1 | “38.533 7.5.8.1.1+7.5.8.2.1 TT.zip” |  |  | 1 NR FR2 Cell, 2 SSBs, 2 time periods, no fading, 2AoA in spherical coverage directions and rough beam |
| Intra_Freq_CLI_ SRS-RSRP_Meas | 5.6.4.1 7.6.4.1 |  | “38.533 5.6.4.1+7.6.4.1 TT.zip” | Cell 1 and Neighbor Cell UE on f1, T1 and T2. |  |
| Intra_Freq_CLI_ SRS-RSRP_Meas_Accu | 5.7.5.1 7.7.5.1 |  | “38.533 5.7.5.1+7.7.5.1 TT.zip” | Cell 1 and Neighbor Cell UE on f1, T1 and T2. |  |
| InterRAT_LTE_Idle_CADC_meas | 8.2.2.2 |  | “38.533 8.2.2.2 TT.zip” | “1 E-UTRA Cell, 1 NR Cell, 3 time periods, no fading” |  |
| UE_Rx_Tx_difference_PDC_02 | 7.6.13.1 7.6.13.2 |  | “38.533 7.6.13.1 TT.zip” | “1 NR FR2 Cell, 2 time periods, no fading” |  |
| RRC_reconfiguration_delay_01 | 7.5.11.1 |  | “38.533 7.5.11.1 TT.zip” | “2 NR Cells, 3 Time Periods, No Fading” |  |
| Intra_Reselection_not_at_cell_edge | 7.1.1.4 |  | “38.533 7.1.1.4 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 2 time periods, 1 AoA in Rx peak and rough beam” |  |
| Intra_Reselection_low_mobility | 7.1.1.3 |  | “38.533 7.1.1.3 TT.zip” | “2 NR FR2 Cells, 2 SSBs, 2 time periods, 1 AoA in Rx peak and rough beam” |  |
| RLM_Out_of_Sync_01 | 5.5.1.1 7.5.1.1 17.5.1.1 |  | 38.533 5.5.1.1+7.5.1.1+17.5.1.1 TT.zip | 1 NR FR2 Cell (1 E-UTRA cell), 2 SSBs, 3 time periods, 2AoA spherical coverage directions and rough beam, fading. |  |
| RLM_Out_of_Sync_02 | 5.5.1.3 7.5.1.3 |  | 38.533 5.5.1.3+7.5.1.3 TT.zip | 1 NR FR2 Cell (1 E-UTRA cell), 2 SSBs, 3 time periods, 1AoA in Rx beam peak and rough beam, fading. |  |
| RLM_InSync_01 | 5.5.1.2 7.5.1.2 17.5.1.2 |  | 38.533 5.5.1.2+7.5.1.2+17.5.1.2 TT_v2.zip | 1 NR FR2 Cell (1 E-UTRA cell), 2 SSBs, 5 time periods, 2AoA in spherical coverage directions and rough beam, fading. |  |
| RLM_InSync_02 | 5.5.1.4 7.5.1.4 17.5.1.4 |  | 38.533 5.5.1.4+7.5.1.4+17.5.1.4 TT.zip | 1 NR FR2 Cell (1 E-UTRA cell), 2 SSBs, 5 time periods, 1AoA in Rx beam peak direction and rough beam, fading. |  |
| RRC_reconfiguration_delay_01 | 7.5.11.1 |  | “38.533 7.5.11.1 TT.zip” | “2 NR Cells, 3 Time Periods, No Fading” |  |
| UE_Rx_Tx_difference_PDC_02 | 7.6.13.1 |  | “38.533 7.6.13.1 TT.zip” | “1 NR FR2 Cell, 2 time periods, no fading” |  |
| MAC-CE_Based_Uplink_Spatial_Switch | 5.5.9.1.1 7.5.9.1.1 |  | “38.533 5.5.9.1.1+7.5.9.1.1 TT.zip” | “1 NR Cell, 2 time periods, no fading” |  |
| RRC_Based_Uplink_Spatial_Switch | 5.5.9.2.1 7.5.9.2.1 |  | “38.533 5.5.9.2.1+7.5.9.2.1 TT.zip” | “1 NR Cell, 2 time periods, no fading” |  |

<End of Changes>

## Add the following attached .zip files to TR 38.903:

## See attachment “38.533 7.5.3.14 TT.zip”