3GPP TSG-RAN5 Meeting #101	R5-236179r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0534 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to condition of FR2 PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The M (Mandatory) columns are set to Yes ("Yes" in the column by "M" indicates the associated feature is mandatory) for PICS A.4.3.2-1/31a, A.4.3.6-1/41a, A.4.3.8-1/3 and A.4.3.8-1/8. A UE supporting only FR1 shall not fill these PICS to True as these PICS are all FR2 related. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set these PICS as optional for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In correct capabilities will be required for FR1 only UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.3.2, A.4.3.6 and A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: update value in the M column. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |
| ... |  |  |  |  |  |  |  |
| 31 | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR1 | No |  |  |
| 31a | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR2 | No |  |  |
| … |  |  |  |  |  |  |  |

[Skip unchanged text]

A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| … |  |  |  |  |  |  |  |
| 41 | Support CSI-RS based Radio Link Monitoring for FR1 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR1 | Yes |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| 41a | Support CSI-RS based Radio Link Monitoring for FR2 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR2 | No |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| … |  |  |  |  |  |  |  |

[Skip unchanged text]

A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item | UE Mobility Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support inter-RAT Handover to EUTRA connected to EPC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_EUTRA_Handover | Yes |  |  |
| 2 | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. | 38.306, 4.2.9 | Rel-15 | pc_handoverInterF | Yes |  |  |
| 3 | Support Handover between FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_FR1toFR2_Handover | No |  |  |
| 4 | Support Handover between FDD and TDD | 38.306, 4.2.9 | Rel-15 | pc_FDDtoTDD_Handover | Yes |  |  |
| 5 | Support inter-RAT Handover to E-UTRA connected to 5GC | 38.306, 4.2.9 | Rel-15 | pc_interRAT_eLTE_Handover | Yes |  |  |
| 6 | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC | 36.306, 4.3.34.9 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 | Yes |  |  |
| 7 | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC | 36.306, 4.3.34.8 | Rel-15 | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 | Yes |  |  |
| 8 | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC | 36.306, 4.3.34.11 | Rel-15 | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 | No |  |  |
| 9 | Support intra-frequency DAPS handover | 38.306, 4.2.7.5 | Rel-16 | pc_intraFreqDAPS_r16 | No |  | It is mandated if the UE supports asynchronous intra-frequency DAPS handover |
| … |  |  |  |  |  |  |  |

[End of change]