3GPP TSG-RAN5 Meeting #102	R5-240045

# Athens, Greece, February 26 – March 1 , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3000 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Common test environments for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Common test environments for ATG needs to be updated as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Common test environments for ATG has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.6, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GMM	5GS Mobility Management

5GS	5G System

5GSM	5GS Session Management

ATG	Air-To-Ground

EN-DC	E-UTRA-NR Dual Connectivity

MCG	Master Cell Group

MR-DC	Multi-RAT Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGC	NG Core Network. Synonym of 5GC.

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NG-RAN	NG Radio Access Network

NR	NR Radio Access

RedCap	Reduced Capability

RRC	Radio Resource Control

SCG	Secondary Cell Group

SS	System Simulator

<< Unchanged Parts Skipped >>

4.6.1	Contents of RRC messages

–	SIB1

Table 4.6.1-28: SIB1

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 dBm | RF OR RRM |
|  |  | -55 | -110 dBm | SIG AND FR1 |
|  |  | ROUND((-110+Delta(NRfs))/2) | -110+Delta(NRfs) | SIG AND FR2 |
| q-RxLevMinOffset |  | Not present |  |  |
| q-RxLevMinSUL |  | Not Present |  |  |
|  |  | -70 | -140 dBm | SUL AND (RF OR RRM) |
|  |  | -55 | -110 dBm | SUL AND SIG |
| q-QualMin |  | -20 | -20dB | QBASED |
|  |  | Not present |  |  |
| q-QualMinOffset |  | Not present |  |  |
| } |  |  |  |  |
| cellAccessRelatedInfo |  | CellAccessRelatedInfo |  |  |
| connEstFailureControl |  | ConnEstFailureControl |  |  |
| si-SchedulingInfo |  | Not present |  | NR_1 |
|  |  | SI-SchedulingInfo |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  |  |
|  |  | ServingCellConfigCommonSIB with condition PEI |  | PEI |
| ims-EmergencySupport |  | Not present |  |  |
|  |  | true | Indicates the cell supports IMS emergency bearer services for UEs in limited service mode. | SIG |
| eCallOverIMS-Support |  | Not present |  |  |
|  |  | true | Support of eCall over IMS services | eCalloverIMSforNR |
| ue-TimersAndConstants |  | UE-TimersAndConstants |  |  |
| uac-BarringInfo |  | Not present |  |  |
| useFullResumeID |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | EMR_EUTRA, EMR_NR, posSIB, pc_supportOfRedCap_r17, SDT |
| idleModeMeasurementsEUTRA-r16 |  | Not present |  |  |
|  |  | true |  | EMR_EUTRA |
| idleModeMeasurementsNR-r16 |  | Not present |  |  |
|  |  | true |  | EMR_NR |
| posSI-SchedulingInfo-r16 |  | PosSI-SchedulingInfo-r16 |  |  |
|  |  | PosSI-SchedulingInfo-r16 |  | posSIB |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| uac-BarringInfo-v1630 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| hsdn-Cell-r17 |  | Not present |  |  |
| uac-BarringInfo-v1700 |  | Not present |  |  |
| sdt-ConfigCommon-r17 |  | Not present |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  | SDT |
| sdt-RSRP-Threshold-r17 |  | 66 |  |  |
| sdt-LogicalChannelSR-DelayTimer-r17 |  | Not present |  |  |
| sdt-DataVolumeThreshold-r17 |  | byte1000 |  |  |
| t319a-r17 |  | ms1000 |  |  |
| } |  |  |  |  |
| redCap-ConfigCommon-r17 |  | Not present |  |  |
| redCap-ConfigCommon-r17 SEQUENCE { |  |  |  | pc_halfDuplexFDD_TypeA_RedCap_r17 AND FDD |
| halfDuplexRedCapAllowed-r17 |  | true |  |  |
| cellBarredRedCap-r17 |  | Not present |  |  |
| } |  |  |  |  |
| featurePriorities-r17 |  | Not present |  |  |
| si-SchedulingInfo-v1700 |  | Not present |  |  |
|  |  | SI-SchedulingInfo-v1700 |  | SIBs_r17 |
| hyperSFN-r17 |  | Not present |  |  |
| eDRX-AllowedIdle-r17 |  | Not present |  |  |
| eDRX-AllowedInactive-r17 |  | Not present |  |  |
| intraFreqReselectionRedCap-r17 |  | Not present |  |  |
|  |  | allowed |  | pc_supportOfRedCap_r17 |
| cellBarredNTN-r17 |  | Not present |  |  |
|  |  | notBarred |  | NTN |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| cellBarredATG-r18 |  | Not present |  |  |
|  |  | notBarred |  | ATG |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the clause 6.1.3.3. NRfs is NR frequency on which SIB1 is broadcasted. NOTE 2: ROUND is rounded off to the nearest integer. As an example, '1 to 1.49' set to '1' while '1.5 to 2' to '2' and '-2.0 to 1.5' set to '-2' while '-1.49 to -1' set to '-1'. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMinSUL. |
| QBASED | This condition applies to Quality based signalling test cases. |
| NR_1 | System information combination NR-1 according table 4.4.3.1.2-1 is applied. |
| SIG | Used for signalling test cases. |
| eCalloverIMSforNR | Used for eCall over IMS test cases (TS 38.523-1[12], TS 34.229-5[47]) |
| posSIB | For test cases using posSIBs in system information. |
| EMR_EUTRA | For E-UTRA idle/inactive measurement test cases. |
| EMR_NR | For NR idle/inactive measurement test cases. |
| SDT | For SDT test cases |
| NTN | For NTN test cases |
| PEI | Paging Early Indication is configured in the cell. |
| SIBs_r17 | For test cases using sibTypes defined in SIB-TypeInfo-v1700. |
| ATG | For ATG test cases |

<< Unchanged Parts Skipped >>

4.6.2	System information blocks

–	SIB21

Table 4.6.2-20: SIB21

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000000’H | entry 1 OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| mbs-FSAI-InterFreqList-r17 | Not present |  |  |
| mbs-FSAI-InterFreqList-r17 SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 { | 1 entry |  | MBS_interFreq |
| MBS-FSAI-InterFreq-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq-r17 | ARFCN-ValueNR with condition DL_SSB |  |  |
| mbs-FSAI-List-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| MBS_interFreq | SIB21 transmitted for a MBS inter-frequency cell environment (MBS service provided on multiple frequencies within a band). |

–	SIB22

Table 4.6.2-21: SIB22

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB22-r18 ::= SEQUENCE { |  |  |  |
| atg-Config-r18 | ATG-Config with condition ATG |  | ATG |
| hs-ATG-cellReselectionSet-r18 | ntn-Config-r17 |  |  |
| atg-NeighCellConfigList-r18 | ntn-Config-r17 |  |  |
| lateNonCriticalExtension | ntn-Config-r17 |  |  |
| } |  |  |  |

<< END OF CHANGES >>