3GPP TSG-RAN5 Meeting #102	R5-240049

# Athens, Greece, February 26 – March 1 , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2585 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of common parts for ATG UE RF test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The "References", "Definitions", "Symbols", "Abbreviations", "Specification suffix information" and "Operating band for ATG" Section needs to be added for NR ATG as per R4-2321919. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add "TS 38.104" into the "Reference" Section. - Add "PMaxOutputPower" into "Definitions" - Add "ATG" and "TAB" into "Abbreviations" - Update cl."Specification suffix information" for ATG - Introduce Operating band for ATG UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 3.3, 4.3, 5.2J (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: " NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: " NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment ".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[7]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[8]	3GPP TS 38.211: "NR; Physical channels and modulation".

[9]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[10]	3GPP TR 38.903: "NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance tests".

[11]	3GPP TR 38.905: "NR; Derivation of test points for radio transmission and reception conformance test cases".

[12]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[13]	3GPP TS 38.521-2: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”.

[14]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[15]	3GPP TS 38.521-4: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”.

[16]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

[17]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Applicability of RF and RRM test cases".

[18]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[19]	3GPP TS 38.133: "NR; Requirements for support of radio resource management ".

[20]	3GPP TS 38.215: "NR; Physical layer measurements".

[21]	3GPP TS 36.521-1: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing”.

[22]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain".

[23]	3GPP TS 38.307: "NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[24]	3GPP TS 37.213: “Physical layer procedures for shared spectrum channel access”.

[25]	3GPP TS 36.508: "Common test environments for User Equipment (UE)"

[26]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[27]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature as defined in clause 4.2.2. from TS38.306 [15].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS 38.306 [15].

Sub-band: For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands. Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Δ Frequency of default TX-RX separation of the FDD operating band

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(BWGB,Channel(k))

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.1 of carrier k

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC, low	The Fc of the lowest carrier, expressed in MHz

FC, high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset., low

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset., high

Fedge, low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the centre frequency of the interferer and the closest edge of the carrier measured)

Foffset	Frequency offset from FC, high to the higher edge or FC, low to the lower edge

Foffset,high	Frequency offset from FC,high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,Shift	RF reference frequency for Supplementary Uplink (SUL) bands and for the uplink for all FDD bands

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj∗2μj for carrier 1 to j, where μ is defined in TS 38.211 [8]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj∗2μj for carrier j, where μ is defined in TS 38.211 [8]

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRB,low	The transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	The transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, c	The configured maximum UE output power for serving cell c

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c in each slot

PEMAX	Maximum allowed UE output power signalled by higher layers

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c

PInterferer	Modulated mean power of the interferer

Plargest BW	Power of the largest transmission bandwidth configuration of the component carriers in the bandwidth combination

PMaxOutputPower 	The rated maximum ATG UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability [RatedMOPATG]

PPowerClass	The nominal UE power (i.e., no tolerance)

P-MPRc	Power Management Maximum Power Reduction for serving cell c

PRB	The transmitted power per allocated RB, measured in dBm

PUMAX	The measured configured maximum UE output power

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBStart	Indicates the lowest RB index of transmitted resource blocks

RBstart_CA	Indicates the lowest RB index of transmitted resource blocks for intra-band contiguous CA

SCSc	SCS for the component carrier c, expressed in kHz

SCSlargest BW	SCS for the largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in kHz

SCSlow	SCS for the lowest assigned component carrier in section 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in section 5.3A.1, expressed in kHz

tp	Transient Period value signalled by the UE

tpstart	Start position of transient period relative to the symbol boundary

T(PCMAX, f, c)	Tolerance for applicable values of PCMAX, f, c for configured maximum UE output power for carrier f of serving cell c

TL,c	Absolute value of the lower tolerance for the applicable operating band as specified in section 6.2.1

SSREF	SS block reference frequency position

UTRAACLR		UTRA ACLR

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

ATG	Air-To-Ground

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands

CC	Component Carriers

CG	Carrier Groups

CM	Cubic Metric

CORESET	Control Resource Set

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DC	Dual Connectivity

DCI	Downlink Control Information

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DL	Downlink

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

EIRP	Equivalent Isotropically Radiated Power

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FFT	Fast Fourier Transformation

FR	Frequency Range

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite System

GSCN	Global Synchronization Channel Number

HARQ	Hybrid Automatic Repeat Request

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

IE	Information Element

ITS	Intelligent Transportation System

ITU-R	Radio communication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MU	Measurement Uncertainty

NR	New Radio

NR/5GC	NR connected to 5GC

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

NSA	Non-Standalone

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

PBCH	Physical Broadcast Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

P-MPR	Power Management Maximum Power Reduction

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RE	Resource Element

RedCap	Reduced Capability

REFSENS	Reference Sensitivity

RF	Radio Frequency

RMC	Reference Measurement Channel

RMS	Root Mean Square (value)

RNTI	Radio Network Tempory Identity

RoT	Rise Over Thermal

RSRP	Reference Signal Receiving Power

Rx	Receiver

SA	Standalone

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SUL	Supplementary uplink

SS	Synchronization Symbol

SS	System Simulator

TAB 	Transceiver Array Boundary

TAE	Time Alignment Error

TAG	Timing Advance Group

TPC	Transmit Power Control

TT	Test Tolerances

Tx	Transmitter

TxD	Tx Diversity

UE	User Equipment

UL	Uplink

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

V2X	Vehicle to Everything

<< Unchanged Parts Skipped >>

4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level subclause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| I | RedCap |
| J | ATG |

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffixes A to J) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional subclause requirements (suffixes A to I) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional subclause.

A terminal which supports more than one feature in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

For a terminal that supports operation in shared spectrum, the current version of this specification assumes in the uplink sub-bands within a wideband channel shall be contiguously allocated to the UE. The uplink requirements for one or more non-transmitted sub-bands between two transmitted sub-bands does not form a part of the current version of this specification.

<< Unchanged Parts Skipped >>

5.2E	Operating band for V2X

5.2E.1	V2X operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 5.2E.1-1.

Table 5.2E.1-1: V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| NOTE 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |  |  |  |  |  |

5.2E.2	V2X operating bands for concurrent operation

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1: Inter-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |

5.2J	Operating band for ATG

NR is designed to support ATG where all of the operating bands are in FR1 defined in Table 5.2J-1.

Table 5.2J-1: NR operating bands for ATG in FR1

NR operating band

n1

n3

n34

n39

n41

n78

n79

5.3	UE channel bandwidth

5.3.1	General

<< END OF CHANGES >>