3GPP TSG-RAN5 Meeting #102	R5-240050

# Athens, Greece, February 26 – March 1 , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2586 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of General description for ATG UE Tx TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - General description for ATG UE Tx TCs needs to be added for NR ATG as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General description for ATG UE Tx TCs has been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1J (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR is related to R5-240049. The reference number [27] for TS 38.104 is introduced by R5-240049. - The names of two ATG UE capabilities are still pending in RAN4 spec. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.1F	General

For wideband operations, the minimum requirements for the transmitter characteristics are specified for transmissions on one scheduled RB set or ≥ 1 scheduled contiguous RB set(s) within the UE channel. The requirements apply with configured UL intra-cell guard bands of non-zero size according to Table 5.3.3-2, with the union of the scheduled RB sets and the intra-cell guard bands.

Unless stated otherwise, when a clause is not present for shared spectrum channel access, the general requirements and the additional clause requirements (suffices A, B, D) in clause 6 apply.

Table 6.F-1: Common uplink configuration for shared spectrum access

| Channel Bandwidth | SCS (kHz) | OFDM | RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Full | [Partial_Left]2 | [Partial_Riggt]3 |
| 20MHz | 15 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 11 | 11 |
|  | 30 | DFT-s | 50@0 | N/A1 | N/A1 |
|  |  | CP | 51@0 | 11 | 11 |
| 40MHz | 15 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 216@0 | 22 | 22 |
|  | 30 | DFT-s | 100@0 | N/A1 | N/A1 |
|  |  | CP | 106@0 | 22 | 22 |
| 60MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | TB |
|  | 30 | DFT-s | 162@0 | N/A1 | N/A1 |
|  |  | CP | 162@0 | 33 | 33 |
| 80MHz | 15 | DFT-s | N/A | N/A1 | N/A1 |
|  |  | CP | N/A | N/A | N/A |
|  | 30 | DFT-s | 216@0 | N/A1 | N/A1 |
|  |  | CP | 217@0 | 44 | 44 |
| Note 1: Partial RB allocation consists of non-contiguous RB allocation in one or several interlaces and therefore DFT-s-OFDM is not applicable. Note 2: A set of distributed RBs over the channel bandwidth for one Interlace only. The lowest RB index in the channel bandwidth is part of the distributed RB set. Note 3: A set of distributed RBs over the channel bandwidth for one Interlace only. The highest RB index in the channel bandwidth is part of the distributed RB set. |  |  |  |  |  |

6.1J	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antenna(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array. The definition about transceiver array boundary (TAB) is specified in clause 4.3.2 of TS 38.104 [27].

For the ATG UE with multiple omni-directional antennas indicating the capability [antennaArrayType-r18], the transmitter RF requirements are defined as the sum of measurement of all antenna connectors.

For the ATG UE with the antenna array indicating the capability [antennaArrayType-r18], the transmitter RF requirements are defined as the sum of measurement of all TAB connectors.

6.2	Transmitter power

6.2.1	UE maximum output power

<< Unchanged Parts Skipped >>

<< END OF CHANGES >>