3GPP TSG-RAN5 Meeting #102	R5-240099

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of reference measurement channels in Annex A.3.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Hefei) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference measurement channels of TC 8.3.1.1.2 are missing in current Annex A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference measurement channels of TC 8.3.1.1.2 in Annex A. (As it’s impossible to add new column with track change mark, delete the whole table and add new one.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference measurement channels of TC 8.3.1.1.2 remain missing in current Annex A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

A.3.12	Reference measurement channels for NPDSCH performance requirements

Editor’s note: The note 2 in Table A.3.12.1.2-1 and note 1 in Table A.3.12.2.1-1 are still under discussion with RAN4.

### A.3.12.1		Standalone

#### A.3.12.1.1 Single-antenna transmission

Table A.3.12.1.1-1: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for FDD

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB.1 FDD | R.NB.2 FDD | R.NB.6 FDD | R.NB.6-1 FDD |
| Carrier Type |  | Anchor | Non-anchor | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 616 | 392 | 616 | 392 |
| For Sub-Frame 0,5 | Bits | N/A | 392 | N/A | 392 |
| For Sub-Frame 4,9 | Bits | Note 2 | 392 | Note 2 | 392 |
| Number of Code Blocks |  |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 3 | 1 | Note 3 | 1 |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 4 | 320 | Note 4 | 320 |
| Max. Average Throughput | Bps | Note 5 | Note 5 | Note 5 | Note 5 |
| UE Category |  | NB1,NB2 | NB1,NB2 | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |  |  |

<<< END OF CHANGES 1>>>