3GPP TSG-RAN WG5 Meeting #102	R5-240103

Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0450 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TC 15.2.10 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The TT analysis for positioning test case 15.2.10 is still missing. - The test requirement are not finished and the configuration tables are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s Note related to TT analysis is removed. - The test requirement tables were added according to the latest TS 38.133 - The LPP response time calculation procedure was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning test case 15.2.10 remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

15.2.10	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

15.2.10.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in TS 38.133 [50] clause 9.9.4.5 in AWGN propagation condition in FR2 in standalone scenario when single positioning frequency layer is configured, and when UE is requested to measure a PRS resource with RxTx TEG.

15.2.10.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports Multi-RTT positioning and nr-UE-RxTxTEG-ID-MaxSupport-r17.

15.2.10.3	Minimum conformance requirements

Same as defined in clause 15.2.8.3.

15.2.10.4	Test description

15.2.10.4.1	Initial conditions

The supported test configuration is listed in Table 15.2.10.4-1.

Table 15.2.10.4-1: Test Configuration

| Test Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 15.2.10.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 15.2.10.5-1 and Table 15.2.10.5-2.

3.	Propagation conditions are set according to clause 4.14.2.

4.	Message contents are defined in clause 15.2.10.4.3.

5.	In the test there are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR2.

15.2.10.4.2	Test procedure

Same as defined in clause 15.2.8.4.2.

15.2.10.4.3	Message contents

Table 15.2.10.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 0 | Multi-RTT |  |

Table 15.2.10.4.3-2: RRCReconfiguration

| Derivation Path: TS 38.508-1 [45],, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { | CellGroupConfig |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config CHOICE { |  |  |  |
| setup | As defined in Table 15.2.10.4.3-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-3: SRS-Config (Table 15.2.10.4.3-2)

| Derivation Path: TS 38.508-1 [45], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList | Not present |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList | Not present |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 { | 1 entry |  |  |
| SRS-PosResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceSetId-r16 | 0 |  |  |
| srs-PosResourceIdList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16 { | 1 entry |  |  |
| SRS-PosResourceId-r16 | 0 | 1 entry |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha-r16 | alpha0 |  |  |
| p0-r16 | 0 |  |  |
| pathlossReferenceRS-Pos-r16 | Not present |  |  |
| } |  |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 { | 1 entry |  |  |
| SRS-PosResource-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 0 |  |  |
| nrofSymbols-r16 | n4 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | Matches NRB,c Table 15.2.10.5-1 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl1280 | 160 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-4: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-Multi-RTT-RequestCapabilities-r16 | TRUE |

Table 15.2.10.4.3-5: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 15.2.10.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 1 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-Multi-RTT-ReportConfig-r16 SEQUENCE { |  |  |  |
| maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 | Not present |  |  |
| timingReportingGranularityFactor-r16 | Not present |  |  |
| } |  |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTxTEG-Request-r17 | case1 |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 | Not present |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 | n0 |  |  |
| reducedDL-PRS-ProcessingSamples-r17 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-6: LPP ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r15 | Not present |  |  |
| tbs-ProvideAssistanceData-r15 | Not present |  |  |
| wlan-ProvideAssistanceData-r15 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData | As defined in Table 15.2.10.4.3-7 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-Multi-RTT-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present |  |  |
| assistanceDataValidityArea-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-7: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.2.10.4.3-8 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 154 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.2.10.4.3-8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-8: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n160-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n4-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.2.10.4.3-9: LPP ProvideLocation Information

| Derivation Path: 37.355 clause 6.5.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| nr-Multi-RTT-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16 { | 2 entries |  |  |
| NR-Multi-RTT-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 |  |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-UE-RxTxTimeDiff-r16 | Present. Any value acceptable. |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-Multi-RTT-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-RxTx-TEG-Info-r17 CHOICE { |  |  |  |
| case1-r17 SEQUENCE { |  |  |  |
| nr-UE-RxTx-TEG-ID-r17 | INTEGER (0..255) |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-Multi-RTT-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| NR-Multi-RTT-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 |  |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-UE-RxTxTimeDiff-r16 | Present. Any value acceptable. |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-Multi-RTT-AdditionalMeasurements-r16 |  |  |  |
| nr-UE-RxTx-TEG-Info-r17 CHOICE { |  |  |  |
| case1-r17 SEQUENCE { |  |  |  |
| nr-UE-RxTx-TEG-ID-r17 | INTEGER (0..255) |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |
| nr-Multi-RTT-AdditionalMeasurementsExt-r17 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-NTA-Offset-r16 |  |  |  |
| nr-SRS-TxTEG-Set-r17 |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |
| nr-UE-TxTEG-TimingErrorMargin-r17 |  |  |  |
| nr-UE-RxTxTEG-TimingErrorMargin-r17 |  |  |  |
| } |  |  |  |
| nr-Multi-RTT-Error-r16 |  |  |  |
| nr-Multi-RTT-SignalMeasurementInstances-r17 |  |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

15.2.10.5	Test requirement

Table 15.2.10.5-1 and Table 15.2.10.5-2 define the primary level settings including test tolerances for the test.

Table 15.2.10.5-1: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [49]. |
| Neighbour cell |  | 1 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [49]. |
| RF Channel Number |  | 1 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 | Synchronous cells |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 20 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table 15.2.10.5-2: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in TS 38.133 [50] clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |
| PRS muting info |  | 1 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.3 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -91 | -Infinity | -99 |
| Io | dBm/95.04 MHz | 1 | N/A | -57.63 | N/A | -57.63 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [50] B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [55], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [55] Table 6.2.1.3-4. |  |  |  |  |  |  |

The UE Rx-Tx time difference measurement period fulfils the requirements specified in clause 15.2.10.3.

The response time is equal to the LPP time IE value plus the test tolerance. The LPP time IE value is derived from the UE Rx-Tx time difference measurement period plus T, where T = 50ms. The UE Rx-Tx time difference measurement period follows the equation:

TUERxTx,i=kmultiTEG,i*CSSFi*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where CSSFPRS,i=1,Kp,PRS,i = 2, NRxBeam,i = 8, CSSFPRS,i=1, NPRS,islot=2, Lavailable_PRS,i=0.071,Nsample= 4. N is the parameter durationOfPRS-ProcessingSysmbols from TS 37.355 [49], N’ is the parameter maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355 [49], Tlast,i = Ti + Tavailable_PRS,i and Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i.

Tavailable_PRS,i= LCMTPRS,i,MGRPi ; where Tprs = 160 ms, and MGRP is 80 (for GP#24) or 40 (for GP#13) depending on UE capabilities. Therefore, Tavailable_PRS,i=160 ms.

Ti depends on the UE parameter durationOfPRS-ProcessingSymbolsInEveryTms from TS 37.355 [49]

Finally, it results in the following equation:

8*2N'0.071N*2*4-1*Teffect+Tlast

Where the remaining parameters depend on the UE capabilities. The LPP time IE ranges between 10.248s and 164s. The value of the LPP time IE is rounded up to the next second (if the value is >128s, it should be rounded up to the next multiple of ten seconds). The result is transmitted in the response time IE in the LPP-RequestLocationInformation in Table 15.2.10.4.3-5. The LPP time IE ranges between 11s and 170s.

The test tolerance for the response time is 300ms. Therefore, the response time ranges between 11.3s and 170.3s.

Table 15.2.10.5-3: value of N’ based on maxNumOfDL-PRS-ResProcessedPerSlot

| maxNumOfDL-PRS-ResProcessedPerSlot | 2N' |
| --- | --- |
| n1 | 2 |
| >=n2 | 1 |

Table 15.2.10.5-4: value of N based on durationOfPRS-ProcessingSysmbols

| durationOfPRS-ProcessingSysmbols | 0.071N |
| --- | --- |
| >= nDot125 | 1 |

Table 15.2.10.5-5: value of Teffect and Tlast based on durationOfPRS-ProcessingSymbolsInEveryTms

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.