3GPP TSG-RAN5 Meeting #102	R5-240232

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2612 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 3DL CA reference sensitivity exceptions TC for CA_n1A-n8A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CU Digital Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CA_n1A-n8A-n78A is still missing in the TC Reference sensitivity power level for 3DL CA exceptions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test configuration and test requirement for 3DL CA congifuration CA_n1A-n8A-n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CA_n1A-n8A-n78A for Reference sensitivity power level will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2_1.4.1, 7.3A.2_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.905 CR 0850 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

7.3A.2_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band NR 3DL CA with UL CA support.

7.3A.2_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2_1.4	Test description

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1945/ DL 2135 | n8 | UL 900/ DL 945 | n78 | DL 3745 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1940/ DL 2130 | n78 | 3380 | n8 | DL 940 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n41 | 2600 | n28 | UL 725/ DL 780 | n79 | 4600 | 10MHz | 5 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4480 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n41 | 2645 | n79 | 4850 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1_1.4.3.

7.3A.2_1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.1_1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.2_1.4.1-1on PCC and SCCs.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3A.2_1.5-1 for PC3 CA. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED and following exception:

For test points with “REFSENS_CA_3” UL configuration in table 7.3A.2_1.4.1-1, message exception in table 7.3A.1_1.4.3-1 applies.

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 15 | -100.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8+28.4+TT |  |  |  |  |  |  |
|  |  |  |  |  | -98.0+28.4+TT4 |  |  |  |  |  |  |
|  | 2 | n1 | 15 | -100.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  | -95.8+11.2+TT |  |  |  |  |  |  |
|  |  |  |  |  | -98.0+11.2+TT |  |  |  |  |  |  |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 15 | -100.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 | -97.0+27.9+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+27.9+TT4 |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8+TT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 15 | -100.0+TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 | -97.0 +TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -99.7 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8+14.9+TT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | -98.0+14.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 15 | -100.0+TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 | -97.0 +3.3+TT |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -99.7 +3.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  | -95.8+TT |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 15 | -98.0+15.6+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+15.6+TT4 |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+16.6+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+16.6+TT4 |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0 +3.8+TT |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 | -98.0+16.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+16.5+TT4 |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+16.0 +TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+16.0 +TT4 |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+32+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+32+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+12.1+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+12.1+TT4 |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 15 | -98.0+28.3+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+28.3+TT4 |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+29.4 +TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+29.4 +TT4 |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  | -95.3+8.9 +TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+8.9 +TT4 |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+29.2+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+29.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5+10.4+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+10.4+TT4 |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
|  | 3 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 | -99.5+4.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+4.0+TT4 |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 | -98.0+32.1+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+32.1+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
|  | 2 | n2 | 15 | -98.0+9.1+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+9.1+TT4 |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
|  | 3 | n2 | 15 | -98.0+2.1+TT |  |  |  |  |  |  |  |
|  |  |  |  | -100.7+2.1+TT4 |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+27.4+TT |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+27.4+TT4 |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |
|  | 2 | n41 | 15 | -94.8+15.9+TT |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+15.9+TT4 |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+26.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+26.0+TT4 |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+TT |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 15 | -97.0+26.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+26.0+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.8+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+15.9+ TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+15.9+ TT4 |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 15 | -97.0+17.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+17.0+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 | -98.5+15.3+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+15.3+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 | -99.0+3.6+TT |  |  |  |  |  |  |  |
|  |  |  |  | -101.2+3.6+TT4 |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+16.1+ TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+16.1+ TT4 |  |  |  |  |  |  |
|  | 2 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  | -95.3+8.2+ TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+8.2+ TT4 |  |  |  |  |  |  |
|  | 3 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  | -95.3+3.3+ TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+3.3+ TT4 |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+13.2+TT |  |  |  |  |  |  |  |
|  |  |  |  | -102.2+13.2TT4 |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+ TT |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |  |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 15 | -98.5+13+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+13+TT |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.9+TT |  |  |  |  |  |  |
|  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.9+TT |  |  |  |  |  |  |
|  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  | -89.6+10.1+TT |
|  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+10.1+TT |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 15 | -98.5+TT |  |  |  |  |  |  |  |
|  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.9+10.4+TT |  |  |  |  |  |  |
|  |  |  |  |  | -94.9-2.7+10.4+TT |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 5: 4 Rx operation is targeted for FWA form factor. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.2_1.5-1a: Reference sensitivity requirement for PC2 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0 +13.6+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 | -98.0+24.8+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+24.8+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+34.7+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+34.7+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 | -98.0+37.6+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+37.6+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+22.2+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+22.2TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+ TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>