3GPP TSG-RAN5 Meeting #102	R5-240310

# Athens , Greece, 26th Feb – 01st Mar. 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2625 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test frequency range for SUL band n83 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to NOTE 9 in table 5.3.5-1 for SUL band n83: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz For 30MHz test channel bandwidth, there’s no mid test frequency range defined. An exception note is needed for n83 to indicate that Low range test frequency will be select for 30MHz CBW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding exception note in test configuration table for SUL band n83. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some test points can't be performed for n83. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.1, 6.2C.3, 6.3C.1, 6.3C.3.1, 6.3C.3.2, 6.3C.4.1, 6.3C.4.3, 6.4C.1, 6.4C.2.1, 6.4C.2.2, 6.4C.2.3, 6.4C.2.4, 6.4C.2.5, 6.5C.1, 6.5C.3.1, 6.5C.3.2, 6.5C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2C.1	Configured transmitted power for SUL

6.2C.1.1	Test purpose

Same test purpose as in clause 6.2.4.1

6.2C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.1.3	Minimum conformance requirements

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier or SUL carrier, the configured transmit power requirements specified in subclause 6.2.4 are applicable for the UL carrier and the SUL carrier, respectively.

For the UE which supports SUL band combination, ΔTIB,c in Table 6.2C.2-1 applies.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.2C.

6.2C.1.4	Test description

Same test description as specified in clause 6.2.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1

Instead of table 6.2.4.4.1-1  use Table 6.2C.1.4-1

Table 6.2C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

<Unchanged Skipped>

6.2C.3	UE maximum output power for SUL

6.2C.3.1	Test purpose

Same test purpose as in clause 6.2.1.1

6.2C.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.2.1.

6.2C.3.4	Test description

Same test description as specified in clause 6.2.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.2.1.4.1-1  use Table 6.2C.3.4-1

Table 6.2C.3.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for Non-SUL carrier. Low, Mid, High range for SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  |  |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

<Unchanged Skipped>

6.3C.1	Minimum output power for SUL

6.3C.1.1	Test purpose

Same test purpose as in clause 6.3.1.1

6.3C.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.1.

6.3C.1.4	Test description

Same test description as specified in clause 6.3.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.1.4.1-1  use Table 6.3C.1.4-1

Table 6.3C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid-range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.3C.3.1	General transmit ON/OFF time mask for SUL

6.3C.3.1.1	Test purpose

Same test purpose as in clause 6.3.3.2.1

6.3C.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.1.

6.3C.3.1.4	Test description

Same test description as specified in clause 6.3.3.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.3.2.4.1-1  use Table 6.3C.3.1.4-1

Table 6.3C.3.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid-range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.3C.3.2	General transmit ON/OFF time mask for switching between two uplink carriers

6.3C.3.2.1	Test purpose

To verify that the time mask for switching between two uplink carriers meets the requirements given in 6.3C.3.0.2.

The time mask for switching between two uplink carriers defines the transient period(s) allowed between two uplink carriers for an uplink band pair of an a SUL configuration when the capability uplinkTxSwitchingPeriod is present

6.3C.3.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support SUL configuration and dynamic UL Tx switching.

6.3C.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3C.3.0.2.

6.3C.3.2.4	Test description

6.3C.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR SUL configuration specified in 5.5C. All of these configurations shall be tested with applicable test parameters for each SUL configuration, and are shown in table 6.3C.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3C.3.2.4.1-1: Test Configuration Table for SUL Tx switching

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for both carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Highest for SUL carrier and Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15kHz for SUL carrier, lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  | SUL Configuration |  |
| 1 | N/A | Modulation | RB allocation (NOTE 2) |  | Modulation | RB allocation (NOTE 2) |
|  |  | DFT-s-OFDM QPSK | Inner Full |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: SUL carrier is configured as Carrier 1 and Non-SUL carrier is configured as Carrier2. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |  |

<Unchanged Skipped>

6.3C.4.1	Absolute power tolerance for SUL

6.3C.4.1.1	Test purpose

Same test purpose as in clause 6.3.4.2.1

6.3C.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.2.

6.3C.4.1.4	Test description

Same test description as specified in clause 6.3.4.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.4.2.4.1-1  use Table 6.3C.4.1.4-1

Table 6.3C.4.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid-range for SUL and Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 2) |
| 1 | N/A | N/A | CP-OFDM QPSK | Outer_Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |

<Unchanged Skipped>

6.3C.4.3	Aggregate power tolerance for SUL

6.3C.4.3.1	Test purpose

Same test purpose as in clause 6.3.4.3.1

6.3C.4.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.3C.4.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.3.4.4.

6.3C.4.3.4	Test description

Same test description as specified in clause 6.3.4.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.3.4.4.4.1-1  use Table 6.3C.4.3.4-1

Instead of table 6.3.4.4.4.1-2  use Table 6.3C.4.3.4-2

Table 6.3C.4.3.4-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for SUL and Non-SUL carrier |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |
|  |  |  | PUCCH format = Format 1 Length in OFDM symbols = 14 |
| 1 | CP-OFDM QPSK Full RB allocation (NOTE 1) | N/A |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |

Table 6.3C.4.3.4-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for SUL and Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration | SUL Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | N/A | N/A | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |

<Unchanged Skipped>

6.4C.1	Frequency error for SUL

6.4C.1.1	Test purpose

Same test purpose as in clause 6.4.1.1

6.4C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.1.

6.4C.1.4	Test description

Same test description as specified in clause 6.4.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.1.4.1-1  use Table 6.4C.1.4-1

Table 6.4C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration |  | UL Configuration | SUL Configuration |  |
|  | Modulation | RB allocation | N/A | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | SUL REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 NOTE 2: SUL REFSENS refers to Table 7.3C.2.4.1-1a which defines SUL RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

<Unchanged Skipped>

6.4C.2.1	Error Vector Magnitude for SUL

6.4C.2.1.1	Test purpose

Same test purpose as in clause 6.4.2.1

6.4C.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.1.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.1.

6.4C.2.1.4	Test description

Same test description as specified in clause 6.4.2.1.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.1.4.1-1, table 6.4.2.1.4.1-2, table 6.4.2.1.4.1-3  use Table 6.4C.2.1.4-1, table 6.4C.2.1.4-2, table 6.4C.2.1.4-3

Table 6.4C.2.1.4-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 13 |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 23 |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| 4 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 5 |  |  |  | DFT-s-OFDM 16 QAM | Inner Full |
| 6 |  |  |  | DFT-s-OFDM 16 QAM | Outer Full |
| 7 |  |  |  | DFT-s-OFDM 64 QAM | Outer Full |
| 8 |  |  |  | DFT-s-OFDM 256 QAM | Outer Full |
| 9 |  |  |  | CP-OFDM QPSK | Inner Full |
| 10 |  |  |  | CP-OFDM QPSK | Outer Full |
| 11 |  |  |  | CP-OFDM 16 QAM | Inner Full |
| 12 |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 13 |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 14 |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.4C.2.2	Carrier leakage for SUL

6.4C.2.2.1	Test purpose

Same test purpose as in clause 6.4.2.2.1.

6.4C.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.2.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.2.

6.4C.2.2.4	Test description

Same test description as specified in clause 6.4.2.2.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.2.4.1-1  use Table 6.4C.2.2.4-1

Table 6.4C.2.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low, Mid, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: When the signalled DC carrier position is at Inner_1RB_Left, use Inner_1RB_Right for UL RB allocation. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.4C.2.3	In-band emissions for SUL

6.4C.2.3.1	Test purpose

Same test purpose as in clause 6.4.2.3.1.

6.4C.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.3.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.4.2.3.

6.4C.2.3.4	Test description

Same test description as specified in clause 6.4.2.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.3.4.1-1  use Table 6.4C.2.3.4-1

Instead of table 6.4.2.3.4.1-2  use Table 6.4C.2.3.4-2

Table 6.4C.2.3.4-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 3 |  |  |  | CP-OFDM QPSK | Inner_1RB_Left |
| 4 |  |  |  | CP-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.4C.2.4	EVM equalizer spectrum flatness for SUL

6.4C.2.4.1	Test purpose

Same test purpose as in clause 6.4.2.4.1.

6.4C.2.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.4C.2.4.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.4.2.4.

6.4C.2.4.4	Test description

Same test description as specified in clause 6.4.2.4.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

Instead of table 6.4.2.4.4.1-1  use Table 6.4C.2.4.4-1

Table 6.4C.2.4.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM QPSK | Outer Full |
| 2 |  |  |  | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.4C.2.5	EVM equalizer spectrum flatness for Pi/2 BPSK for SUL

6.4C.2.5.1	Test purpose

Same test purpose as in clause 6.4.2.5.1.

6.4C.2.5.2	Test applicability

This test applies to all types of NR UE release 16 and forward that support SUL operating on the SUL bands and indicate support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.

6.4C.2.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3 and 6.4.2.4.1.

6.4C.2.5.4	Test description

6.4C.2.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5C-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4C.2.5.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4C.2.5.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.5C.1	Occupied bandwidth for SUL

6.5C.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE supporting SUL are less than their specific limits when UE is configured using SUL transmission.

6.5C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3

6.5C.1.4	Test description

Same test description as specified in clause 6.5.1.4 with following exceptions:

-	Instead of Table 5.3.5-1  use Table 5.5C-1

-	Instead of Table 6.5.1.4.1-1 use Table 6.5C.1.4-1

Table 6.5C.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range by default for SUL carrier Mid Range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | All for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
| Test ID | N/A | N/A |  | Modulation | RB allocation |
| 1 |  |  |  | CP-OFDM QPSK | Outer_full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RB allocation is defined in Table 6.1-1. Note 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

<Unchanged Skipped>

6.5C.3.1	General spurious emissions for SUL

6.5C.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5C.3.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.1.3	Minimum conformance requirements

The general spurious emission requirement specified in clause 6.5.3.1.3 applies to the UE that support SUL operating on the SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] subclauses 6.5.3.1

6.5C.3.1.4	Test description

Same test description as specified in clause 6.5.3.1.4 with following exceptions:

-	Instead of Table 5.3.5-1  use Table 5.5C-1

-	Instead of Table 6.5.3.1.4.1-1 use Table 6.5C.3.1.4-1

Table 6.5C.3.1.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: Void NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.5C.3.2	Spurious emissions for UE co-existence for SUL

6.5C.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5C.3.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.2.3	Minimum conformance requirements

The requirements for NR bands for coexistence with protected bands specified in subclause 6.5.3.2.3 apply to the UE that support SUL operating on the SUL bands

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5.3.2.

6.5C.3.2.4	Test description

Same test description as specified in clause 6.5.3.2.4 with following exceptions:

-	Instead of Table 5.3.5-1  use Table 5.5C-1

-	Instead of Table 6.5.3.2.4.1-1 use Table 6.5C.3.2.4-1

Table 6.5C.3.2.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 1) |
| 1 |  |  |  | CP-OFDM QPSK | OuterFull |
| 2 |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration. NOTE 2: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 3: For NR band n83, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |  |  |

<Unchanged Skipped>

6.5C.4	Transmit intermodulation for SUL

6.5C.4.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5C.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.4.3	Minimum conformance requirements

The requirements in subclause 6.5.4 apply to the UE operating on SUL bands

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.4.

6.5C.4.4	Test description

Same test description as specified in clause 6.2.4.4 with following exceptions:

-	Instead of Table 5.3.5-1  use Table 5.5C-1

-	Instead of Table 6.5.4.4.1-1 use Table 6.5C.4.4-1

Table 6.5C.4.4-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range for both SUL carrier and Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for SUL carrier and Lowest supported SCS for Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
|  | N/A | N/A |  | Modulation | RB allocation (NOTE 2) |
| 1 |  |  |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  |  |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: For NR band n83, 30MHz test channel bandwidth is tested with Low range test frequency. |  |  |  |  |  |

<End of Changes>