3GPP TSG-RAN5 Meeting #102	R5-240424

# Athens, Greece, 26 February-1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1437 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS and test applicability for SENSE TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SENSE-UEContest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the all the conditions listed in 23.122 3.11 are fulfilled by IoT UE, SENSE(signal level enhanced network selection) shall apply to this UE. The condtion 2) indicated that “Operator controlled signal threshold per access technology” is an optional feature by UE. Therefore new PICS is need for it. [23.122 3.11] Signal level enhanced network selection applies only to NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA. An MS supporting any, or a combination, of NB-IoT, GERAN EC-GSM-IoT and Category M1 or M2 of E-UTRA shall apply signal level enhanced network selection if the following conditions are fulfilled: 1) The MS is in automatic PLMN selection mode; 2) The MS supports the "Operator controlled signal threshold per access technology" as specified in 3GPP TS 22.011 [19]; 3) The MS is configured for using signal level enhanced network selection as specified in 3GPP TS 24.368 [50]; 4) The MS is configured for using signal level enhanced network selection as specified in 3GPP TS 31.102 [40]; and 5) The "Operator controlled signal threshold per access technology" is configured in the USIM as specified in 3GPP TS 31.102 [40]. NOTE 1: The usage of the "Operator controlled signal threshold per access technology" is intended only for IoT stationary devices (see 3GPP TS 22.011 [19]). NOTE 2: "Operator controlled signal threshold per access technology" is not expected to be supported by non-IoT devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new PICS “pc_operator_controlled_signal_threshold_per_access_technology” - Add test applicability for SENSE TC 6.1.1.12, 6.1.1.13, 6.1.1.14, 22.2.14, 22.2.15,22.1.16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SENSE-UEContest will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4, 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 4-1: Applicability of tests and additional information for testing

| Clause | TC Title | Release | Applicability Condition | Applicability Comment | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Idle mode operations |  |  |  |  |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.1a | PLMN selection / Automatic mode / between FDD and TDD | Rel-8 | C142 | UEs supporting E-UTRA FDD and E-UTRA TDD |  |  |  |  |
| 6.1.1.1b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of TC 6.1.1.1 | pc_eFDD |  | Either TC 6.1.1.1 or TC 6.1.1.1b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.2a | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA This test is 'cells on single frequency only' equivalent of 6.1.1.2 | pc_eFDD |  | Either TC 6.1.1.2 or TC 6.1.1.2a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.3 | Cell reselection of ePLMN in manual mode | Rel-8 | C388 | UEs supporting E-UTRA and (( NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.3a | Cell reselection of ePLMN in manual mode / between FDD and TDD | Rel-9 (Note 3) | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |
| 6.1.1.3b | Cell reselection of ePLMN in manual mode / Single Frequency operation | Rel-8 | R | UEs supporting E-UTRA. This test is 'cells on single frequency only' equivalent of 6.1.1.3 | pc_eFDD |  | Either TC 6.1.1.3 or TC 6.1.1.3b shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.4 | PLMN selection in shared network environment / Automatic mode | Rel-8 | R | UEs supporting E-UTRA | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Between FDD and TDD | Rel-8 | C389 | UEs supporting E-UTRA FDD and E-UTRA TDD and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) |  |  |  |  |
| 6.1.1.5 | Void |  |  |  |  |  |  |  |
| 6.1.1.6 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection | Rel-8 | C157a | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.6a | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation | Rel-8 | C157 | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on single frequency only' equivalent of 6.1.1.6 | pc_eFDD |  | Either TC 6.1.1.6 or TC 6.1.1.6a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.6b | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation | Rel-13 | C157b | UEs supporting E-UTRA and user initiated PLMN reselection in automatic mode. This test is 'cells on two frequencies only' and 'TDD cat.1bis UE only' equivalent of 6.1.1.6 | pc_eTDD |  | Either TC 6.1.1.6 or TC 6.1.1.6b shall be executed. (Note 21) |  |
| 6.1.1.7 | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-10 | C179a | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search" and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.7a | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer / Single Frequency operation | Rel-10 | C179 | UEs supporting E-UTRA and MinimumPeriodicSearchTimer and not supporting "Fast First Higher Priority PLMN search". This test is 'cells on single frequency only' equivalent of 6.1.1.7 | pc_eFDD |  | Either TC 6.1.1.7 or TC 6.1.1.7a shall be executed. (Note 4) |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.8 | PLMN selection of RPLMN or (E)HPLMN; Automatic mode | Rel-8 | C212a | UEs supporting E-UTRA and EF_LRPLMSI_Exception and ((NOT Category M1) OR (Category M1 AND (intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED))) | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.9 | PLMN selection of RPLMN or (E)HPLMN; Manual mode | Rel-8 | C213 | UEs supporting E-UTRA and ManualModeNetworkSelectionException | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.10 | eMTC / NTN | Rel-17 | C414 | UEs supporting E-UTRA and Category M1 and NTN access in CE Mode A | pc_eFDD |  | Note 22 |  |
| 6.1.1.11 | eMTC / NTN / Multi-TAC | Rel-17 | C414 | UEs supporting E-UTRA and Category M1 and NTN access in CE Mode A | pc_eFDD |  | Note 22 |  |
| 6.1.1.12 | eMTC / SENSE/ PLMN selection of RPLMN, HPLMN, UPLMN, OPLMN and Other PLMN / Automatic mode | Rel-18 | CXXX | UEs supporting E-UTRA and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.13 | eMTC / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode | Rel-18 | CXXXX | UEs supporting E-UTRA and EF_LRPLMSI_Exception and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.1.14 | eMTC / SENSE/ Periodic attempts for signal level enhanced network selection; Automatic mode | Rel-18 | CXXX | UEs supporting E-UTRA and Category M1 and operator controlled signal threshold per access technology. | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |
| 6.1.2.1 | Void |  |  |  |  |  |  |  |
| 6.1.2.2 | Cell selection / Qrxlevmin | Rel-8 | C224c | UEs supporting E-UTRA and NOT Category M1 | pc_eFDD |  |  |  |
|  |  |  |  |  | pc_eTDD |  |  |  |

<Skip unchanged part>

| 22 | NB-IoT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22.1.1 | NB-IoT / Control Plane CIoT EPS optimisation for EPS services | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD, pc_NonIP_PDN, pc_IP_PDN, pc_NB_S1_only pc_NonIP_Link_MTU_Parameter pc_IPv4_Link_MTU_Parameter pc_APN_RateControl | px_DoAttachWithoutPDN, px_nonSMSTransport_CP_CIoT, px_SMSTransport_CP_CIoT, px_ModifyBearerResources | Note 18 Note 23 |  |
|  |  |  |  |  | pc_NB_TDD, pc_NonIP_PDN, pc_IP_PDN, pc_NB_S1_only pc_NonIP_Link_MTU_Parameter pc_IPv4_Link_MTU_Parameter pc_APN_RateControl | px_DoAttachWithoutPDN, px_nonSMSTransport_CP_CIoT, px_SMSTransport_CP_CIoT, px_ModifyBearerResources | Note 18 Note 23 |  |
| 22.1.2 | NB-IoT / NTN | Rel-17 | C412 | UEs supporting NB-IoT and NTN access in NB-IoT | pc_NB_FDD, pc_NonIP_PDN, pc_IP_PDN, pc_NB_S1_only pc_NonIP_Link_MTU_Parameter pc_IPv4_Link_MTU_Parameter pc_APN_RateControl pc_NB_ntn_Connectivity_EPC pc_NB_ntn_only_Connectivity_EPC pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport | px_DoAttachWithoutPDN, px_nonSMSTransport_CP_CIoT, px_SMSTransport_CP_CIoT, px_ModifyBearerResources | Note 18 Note 22 |  |
| 22.2.1 | NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.2 | NB-IoT / PLMN selection of RPLMN, HPLMN / EHPLMN, UPLMN and OPLMN / Manual mode | Rel-13 | C266a | UEs supporting NB-IoT and Manual Mode PLMN Selection exception | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.3 | NB-IoT / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.4 | NB-IoT / Cell selection / Qrxlevmin and Qqualmin / Serving cell becomes non-suitable (S<0 or barred or Srxlev > 0 and Squal < 0) | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.5 | NB-IoT / Intra-frequency Cell reselection / Qhyst, Qoffset, Treselection and Cell-specific reselection parameters | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.6 | NB-IoT / Cell reselection using cell status and cell reservations / Access control class 0 to 9 | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.7 | NB-IoT / Cell reselection using cell status and cell reservations / Access control class 11 to 15 | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.8 | NB-IoT / Cell reselection in shared network environment | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.9 | NB-IoT / Inter-frequency cell reselection | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.10 | NB-IoT / Cell reselection / MFBI | Rel-13 | C266 | UEs supporting NB-IoT | pc_NB_FDD |  | Note 23 |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.11 | Void |  |  |  |  |  |  |  |
| 22.2.12 | Void |  |  |  |  |  |  |  |
| 22.2.13 | NB-IoT / NTN / Multi-TAC | Rel-17 | C412 | UEs supporting NB-IoT and NTN access in NB-IoT | pc_NB_FDD, pc_NB_ntn_Connectivity_EPC pc_NB_ntn_only_Connectivity_EPC pc_NB_ntn_GSO_ScenarioSupport pc_NB_ntn_NGSO_ScenarioSupport |  | Note 22 |  |
| 22.2.14 | NB-IoT / SENSE/ PLMN selection of RPLMN, HPLMN, UPLMN, OPLMN and Other PLMN / Automatic mode | Rel-18 | CYYY | UEs supporting NB-IoT and operator controlled signal threshold per access technology | pc_NB_FDD |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.15 | NB-IoT / SENSE/ PLMN selection of RPLMN or (E)HPLMN / Automatic mode | Rel-18 | CYYYY | UEs supporting NB-IoT and operator controlled signal threshold per access technology and EF_LRPLMSI_Exception and | pc_NB_FDD |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |
| 22.2.16 | NB-IoT / SENSE/ Periodic attempts for signal level enhanced network selection/ Automatic mode | Rel-18 | CYYY | UEs supporting NB-IoT and operator controlled signal threshold per access technology | pc_NB_FDD |  |  |  |
|  |  |  |  |  | pc_NB_TDD |  |  |  |

<End of modified section 1>

<Start of modified section 2>

Table 4-1a: Applicability of tests Conditions

C01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01a IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND (A.4.5-2/3 OR A.4.5-2/4) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C01b IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/1 AND A.4.5-2/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C02 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 THEN R ELSE N/A

C02a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C03 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/1 THEN R ELSE N/A

C04 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 THEN R ELSE N/A

C05 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C06 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C07 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C08F IF A.4.1-1/1 AND A.4.5-1a/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C08aF IF A.4.1-1/1 AND A.4.5-1a/5 AND A.4.4-1/122 THEN R ELSE N/A

C08bF IF A.4.1-1/1 AND A.4.5-1a/5 THEN R ELSE N/A

C08T IF A.4.1-1/2 AND A.4.5-1b/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C08aT IF A.4.1-1/2 AND A.4.5-1b/5 AND A.4.4-1/122 THEN R ELSE N/A

C08bT IF A.4.1-1/2 AND A.4.5-1b/5 THEN R ELSE N/A

C09F IF (A.4.1-1/1 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A

C09T IF (A.4.1-1/2 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A

C10F IF A.4.1-1/1 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C10T IF A.4.1-1/2 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C11F IF (A.4.1-1/1 AND A.4.5-1a/16 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A

C11T IF (A.4.1-1/2 AND A.4.5-1b/16 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A

C12 IF ((A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1) OR ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15) THEN R ELSE N/A

C13F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/16 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C13T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/16 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C14F IF A.4.1-1/1 AND A.4.5-1a/5 AND A.4.5-1a/17 THEN R ELSE N/A

C14T IF A.4.1-1/2 AND A.4.5-1b/5 AND A.4.5-1b/17 THEN R ELSE N/A

C15F IF A.4.1-1/1 AND A.4.5-1a/3 AND A.4.5-1a/7 THEN R ELSE N/A

C15T IF A.4.1-1/2 AND A.4.5-1b/3 AND A.4.5-1b/7 THEN R ELSE N/A

C16F IF A.4.1-1/1 AND A.4.5-1a/7 THEN R ELSE N/A

C16aF IF A.4.1-1/1 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C16T IF A.4.1-1/2 AND A.4.5-1b/7 THEN R ELSE N/A

C16aT IF A.4.1-1/2 AND A.4.5-1b/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C17F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1a/22 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C17T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1b/22 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C18 IF (A.4.1-1/1 OR A.4.1-1/2) OR (A.4.4-1/122 AND A.4.4-1A/14) THEN R ELSE N/A

C19F IF A.4.1-1/1 AND A.4.5-1a/6 AND A.4.5-1a/7 AND NOT (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C19aF IF A.4.1-1/1 AND A.4.5-1a/6 AND A.4.5-1a/7 AND (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C19T IF A.4.1-1/2 AND A.4.5-1b/6 AND A.4.5-1b/7 AND NOT (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C19aT IF A.4.1-1/2 AND A.4.5-1b/6 AND A.4.5-1b/7 AND (A.4.3.2-2/1 OR A.4.3.2-1/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C20F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/16 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C20T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/16 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C21F IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C21T IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C21aF IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C21aT IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C22 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/3 AND A.4.4-2/2 AND NOT (A.4.4-2/32) THEN R ELSE N/A

C23 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/4 AND A.4.4-2/2 AND NOT (A.4.4-2/32) THEN R ELSE N/A

C24F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/16 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C24T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/16 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C25F IF A.4.1-1/1 AND A.4.1-1/4 AND A.4.5-1a/16 AND A.4.5-1a/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C25T IF A.4.1-1/2 AND A.4.1-1/4 AND A.4.5-1b/16 AND A.4.5-1b/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C26 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C27 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C28F IF (A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A

C28T IF (A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A

C29F IF A.4.1-1/1 AND A.4.5-1a/7 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C29T IF A.4.1-1/2 AND A.4.5-1b/7 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C30F IF A.4.1-1/1 AND A.4.5-1a/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C30T IF A.4.1-1/2 AND A.4.5-1b/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C31F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.5-1a/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C31T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.5-1b/20 AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C32F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.5-1a/20 THEN R ELSE N/A

C32T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.5-1b/20 THEN R ELSE N/A

C33F IF A.4.1-1/1 AND A.4.5-1a/20 THEN R ELSE N/A

C33T IF A.4.1-1/2 AND A.4.5-1b/20 THEN R ELSE N/A

C34 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/7 THEN R ELSE N/A

C35 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A

C36F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/8 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C36T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/8 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C37 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.5-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C38F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/10 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C38T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/10 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C39F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C39T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C40F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C40T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C41 Void

C42F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/12 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C42T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/12 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C44F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/5 AND A.4.5-1a/19 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C44T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/5 AND A.4.5-1b/19 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C45F IF (A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A

C45T IF (A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A

C46 IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.4-1/9 THEN R ELSE N/A

C47 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-2/1 THEN R ELSE N/A

C47a Void

C48 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C49 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/10 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C50 Void

C51 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/9 AND (A.4.4-1/12 OR A.4.4-1/13 OR A.4.4-1/14 OR A.4.4-1/15 OR A.4.4-1/93) THEN R ELSE N/A

C52 Void

C53 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.20/35 THEN R ELSE N/A

C54 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/18 THEN R ELSE N/A

C55 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/19 AND A.4.4-1/54 THEN R ELSE N/A

C56 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/2 OR A.4.3.2-1/3 OR A.4.3.2-1/4 OR A.4.3.2-1/5) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C57 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C58F IF A.4.1-1/1 AND A.4.5-1a/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C58T IF A.4.1-1/2 AND A.4.5-1b/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C59 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C60 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C61F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1a/16 AND A.4.5-1a/22 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C61T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.1-1/7 AND A.4.5-1b/16 AND A.4.5-1b/22 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C62 Void

C63 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1a/25 AND A.4.5-1a/30 AND A.4.5-1b/25 AND A.4.5-1b/30 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C64 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/20 THEN R ELSE N/A

C64a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/20 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C65 Void

C66 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.1/4 AND A.4.4-1/21 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C67 Void

C68 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C69 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/6 AND A.4.4-1/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C70 Void

C71 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 THEN R ELSE N/A

C71a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C71b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C72 Void

C73 Void

C74 Void

C75 Void

C76 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C77 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.5-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C78 Void

C79 Void

C80 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C80a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-1/49 AND A.4.4-1/103 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C81F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1a/8 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C81T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1b/8 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C82 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.5-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C83 Void

C84 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND [8]A.3/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C85 Void

C86 Void

C86a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C87 Void

C87a Void

C87b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C88 Void

C89 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C90F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C90T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C91F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C91T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C92F IF A.4.1-1/1 AND A.4.1-1/3 AND A.4.5-1a/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C92T IF A.4.1-1/2 AND A.4.1-1/3 AND A.4.5-1b/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C93F IF A.4.1-1/1 AND A.4.1-1/4 AND A.4.5-1a/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C93T IF A.4.1-1/2 AND A.4.1-1/4 AND A.4.5-1b/24 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C94 Void

C95 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/2 AND A.4.4-1/49 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C96F IF A.4.1-1/1 AND A.4.5-1a/10 AND A.4.4-2/2 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C96T IF A.4.1-1/2 AND A.4.5-1b/10 AND A.4.4-2/2 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C97 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 THEN R ELSE N/A

C97A IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 AND A.4.4-2/16 THEN R ELSE N/A

C98 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/18 AND A.4.4-1/30 THEN R ELSE N/A

C99F IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C99T IF A.4.1-1/2 AND A.4.4-1/51 AND A.4.5-1b/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C100F IF A.4.1-1/1 AND A.4.4-1/50 AND A.4.5-1a/7 THEN R ELSE N/A

C100T IF A.4.1-1/2 AND A.4.4-1/50 AND A.4.5-1b/7 THEN R ELSE N/A

C101 Void

C102 Void

C103 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/1 OR A.4.3.2-2/1) THEN R ELSE N/A

C104 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-1/31 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C105F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1a/8 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C105T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.2.1.1-1/1 AND A.4.4-2/2 AND A.4.5-1b/8 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C106 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/34 AND A.4.4-2/2 THEN R ELSE N/A

C107F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.4-1/52 AND A.4.5-1a/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C107T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.4-1/52 AND A.4.5-1b/23 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C108 Void

C109 Void

C109a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND (A.4.4-1/35 OR A.4.4-1/36) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C110F IF A.4.1-1/1 AND A.4.4-1/52 AND A.4.4-2/2 AND A.4.5-1a/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C110T IF A.4.1-1/2 AND A.4.4-1/52 AND A.4.4-2/2 AND A.4.5-1b/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C111F IF A.4.1-1/1 AND A.4.4-1/38 AND A.4.4-2/2 AND A.4.4-1/52 AND A.4.5-1a/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C111T IF A.4.1-1/2 AND A.4.4-1/38 AND A.4.4-2/2 AND A.4.4-1/52 AND A.4.5-1b/23 AND A.4.1-1/7 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C112F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/7 AND A.4.5-1a/8 AND A.4.5-1a/22 AND A.4.5-1a/27 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C112T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/7 AND A.4.5-1b/8 AND A.4.5-1b/22 AND A.4.5-1b/27 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C113 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 THEN R ELSE N/A

C113a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113bF IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113bT IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113cF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113cT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113dF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113dT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113e IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 THEN R ELSE N/A

C113gF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 AND A.4.3.3.3-2/2 THEN R ELSE N/A

C113gT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/5 AND A.4.2.1.1-1/7 AND A.4.3.3.3-2/2 THEN R ELSE N/A

C114 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/39 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C115 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C116 Void

C117F IF A.4.1-1/1 AND A.4.1-1/6 AND (([8]A.18a/14 AND [8]A.18a/18 AND [8]A.18a/22) OR ([8]A.18b/10 AND [8]A.18b/14)) AND A.4.5-1a/8 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C117T IF A.4.1-1/2 AND A.4.1-1/6 AND (([8]A.18a/14 AND [8]A.18a/18) OR ([8]A.18b/10 AND [8]A.18b/14)) AND A.4.5-1b/8 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C118F IF A.4.1-1/1 AND A.4.4-1/2 AND A.4.4-1/104 AND A.4.5-1a/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C118T IF A.4.1-1/2 AND A.4.4-1/2 AND A.4.4-1/104 AND A.4.5-1b/25 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C119F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/100 AND A.4.5-1a/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C119T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.4-1/2 AND A.4.4-1/100 AND A.4.5-1b/22 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C120F IF A.4.1-1/1 AND A.4.5-1a/7 AND A.4.4-1/40 AND A.4.4-1/41 THEN R ELSE N/A

C120T IF A.4.1-1/2 AND A.4.5-1b/7 AND A.4.4-1/40 AND A.4.4-1/41 THEN R ELSE N/A

C121 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C122 Void

C123 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND A.4.4-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C124 Void

C125 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND (A.4.4-2/5 OR (A.4.4-2/4 AND A.4.4-1/33)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C126 IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/56 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C127 IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.4-1/57 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C128 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND (A.4.1-1/6 OR A.4.1-1/7) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C129 IF A.4.1-1/1 AND A.4.4-1/58 THEN R ELSE N/A

C129a IF A.4.1-1/1 AND A.4.4-1/58 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C130 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.5-1a/25 AND A.4.5-1b/25 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C131 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND NOT A.4.4-1/57 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C132 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C132a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 THEN R ELSE N/A

C133 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-2/1 OR A.4.3.3.1-2/2) THEN R ELSE N/A

C134F IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-3a/11 THEN R ELSE N/A

C134T IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-3b/11 THEN R ELSE N/A

C134aF IF A.4.1-1/1 AND A.4.3.3.2-1/1 AND A.4.5-3a/11 THEN R ELSE N/A

C134aT IF A.4.1-1/2 AND A.4.3.3.2-1/1 AND A.4.5-3b/11 THEN R ELSE N/A

C135 Void

C136 Void

C137 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C138 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.4-1/62 AND A.4.5-2/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C139 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/27 or A.4.5-1b/27) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C140 Void

C141 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/2 AND A.4.4-2/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C142 IF A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A

C142a Void

C143 IF A.4.4-1/2 AND A.4.4-1/49 AND A.4.4-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C144F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C144T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C145 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/65 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C146 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C147 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C148F IF A.4.1-1/1 AND A.4.5-1a/23 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C148T IF A.4.1-1/2 AND A.4.5-1b/23 AND A.4.4-1/29 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C149 Void

C150 IF (((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6) OR ((A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.1-1/7)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C151 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 THEN R ELSE N/A

C152F IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-3a/11 THEN R ELSE N/A

C152T IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-3b/11 THEN R ELSE N/A

C153 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-2/2 AND A.4.4-1/26 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C154F IF A.4.1-1/1 AND A.4.5-3a/15 THEN R ELSE N/A

C154T IF A.4.1-1/2 AND A.4.5-3b/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155aF IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND A.4.3.3.3-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155aT IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND A.4.3.3.3-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155bF IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-3a/12 AND A.4.4-1/8 AND A.4.5-2/2 AND A.4.3.3.2-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C155bT IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-3b/12 AND A.4.4-1/53 AND A.4.5-2/2 AND A.4.3.3.2-1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C156 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/2 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C157 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/69 THEN R ELSE N/A

C157a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/69 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16))THEN R ELSE N/A

C157b IF A.4.1-1/2 AND A.4.4-1/69 AND A.4.3.2-2A/2 AND A.4.3.2-3A/2 THEN R ELSE N/A

C158 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/70 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C159F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C159T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C160F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/7 AND A.4.5-1a/8 AND A.4.5-1a/22 AND A.4.5-1a/27 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C160T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/7 AND A.4.5-1b/8 AND A.4.5-1b/22 AND A.4.5-1b/27 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C161F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C161T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C162 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/1 AND A.4.3.3.3-2/2 THEN R ELSE N/A

C163 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/29 AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C164 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/72 AND A.4.4-2/2 AND NOT A.4.4-2/32 THEN R ELSE N/A

C165 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/3 AND A.4.4-1/62 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C166F IF A.4.1-1/1 AND A.4.5-1a/14 THEN R ELSE N/A

C166T IF A.4.1-1/2 AND A.4.5-1b/14 THEN R ELSE N/A

C167F IF A.4.1-1/1 AND A.4.5-1a/14 AND A.4.5-1a/25 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C167T IF A.4.1-1/2 AND A.4.5-1b/14 AND A.4.5-1b/25 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C168F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C168T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/15 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C169 Void

C170 IF A.4.1-1/1 AND A.4.4-1/76 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C171 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/79 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C172 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND A.4.4-1/37 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C173 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/80 AND A.4.4-2/1 THEN R ELSE N/A

C174 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/81 THEN R ELSE N/A

C175 IF A.4.1-1/2 AND A.4.4-1A/2 THEN R ELSE N/A

C176 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND NOT A.4.3.2-1/1 THEN R ELSE N/A

C177 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND NOT A.4.3.2-1/1 THEN R ELSE N/A

C178 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 THEN R ELSE N/A

C179 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/84 AND NOT A.4.4-1/138 THEN R ELSE N/A

C179a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/84 AND NOT A.4.4-1/138 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16))THEN R ELSE N/A

C180 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C181 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/85 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C182 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [8]A.2/2 AND NOT A.4.2.1.1-1/4 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C183 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/33 OR A.4.4-1/145) THEN R ELSE N/A

C184 IF ((A.4.1-1/1 AND A.4.1-2/1) OR (A.4.1-1/2 AND A.4.1-2/2)) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C184a IF ((A.4.1-1/1 AND A.4.1-2/1) OR (A.4.1-1/2 AND A.4.1-2/2)) AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C185F IF A.4.1-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.1-2/1 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C185T IF A.4.1-1/2 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.1-2/2 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C186F IF (A.4.1-1/1 AND A.4.5-1a/25 AND A.4.1-2/1) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/25) THEN R ELSE N/A

C186T IF (A.4.1-1/2 AND A.4.5-1b/25 AND A.4.1-2/2) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/25) THEN R ELSE N/A

C187 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/86 THEN R ELSE N/A

C188 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/87 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C189F IF A.4.1-1/1 AND A.4.5-1a/31 THEN R ELSE N/A

C189T IF A.4.1-1/2 AND A.4.5-1b/31 THEN R ELSE N/A

C189aF IF A.4.1-1/1 AND A.4.5-1a/31 AND [8]A.1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C189aT IF A.4.1-1/2 AND A.4.5-1b/31 AND [8]A.1/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C189bF IF A.4.1-1/1 AND A.4.5-1a/31 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C189bT IF A.4.1-1/2 AND A.4.5-1b/31 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C189cF IF A.4.1-1/1 AND A.4.5-1a/31 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C189cT IF A.4.1-1/2 AND A.4.5-1b/31 AND A.4.1-1/6 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C190 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-2/1 OR A.4.3.3.1-2/2) AND A.4.4-1A/3 THEN R ELSE N/A

C191 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/1 AND A.4.4-1A/3 AND A.4.3.3.3-2/2 THEN R ELSE N/A

C192 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 AND A.4.3.3.2-2/1 AND A.4.4-1A/3 THEN R ELSE N/A

C193F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C193T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C194 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND A.4.4-1A/4 THEN R ELSE N/A

C195 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND [8]A.10/37 AND A.4.4-2/1 THEN R ELSE N/A

C196 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/19 AND A.4.4-1/54 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A

C197 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND [8]A.10/31 AND A.4.4-1/91 AND A.4.4-2/1 THEN R ELSE N/A

C198F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C198T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C199F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C199T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C200F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/7 AND A.4.5-1a/9 AND A.4.5-1a/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C200T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/7 AND A.4.5-1b/9 AND A.4.5-1b/23 AND A.4.4-1/32 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/34 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C201F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C201T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C202F IF A.4.1-1/1 AND A.4.1-1/6 AND A.4.5-1a/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C202T IF A.4.1-1/2 AND A.4.1-1/6 AND A.4.5-1b/27 AND A.4.4-1/33 AND A.4.4-1/71 AND [45]A.12/36 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C203 Void

C203a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/62 AND A.4.4-1/63 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C204 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/30 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A

C205 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND (A.4.4-1/8 OR A.4.4-1/53) AND A.4.4-1/94 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C206F IF A.4.1-1/1 AND A.4.1-1/7 AND A.4.5-1a/5 AND A.4.5-1d/2 AND A.4.5-1a/23 THEN R ELSE N/A

C206T IF A.4.1-1/2 AND A.4.1-1/7 AND A.4.5-1b/5 AND A.4.5-1e/2 AND A.4.5-1b/23 THEN R ELSE N/A

C207 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.2-1/1 AND A.4.3.3.2-2/1 THEN R ELSE N/A

C208 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C209 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND (A.4.4-2/14 OR A.4.4-2/15) THEN R ELSE N/A

C210 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND (A.4.4-2/11 OR A.4.4-2/13) AND NOT A.4.4-2/14 THEN R ELSE N/A

C211 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/33 AND A.4.4-2/14 THEN R ELSE N/A

C212 Void

C212a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/97 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C213 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/98 THEN R ELSE N/A

C214 Void

C215 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/99 THEN R ELSE N/A

C216F IF A.4.1-1/1 AND A.4.5-1a/4 AND A.4.5-1a/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C216T IF A.4.1-1/2 AND A.4.5-1b/4 AND A.4.5-1b/5 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C217 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C218 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND [45]A.12/41 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C219 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/33 AND [45]A.12/40 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C220 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/33 AND [45]A.12/40 AND [45]A.12/41 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C221 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.4-1/101 AND NOT A.4.4-1/102 THEN R ELSE N/A

C222 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.4-1/101 AND A.4.4-1/102 THEN R ELSE N/A

C223 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C224 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-2/1 THEN R ELSE N/A

C224a IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT (A.4.3.2-2/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C224b IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-2/1 OR A.4.3.2-2A/1) THEN R ELSE N/A

C224c IF (A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C224d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/183 THEN R ELSE N/A

C225 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/8 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C225a IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2 OR A.4.3.3.2-1/1 OR A.4.3.3.3-1/1) AND A.4.2.1.1-1/8 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C226 Void

C227 IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.4-1/107 AND A.4.5-1a/7 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C228 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/51 AND NOT A.4.3.2-2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C228a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/51 AND A.4.3.2-2/1 THEN R ELSE N/A

C229 Void

C229a IF A.4.1-1/1 AND NOT A.4.5-1a/31 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C230 Void

C230a IF A.4.1-1/2 AND NOT A.4.5-1b/31 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C231 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND A.4.4-1/32 AND A.4.2.1.1-1/4 AND (A.4.5-1a/9 or A.4.5-1b/9) AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C232 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND A.4.4-1/33 AND [45]A.12/40 AND A.4.4-1/30 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C233 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/2 AND A.4.3.3-2/2 AND (A.4.4-1/108 OR A.4.4-1/109) AND A.4.4-1A/3 THEN R ELSE N/A

C234 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.3.3-2/1 AND A.4.4-1/108 THEN R ELSE N/A

C234a IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/108 THEN R ELSE N/A

C235 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.3.3-2/1 AND A.4.4-1/109 THEN R ELSE N/A

C235a IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/109 THEN R ELSE N/A

C236 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/1 THEN R ELSE N/A

C237 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.3A/50 AND [45]A.4/2B AND [45]A.15/1 AND [45]A.15/3 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C238 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/110 THEN R ELSE N/A

C239 Void

C240 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/120 THEN R ELSE N/A

C241 Void

C242 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.3.3.3-2/2 THEN R ELSE N/A

C243 Void

C244 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/9 THEN R ELSE N/A

C245 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/10 THEN R ELSE N/A

C246 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/9 AND A.4.2.1.1-1/10 THEN R ELSE N/A

C247 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/115 THEN R ELSE N/A

C248 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/6 OR A.4.3.2-2/7 OR A.4.3.2-2/8 OR A.4.3.2-2/9 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/12 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/116 THEN R ELSE N/A

C249 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND A.4.4-1/33 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND [8]A.2/1 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C250 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.10/31 AND A.4.4-2/1 THEN R ELSE N/A

C251 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/118 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C252 VOID

C253 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 AND A.4.4-1/115 THEN R ELSE N/A

C254 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A

C254a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 THEN R ELSE N/A

C254b IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.4-1/122 OR A.4.4-1/123) AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C254c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 THEN R ELSE N/A

C254d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND NOT A.4.3.2-2A/3 THEN R ELSE N/A

C254e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-2A/3 THEN R ELSE N/A

C255 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 THEN R ELSE N/A

C255a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 AND NOT A.4.3.2-2A/3 THEN R ELSE N/A

C255b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/123 AND A.4.3.2-2A/3 THEN R ELSE N/A

C256 IF A.4.1-1/2 AND A.4.4-1/124 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C257 IF A.4.1-1/1 AND A.4.5-1a/31 AND A.4.4-1/125 AND A.4.3.3.3-1/1 THEN R ELSE N/A

C258 IF A.4.1-1/2 AND A.4.5-1b/31 AND A.4.4-1/125 AND A.4.3.3.3-1/1 THEN R ELSE N/A

C259 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 THEN R ELSE N/A

C259cF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259cT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259dF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259dT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259e IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.3.3-1/1 AND A.4.2.1.1-1/11 AND A.4.4-1/126 AND A.4.4-1/127 THEN R ELSE N/A

C259gF IF A.4.1-1/1 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A

C259gT IF A.4.1-1/2 AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A

C259hF IF A.4.1-1/1 AND A.4.3.3.3-1/1 AND A.4.5-1a/13 AND A.4.5-1a/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A

C259hT IF A.4.1-1/2 AND A.4.3.3.3-1/1 AND A.4.5-1b/13 AND A.4.5-1b/25 AND A.4.2.1.1-1/11 AND A.4.4-1/126 THEN R ELSE N/A

C260 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/128 THEN R ELSE N/A

C261 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND [8]A.10/31 AND A.4.4-2/1 THEN R ELSE N/A

C262 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 THEN R ELSE N/A

C263 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/121 AND A.4.2.1.1-1/4 THEN R ELSE N/A

C264 IF A.4.1-1/2 AND A.4.4-1/124 AND A.4.3.3.3-1/1 THEN R ELSE N/A

C265 IF A.4.1-1/2 AND A.4.4-1/124 AND A.4.3.3.3-2/1 THEN R ELSE N/A

C266 IF A.4.1-1/8 OR A.4.1-1/9 THEN R ELSE N/A

C266a IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/98 THEN R ELSE N/A

C267 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/12 THEN R ELSE N/A

C268 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/7 AND A.4.4-1A/8 THEN R ELSE N/A

C269 IF A.4.1-1/5 AND A.4.4-1/117 THEN R ELSE N/A

C270 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/131 THEN R ELSE N/A

C271 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/199 THEN R ELSE N/A

C272 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/99 THEN R ELSE N/A

C273 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/121 THEN R ELSE N/A

C274 IF (A.4.1-1/1 OR A.4.1-1/2 ) AND A.4.2.1.1-1/13 THEN R ELSE N/A

C275 IF (A.4.1-1/8 OR A.4.1-1/9) AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A

C276 Void

C277 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/30 AND [8]A.10/31 AND [8]A.10/37 THEN R ELSE N/A

C278 Void

C279 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/129 AND A.4.4-1/130 THEN R ELSE N/A

C280 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/129 THEN R ELSE N/A

C281 IF A.4.1-1/1 AND A.4.1-1/2 AND A.4.4-1/139 AND NOT A.4.3.2-2A/1 THEN R ELSE N/A

C282 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/140 THEN R ELSE N/A

C283 IF (A.4.1-1/1 OR A.4.1-1/2) AND [8]A.20/35 AND NOT A.4.4-1/25 THEN R ELSE N/A

C284 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 THEN R ELSE N/A

C285 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/132 THEN R ELSE N/A

C286 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1 AND A.4.4-1/2 AND A.4.4-2/1 THEN R ELSE N/A

C287 IF(A.4.1-1/1 OR A.4.1-1/2) AND NOT A.4.3.2-2A/1 AND A.4.1-1/6 AND A.4.4-2/2 AND A.4.2.1.1-1/1 AND A.4.4-2/5 THEN R ELSE N/A

C288 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1A/10 THEN R ELSE N/A

C289 Void

C290 IF (A.4.1-1/8 OR A.4.1-1/9) AND (A.4.4-1/132 OR A.4.4-1/144) THEN R ELSE N/A

C291 IF (A.4.1-1/8 OR A.4.1-1/9) AND (A.4.4-1/132 OR A.4.4-1/144) AND A.4.4-1/99 THEN R ELSE N/A

C292 Void

C293 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-2/24 AND A.4.4-1/19 THEN R ELSE N/A

C294 Void

C295 IF(A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/14 THEN R ELSE N/A

C296 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/5 OR A.4.3.2-1/6 OR A.4.3.2-1/7 OR A.4.3.2-1/9 OR A.4.3.2-1/10 OR A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/159 THEN R ELSE N/A

C297 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.2-1/11 OR A.4.3.2-1/12 OR A.4.3.2-2/8 OR A.4.3.2-2/10 OR A.4.3.2-2/11 OR A.4.3.2-2/13 OR A.4.3.2-2/14 OR A.4.3.2-2/15 OR A.4.3.2-2/16) AND A.4.4-1/159 AND A.4.4-1/116 THEN R ELSE N/A

C298 Void

C299 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/161 THEN R ELSE N/A

C300 Void

C301 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3-1/1 OR A.4.3.3-1/2) AND (A.4.3.3-2/1 OR A.4.3.3-2/2) AND A.4.4-1/163 THEN R ELSE N/A

C302 IF A.4.4-1/148 THEN R ELSE N/A

C303 IF A.4.4-1/148 AND A.4.4-1/153 THEN R ELSE N/A

C304 IF A.4.4-1/148 AND A.4.4-1/155 THEN R ELSE N/A

C305 IF A.4.4-1/148 AND A.4.4-1/156 THEN R ELSE N/A

C306 IF A.4.4-1/148 AND A.4.4-1/157 THEN R ELSE N/A

C307 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 THEN R ELSE N/A

C308 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/152 THEN R ELSE N/A

C309 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/153 THEN R ELSE N/A

C310 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/155 THEN R ELSE N/A

C311 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/156 THEN R ELSE N/A

C312 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/157 THEN R ELSE N/A

C313 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/164

C314 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

C314a IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A

C315 IF (A.4.1-1/1 OR A.4.1-1/2) AND [45]A.12/55 AND [8]A.10/16 AND [8]A.10/19 THEN R ELSE N/A

C316 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/54 AND [8]A.10/17 AND A.4.2.1.1-1/4 THEN R ELSE N/A

C317 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/6 OR A.4.1-1/7) AND [45]A.12/55 AND [8]A.10/17 THEN R ELSE N/A

C318 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/6 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

C319 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-1/7 AND [45]A.12/55 AND [8]A.10/16 THEN R ELSE N/A

C320 IF A.4.1-1/1 AND A.4.3.3-1/1 AND A.4.4-1/109 AND A.4.4-1/166 THEN R ELSE N/A

C321 IF A.4.1-1/2 AND A.4.3.3-1/1 AND A.4.4-1/166 THEN R ELSE N/A

C322 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/165 THEN R ELSE N/A

C323 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-1/8 OR A.4.1-1/9) THEN R ELSE N/A

C324 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/120 AND A.4.4-1/169 THEN R ELSE N/A

C325 IF A.4.4-1/173 THEN R ELSE N/A

C326 IF A.4.4-1/172 THEN R ELSE N/A

C327 IF A.4.4-1/170 OR A.4.4-1/171 THEN R ELSE N/A

C328 IF A.4.4-1/148 AND A.4.4-1/153 AND A.4.4-1/156 THEN R ELSE N/A

C329 Void

C330 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 THEN R ELSE N/A

C331 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/70 THEN R ELSE N/A

C332 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/176 THEN R ELSE N/A

C333 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/174 AND A.4.4-1/70 AND A.4.4-1/176 THEN R ELSE N/A

C334 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/161 THEN R ELSE N/A

C335 Void

C336 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/149 AND A.4.4-1/177 THEN R ELSE N/A

C337 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/5 AND A.4.4-1/149 THEN R ELSE N/A

C338 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 AND A.4.4-1/149 THEN R ELSE N/A

C339 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/167 AND A.4.3.2-1A/2 THEN R ELSE N/A

C340 Void

C341 Void

C342 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-2/27 AND A.4.4-2/31 THEN R ELSE N/A

C343 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/154 AND A.4.4-1/178 THEN R ELSE N/A

C344 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND NOT A.4.4-1/154 AND A.4.4-1/178 THEN R ELSE N/A

C345 IF A.4.4-1/148 AND A.4.4-1/178 THEN R ELSE N/A

C346 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/148 AND A.4.4-1/178 THEN R ELSE N/A

C347 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.3.2-1A/2 THEN R ELSE N/A

C348 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1A/11 THEN R ELSE N/A

C349 IF A.4.1-1/8 AND A.4.4-1A/12 THEN R ELSE N/A

C350 IF A.4.1-1/8 AND A.4.2.1.1-1/15 THEN R ELSE N/A

C351 IF A.4.1-1/8 AND A.4.2.1.1-1/11 AND (A.4.5-1a/3 or A.4.5-1b/3) AND (A.4.5-1a/7 or A.4.5-1b/7) THEN R ELSE N/A

C352 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/179 THEN R ELSE N/A

C353 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/179 AND A.4.4-1/180 THEN R ELSE N/A

C354 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/11 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A

C355 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/181 THEN R ELSE N/A

C356 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/182 THEN R ELSE N/A

C357 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/33 THEN R ELSE N/A

C358 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/184 THEN R ELSE N/A

C359 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/185 THEN R ELSE N/A

C360 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/186 THEN R ELSE N/A

C361 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/187 THEN R ELSE N/A

C362 IF A.4.1-1/1 AND A.4.5-1a/7 OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1a/7) THEN R ELSE N/A

C363 IF A.4.1-1/2 AND A.4.5-1b/7 OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.4-1A/15 AND A.4.5-1b/7) THEN R ELSE N/A

C364 IF (A.4.1-1/1 AND A.4.5-1a/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.5-1a/25) THEN R ELSE N/A

C365 IF (A.4.1-1/2 AND A.4.5-1b/25) OR (A.4.4-1/122 AND A.4.4-1A/14 AND A.4.5-1b/25) THEN R ELSE N/A

C366 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.2.1.1-1/4 AND [8] A.20/90 THEN R ELSE N/A

C367 IF A.4.1-1/1 AND A.4.4-1/122 AND A.4.4-1/188 THEN R ELSE N/A

C368 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/189 AND A.4.4-1/190 THEN R ELSE N/A

C369 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/191 THEN R ELSE N/A

C370 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/192 THEN R ELSE N/A

C371 Void

C372 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/195 THEN R ELSE N/A

C373 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND (A.4.4-1/196 OR A.4.4-1/197) THEN R ELSE N/A

C374 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.4-1/197 THEN R ELSE N/A

C375 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.3.3.1-1/1 OR A.4.3.3.1-1/2) AND A.4.4-1/198 THEN R ELSE N/A

C376 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 AND A.4.4-1/200 THEN R ELSE N/A

C377 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/202 AND (A.4.4-1/132 OR A.4.4-1/144) THEN R ELSE N/A

C378 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 THEN R ELSE N/A

C379 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 AND NOT A.4.4-1/206 THEN R ELSE N/A

C379a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/205 THEN R ELSE N/A

C380 IF A.4.1-1/1 AND A.4.4-1/203 AND A.4.4-1/206 THEN R ELSE N/A

C381 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/205 AND A.4.4-1/207 THEN R ELSE N/A

C382 IF A.4.1-1/2 AND A.4.4-1/203 AND A.4.4-1/205 AND A.4.4-1/208 THEN R ELSE N/A

C383 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/203 AND A.4.4-1/204 AND A.4.4-1/205 AND A.4.4-1/209 THEN R ELSE N/A

C384 IF A.4.1-1/1 AND (A.4.4-1/122 OR A.4.4-1/123) AND A.4.4-1/210 THEN R ELSE N/A

C385 IF A.4.1-1/1 AND (A.4.4-1/122 OR A.4.4-1/123) AND A.4.4-1/121 AND A.4.4-1/210 THEN R ELSE N/A

C386 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1A/4 AND A.4.4-2/1 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A

C387 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/143 AND A.4.4-1/201 THEN R ELSE N/A

C388 IF (A.4.1-1/1 OR A.4.1-1/2) AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C389 IF A.4.1-1/1 AND A.4.1-1/2 AND (NOT A.4.3.2-2A/1 OR (A.4.3.2-2A/1 AND A.4.4-1A/16)) THEN R ELSE N/A

C390 IF A.4.1-1/8 AND A.4.4-1/210 THEN R ELSE N/A

C391 IF A.4.1-1/8 AND A.4.4-1/121 AND A.4.4-1/210 THEN R ELSE N/A

C392 IF A.4.1-1/8 AND A.4.4-1/212 THEN R ELSE N/A

C393 IF A.4.1-1/1 AND A.4.4-1/51 AND A.4.5-1a/7 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A

C394 IF A.4.1-1/2 AND A.4.4-1/51 AND A.4.5-1b/7 AND (A.4.4-1/122 OR A.4.4-1/123) THEN R ELSE N/A

C395 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.5-1a/7 AND A.4.4-1/213 THEN R ELSE N/A

C396 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/199 AND (A.4.4-1/41 OR A.4.4-1/42 OR A.4.4-1/43 OR A.4.4-1/44 OR A.4.4-1/46 OR A.4.4-1/47 OR A.4.4-1/48) THEN R ELSE N/A

C397 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/1 THEN R ELSE N/A

C398 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/214 THEN R ELSE N/A

C399 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/217 THEN R ELSE N/A

C400 IF A.4.1-1/8 AND A.4.4-1/218 THEN R ELSE N/A

C401 Void

C402 IF A.4.1-1/8 AND A.4.4-1/219THEN R ELSE N/A

C403 IF A.4.1-1/9 AND A.4.4-1A/17 THEN R ELSE N/A

C404 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/220 THEN R ELSE N/A

C405 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/216

C406 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/222 THEN R ELSE N/A

C407 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/122 AND A.4.4-1/223 THEN R ELSE N/A

C408 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/215 THEN R ELSE N/A

C409 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/3 THEN R ELSE N/A

C410 IF (A.4.1-1/1 OR A.4.1-1/2) AND [56] A.4.1-1/4 THEN R ELSE N/A

C411 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/224 THEN R ELSE N/A

C412 IF A.4.4-1/230 OR A.4.4-1/240 THEN R ELSE N/A

C412a Void

C413 IF (A.4.4-1/230 OR A.4.4-1/240) AND A.4.4-1/231 AND A.4.4-1/233 THEN R ELSE N/A

C414 IF A.4.3.2-2A/1 AND (A.4.4-1/242 OR A.4.4-1/241) THEN R ELSE N/A

C414a Void

C415 IF A.4.3.2-2A/1 AND (A.4.4-1/242 OR A.4.4-1/241) AND A.4.4-1/243 AND A.4.4-1/245 THEN R ELSE N/A

C416 IF A.4.4-1/25 AND A.4.4-1/235 AND A.4.4-1/117 THEN R ELSE N/A

C417 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/225 THEN R ELSE N/A

C418 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-2/1 AND A.4.4-1/227 THEN R ELSE N/A

C419 Void

C420 IF A.4.4-1/117 AND A.4.4-1/239 AND A.4.4-1/25 THEN R ELSE N/A

C421 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/248 THEN R ELSE N/A

C422 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/249 THEN R ELSE N/A

C423 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/115 THEN R ELSE N/A

C424 IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/115 AND A.4.4-1/121 THEN R ELSE N/A

CXXX IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.2-2A/1 AND A.4.4-1/ZZZ THEN R ELSE N/A

CXXXX IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.4-1/97 AND A.4.3.2-2A/1 AND A.4.4-1/ZZZ THEN R ELSE N/A

CYYY IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/ZZZ THEN R ELSE N/A

CYYYY IF (A.4.1-1/8 OR A.4.1-1/9) AND A.4.4-1/ZZZ AND A.4.4-1/97 THEN R ELSE N/A

<End of modified section 2>

<Start of modified section 3>

A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of USIM removal without power down |  | Rel-8 | pc_USIM_Removal |  |
| 2 | Support of Allowed CSG list | 36.331 Annex B.2 | Rel-8 | pc_Allowed_CSG_list | For Rel-8: CSG autonomous search is optional. For Rel-9 or later releases: CSG autonomous search is mandatory for UEs supporting CSG full functionality. |
| 3 | Support of Short Message Service (SMS) MT over SGs | 23.272, 8.2.4, 8.2.5 | Rel-8 | pc_SMS_SGs_MT |  |
| 4 | Support of Short Message Service (SMS) MO over SGs | 23.272, 8.2.2, 8.2.3 | Rel-8 | pc_SMS_SGs_MO |  |
| 5 | Support of ISR | 23.401, 4.3.5.6 | Rel-8 | pc_ISR |  |
| 6 | Support of Mobility management based on Dual-Stack Mobile IPv6 | 24.303 | Rel-8 | pc_DSMIPv6 |  |
| 7 | Support for being configured to discover the Home Agent address via DNS | 24.303 | Rel-8 | pc_HAAddress_via_DNS |  |
| 8 | Support of inter-RAT PS handover to E-UTRA (FDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eFDD |  |
| 9 | Support of EMM information message | 24.301, 5.4.5.3 | Rel-8 | pc_EMM_Information |  |
| 10 | Support for being configured to discover the Home Agent address via DHCPv6 | 24.303 | Rel-8 | pc_HAAddress_via_DHCPv6 |  |
| 11 | Void |  |  |  |  |
| 12 | Upon reception of ‘Full name for network’ information the UE stores/updates the network full name | 24.301, 8.2.13 | Rel-8 | pc_FullNameNetwork |  |
| 13 | Upon reception of ‘Short name for network’ information the UE stores/updates the network short name | 24.301, 8.2.13 | Rel-8 | pc_ShortNameNetwork |  |
| 14 | Upon reception of ‘Local time zone’ information the UE stores/updates the local time zone | 24.301, 8.2.13 | Rel-8 | pc_LocalTimeZone |  |
| 15 | Upon reception of ‘Universal time and local time zone’ information the UE stores/updates the universal time and local time zone | 24.301, 8.2.13 | Rel-8 | pc_UniversalAndLocalTimeZone |  |
| 16 | Void |  |  |  |  |
| 17 | Void |  |  |  |  |
| 18 | Support of ESM UE requested bearer resource allocation procedure | 24.301, 6.5.3 | Rel-8 | pc_ESM_MO_Bearer_Allocation |  |
| 19 | Support of ESM UE requested bearer resource modification procedure | 24.301, 6.5.4 | Rel-8 | pc_ESM_MO_Bearer_Modification |  |
| 20 | Support of ETWS message | 23.401, 5.12.2 | Rel-8 | pc_ETWS_message |  |
| 21 | Supports E-UTRAN Neighbour Cell measurements and MS autonomous cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_meas |  |
| 22 | Support for being configured to request the IPv6 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv6HAAddress_DuringAttach |  |
| 23 | Support for being configured to request the IPv4 address of the Home Agent during Attach procedure | 24.303 | Rel-8 | pc_RequestIPv4HAAddress_DuringAttach |  |
| 24 | Void |  |  |  |  |
| 25 | Support of IMS | 24.229 | Rel-8 | pc_IMS |  |
| 26 | Supports of disabling the EPS services | 24.301, 3.1, 5.5.2.1 | Rel-8 | pc_EPS_Services_Disable |  |
| 27 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach with detach type set to "re-attach required" | 24.301, 5.5.2.3.2 | Rel-8 | pc_Automatic_Re_Attach |  |
| 28 | Support of Compressed mode | 25.306 | Rel-8 | pc_UTRA_CompressedModeRequired |  |
| 29 | Support of GERAN to E-UTRAN PS Handover | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_PSHO |  |
| 30 | Support for multiple PDN connections | 23.401, 5.10 | Rel-8 | pc_Multiple_PDN |  |
| 31 | Support of use of the UTRA system information provided by RRCConnectionRelease upon redirection | 36.306 | Rel-9 | pc_eRedirectionUTRA |  |
| 32 | Support for SRVCC from E-UTRAN to GERAN/UTRAN | 24.301, 8.2.4 | Rel-8 | pc_SRVCC_GERAN_UTRAN |  |
| 33 | Support for VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS" | 24.173, 24.229, 26.114, 5.2.1, GSMA PRD IR.92 | Rel-8 | pc_VoLTE | Multimedia telephony service participant initiating a speech session. UE supports sending DTMF events over RTP. |
| 34 | Support of detach for non-EPS services | 24.301, 5.5.2.1 | Rel-8 | pc_IMSI_Detach |  |
| 35 | Support for establishing the emergency call using the CS domain in UTRA after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_UTRA |  |
| 36 | Support for establishing the emergency call using the CS domain in GERAN after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_GERAN |  |
| 37 | Support for establishing the emergency call using the CS domain in 1xRTT after ATTACH REJECT to emergency bearer service | 24.301, 5.5.1.2.5A | Rel-9 | pc_CS_Em_Call_in_1xRTT |  |
| 38 | Support for EDTM | 44.060 8.9.1.2 | Rel-8 | pc_EDTM |  |
| 39 | Supports CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN | 24.008, 10.5.5.12a | Rel-8 | pc_GERAN_2_E_UTRAN_measreporting_CCN |  |
| 40 | Support for ROHC profile0x0001 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0001 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 41 | Support for ROHC profile0x0002 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0002 | 'IMS capable UEs supporting voice' shall set this PICS to true. |
| 42 | Support for ROHC profile0x0003 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0003 |  |
| 43 | Support for ROHC profile0x0004 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0004 |  |
| 44 | Support for ROHC profile0x0006 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0006 |  |
| 45 | Support for ROHC profile0x0101 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0101 |  |
| 46 | Support for ROHC profile0x0102 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0102 |  |
| 47 | Support for ROHC profile0x0103 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0103 |  |
| 48 | Support for ROHC profile0x0104 | 36.306, 4.3.1.1 | Rel-8 | pc_ROHC_profile0x0104 |  |
| 49 | Support of manual CSG selection | 36.331, Annex B2 | Rel-8 | pc_Manual_CSG_Selection | For Rel-8: manual CSG selection is optional. For Rel-9 or later releases: manual CSG selection is mandatory for UEs supporting CSG full functionality. |
| 50 | Support of semi-persistence scheduling | 36.331, Annex B1 | Rel-8 | pc_Semi_Persistence_Scheduling | For Rel-8: semi-persistence scheduling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases: semi-persistence scheduling is mandatory if pc_FeatrGrp_29 is set to true. |
| 51 | Support of TTI bundling | 36.331, Annex B1 | Rel-8 | pc_TTI_Bundling | For Rel-8: TTI bundling is mandatory if pc_FeatrGrp_3 is set to true. For Rel-9 or later releases TDD: TTI bundling is mandatory if pc_FeatrGrp_28 is set to true. For Rel-9 or later releases FDD: TTI bundling is mandatory. |
| 52 | Support for inter-RAT PS handover from E-UTRAN to GERAN. | 36.306, 4.3.7.11 | Rel-8 | pc_E_UTRAN_2_GERAN_PSHO |  |
| 53 | Support of inter-RAT PS handover to E-UTRA (TDD) from UTRA | 25.306, 4.7 | Rel-8 | pc_HO_from_UTRA_to_eTDD |  |
| 54 | Support for UE requested modification of network allocated TFTs | 24.301, 6.5.4 | Rel-8 | pc_ESM_UE_Modification_NW_TFT |  |
| 55 | Support of automatic re-activation of the EPS bearer(s) during Network Initiated Detach even though UE has initiated a detach procedure with detach type set to "EPS detach" or "combined EPS/IMSI detach" | 24.301, 5.5.2.2.4 | Rel-8 | pc_Re_Attach_AfterDetachColl |  |
| 56 | Support of Squal based cell reselection to UTRAN from E-UTRAN | 25.304, 5.2.6.1.4a | Rel-9 | pc_Squal_based_CellReselection_to_UTRAN_from_E_UTRAN |  |
| 57 | Support of Squal based cell reselection to E-UTRAN from UTRAN | 36.304, 5.2.4.5 | Rel-9 | pc_Squal_based_CellReselection_to_E_UTRAN_from_UTRAN |  |
| 58 | Support of CMAS message | 36.331, 5.2.1.5 | Rel-9 | pc_CMAS_Message |  |
| 59 | Void |  |  |  |  |
| 60 | Void |  |  |  |  |
| 61 | Void |  |  |  |  |
| 62 | Support of logged measurements in RRC_IDLE | 36.306, 4.3.13.1 | Rel-10 | pc_LoggedMeasurementsIdle |  |
| 63 | Support of standalone GNSS receiver to provide detailed location information in RRC measurement report and logged measurements in RRC_IDLE | 36.306, 4.3.13.2 | Rel-10 | pc_StandaloneGNSS_Location |  |
| 64 | Support of automatic re-activation of the EPS bearer(s) | 24.301 | Rel-8 | pc_Automatic_EPS_Re_Attach |  |
| 65 | Support of UTRAN ANR | 25.306, 4.15 | Rel-10 | pc_UTRAN_ANR |  |
| 66 | Void |  |  |  |  |
| 67 | Support of PWS upper layer | 23.041 clause 9.1.3.4.2 | Rel-9 | pc_PWS_UpperLayer |  |
| 68 | Support of automatic PDN connectivity in EUTRAN (i.e. UE upper layer provides PDN connectivity parameters) | 24.301, 6.5.1.1 | Rel-8 | pc_Auto_PDN_Connectivity |  |
| 69 | Support user initiated PLMN reselection in automatic mode | 23.122 | Rel-8 | pc_UserInitiatedPLMN_Reselection |  |
| 70 | Support of UL MIMO | 36.306, clause 4.3.4.6 | Rel-10 | pc_UL_MIMO |  |
| 71 | Support of ESM Notification procedure | 24.301, 6.6.2 | Rel-9 | pc_ESM_Notification |  |
| 72 | Support of sending concatenated multiple Short Message over SGs | 23.272, 8.2.3a | Rel-9 | pc_SMS_SGs_Multi_MO |  |
| 73 | Support TAU in connected mode | 23.221, 7.2a | Rel-8 | pc_TAU_connected_in_IMS | Applicable when configured to pc_voice_PS_1_CS_2 and pc_Attach |
| 74 | Support TAU in idle mode | 23.221, 7.2a | Rel-8 | pc_TAU_idle_in_IMS | Applicable when configured to pc_voice_PS_1_CS_2 and pc_Attach |
| 75 | Support of Intra Frequency Proximity Indication | 36.306, clause 4.3.10.1 | Rel-9 | pc_IntraFreq_ProximityIndication |  |
| 76 | Support of Inter Frequency Proximity Indication | 36.306, clause 4.3.10.2 | Rel-9 | pc_InterFreq_ProximityIndication |  |
| 77 | Support of UTRAN Proximity Indication | 36.306, clause 4.3.10.3 | Rel-9 | pc_UTRAN_ProximityIndication |  |
| 78 | Support of Access Technology Indication in available PLMNs list | 23.122, clause 4.4.3.1.2 | Rel-8 | pc_Available_PLMNs_AcT_Ind |  |
| 79 | Support of Squal based cell reselection between E-UTRAN and GERAN | 36.304, clause 5.2.4.5, 45.008, clause 6.6.6 | Rel-9 | pc_Squal_based_CellReselection_between_E_UTRAN_and_GERAN |  |
| 80 | Support of AttachWithIMSI | 24.368, 5.4 | Rel-10 | pc_eAttachWithIMSI |  |
| 81 | Support of T3412 extended value IE | 24.301, 8.2.1.12, 8.2.26.15 | Rel-10 | pc_T3412Extended |  |
| 82 | Void |  |  |  |  |
| 83 | Void |  |  |  |  |
| 84 | Support of MinimumPeriodicSearchTimer | 23.122, 4.4.3.3 | Rel-10 | pc_eMinimumPeriodicSearchTimer |  |
| 85 | Support of delivery of rachReport upon request from the network | 36.306, 4.3.12.1 | Rel-9 | pc_Rach_Report |  |
| 86 | Support of Power Preference Indication | 36.306 4.3.15.3, 36.331, 5.6.10 | Rel-11 | pc_PPI_Support |  |
| 87 | Support of ePDCCH | 36.306, 4.3.4.18 36.331, 6.3.6 | Rel-11 | pc_ePDCCH |  |
| 88 | Void |  |  |  |  |
| 89 | Void |  |  |  |  |
| 90 | Void |  |  |  |  |
| 91 | Support of Extended Access Barring Override | 24.368, 5.10, 31.102, 4.2.94 | Rel-11 | pc_EAB_override |  |
| 92 | Void |  |  |  |  |
| 93 | Upon reception of ‘Daylight saving time’ information the UE stores/updates the daylight saving time | 24.301, 8.2.13 | Rel-8 | pc_DaylightSavingTime |  |
| 94 | Support of Radio Link Failure Report for inter-RAT MRO | 36.306, clause 6.10.1 | Rel-11 | pc_RLF_ReportForInterRAT_MRO |  |
| 95 | Support of IPv4 | 23.221, 5.1 | Rel-5 | pc_IPv4 |  |
| 96 | Support of IPv6 | 23.221, 5.1 | Rel-5 | pc_IPv6 |  |
| 97 | Support of Automatic Mode EF_LRPLMSI PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_EF_LRPLMNSI_Automatic_Mode_Exception |  |
| 98 | Support of Manual Mode PLMN Selection exception | 23.122, 4.4.3.1 | Rel-8 | pc_PLMN_Manual_Mode_Exception |  |
| 99 | Support of ZUC algorithm | 33.401,5.1.3.2 | Rel-11 | pc_ZUC |  |
| 100 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring UMTS cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.3 | Rel-9 | pc_SI_Neighbour_UMTS_Autonomous_Gaps |  |
| 101 | Support of reception of requestedFrequencyBands | 36.306, 4.3.5.6 | Rel-11 | pc_reqFreqBands |  |
| 102 | Support of more than 128 CA Band Combinations | 36.331, 5.6.3.3, 6.4 | Rel-11 | pc_More_Than_128_CAbandComb |  |
| 103 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring intra-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.1 | Rel-9 | pc_SI_Neighbour_intraFreq_Autonomous_Gaps |  |
| 104 | Supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring inter-frequency cell by reading the SI of the neighbouring cell using autonomous gaps and reporting | 36.306, 4.3.11.2 | Rel-9 | pc_SI_Neighbour_interFreq_Autonomous_Gaps |  |
| 105 | Support of Type B Half-duplex FDD operation | 36.211, 6.2.5 36.306, 4.2.6 | Rel-12 | pc_FDD_TypeB_HalfDuplex | Only applicable for UE supporting Category 0 and Category M1 and M2. When set transmission scheduling is performed in accordance to Half-Duplex operation Type B else in accordance to Full-Duplex operation. |
| 106 | Void |  |  |  |  |
| 107 | Support of enhanced HARQ pattern for TTI bundling operation for FDD | 36.306, 4.3.4.27 | Rel-12 | pc_eHARQ_Pattern_for_TTI_bundling |  |
| 108 | Support of tdd-FDD-CA-PCellDuplex-r12 with the first bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_TDD_PCell |  |
| 109 | Support of tdd-FDD-CA-PCellDuplex-r12 with the second bit setting to "1" | 36.306, 4.3.4.28 | Rel-12 | pc_tdd_FDD_CA_FDD_PCell |  |
| 110 | Support of ProSe direct communication | 36.306, 4.3.21.1 | Rel-12 | pc_commSupportedBands | 36.306, 4.3.21.1: If a UE supports sidelink communication on at least one band, the UE shall support sidelink communication transmission based on UE autonomous resource selection and eNB scheduled resource allocation. |
| 111 | Support of ProSe direct discovery | 36.306, 4.3.21.3 | Rel-12 | pc_discSupportedBands |  |
| 112 | Support of ProSe EPC level discovery | 24.334, 7.2 | Rel-12 | pc_Prose_EPC_Discovery |  |
| 113 | Support of ProSe discovery SLSS transmission and reception | 36.306, 4.3.21.6 | Rel-12 | pc_discSLSS |  |
| 114 | Support of uplink 64QAM | 36.306, 4.3.4.39 | Rel-12 | pc_UL_64QAM |  |
| 115 | Support of Power Saving Mode | 24.301, 5.3.11 | Rel-12 | pc_ePSM |  |
| 116 | Support of downlink 256QAM | 36.306, 4.1, 4.1A | Rel-12 | pc_DL_256QAM | Applicable for UEs of category 11-12 and UEs of DL category 11 and onwards. It is mandatory for UEs of DL category 13-14. |
| 117 | Support for GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" | IEEE Std 802.11 GSMA PRD IR.51 | Rel-11 | pc_WLAN_voice | The IR.51 is based on 3GPP Rel-11. |
| 118 | Support of CSI-RS based discovery signals measurement | 36.306 4.3.6.10 | Rel-12 | pc_CSI_RS_DS_Meas |  |
| 119 | Support of simultaneous transmission of EUTRA and sidelink communication (on different carriers) in all bands for which the UE indicated simultaneous sidelink and EUTRA support in a band combination (using commSupportedBandsPerBC) | 36.306, 4.3.21.2 | Rel-12 | pc_commSimultaneousTx |  |
| 120 | ProSe Discovery for Public Safety supported | 24.334, 4.1 | Rel-12 | pc_disc_public_safety | If Support of ProSe direct discovery (entry 111) is indicated then if the present entry is set to FALSE this shall be understood as ProSe Discovery for non-Public Safety supported |
| 121 | Support of extended DRX | 24.301, 5.3.12 | Rel-13 | pc_edrx |  |
| 122 | Support of CE mode A | 36.306, 4.3.29.1 | Rel-13 | pc_CEmodeA | Mandatory for CAT M1 and M2 UEs |
| 123 | Support of CE mode B | 36.306, 4.3.29.2 | Rel-13 | pc_CEmodeB |  |
| 124 | Support of TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations | 36.306, 4.3.4.31 | Rel-12 | pc_eIMTA_TDD |  |
| 125 | Support of prioritization of the frequency bands in multiBandInfoList over the band in freqBandIndicator as defined by freqBandIndicatorPriority-r12 | 36.306, 4.3.5.11 | Rel-12 | pc_freqBandPriorityAdjustment |  |
| 126 | Support of MBMS reception via SC-PTM on configured SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_SCell |  |
| 127 | Support of MBMS reception via SC-PTM on a cell that may be additionally configured as an SCell | 36.306, 4.3.5.2 | Rel-13 | pc_scptm_NonServingCell |  |
| 128 | Support of extended Long DRX cycle | 36.306, 4.3.19.4 | Rel-13 | pc_extendedLongDRX |  |
| 129 | Supports downlink LAA operation | 36.306, 4.3.23.1 | Rel-13 | pc_downlink_LAA |  |
| 130 | Supports measurement and reporting for RSSI and channel occupancy | 36.306, 4.3.6.19 | Rel-13 | pc_rssiAndChannelOccupancyReporting |  |
| 131 | Support of QCI1 indication in Radio Link Failure Report | 36.306, 6.8.2 | Rel-13 | pc_qci1Indication_inRLF |  |
| 132 | Support of user plane CIoT optimisation in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_User_Plane_CIoT_Optimisation |  |
| 133 | Support of EMM-REGISTERED without PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithoutPDN |  |
| 134 | Support of EMM-REGISTERED with PDN | 24.301, 5.3.15 | Rel-13 | pc_AttachWithPDN |  |
| 135 | Void |  |  |  |  |
| 136 | Void |  |  |  |  |
| 137 | Support of multiple DRBs in NB-IoT | 36.306, 4.3.8.5 | Rel-13 | pc_NB_MultiDRB |  |
| 138 | Support of Fast First Higher Priority PLMN search | 23.122, 4.4.3.3.1 | Rel-12 | pc_Fast_First_HPPLMN_Search |  |
| 139 | Support of TDD Bands38, 40, 41 or 42 Power class 2 operation | 36.101, 6.2.2 | Rel-14 | pc_TDD_band_UE_PC2 |  |
| 140 | Support for PDCP Packet Delay per QCI | 36.331, 5.5.2 | Rel-13 | pc_PDCP_PktDelay |  |
| 141 | Void |  |  |  |  |
| 142 |  |  |  |  |  |
| 143 | Support of Control plane CIoT in WB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_Control_Plane_CIoT_Optimisation |  |
| 144 | Support of S1-U data transfer | 24.301, 5.3.15 | Rel-13 | pc_S1_U_DataTransfer | An UE supporting user plane CIoT optimization shall set this PICS to true. |
| 145 | Support for GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1" | GSMA PRD NG.108 | Rel-13 | pc_Category_M1_voice |  |
| 146 | Support of automatic PDN connection trigger on HRPD cell reselection | X.s0057, 6.4.1 | Rel-8 | pc_AutomaticHRPD_PDN_Connection |  |
| 147 | Support for Dual RM Coding | 36.331, 6.3.6 | Rel-10 | pc_DualRM_Coding |  |
| 148 | Support of V2X sidelink communication | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommSidelink |  |
| 149 | Support of V2X communication Via Uu | 36.300, 23.14.1.1 | Rel-14 | pc_v2xCommUu |  |
| 150 | Support of simultaneous transmission of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousTx |  |
| 151 | Support of simultaneous reception of EUTRA and V2X sidelink communication | 36.306, 4.3.5.27 | Rel-14 | pc_v2xSimultaneousRx |  |
| 152 | Support of transmitting PSCCH/PSSCH using dynamic scheduling | 36.306, 4.3.21.14 | Rel-14 | pc_v2xScheduling |  |
| 153 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing | 36.306, 4.3.21.15 | Rel-14 | pc_v2xFullSensing |  |
| 154 | Support of transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing | 36.306, 4.3.21.16 | Rel-14 | pc_v2xPartialSensing |  |
| 155 | Support of SLSS transmission and reception for V2X sidelink communication | 36.306, 4.3.21.17 | Rel-14 | pc_v2xSLSS |  |
| 156 | Support of CBR measurement and reporting | 36.306, 4.3.21.18 | Rel-14 | pc_v2xCBRMeas |  |
| 157 | Support of zone based transmission resource pool selection for V2X sidelink communication | 36.306, 4.3.21.12 | Rel-14 | pc_v2xZoneBasedPoolSelection |  |
| 158 | Require intra-frequency measurement gaps for operating in CE Mode A or CE Mode B | 36.306, 4.3.5.1.2 | Rel-13 | pc_intraFreq_CE_NeedForGaps |  |
| 159 | Support of 4 layer spatial multiplexing with transmission mode 3 and transmission mode 4 | 36.306, 4.3.4.7 | Rel-10 | pc_4Layer_spatial_mux_tm3_tm4 |  |
| 160 | Support of delay budget reporting for MMTEL voice and video enhancements | 36.306, 4.3.32.1 | Rel-14 | pc_delayBudgetReporting |  |
| 161 | Support of PUSCH enhancement for MMTEL voice and video enhancements mode | 36.306, 4.3.32.2 | Rel-14 | pc_PUSCH_Ehn_MMTEL |  |
| 162 | Void |  |  |  |  |
| 163 | Support of PUCCH transmission on SCell in CA | 36.306, 4.3.4.47 | Rel-13 | pc_PUCCH_SCell |  |
| 164 | Support high speed enhancement for random access preambles generated from restricted set type B in high speed scenoario as specified in TS 36.211 | 36.306 | Rel-14 | pc_Highspeed_Enh_Prach |  |
| 165 | Support of RRC connection re-establishment | 36.306, 6.7.5 | Rel-14 | pc_RRC_re_establishment_CP_CIoT | An UE supporting S1-U data transfer shall set this PICS to true. |
| 166 | Support of SRS switching between a band pair | 36.306, 4.3.5.24, 4.3.5.25 | Rel-14 | pc_SRS_switching | Support of SRS switching between a band pair |
| 167 | Support of 2 HARQ processes in DL and UL in NB-IoT | 36.306, 4.3.4.62 | Rel-14 | pc_NB_TwoHARQ_Processes |  |
| 168 | Support of Release Assistance Indication (RAI) in NB-IoT | 36.306, 4.3.19.10 | Rel-14 | pc_NB_Rai_Support |  |
| 169 | Support of Announcing for ProSe Group Member Discovery | 24.334, 10A.2.6 | Rel-13 | pc_ProSeAnnForGroupMemberDiscovery |  |
| 170 | Support of SPS interval shorter than 10 subframes in FDD mode | 36.306, 4.3.19.5 | Rel-14 | pc_shortSPS_intervalFDD |  |
| 171 | Support of SPS interval shorter than 10 subframes in TDD mode | 36.306, 4.3.19.6 | Rel-14 | pc_shortSPS_intervalTDD |  |
| 172 | Support of skipping SPS UL transmissions if no data is available | 36.306, 4.3.19.8 | Rel-14 | pc_skipUplinkSPS | An UE supporting SPS interval shorter than 10 (pc_shortSPS_intervalFDD or pc_shortSPS_intervalTDD) shall set this PICS to true. |
| 173 | Support of skipping UL transmissions if no data is available | 36.306, 4.3.19.7 | Rel-14 | pc_skipUplinkDynamic |  |
| 174 | Supports uplink LAA operation | 36.306, 4.3.23.8 | Rel-14 | pc_uplink_LAA | Support of Enhanced LAA operations |
| 175 | Void |  |  |  |  |
| 176 | Supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B | 36.306, 4.3.23.10 | Rel-14 | pc_twoStepScheduling_uplink_LAA | UE supports two step uplink scheduling using PUSCH trigger A and PUSCH trigger B, applying to the UE supports uplink LAA operation |
| 177 | Supports multiple uplink SPS and reporting SPS assistance information | 36.306, 4.3.19.11 | Rel-14 | pc_multipleUplinkSPS | Support of multiple uplink SPS and reporting SPS assistance information |
| 178 | Support of V2X communication as Pedestrian UE | 36.300, 23.14.1.1 | Rel-14 | pc_P2X_UE |  |
| 179 | Support of the uplink data compression operation | 36.306, 4.3.1.7 | Rel-15 | pc_UDC |  |
| 180 | Support of UL data compression with SIP static dictionary | 36.306, 4.3.1.8 | Rel-15 | pc_UDC_SIP |  |
| 181 | Support of QoE Measurement Collection for Streaming Service | 36.306, 4.36.30 | Rel-15 | pc_qoe_MeasReport |  |
| 182 | Support of QoE Measurement Collection for MTSI Service | 36.306, 4.36.33 | Rel-15 | pc_qoe_MTSI_MeasReport |  |
| 183 | Support of 256QAM in UL | 36.306, 4.3.4.73 | Rel-14 | pc_UL_256QAM |  |
| 184 | Support of Bluetooth Measurement Collection in logged MDT | 36.306, 4.3.13.6 | Rel-15 | pc_BT_Meas_logged_MDT |  |
| 185 | Support of WLAN Measurement Collection in logged MDT | 36.306, 4.3.13.7 | Rel-15 | pc_WLAN_Meas_logged_MDT |  |
| 186 | Support of Bluetooth Measurement Collection in Immediate MDT | 36.306, 4.3.13.8 | Rel-15 | pc_BT_Meas_Imm_MDT |  |
| 187 | Support of WLAN Measurement Collection in Immediate MDT | 36.306, 4.3.13.9 | Rel-15 | pc_WLAN_Meas_Imm_MDT |  |
| 188 | Support of ce-PUSCH-NB-MaxTBS-r14 | 36.306, 4.3.4.63 | Rel-15 | pc_ce_PUSCH_NB_MaxTBS |  |
| 189 | Support of height-based measurement reporting | 36.306, 4.3.6.35 | Rel-15 | pc_heightMeas |  |
| 190 | Support of GNSS for height measurement |  | Rel-15 | pc_gnss_heightMeas |  |
| 191 | Support of measurement reporting triggered based on a number of cells | 36.306, 4.3.6.34 | Rel-15 | pc_Multiple_Cells_Meas_Ext |  |
| 192 | Support of flight path plan reporting | 36.306, 4.3.15.14 | Rel-15 | pc_FlightPathPlan |  |
| 193 | Void |  |  |  |  |
| 194 | Support of HARQ-ACK bundling | 36.213, 7.3.1 | Rel-14 | pc_ce_HARQ_AckBundling | Support of HARQ-ACK bundling |
| 195 | Support of eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode. | 36.306, 4.3.6.31 | Rel-15 | pc_idleModeMeasurement |  |
| 196 | Support of the dormant SCell state. | 36.306, 4.3.19.16 | Rel-15 | pc_dormantSCellState |  |
| 197 | Support of having SCell configured in dormant SCell state | 36.306 4.3.19.18 | Rel-15 | pc_directSCellHibernation |  |
| 198 | Support of having SCell configured in activated SCell state | 36.306, 4.3.19.17 | Rel-15 | pc_directSCellActivation |  |
| 199 | Support of user plane CIoT optimisation in NB-S1 mode | 24.301, 5.3.15 | Rel-13 | pc_NB_User_Plane_CIoT_Optimisation |  |
| 200 | Support of Control Plane Early Data Transmission | 36.306, 6.8.4 | Rel-15 | pc_Control_Plane_CIoT_Optimisation_EDT |  |
| 201 | Support of User Plane Early Data Transmission | 36.306, 4.3.8.7 | Rel-15 | pc_User_Plane_CIoT_Optimisation_EDT |  |
| 202 | Support of RLC UM mode in NB-IoT | 36.306, 4.3.2.5 | Rel-15 | pc_NB_RLC_UM |  |
| 203 | Support of short TTI and/or short processing time | 36.306, 4.3.4.150 | Rel-15 | pc_sTTI_SPT |  |
| 204 | Support of short processing time for the corresponding frame structure types | 36.306, 4.3.4.100 | Rel-15 | pc_spt_Parameters |  |
| 205 | Support of sTTI in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_sTTI_Combinations |  |
| 206 | Support of {subslot, subslot} combinations in downlink CCs and uplink CCs | 36.306, 4.3.4.103 | Rel-15 | pc_subslot_Combinations |  |
| 207 | Support of L1-based SPDCCH reuse | 36.306, 4.3.4.147 | Rel-15 | pc_SPDCCH_Reuse |  |
| 208 | Support of SRS trigerring via DCI format 7 for FS2 | 36.306, 4.3.4.181 | Rel-15 | pc_SRS_DCI7_Triggering |  |
| 209 | Support of UL asynchronous HARQ sharing between different TTI lengths for an UL serving cell. | 36.306, 4.3.4.156 | Rel-15 | pc_ul_AsyncHarqSharingDiffTTI |  |
| 210 | Support of Wake Up Signal | 36.306, 4.3.4.113 | Rel-15 | pc_wakeUpSignal |  |
| 211 | Support of physical layer SR with HARQ ACK | 36.306, 4.3.4.117 | Rel-15 | pc_SR_WithHARQ_ACK |  |
| 212 | Support of physical layer SR without HARQ ACK | 36.306, 4.3.4.118 | Rel-15 | pc_SR_WithoutHARQ_ACK |  |
| 213 | UE supports Ethernet header compression and decompression using EHC protocol | 36.306, 4.3.1.12 | Rel-16 | pc_EUTRAN_EHC |  |
| 214 | UE supports DAPS handover in source PCell and intra-frequency target PCell | 36.306, 4.3.5.40 | Rel-16 | pc_EUTRA_intraFreqDAPS |  |
| 215 | Support of RACS | 24.301, 5.3.20 | Rel-16 | pc_EPC_RACS |  |
| 216 | Support of RRC message Segmentation in the UL | 36.306, 6.8.12 | Rel-16 | pc_LTE_UL_Segmentation | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |
| 217 | UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. | 36.306, 4.3.30.3 | Rel-16 | pc_EUTRA_cho_r16 |  |
| 218 | Support of Mixed Operation Mode in NB-IoT | 36.306, 4.3.4.115 | Rel-15 | pc_NB_mixedOperationMode |  |
| 219 | Support of NPRACH resources using preamble format 2 for FDD in NB-IoT | 36.306, 4.3.4.119 | Rel-15 | pc_NB_nprach_Format2 |  |
| 220 | UE supports DAPS handover in source PCell and inter-frequency target PCell | 36.306, 4.3.5.43 | Rel-16 | pc_EUTRA_interFreqDAPS |  |
| 221 | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over LTE | 36.509, 5.10 | Rel-16 | pc_Set_UE_Cap_Info_LTE | This test function is mandatory for UEs supporting UL segmentation whose maximum UECapabilityInformation message size is less than the allowed maximum supported size of a PDCP SDU. |
| 222 | Support of flexible starting PRB for PDSCH | 36.306, 4.3.4.121 and 4.3.4.122 | Rel-15 | pc_FlexibleStartPRB_PDSCH |  |
| 223 | Support of flexible starting PRB for PUSCH | 36.306, 4.3.4.123 and 4.3.4.124 | Rel-15 | pc_FlexibleStartPRB_PUSCH |  |
| 224 | Support one or more Multi-SIM features include NAS signalling connection release/Paging indication for voice services/Reject paging request/Paging restriction/Paging timing collision control and so on. | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM |  |
| 225 | Support of Multi-SIM NAS signalling connection release | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM_NCR |  |
| 226 | Support of Multi-SIM Paging indication for voice services | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM_PIV |  |
| 227 | Support of Multi-SIM Reject paging request | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM_RPR |  |
| 228 | Support of Multi-SIM Paging restriction | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM_PR | A UE support Pging restriction shall support: - NAS signalling connection release or - Reject paging request or - both of them |
| 229 | Support of Multi-SIM Paging time collision control | 24.301,5.5.1 | Rel-17 | pc_EPC_MUSIM_PTCC |  |
| 230 | Support of NTN access in NB-IoT | 36.306, 4.3.38.1 | Rel-17 | pc_NB_ntn_Connectivity_EPC | Note 1 |
| 231 | Support of Timing advance reporting in NTN cell in NB-IoT | 36.306, 4.3.38.2 | Rel-17 | pc_NB_ntn_TA_Report |  |
| 232 | Support of modified timer value for PUR operation required for NTN operation in NB-IoT | 36.306, 4.3.38.3 | Rel-17 | pc_NB_ntn_PUR_TimerEnhancement |  |
| 233 | Support of timing relationship enhancements using Differential Koffset in NB-IoT | 36.306, 4.3.38.4 | Rel-17 | pc_NB_ntn_OffsetTimingEnh |  |
| 234 | Support of NTN features in GSO scenario in NB-IoT | 36.306, 4.3.38.5 | Rel-17 | pc_NB_ntn_GSO_ScenarioSupport |  |
| 235 | Support handover from E-UTRAN/EPC to EPC/ePDG | 23.402, 8.2.3 | Rel-15 | pc_HO_from_E_UTRAN_EPC_to_EPC_ePDG |  |
| 236 | Supports reception of segmented DL RRC messages | 36.306, 4.3.8.14 | Rel-16 | pc_dl_DedicatedMessageSegmentation |  |
| 237 | Support of NTN features in NGSO scenario in NB-IoT | 36.306, 4.3.38.5 | Rel-17 | pc_NB_ntn_NGSO_ScenarioSupport |  |
| 238 | Support of gap length between segments for PUSCH and PUCCH required by a UE supporting ce-ModeA-r13 or for NPUSCH required by a UE supporting ue-category-NB, for TA pre-compensation | 36.306, 4.3.38.6 | Rel-17 | pc_ntn_SegmentedPrecompensationGaps |  |
| 239 | Support handover from ePDG/EPC to E-UTRAN/EPC | 23.402, 8.2.1 | Rel-15 | pc_HO_from_ePDG_EPC_to_E_UTRAN_EPC |  |
| 240 | Support of NTN only access in NB-IoT | 36.306, 4.3.38.1 | Rel-17 | pc_NB_ntn_only_Connectivity_EPC | Note 1 |
| 241 | Support of NTN only access in CE Mode A | 36.306, 4.3.38.1 | Rel-17 | pc_ntn_only_Connectivity_EPC_CE_ModeA | Note 1 |
| 242 | Support of NTN access in CE Mode A | 36.306, 4.3.38.1 | Rel-17 | pc_ntn_Connectivity_EPC_CE_ModeA | Note 1 |
| 243 | Support of Timing advance reporting in NTN cell in CE Mode A | 36.306, 4.3.38.2 | Rel-17 | pc_ntn_TA_Report_CE_ModeA |  |
| 244 | Support of modified timer value for PUR operation required for NTN operation in CE Mode A | 36.306, 4.3.38.3 | Rel-17 | pc_ntn_PUR_TimerEnhancement_CE_ModeA |  |
| 245 | Support of timing relationship enhancements using Differential Koffset in CE Mode A | 36.306, 4.3.38.4 | Rel-17 | pc_ntn_OffsetTimingEnh_CE_ModeA |  |
| 246 | Support of NTN features in GSO scenario in CE Mode A | 36.306, 4.3.38.5 | Rel-17 | pc_ntn_GSO_ScenarioSupport_CE_ModeA |  |
| 247 | Support of NTN features in NGSO scenario in CE Mode A | 36.306, 4.3.38.5 | Rel-17 | pc_ntn_NGSO_ScenarioSupport_CE_ModeA |  |
| 248 | Support of mpsPriorityIndication on RRC release with redirect | 36.306, 4.3.15.23 | Rel-16 | pc_EUTRA_mpspriorityindication_r16 |  |
| 249 | Support of UAS Services | 24.301, 3.1, 6.3.13 | Rel-17 | pc_EPS_UAS | A UE supporting UAS services |
| ZZZ | Support of operator controlled signal threshold per access technology | 23.122, 3.11 | Rel-18 | pc_operator_controlled_signal_threshold_per_access_technology | Only IoT stationary UE can support the "Operator controlled signal threshold per access technology". |
| Note 1: A UE supporting this PICS shall set pc_StandaloneGNSS_Location to true. |  |  |  |  |  |

<End of modified section 3>