3GPP TSG-RAN5 Meeting #102	R5-240429

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4225 | rev |  | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MUSIM TC 8.1.2.1.6-MUSIM gap configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 38.133 Table 9.1.10-1, the MGL for aperiodic MUSIM gap cannot be 4ms. It can be 10ms or 20ms. - Based on the discussion paper R5-240431, network only allow to configure the gap indicated by UE in the MUSIM UAI. If TC directly configure MUSIM gap to UE, the UE behaviour is not specified by 38.331.In addition, this TC may test the gap pattern not supported by UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update MGL for aperiodic MUSIM gap - add MUSIM UAI trigger procedure (MUSIM UAI triggered by “Set MUSIM UAI test function” or other method) before MUSIM gap configuration - only test the gap pattern indicated in the PIXIT :px_NR_SupportPeriodicMusimGapPattern_First/px_NR_SupportPeriodicMusimGapPattern_Second/px_NR_SupportAperiodicMusimGapPattern - Correct SS1 to NR Cell 1 in the TP - Correct “paging” to “UECapabilityEnquiry” in the Table 8.1.2.1.6.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformat UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-3 CR 3373 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.2.1.6	RRC reconfiguration/ MUSIM / MUSIM- gap / Addition / Modification / Release

8.1.2.1.6.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToAddModList and the gap configuration is active in NR Cell 1 }

then { UE stops all interworking with NR Cell 1 during the configured gaps periodically }

}

(2)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToAddModList and an entry is matched with musim-GapID that exists in the musim-GapConfigLis in NR Cell 1}

then { UE stops all interworking with NR Cell 1 during the replaced new configured gaps periodically }

}

(3)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with periodic musim-GapID involved in musim-GapToReleaseList in NR Cell 1}

then { UE performs normal interworking with NR Cell 1 during the released gaps periodically }

}

(4)

with { UE in NR RRC_CONNECTED state}

ensure that

when { UE receives the RRCReconfiguration message includes the musim-GapConfig with musim-AperiodicGap-r17}

then { UE stops all interworking with NR Cell 1 temporarily with configuration MUSIM-GapInfo-r17 }

}

8.1.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses  5.3.5.3, 5.3.5.9a and TS 38.133 clause 9.1.10. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

[TS 38.331, clause 5.3.5.9a]

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

1>	else if musim-GapConfig is set to release:

[TS 38.133, clause 9.1.10]

If the UE requires gap patterns for MUSIM purpose, such as cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network, then the network may provide one or more per-UE MUSIM gap pattern(s) for concurrent monitoring of all frequency layers for MUSIM via MUSIM-GapConfig [2]. The UE can be configured with no more than three periodic MUSIM gap patterns and/or one aperiodic MUSIM gap pattern for MUSIM via MUSIM-GapConfig [2]. The MUSIM gap patterns specified in Table 9.1.10-1 are applicable only for MUSIM operation.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 9.1.10-1 based on UE’s capability specified in TS38.306[14] and the applicability specified in Table 9.1.10-2

UE determines MUSIM gap timing based on gap offset configuration from serving cell provided by higher layer signalling as specified in TS 38.331 [2].

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Table 9.1.10-2: Applicability for MUSIM Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA configuration)

| MUSIM gap pattern configuration | Serving cell | Gap Purpose | Applicable MUSIM Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | FR1, FR2, or | MUSIM Note1 | 0-13, 14-26, 27, 28 |
| MUSIM gap | FR1 + FR2 |  |  |
| NOTE 1: Inclusion of MUSIM procedures for per-UE MUSIM gaps only in NR single carrier, NR CA mode: MUSIM purpose which includes cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network. |  |  |  |

8.1.2.1.6.3	Test description

8.1.2.1.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-	NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-	NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell''.

UE:

-	the UE is a MUSIM UE;

-	the UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4] with following exceptions.

-	NGC Cell A is replaced by NR Cell 1

-	NGC Cell G is replaced by NR Cell 12

8.1.2.1.6.3.2	Test procedure sequence

Table 8.1.2.1.6.3.2-1 illustrates a paging message to be sent for NR Cell 1 at various time instants of the test execution.

Table 8.1.2.1.6.3.2-1: Time instances of sending a UECapabilityEnquiry message in conducted test environment

|  | Remark | Comment |
| --- | --- | --- |
| T0 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T0 = SFN + subframe + (X- 1)ms X = musim-GapLength-r17 configured in step9 | When musim-GapLength-r17is configured in step 9, T0 is in the gap; |
| T1 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T1 = SFN + subframe + (X+2)ms X= musim-GapLength-r17 configured in step 9 for step13; X= musim-GapLength-r17 configured in step 13 for step17 and step 21; | T1 is always outside the gap in Step 13,17,21. |
| T2 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T2 = SFN + subframe + 2ms | If the gap is not released successfully, T2 is in the gap in step 19. |
| T3 | SFN = starting-SFN; subframe = startingSubframe; T3 = SFN + subframe + 2ms | T3 is in the gap in step 23. |
| T4 | SFN mod T = FLOOR(Offset/10); subframe = Offset mod 10; with T = musim-GapRepetition/10; T0 = SFN + subframe + (X+ 1)ms X = musim-GapLength-r17 configured in step13 | When musim-GapLength-r17 is configured in step 13, T0 is outside the gap. |

Table 8.1.2.1.6.3.2-2: Main behaviour

| St | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S |  | Message |  |  |  |  |  |
| 1-8 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 on NR Cell 1 |  | - |  | - |  | - |  | - |  |
| 8A |  | The SS transmits an RRCReconfiguration message to configure musim-GapAssistanceConfig-r17 IE. |  | <-- |  | NR RRC: RRCReconfiguration |  | - |  | - |
| 8B |  | The UE transmits an RRCReconfigurationComplete message. |  | --> |  | NR RRC: RRCReconfigurationComplete |  | - |  | - |
| - |  | EXCEPTION: Steps 8Ca1 and 8Ca2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 8Cb1 is executed(Note 1). |  | - |  | - |  | - |  | - |
| 8Ca1 |  | The SS transmits an SET MUSIM UAI REQUEST message. |  | <-- |  | SET MUSIM UAI REQUEST |  | - |  | - |
| 8Ca2 |  | The UE transmits an SET MUSIM UAI RESPONSE message. |  | --> |  | SET MUSIM UAI RESPONSE |  | - |  | - |
| 8Cb1 |  | Make the UE send UEAssistanceinformation |  | - |  | - |  | - |  | - |
| 8D |  | The UE transmit an UEAssistanceInformation message to indicate MUSIM-GapPreferenceList-r17. |  | --> |  | NR RRC: UEAssistanceInformation |  | - |  | - |
| 9 | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 with musim-GapToAddModList-r17 IE on NR Cell 1 |  | <-- |  | NR RRC: RRCReconfiguration |  | - |  | - |  |
| 10 | UE transmits an RRCReconfigurationComplete message on NR Cell 1. |  | --> |  | NR RRC: RRCReconfigurationComplete |  | - |  | - |  |
| 11 | The SS sends UECapabilityEnquiry message on Cell 1 at T0. |  | <-- |  | NR RRC: UECapabilityEnquiry |  | - |  | - |  |
| 12 | Check: Does the UE send a UECapabilityInformation message? |  | --> |  | NR RRC: UECapabilityInformation |  | 1 |  | F |  |
| 13 | The SS transmits an RRCReconfiguration message to configure musim-GapLength-r17 with musim-GapToAddModList-r17 IE on Cell 1 at T1. |  | <-- |  | NR RRC: RRCReconfiguration |  | - |  | - |  |
| 14 | The UE transmits an RRCReconfigurationComplete message on Cell 1. |  | --> |  | NR RRC: RRCReconfigurationComplete |  | - |  | - |  |
| 15 | The SS sends UECapabilityEnquiry message to the UE on Cell 1 at T4. |  | <-- |  | NR RRC: UECapabilityEnquiry |  | - |  | - |  |
| 16 | Check: Does the UE send a UECapabilityInformation message? |  | --> |  | NR RRC: UECapabilityInformation |  | 2 |  | P |  |
| 17 | The SS transmits an RRCReconfiguration message to USIM A to configure musim-GapId-r17 = ‘0’ with musim-GapToReleaseList-r17 IE to UE on Cell 1 at T1. |  | <-- |  | NR RRC: RRCReconfiguration |  | - |  | - |  |
| 18 | The UE transmits an RRCReconfigurationComplete message on Cell 1. |  | --> |  | NR RRC: RRCReconfigurationComplete |  | - |  | - |  |
| 19 | The SS sends UECapabilityEnquiry message to the UE on Cell 1at T2. |  | <-- |  | NR RRC: UECapabilityEnquiry |  | - |  | - |  |
| 20 | Check: Does the UE send a UECapabilityInformation message? |  | --> |  | NR RRC: UECapabilityInformation |  | 3 |  | P |  |
| 21 | The SS transmits an RRCReconfiguration message to USIM A to configure starting-SFN-r17 = ‘0’, startingSubframe-r17 = ‘0’ and musim-GapLength-r17 with musim-AperiodicGap-r17 IE to UE on Cell 1 at T1. |  | <-- |  | NR RRC: RRCReconfiguration |  | - |  | - |  |
| 22 | The UE transmits an RRCReconfigurationComplete message on Cell 1. |  | --> |  | NR RRC: RRCReconfigurationComplete |  | - |  | - |  |
| 23 | The SS sends UECapabilityEnquiry message to the UE on Cell 1 at T3. |  | <-- |  | NR RRC: UECapabilityEnquiry |  | - |  | - |  |
| 24 | Check: Does the UE send a UECapabilityInformation message? |  | --> |  | NR RRC: UECapabilityInformation |  | 4 |  | F |  |
| Note1: The test function is optional, it can be replaced by alternative solutions (e.g AT command or MMI) due to UE implementation specific behavior. |  |  |  |  |  |

8.1.2.1.6.3.3	Specific message contents

Table 8.1.2.1.6.3.3-0: RRCReconfiguration (step 8A, Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-GapAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-GapProhibitTimer-r17 | s10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-0A: SET MUSIM UAI REQUEST (step 8Ca1, Table 8.1.2.1.6.3.2-2)

| Derivation Path: 38.509 clause 6.13.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment |  |  |  | Condition |  |
| Protocol discriminator |  | 1 1 1 1’B |  |  |  |  |  |  |  |  |
| Skip indicator |  | 0 0 0 0’B |  |  |  |  |  |  |  |  |
| Message type |  | 1 0 1 1 0 0 1 0’B |  |  |  |  |  |  |  |  |
| musim-PreferredRRC-State |  | 0 0 0 0 0 0 1 1’B |  |  | Reserved |  |  |  |  |  |
| musim-GapPreferenceList |  |  |  |  |  |  |  |  |  |  |
| musim-GapPreferenceList Value type |  | 0 0 0 0 0 0 0 1’B |  |  |  |  |  |  |  |  |
| Length of musim-GapPreferenceList Value contents in bytes |  | 0 0 0 0 1 0 0 1’B |  |  | 9 bytes |  |  |  |  |  |
| musim-GapPreferenceList IE #0 |  |  |  |  |  |  |  |  |  |  |
| C2..C0 |  |  | 0 0 0’B |  |  | MGL=3ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=2 OR 3 OR 10 OR 11 |  |
|  |  |  | 0 0 1’B |  |  | MGL=4ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=6 OR 7 OR 8 OR 9 |  |
|  |  |  | 0 1 0’B |  |  | MGL=6ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=0 OR 1 OR 4 OR 5 OR 14 OR 15 OR 16 OR 17 |  |
|  |  |  | 0 1 1’B |  |  | MGL=10ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=12 OR 18 OR 19 OR 20 OR 21 |  |
|  |  |  | 1 0 0’B |  |  | MGL=20ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=13 OR 22 OR 23 OR 24 OR 25 OR 26 |  |
| D3..D0 | 0 0 0 0’B | MGRP=20ms | px_NR_SupportPeriodicMusimGapPattern_First=1 OR 3 OR 8 OR 12 |
|  | 0 0 0 1’B | MGRP=40ms | px_NR_SupportPeriodicMusimGapPattern_First=0 OR 2 OR 7 |
|  |  |  | 0 0 1 0’B |  |  | MGRP=80ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=4 OR 6 OR 10 |  |
|  |  |  | 0 0 1 1’B |  |  | MGRP=160ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=5 OR 9 OR 11 OR 13 |  |
|  |  |  | 0 1 0 0’B |  |  | MGRP=320ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=14 OR 18 OR 22 |  |
|  |  |  | 0 1 0 1’B |  |  | MGRP=640ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=15 OR 19 OR 23 |  |
|  |  |  | 0 1 1 0’B |  |  | MGRP=1280ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=16 OR 20 OR 24 |  |
|  |  | 0 1 1 0’B |  |  | MGRP=2560ms |  |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=17 OR 21 OR 25 |
|  |  |  | 1 0 0 0’B |  |  | MGRP=5120ms |  |  | px_NR_SupportPeriodicMusimGapPattern_First=26 |  |
| F0 | 0’B | Periodic MUSIM gap |  |
| E13..E0 |  |  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0’B |  |  | Gap offset=0 |  |  |  |  |
| musim-GapPreferenceList IE #1 |  |  |  |  |  |  |  |  |  |  |
| C2..C0 |  |  | 0 0 0’B |  |  | MGL=3ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =2 OR 3 OR 10 OR 11 |  |
|  |  |  | 0 0 1’B |  |  | MGL=4ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =6 OR 7 OR 8 OR 9 |  |
|  |  |  | 0 1 0’B |  |  | MGL=6ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =0 OR 1 OR 4 OR 5 OR 14 OR 15 OR 16 OR 17 |  |
|  |  |  | 0 1 1’B |  |  | MGL=10ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =12 OR 18 OR 19 OR 20 OR 21 |  |
|  |  |  | 1 0 0’B |  |  | MGL=20ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =13 OR 22 OR 23 OR 24 OR 25 OR 26 |  |
| D3..D0 |  |  | 0 0 0 0’B |  |  | MGRP=20ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =1 OR 3 OR 8 OR 12 |  |
|  |  |  | 0 0 0 1’B |  |  | MGRP=40ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =0 OR 2 OR 7 |  |
|  |  |  | 0 0 1 0’B |  |  | MGRP=80ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =4 OR 6 OR 10 |  |
|  |  |  | 0 0 1 1’B |  |  | MGRP=160ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =5 OR 9 OR 11 OR 13 |  |
|  |  |  | 0 1 0 0’B |  |  | MGRP=320ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =14 OR 18 OR 22 |  |
|  |  |  | 0 1 0 1’B |  |  | MGRP=640ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =15 OR 19 OR 23 |  |
|  |  |  | 0 1 1 0’B |  |  | MGRP=1280ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =16 OR 20 OR 24 |  |
|  |  |  | 0 1 1 0’B |  |  | MGRP=2560ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =17 OR 21 OR 25 |  |
|  |  |  | 1 0 0 0’B |  |  | MGRP=5120ms |  |  | px_NR_SupportPeriodicMusimGapPattern_Second =26 |  |
| F0 | 0’B | Periodic MUSIM gap |  |
| E13..E0 |  |  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0’B |  |  | Gap offset=0 |  |  |  |  |
| musim-GapPreferenceList IE #2 |  |  |  |  |  |  |  |  |  |  |
| C2..C0 |  | 0 1 1’B |  |  | MGL=10ms |  |  | px_NR_SupportAperiodicMusimGapPattern=27 |  |  |
|  |  | 1 0 0’B |  |  | MGL=20ms |  |  | px_NR_SupportAperiodicMusimGapPattern=28 |  |  |
| D3..D0 |  | 1 1 1 1’B |  |  | MGRP=NA |  |  |  |  |  |
| F0 | 1’B | Aperiodic MUSIM gap |  |
| E13..E0 |  | 1 1 1 1 1 1 1 1 1 1 1 1 1 1’B |  |  | The gap starting subframe number and starting SFN number are not valid. |  |  |  |  |  |

Table 8.1.2.1.6.3.3-1: RRCReconfiguration (step 9, step 13, step 17, step 21,Table 8.1.2.1.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 CHOICE { |  |  |  |
| setup | MUSIM-GapConfig-r17 for PeriodicGap1 |  | step 9 |
|  | MUSIM-GapConfig-r17- for PeriodicGap2 |  | Step13 |
|  | MUSIM-GapConfig -r17 for Release Gap |  | step 17 |
|  | MUSIM-GapConfig- -r17 for AperiodicGap |  | step 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-2:MUSIM-GapConfig-r17 for PeriodicGap1 (Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not Present |  |  |
| musim-GapLength-r17 | ms6 |  | px_NR_SupportPeriodicMusimGapPattern_First=0 OR 1 OR 4 OR 5 OR 14 OR 15 OR 16 OR 17 |
|  | ms3 |  | px_NR_SupportPeriodicMusimGapPattern_First=2 OR 3 OR 10 OR 11 |
|  | ms10 |  | px_NR_SupportPeriodicMusimGapPattern_First=12 OR 18 OR 19 OR 20 OR 21 |
|  | ms20 |  | px_NR_SupportPeriodicMusimGapPattern_First=13 OR 22 OR 23 OR 24 OR 25 OR 26 |
|  |  | ms4 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=6 OR 7 OR 8 OR 9 |  |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |  |  |  |  |
| ms80-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=1 OR 3 OR 8 OR 12 |  |
| ms40-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=0 OR 2 OR 7 |  |
| ms20-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=4 OR 6 OR 10 |  |
| ms160-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=5 OR 9 OR 11 OR 13 |  |
| ms320-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=14 OR 18 OR 22 |  |
| ms640-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=15 OR 19 OR 23 |  |
| ms1280-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=16 OR 20 OR 24 |  |
| ms2560-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=17 OR 21 OR 25 |  |
| ms5120-r17 |  | 0 |  |  |  | px_NR_SupportPeriodicMusimGapPattern_First=26 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| musim-AperiodicGap-r17 |  | Not Present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.2.1.6.3.3-2A:MUSIM-GapConfig-r17 for PeriodicGap2(Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17{ |  |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  |  |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not Present |  |  |
| musim-GapLength-r17 | ms6 |  | px_NR_SupportPeriodicMusimGapPattern_Second=0 OR 1 OR 4 OR 5 OR 14 OR 15 OR 16 OR 17 |
|  | ms3 |  | px_NR_SupportPeriodicMusimGapPattern_Second =2 OR 3 OR 10 OR 11 |
|  | ms10 |  | px_NR_SupportPeriodicMusimGapPattern_Second =12 OR 18 OR 19 OR 20 OR 21 |
|  | ms20 |  | px_NR_SupportPeriodicMusimGapPattern_Second =13 OR 22 OR 23 OR 24 OR 25 OR 26 |
|  | ms4 |  | px_NR_SupportPeriodicMusimGapPattern_Second =6 OR 7 OR 8 OR 9 |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |
| ms80-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =1 OR 3 OR 8 OR 12 |
| ms40-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =0 OR 2 OR 7 |
| ms20-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =4 OR 6 OR 10 |
| ms160-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =5 OR 9 OR 11 OR 13 |
| ms320-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =14 OR 18 OR 22 |
| ms640-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =15 OR 19 OR 23 |
| ms1280-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =16 OR 20 OR 24 |
| ms2560-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =17 OR 21 OR 25 |
| ms5120-r17 | 0 |  | px_NR_SupportPeriodicMusimGapPattern_Second =26 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.6.3.3-3: MUSIM-GapConfig -r17 for Release Gap (Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |  |
| musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 { |  |  |  |  |
| MUSIM-GapId-r17[1] |  | 0 |  |  |
| } |  |  |  |  |
| musim-GapToAddModList-r17 |  | Not Present |  |  |
| musim-AperiodicGap-r17 |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.2.1.6.3.3-4: MUSIM-GapConfig -r17 for Aperiodic Gap (Table 8.1.2.1.6.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | Not Present |  |  |
| musim-GapToAddModList-r17 | Not Present |  |  |
| musim-AperiodicGap-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 SEQUENCE { |  |  |  |
| starting-SFN-r17 | 0 |  |  |
| startingSubframe-r17 | 0 |  |  |
| } |  |  |  |  |  |  |  |
| musim-GapLength-r17 |  | ms10 |  |  |  | px_NR_SupportAperiodicMusimGapPattern=27 |  |
|  |  | ms20 |  |  |  | px_NR_SupportAperiodicMusimGapPattern=28 |  |
| musim-GapRepetitionAndOffset-r17 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>