3GPP TSG-RAN5 Meeting #102	R5-240432

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3030 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Move all V2X sub-clauses from 4.7 into a new clause 4.7D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | V2X message is not NAS message and clause 4.7 is used for 5G NAS message |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new clause 4.7D for Defalut V2X message and information elements contents. - Move “4.7.4 contents of V2X message” to “4.7D.1 contents of V2X message” - Move “4.7.5 V2X information elements” to “4.7D.2 V2X information elements” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G NAS message clause include non-NAS message |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7,4.9.22,4.9.23,4.9.30,4.11.2,4.8.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.7.4	Void

4.7.5	Void

4.7.6	Contents of UE Policy Delivery messages

–	MANAGE UE POLICY COMMAND

Table 4.7.6-1: MANAGE UE POLICY COMMAND

| Derivation Path: TS 24.501 Table D.5.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PTI |  | Any value from 1 to 254 |  |  |
| MANAGE UE POLICY COMMAND message identity |  | ’0000 0001’B | MANAGE UE POLICY COMMAND message |  |
| UE policy section management list |  |  |  |  |
| Length of UE policy section management list contents |  | Set to the actual length of 'UE policy section management list contents' in bytes |  |  |
| UE policy section management list contents |  | 1 entry |  |  |
| UE policy section management sublist (PLMN-1) |  |  |  |  |
| Length of UE policy section management sublist |  | Set to the actual length of 'UE policy section management sublist' in bytes |  |  |
| PLMN ID |  | Set to the PLMN value used in the test case |  |  |
| UE policy section management sublist contents |  |  |  |  |
| Instruction 1 |  |  |  |  |
| Instruction contents length |  | Set to the actual length of 'Instruction contents' in bytes |  |  |
| UPSC |  | '00 01'H | 2 bytes, value set by PCF |  |
| UE policy section contents |  |  |  |  |
| UE policy part 1 |  |  |  |  |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part type |  | ‘0001’B |  | URSP |
|  |  | ‘0010’B |  | ANDSP |
|  |  | ‘0011’B |  | V2XP |
|  |  | ‘0100’B |  | ProSeP |
| UE policy part contents |  | See Table 4.8.5.1-1 |  | URSP |
|  |  | FFS |  | ANDSP |
|  |  | See Table 4.7D.2.1-1 |  | V2XP |
|  |  | See Table 4.7C.1.1-1 |  | ProSeP |

| Condition | Explanation |
| --- | --- |
| URSP | UE Route Selection Policy (URSP) |
| ANDSP | UE Access Network Discovery and Selection Policy (ANDSP) |
| V2XP | UE policies for V2X (V2XP) |
| ProSeP | UE policies for ProSe (ProSeP) |

<End of modified section 1>

<Start of modified section 2>

4.7D	Default V2X message and information elements contents

4.7D.1	Contents of V2X messages

Table 4.7D.1-1 to -6: Void

–	DIRECT LINK ESTABLISHMENT REQUEST

Table 4.7D.1-7: DIRECT LINK ESTABLISHMENT REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK ESTABLISHMENT REQUEST message identity |  | '0000 0001'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| V2X service identifiers |  |  |  |  |
| Length of V2X service identifier contents |  | '04'H |  |  |
| V2X service identifier 1 |  | '00 00 00 01'H |  |  |
| Source user info |  |  |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| UE PC5 unicast signalling security policy |  | '0000 0000'B | Signalling integrity protection not needed, Signalling ciphering not needed. |  |
| Key establishment information container |  | Not Present |  |  |
| Nonce_1 |  | Not Present |  |  |
| MSBs of KNRP-sess ID |  | Not Present |  |  |
| Target user info |  | Not Checked |  | Tx |
| Target user info |  |  |  | Rx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application layer ID contents |  | '00 00 11 00'H |  |  |
| KNRP ID |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK ESTABLISHMENT ACCEPT

Table 4.7D.1-8: DIRECT LINK ESTABLISHMENT ACCEPT

| Derivation Path: TS 24.587 [54] Table 7.3.1.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DIRECT LINK ESTABLISHMENT ACCEPT message identity | '0000 0010'B |  |  |
| Sequence number | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Source user info |  |  |  |
| Length of Application layer ID contents | '04'H |  |  |
| Application Layer ID 1 | '00 00 02 00'H | Application Layer ID in target UE side |  |
| QoS flow descriptions |  |  |  |
| Length of PC5 QoS flow descriptions contents | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0001'B |  |  |
| Operation Code | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters | 5 |  |  |
| E | 1 | parameters list is included |  |
| Associated V2X service identifiers |  |  |  |
| Length of V2X service identifier contents | '04'H |  |  |
| V2X service identifier 1 | '00 00 00 01'H |  |  |
| Parameters list |  |  |  |
| Parameter 1 |  |  |  |
| Parameter identifier | '01'H | PQI |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 22 | Sensor sharing, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |
| Parameter identifier | '02'H | GFBR |  |
| Length of parameter contents | 3 |  |  |
| Parameter contents | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Parameter 3 |  |  |  |
| Parameter identifier | '03'H | MFBR |  |
| Length of parameter contents | 3 |  |  |
| Parameter contents | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Parameter 4 |  |  |  |
| Parameter identifier | '04'H | Averaging window |  |
| Length of parameter contents | 2 |  |  |
| Parameter contents | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |
| Parameter identifier | '06'H | Default priority level |  |
| Length of parameter contents | 1 |  |  |
| Parameter contents | 4 |  |  |
| Configuration of UE PC5 unicast user plane security protection | '0000 0000'B | User plane integrity protection and ciphering is off |  |
| IP address configuration | Not Checked |  | Tx |
| IP address configuration |  |  | Rx |
| IP address configuration IEI | '57'H |  |  |
| IP address configuration content | '0000 0001'B | IPv6 Router |  |
| Link local IPv6 address | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK MODIFICATION REQUEST

Table 4.7D.1-9: DIRECT LINK MODIFICATION REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.4.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK MODIFICATION REQUEST message identity |  | '0000 0100'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Link modification operation code |  | '0000 0011'B | Add new PC5 QoS flow(s) to the existing PC5 unicast link |  |
| QoS flow descriptions |  |  |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0010'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated V2X service identifiers |  |  |  |  |
| Length of V2X service identifier contents |  | '04'H |  |  |
| V2X service identifier 1 |  | '00 00 00 02'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 23 | Platooning between UEs, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0000 1010'B | 10 * 4Mbps = 40Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0001 0100'B | 20 * 4Mbps = 80Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 3 |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK MODIFICATION ACCEPT

Table 4.7D.1-10: DIRECT LINK MODIFICATION ACCEPT

| Derivation Path: TS 24.587 [54] Table 7.3.5.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK MODIFICATION ACCEPT message identity |  | '0000 0101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| QoS flow descriptions |  |  |  |  |
| PC5 QoS flow descriptions IEI |  | '79'H |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0010'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated V2X service identifiers |  |  |  |  |
| Length of V2X service identifier contents |  | '04'H |  |  |
| V2X service identifier 1 |  | '00 00 00 02'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 23 | Platooning between UEs, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0000 1010'B | 10 * 4Mbps = 40Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0111 0000 0000 0001 0100'B | 20 * 4Mbps = 80Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 3 |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK RELEASE REQUEST

Table 4.7D.1-11: DIRECT LINK RELEASE REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.6.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK RELEASE REQUEST message identity |  | '0000 0111'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified | Rx |
|  |  | Not Checked |  | Tx |
| MSB of KNRP ID |  | Not Checked |  | Tx |
|  |  | '00 00'H |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK RELEASE ACCEPT

Table 4.7D.1-12: DIRECT LINK RELEASE ACCEPT

| Derivation Path: TS 24.587 [54] Table 7.3.7.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK RELEASE ACCEPT message identity |  | '0000 1000'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP ID |  | Not Checked |  | Tx |
|  |  | '00 00'H |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK KEEPALIVE REQUEST

Table 4.7D.1-13: DIRECT LINK KEEPALIVE REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.8.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK KEEPALIVE REQUEST message identity |  | '0000 1001'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Keep-alive counter |  | '00 00 00 00'H | Increase by 1 after each keep-alive procedure. |  |
| Maximum inactivity period |  | Not Checked |  | Tx |
| Maximum inactivity period |  |  |  | Rx |
| Maximum inactivity period IEI |  | '55'H |  |  |
| Maximum inactivity period contents |  | '00 00 00 06'H | 6 seconds, higher value than T5003=5s |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK KEEPALIVE RESPONSE

Table 4.7D.1-14: DIRECT LINK KEEPALIVE RESPONSE

| Derivation Path: TS 24.587 [54] Table 7.3.9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK KEEPALIVE RESPONSE message identity |  | '0000 1010'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Keep-alive counter |  | '00 00 00 01'H |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK AUTHENTICATION REQUEST

Table 4.7D.1-15: DIRECT LINK AUTHENTICATION REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK AUTHENTICATION REQUEST message identity |  | '0000 1011'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Key establishment information container |  |  |  |  |
| Length of key establishment information container contents |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Key establishment information container contents |  | Not checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK AUTHENTICATION RESPONSE

Table 4.7D.1-16: DIRECT LINK AUTHENTICATION RESPONSE

| Derivation Path: TS 24.587 [54] Table 7.3.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK AUTHENTICATION RESPONSE message identity |  | '0000 1100'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Key establishment information container |  |  |  |  |
| Length of key establishment information container contents |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Key establishment information container contents |  | Not checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK AUTHENTICATION REJECT

Table 4.7D.1-17: DIRECT LINK AUTHENTICATION REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.12.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK AUTHENTICATION REJECT message identity |  | '0000 1101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause value |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK SECURITY MODE COMMAND

Table 4.7D.1-18: DIRECT LINK SECURITY MODE COMMAND

| Derivation Path: TS 24.587 [54] Table 7.3.13.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK SECURITY MODE COMMAND message identity |  | '0000 1110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| Selected security algorithms |  | '0000 0000'B |  |  |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| UE PC5 unicast signalling security policy |  |  |  |  |
| UE PC5 unicast signalling security policy IEI |  | '59'H |  |  |
| Signalling integrity protection policy |  | '000'B | Signalling integrity protection not needed |  |
| Signalling ciphering policy |  | '000'B | Signalling ciphering not needed. |  |
| Nonce_2 |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| LSBs of KNRP-sess ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| Key establishment information container |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |
| MSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not Present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK SECURITY MODE COMPLETE

Table 4.7D.1-19: DIRECT LINK SECURITY MODE COMPLETE

| Derivation Path: TS 24.587 [54] Table 7.3.14.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK SECURITY MODE COMPLETE message identity |  | '0000 1111'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| QoS flow descriptions |  |  |  |  |
| Length of PC5 QoS flow descriptions contents |  | Set to the actual length of 'PC5 QoS flow descriptions contents' in bytes |  |  |
| PC5 QoS flow description 1 |  |  |  |  |
| PQFI |  | '00 0001'B |  |  |
| Operation Code |  | '001'B | Create new PC5 QoS flow description |  |
| Number of parameters |  | 5 |  |  |
| E |  | 1 | parameters list is included |  |
| Associated V2X service identifiers |  |  |  |  |
| Length of V2X service identifier contents |  | '04'H |  |  |
| V2X service identifier 1 |  | '00 00 00 01'H |  |  |
| Parameters list |  |  |  |  |
| Parameter 1 |  |  |  |  |
| Parameter identifier |  | '01'H | PQI |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 22 | Sensor sharing, See Table 5.4.4-1 in TS 23.287[xx] |  |
| Parameter 2 |  |  |  |  |
| Parameter identifier |  | '02'H | GFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Parameter 3 |  |  |  |  |
| Parameter identifier |  | '03'H | MFBR |  |
| Length of parameter contents |  | 3 |  |  |
| Parameter contents |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Parameter 4 |  |  |  |  |
| Parameter identifier |  | '04'H | Averaging window |  |
| Length of parameter contents |  | 2 |  |  |
| Parameter contents |  | '0000 0111 1101 0000'B | 2000ms |  |
| Parameter 5 |  |  |  |  |
| Parameter identifier |  | '06'H | Default priority level |  |
| Length of parameter contents |  | 1 |  |  |
| Parameter contents |  | 4 |  |  |
| UE PC5 unicast user plane security policy |  | '0000 0000'B | Signalling integrity protection not needed, Signalling ciphering not needed. |  |
| IP address configuration |  | Not Checked |  | Tx |
| IP address configuration |  |  |  | Rx |
| IP address configuration IEI |  | '57'H |  |  |
| IP address configuration content |  | '0000 0001'B | IPv6 Router |  |
| Link local IPv6 address |  | Not Present |  | Rx |
|  |  | Not Checked |  | Tx |
| LSBs of KNRP ID |  | Not Checked |  | Tx |
|  |  | Not present |  | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK SECURITY MODE REJECT

Table 4.7D.1-20: DIRECT LINK SECURITY MODE REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.15.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK SECURITY MODE REJECT message identity |  | '0001 0000'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK REKEYING REQUEST

Table 4.7D.1-21: DIRECT LINK REKEYING REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.16.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK REKEYING REQUEST message identity |  | '0001 0001'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| UE security capabilities |  |  |  |  |
| Length of UE security capabilities contents |  | '02'H |  |  |
| 5G-EA algorithms |  | '1000 0000'B | 5G-EA0 supported |  |
| 5G-IA algorithms |  | '1000 0000'B | 5G-IA0 supported |  |
| Key establishment information container |  | Not Checked |  |  |
| Nonce_1 |  | Not Checked |  |  |
| MSBs of KNRP-sess ID |  | Not Checked |  |  |
| Re-authentication indication |  |  |  |  |
| Re-authentication indication IEI |  | '56'H |  |  |
| Re-authentication indication contents |  | '0000 0000'B | KNRP is not requested to be refreshed |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK REKEYING RESPONSE

Table 4.7D.1-22: DIRECT LINK REKEYING RESPONSE

| Derivation Path: TS 24.587 [54] Table 7.3.17.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK REKEYING RESPONSE message identity |  | '0001 0010'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK IDENTIFIER UPDATE REQUEST

Table 4.7D.1-23: DIRECT LINK IDENTIFIER UPDATE REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.18.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE REQUEST message identity |  | '0001 0011'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| MSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Source layer-2 ID |  | Not checked |  | Tx |
|  |  | '00 00 10'H | New Layer-2 ID in initiating UE side | Rx |
| Source user info |  | Not checked |  | Tx |
| Source layer-2 ID |  | Not checked |  | Tx |
| Application layer ID IEI |  | '57'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H | New application Layer ID in initiating UE side |  |
| Source link local IPv6 address |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK IDENTIFIER UPDATE ACCEPT

Table 4.7D.1-24: DIRECT LINK IDENTIFIER UPDATE ACCEPT

| Derivation Path: TS 24.587 [54] Table 7.3.19.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE ACCEPT message identity |  | '0001 0100'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| MSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Source layer-2 ID |  | Not checked |  | Tx |
|  |  | '00 00 20'H |  | Rx |
| Target layer-2 ID |  | '00 00 10'H |  | Tx |
|  |  | Same as the source layer-2 ID in DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition |  | Rx |
| Target user info |  |  |  | Tx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H | New application Layer ID in initiating UE side |  |
| Target user info |  | Not Present |  | Rx AND (NOT Update_user_info) |
| Target user info |  |  |  | Rx AND Update_user_info |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | Same as the source user info in DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition | New application Layer ID in initiating UE side |  |
| Target link local IPv6 address |  | Not Present |  |  |
| Source user info |  | Not checked |  | Tx |
| Source user info |  |  |  | Rx |
| Application layer ID IEI |  | '57'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H | New application Layer ID in target UE side |  |
| Source link local IPv6 address |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the DIRECT LINK IDENTIFIER UPDATE REQUEST. |

–	DIRECT LINK IDENTIFIER UPDATE ACK

Table 4.7D.1-25: DIRECT LINK IDENTIFIER UPDATE ACK

| Derivation Path: TS 24.587 [54] Table 7.3.20.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE ACK message identity |  | '0001 0101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Target layer-2 ID |  | '00 00 20'H |  | Tx |
|  |  | Same as the source layer-2 ID in DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition |  | Rx |
| Target user info |  |  |  | Tx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H | New application Layer ID in target UE side |  |
| Target user info |  | Not Present |  | Rx AND (NOT Update_user_info) |
| Target user info |  |  |  | Rx AND Update_user_info |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | Same as the source user info in DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition | New application Layer ID in target UE side |  |
| Target link local IPv6 address |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the DIRECT LINK IDENTIFIER UPDATE ACCEPT |

–	DIRECT LINK IDENTIFIER UPDATE REJECT

Table 4.7D.1-26: DIRECT LINK IDENTIFIER UPDATE REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.21.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE REJECT message identity |  | '0001 0110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK MODIFICATION REJECT

Table 4.7D.1-27: DIRECT LINK MODIFICATION REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.22.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK MODIFICATION REJECT message identity |  | '0000 0110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK ESTABLISHMENT REJECT

Table 4.7D.1-28: DIRECT LINK ESTABLISHMENT REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.23.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK ESTABLISHMENT REJECT message identity |  | '0000 0011'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

4.7D.2	V2X information elements

4.7D.2.1	V2X information elements for UE policy part

–	UE policy part when UE policy part type = {V2XP}

Table 4.7D.2.1-1: UE policy part when UE policy part type = {V2XP}

| Derivation Path: TS 24.588 Figure 5.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part contents length |  | Set to the actual length of 'UE policy part contents' in bytes |  |  |
| UE policy part type |  | ‘0001’B | UE policy part type={ V2XP } |  |
| Spare |  | ‘0000’B |  |  |
| UE policy part contents={V2XP contents} |  | See Table 4.7D.2.1-2 |  |  |

–	V2XP contents

Table 4.7D.2.1-2: V2XP contents

| Derivation Path: TS 24.588 Figure 5.2.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| V2XP info #1 |  | See Table 4.7D.2.1-3 |  |  |
| V2XP info #2 |  | FFS |  |  |

–	V2XP info

Table 4.7D.2.1-3: V2XP info

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Spare |  | ‘0000’B |  |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| V2XP info contents |  | See Table 4.7D.2.2-1 |  |  |

4.7D.2.2	V2X information elements of UE policies for V2X communication over PC5

–	V2XP info = {UE policies for V2X communication over PC5}

Table 4.7D.2.2-1: V2XP info = {UE policies for V2X communication over PC5}

| Derivation Path: TS 24.588 Figure 5.3.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| V2XP info type |  | ‘0001’B | UE policies for V2X communication over PC5 |  |
| Length of V2XP info contents |  | Set to the actual length of 'V2XP info contents' in bytes |  |  |
| Validity timer |  | 'FF FF FF FF FF FF FF FF FF FF'H | 5 bytes, Expiration UTC time of validity of the UE policies, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| VSITPMRI |  | ‘1’B | ‘V2X service identifier to PC5 RAT and Tx profiles mapping rules’ is present |  |
| Served by E-UTRA or served by NR |  | See Table 4.7D.2.2-2 |  |  |
| Not served by E-UTRA and not served by NR |  | See Table 4.7D.2.2-6 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rules |  | See Table 4.7D.2.2-12 |  |  |
| Privacy config |  | See Table 4.7D.2.2-15 |  |  |
| V2X communication over PC5 in E-UTRA-PC5 |  | See Table 4.7D.2.2-19 |  |  |
| V2X communication over PC5 in NR-PC5 |  | See Table 4.7D.2.2-31 |  |  |

–	Served by E-UTRA or served by NR

Table 4.7D.2.2-2: Served by E-UTRA or served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of served by E-UTRA or served by NR contents |  | Set to the actual length of 'served by E-UTRA or served by NR contents' in bytes |  |  |
| Authorized PLMN and RATs combinations |  | See Table 4.7D.2.2-3 |  |  |

–	Authorized PLMN and RATs combinations

Table 4.7D.2.2-3: Authorized PLMN and RATs combinations

| Derivation Path: TS 24.588 Figure 5.3.1.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of authorized PLMN and RATs combinations contents |  | Set to the actual length of 'authorized PLMN and RATs combinations contents' in bytes |  |  |
| Authorized PLMN and RATs combination 1 |  | See Table 4.7D.2.2-4 |  |  |
| Authorized PLMN and RATs combination 2 |  | FFS |  |  |

–	Authorized PLMN and RATs combination

Table 4.7D.2.2-4: Authorized PLMN and RATs combination

| Derivation Path: TS 24.588 Figure 5.3.1.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PLMN ID |  | Set according to parameter given in test case |  |  |
| NPIEN |  | '1'B | UE is authorized to use V2X communication over NR-PC5 when served by E-UTRA or served by NR |  |
| EPIEN |  | '1'B | UE is authorized to use V2X communication over E-UTRA-PC5 when served by E-UTRA or served by NR |  |

–	PLMN ID

Table 4.7D.2.2-5: PLMN ID

| Derivation Path: TS 24.588 Figure 5.3.1.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MCC digit 1 |  | Set according to parameter given in test case |  |  |
| MCC digit 2 |  | Set according to parameter given in test case |  |  |
| MCC digit 3 |  | Set according to parameter given in test case |  |  |
| MNC digit 3 |  | Set according to parameter given in test case |  |  |
| MNC digit 1 |  | Set according to parameter given in test case |  |  |
| MNC digit 2 |  | Set according to parameter given in test case |  |  |

–	Not served by E-UTRA and not served by NR

Table 4.7D.2.2-6: Not served by E-UTRA and not served by NR

| Derivation Path: TS 24.588 Figure 5.3.1.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of not served by E-UTRA and not served by NR contents |  | Set to the actual length of 'not served by E-UTRA and not served by NR contents' in bytes |  |  |
| VPNENNI |  | '1'B | UE is authorized to use V2X communication over PC5 |  |
| NPINENN |  | '1'B | UE is authorized to use V2X communication over NR-PC5 when not served by E-UTRA and not served by NR |  |
| EPINENN |  | '1'B | UE is authorized to use V2X communication over E-UTRA-PC5 when not served by E-UTRA and not served by NR |  |
| E-UTRA radio parameters per geographical area list |  | See Table 4.7D.2.2-7 |  |  |
| NR radio parameters per geographical area list |  | See Table 4.7D.2.2-7 |  |  |

–	Radio parameters per geographical area list

Table 4.7D.2.2-7: Radio parameters per geographical area list

| Derivation Path: TS 24.588 Figure 5.3.1.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area list contents |  | Set to the actual length of 'radio parameters per geographical area list contents' in bytes |  |  |
| Radio parameters per geographical area info 1 |  | See Table 4.7D.2.2-8 |  |  |
| Radio parameters per geographical area info 2 |  | FFS |  |  |

–	Radio parameters per geographical area info

Table 4.7D.2.2-8: Radio parameters per geographical area info

| Derivation TS 24.588 Figure 5.3.1.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents |  | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area |  | See Table 4.7D.2.2-9 |  |  |
| Radio parameters |  | See Table 4.7D.2.2-11 |  |  |
| MI |  | '1'B | Operator managed |  |

–	Geographical area

Table 4.7D.2.2-9: Geographical area

| Derivation Path: TS 24.588 Figure 5.3.1.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical area contents |  | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 |  | See Table 4.7D.2.2-10 |  |  |
| Coordinate 2 |  | See Table 4.7D.2.2-10 |  |  |
| Coordinate 3 |  | See Table 4.7D.2.2-10 |  |  |

–	Coordinate area

Table 4.7D.2.2-10: Coordinate area

| Derivation Path: TS 24.588 Figure 5.3.1.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Latitude |  | 35.753056 |  | Coordinate 1 |
|  |  | 35.735278 |  | Coordinate 2 |
|  |  | 35.744167 |  | Coordinate 3 |
| Longitude |  | 139.689167 |  | Coordinate 1 |
|  |  | 139.689167 |  | Coordinate 2 |
|  |  | 139.709167 |  | Coordinate 3 |

–	Radio parameters

Table 4.7D.2.2-11: Radio parameters

| Derivation Path: TS 24.588 Figure 5.3.1.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of radio parameters contents |  | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents |  | See Table 4.10.1.1-1: SL-V2X-Preconfiguration in TS 36.508 [2] |  | E-UTRAN V2X |
|  |  | See Table 4.10.1-1: SL-PreconfigurationNR |  | NR V2X |

–	V2X service identifier to PC5 RAT and Tx profiles mapping rules

Table 4.7D.2.2-12: V2X service identifier to PC5 RAT and Tx profiles mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 1 |  | See Table 4.7D.2.2-13 |  |  |
| V2X service identifier to PC5 RAT and Tx profiles mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 RAT and Tx profiles mapping rule

Table 4.7D.2.2-13: V2X service identifier to PC5 RAT and Tx profiles mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 RAT and Tx profiles mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 RAT and Tx profiles mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| PC5 RAT |  | ‘00’B |  | E-UTRA-PC5 |
|  |  | ‘01’B |  | NR-PC5 |
| Length of E-UTRA-PC5 Tx profiles |  | Set to the actual length of 'E-UTRA-PC5 Tx profiles' in bytes |  | E-UTRA-PC5 |
| E-UTRA-PC5 Tx profiles |  | rel14 | UE shall use Release 14 compatible format (i.e. using MCS table in Table 8.6.1-1 with 64 QAM indices overridden by 16QAM in TS 36.213 [23] and not Rel-15 feature) to transmit the corresponding V2X packet, Refer to IE v2x-TxProfileList in TS 36.331 | E-UTRA-PC5 |

–	V2X service identifiers

Table 4.7D.2.2-14: V2X service identifiers

| Derivation Path: TS 24.588 Figure 5.3.1.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifiers contents |  | Set to the actual length of 'V2X service identifiers contents' in bytes |  |  |
| V2X service identifier 1 |  | '00 00 00 01'H | 4 bytes for each V2X service identifier |  |
| V2X service identifier 2 |  | '00 00 00 02'H |  |  |

–	Privacy config

Table 4.7D.2.2-15: Privacy config

| Derivation Path: TS 24.588 Figure 5.3.1.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of privacy config contents |  | Set to the actual length of 'privacy config contents' in bytes |  |  |
| V2X services requiring privacy |  | See Table 4.7D.2.2-16 |  |  |
| Privacy timer |  | 'FF FF'H | Expressed in units of seconds, after which the UE shall change the source Layer-2 ID self-assigned by the UE |  |

–	V2X services requiring privacy

Table 4.7D.2.2-16: V2X services requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services requiring privacy contents |  | Set to the actual length of 'V2X services requiring privacy contents' in bytes |  |  |
| V2X service requiring privacy 1 |  | See Table 4.7D.2.2-17 |  |  |
| V2X service requiring privacy 2 |  | FFS |  |  |

–	V2X service requiring privacy

Table 4.7D.2.2-17: V2X service requiring privacy

| Derivation Path: TS 24.588 Figure 5.3.1.17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service requiring privacy contents |  | Set to the actual length of 'V2X service requiring privacy contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Geographical areas |  | See Table 4.7D.2.2-18 |  |  |

–	Geographical areas

Table 4.7D.2.2-18: Geographical areas

| Derivation Path: TS 24.588 Figure 5.3.1.18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of geographical areas contents |  | Set to the actual length of 'geographical areas contents' in bytes |  |  |
| Geographical area 1 |  | See Table 4.7D.2.2-9 |  |  |
| Geographical area 2 |  | FFS |  |  |

–	V2X communication over PC5 in E-UTRA-PC5

Table 4.7D.2.2-19: V2X communication over PC5 in E-UTRA-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in E-UTRA-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in E-UTRA-PC5 contents' in bytes |  |  |
| PPMR |  | '1'B | PPPP to PDB mapping rules field is present |  |
| VSAPI |  | '1'B | V2X services authorized for PPPR field is present |  |
| VSIEFMRI |  | '1'B | V2X service identifier to V2X E-UTRA frequency mapping rules field is present |  |
| DDL2II |  | '1'B | Default destination layer-2 ID field is present |  |
| V2X service identifier to destination layer-2 ID mapping rules |  | See Table 4.7D.2.2-20 |  |  |
| PPPP to PDB mapping rules |  | See Table 4.7D.2.2-22 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rules |  | See Table 4.7D.2.2-24 |  |  |
| V2X services authorized for PPPR |  | See Table 4.7D.2.2-29 |  |  |
| Default destination layer-2 ID |  | '00 00 40'H | 3 bytes, Random value |  |

–	V2X service identifier to destination layer-2 ID mapping rules

Table 4.7D.2.2-20: V2X service identifier to destination layer-2 ID mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 1 |  | See Table 4.7D.2.2-21 |  |  |
| V2X service identifier to destination layer-2 ID mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID mapping rule

Table 4.7D.2.2-21: V2X service identifier to destination layer-2 ID mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Destination layer-2 ID |  | '00 00 41'H |  |  |

–	PPPP to PDB mapping rules

Table 4.7D.2.2-22: PPPP to PDB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PPPP to PDB mapping rules contents |  | Set to the actual length of 'PPPP to PDB mapping rules contents' in bytes |  |  |
| PPPP to PDB mapping rule 1 |  | See Table 4.7D.2.2-23 |  |  |
| PPPP to PDB mapping rule 2 |  | FFS |  |  |

–	PPPP to PDB mapping rule

Table 4.7D.2.2-23: PPPP to PDB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PPPP |  | '000'B | PPPP value 1 |  |
| PDB |  | '00 32'H | 50ms |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rules

Table 4.7D.2.2-24: V2X service identifier to V2X E-UTRA frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.24 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 1 |  | See Table 4.7D.2.2-25 |  |  |
| V2X service identifier to V2X E-UTRA frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X E-UTRA frequency mapping rule

Table 4.7D.2.2-25: V2X service identifier to V2X E-UTRA frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X E-UTRA frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X E-UTRA frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| V2X E-UTRA frequencies with geographical areas list |  | See Table 4.7D.2.2-26 |  |  |

–	V2X E-UTRA frequencies with geographical areas list

Table 4.7D.2.2-26: V2X E-UTRA frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.26 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas list contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas list contents' in bytes |  |  |
| V2X E-UTRA frequencies with geographical areas info 1 |  | See Table 4.7D.2.2-27 |  |  |
| V2X E-UTRA frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X E-UTRA frequencies with geographical areas info

Table 4.7D.2.2-27: V2X E-UTRA frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.27 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies with geographical areas info contents |  | Set to the actual length of 'V2X E-UTRA frequencies with geographical areas info contents' in bytes |  |  |
| V2X E-UTRA frequencies |  | See Table 4.7D.2.2-28 |  |  |
| Geographical areas |  | See Table 4.7D.2.2-18 |  |  |

–	V2X E-UTRA frequencies

Table 4.7D.2.2-28: V2X E-UTRA frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X E-UTRA frequencies contents |  | Set to the actual length of 'V2X E-UTRA frequencies contents' in bytes |  |  |
| V2X E-UTRA frequency 1 |  | Downlink E-UTRA ARFCN under test. f5 of Table 6.2.3.5-1 in TS 36.508 [2] |  | SIG |
|  |  | Downlink E-UTRA ARFCN under test. Mid Range EARFCN value of Table 4.3.1.2.15-1 in TS 36.508 [2]. |  |  |
| V2X E-UTRA frequency 2 |  | FFS |  |  |

–	V2X services authorized for PPPR

Table 4.7D.2.2-29: V2X services authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.29 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X services authorized for PPPR contents |  | Set to the actual length of 'V2X services authorized for PPPR contents' in bytes |  |  |
| V2X service authorized for PPPR 1 |  | See Table 4.7D.2.2-30 |  |  |
| V2X service authorized for PPPR 2 |  | FFS |  |  |

–	V2X service authorized for PPPR

Table 4.7D.2.2-30: V2X service authorized for PPPR

| Derivation Path: TS 24.588 Figure 5.3.1.30 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service authorized for PPPR contents |  | Set to the actual length of 'V2X service authorized for PPPR contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| PPPR |  | '000'B | PPPR value 1 |  |

–	V2X communication over PC5 in NR-PC5

Table 4.7D.2.2-31: V2X communication over PC5 in NR-PC5

| Derivation Path: TS 24.588 Figure 5.3.1.31 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X communication over PC5 in NR-PC5 contents |  | Set to the actual length of 'V2X communication over PC5 in NR-PC5 contents' in bytes |  |  |
| VSINFMRI |  | '1'B | V2X service identifier to V2X NR frequency mapping rules field is present |  |
| DDL2IBI |  | '1'B | Default destination layer-2 ID for broadcast field is present |  |
| V2X service identifier to V2X NR frequency mapping rules |  | See Table 4.7D.2.2-32 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rules |  | See Table 4.7D.2.2-37 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rules |  | See Table 4.7D.2.2-39 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules |  | See Table 4.7D.2.2-41 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rules |  | See Table 4.7D.2.2-43 |  |  |
| AS configuration |  | See Table 4.7D.2.2-45 |  |  |
| Default destination layer-2 ID for broadcast |  | '00 00 50'H |  |  |
| NR-PC5 unicast security policies |  | See Table 4.7D.2.2-49 |  |  |
| V2X service identifier to default mode of communication mapping rules |  | See Table 4.7D.2.2-52 |  |  |

–	V2X service identifier to V2X NR frequency mapping rules

Table 4.7D.2.2-32: V2X service identifier to V2X NR frequency mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.32 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rules contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rules contents' in bytes |  |  |
| V2X service identifier to V2X NR frequency mapping rule 1 |  | See Table 4.7D.2.2-33 |  |  |
| V2X service identifier to V2X NR frequency mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to V2X NR frequency mapping rule

Table 4.7D.2.2-33: V2X service identifier to V2X NR frequency mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to V2X NR frequency mapping rule contents |  | Set to the actual length of 'V2X service identifier to V2X NR frequency mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| V2X NR frequencies with geographical areas list |  | See Table 4.7D.2.2-34 |  |  |

–	V2X NR frequencies with geographical areas list

Table 4.7D.2.2-34: V2X NR frequencies with geographical areas list

| Derivation Path: TS 24.588 Figure 5.3.1.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas list contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas list contents' in bytes |  |  |
| V2X NR frequencies with geographical areas info 1 |  | See Table 4.7D.2.2-35 |  |  |
| V2X NR frequencies with geographical areas info 2 |  | FFS |  |  |

–	V2X NR frequencies with geographical areas info

Table 4.7D.2.2-35: V2X NR frequencies with geographical areas info

| Derivation Path: TS 24.588 Figure 5.3.1.35 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies with geographical areas info contents |  | Set to the actual length of 'V2X NR frequencies with geographical areas info contents' in bytes |  |  |
| V2X NR frequencies |  | See Table 4.7D.2.2-36 |  |  |
| Geographical areas |  | See Table 4.7D.2.2-18 |  |  |

–	V2X NR frequencies

Table 4.7D.2.2-36: V2X NR frequencies

| Derivation Path: TS 24.588 Figure 5.3.1.36 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X NR frequencies contents |  | Set to the actual length of 'V2X NR frequencies contents' in bytes |  |  |
| V2X NR frequency 1 |  | ARFCN-ValueNR with condition SL_SSB in Table 4.6.3-5 |  |  |
| V2X NR frequency 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rules

Table 4.7D.2.2-37: V2X service identifier to destination layer-2 ID for broadcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.37 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 1 |  | See Table 4.7D.2.2-38 |  |  |
| V2X service identifier to destination layer-2 ID for broadcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for broadcast mapping rule

Table 4.7D.2.2-38: V2X service identifier to destination layer-2 ID for broadcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.38 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for broadcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for broadcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Destination layer-2 ID for broadcast |  | '00 00 51'H |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rules

Table 4.7D.2.2-39: V2X service identifier to destination layer-2 ID for groupcast mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.39 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 1 |  | See Table 4.7D.2.2-40 |  |  |
| V2X service identifier to destination layer-2 ID for groupcast mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for groupcast mapping rule

Table 4.7D.2.2-40: V2X service identifier to destination layer-2 ID for groupcast mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.40 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for groupcast mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for groupcast mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Destination layer-2 ID for groupcast |  | '00 00 52'H |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

Table 4.7D.2.2-41: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.41 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rules contents' in bytes |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 1 |  | See Table 4.7D.2.2-42 |  |  |
| V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

Table 4.7D.2.2-42: V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.42 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents |  | Set to the actual length of 'V2X service identifier to destination layer-2 ID for unicast initial signalling mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Destination layer-2 ID for unicast initial signalling |  | '00 00 53'H |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rules

Table 4.7D.2.2-43: V2X service identifier to PC5 QoS parameters mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.43 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rules contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rules contents' in bytes |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 1 |  | See Table 4.7D.2.2-44 |  |  |
| V2X service identifier to PC5 QoS parameters mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to PC5 QoS parameters mapping rule

Table 4.7D.2.2-44: V2X service identifier to PC5 QoS parameters mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.46 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to PC5 QoS parameters mapping rule contents |  | Set to the actual length of 'V2X service identifier to PC5 QoS parameters mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| RI |  | '1'B | Range field is present |  |
| PLAMBRI |  | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI |  | '1'B | Maximum flow bit rate field is present |  |
| GFBRI |  | '1'B | Guaranteed flow bit rate field is present |  |
| PQI |  | 22 | See Table 5.4.4-1 in TS 23.287 [xx] |  |
| Guaranteed flow bit rate |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate |  | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range |  | '0000 0001 1111 0100'B | 500 meters |  |

–	AS configuration

Table 4.7D.2.2-45: AS configuration

| Derivation Path: TS 24.588 Figure 5.3.1.46a |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of AS configuration contents |  | Set to the actual length of 'AS configuration contents' in bytes |  |  |
| SLRB mapping rules |  | See Table 4.7D.2.2-46 |  |  |

–	SLRB mapping rules

Table 4.7D.2.2-46: SLRB mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.47 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rules contents |  | Set to the actual length of 'SLRB mapping rules contents' in bytes |  |  |
| SLRB mapping rule 1 |  | See Table 4.7D.2.2-47 |  |  |
| SLRB mapping rule 2 |  | FFS |  |  |

–	SLRB mapping rule

Table 4.7D.2.2-47: SLRB mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.48 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of SLRB mapping rule contents |  | Set to the actual length of 'SLRB mapping rule contents' in bytes |  |  |
| PC5 QoS profile |  | See Table 4.7D.2.2-48 |  |  |
| Length of SLRB |  | Set to the actual length of 'SLRB' in bytes |  |  |
| SLRB |  | See Table 4.10.1-1 |  |  |

–	PC5 QoS profile

Table 4.7D.2.2-48: PC5 QoS profile

| Derivation Path: TS 24.588 Figure 5.3.1.49 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of PC5 QoS profile contents |  | Set to the actual length of 'PC5 QoS profile contents' in bytes |  |  |
| MDBVI |  | '0'B | Maximum data burst volume field is absent |  |
| AWI |  | '1'B | Averaging window field is present |  |
| PLOI |  | '1'B | The octet of the priority level is present |  |
| RI |  | '1'B | Range field is present |  |
| PLAMBRI |  | '1'B | Per-link aggregate maximum bit rate field is present |  |
| MFBRI |  | '1'B | Maximum flow bit rate field is present |  |
| GFBRI |  | '1'B | Guaranteed flow bit rate field is present |  |
| PQI |  | 22 | See Table 5.4.4-1 in TS 23.287[xx] |  |
| Guaranteed flow bit rate |  | '0000 0110 0000 0000 0000 1010'B | 10 * 1Mbps = 10Mbps. |  |
| Maximum flow bit rate |  | '0000 0110 0000 0000 0001 0100'B | 20 * 1Mbps = 20Mbps. |  |
| Per-link aggregate maximum bit rate |  | '0000 0110 0000 0000 0000 0010'B | 2 * 1Mbps = 2Mbps. |  |
| Range |  | '0000 0001 1111 0100'B | 500 meters |  |
| Priority level |  | 4 |  |  |
| Averaging window |  | '0000 0111 1101 0000'B | 2000ms |  |
| Maximum data burst volume |  | Not Present |  |  |

–	NR-PC5 unicast security policies

Table 4.7D.2.2-49: NR-PC5 unicast security policies

| Derivation Path: TS 24.588 Figure 5.3.1.50 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents |  | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 |  | See Table 4.7D.2.2-50 |  |  |
| NR-PC5 unicast security policy 2 |  | FFS |  |  |

–	NR-PC5 unicast security policy

Table 4.7D.2.2-50: NR-PC5 unicast security policy

| Derivation Path: TS 24.588 Figure 5.3.1.51 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policy contents |  | Set to the actual length of 'NR-PC5 unicast security policy contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| Security policy |  | See Table 4.7D.2.2-51 |  |  |
| Geographical areas |  | See Table 4.7D.2.2-18 |  |  |

–	Security policy

Table 4.7D.2.2-51: Security policy

| Derivation Path: TS 24.588 Figure 5.3.1.52 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Signalling integrity protection policy |  | '000'B | Signalling integrity protection not needed |  |
| Spare |  | ‘0’B |  |  |
| Signalling ciphering policy |  | '000'B | Signalling ciphering not needed |  |
| Spare |  | ‘0’B |  |  |
| User plane integrity protection policy |  | '000'B | User plane integrity protection not needed |  |
| Spare |  | ‘0’B |  |  |
| User plane ciphering policy |  | '000'B | User plane ciphering not needed |  |
| Spare |  | ‘0’B |  |  |

–	V2X service identifier to default mode of communication mapping rules

Table 4.7D.2.2-52: V2X service identifier to default mode of communication mapping rules

| Derivation Path: TS 24.588 Figure 5.3.1.53 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rules contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rules contents' in bytes |  |  |
| V2X service identifier to default mode of communication mapping rule 1 |  | See Table 4.7D.2.2-53 |  |  |
| V2X service identifier to default mode of communication mapping rule 2 |  | FFS |  |  |

–	V2X service identifier to default mode of communication mapping rule

Table 4.7D.2.2-53: V2X service identifier to default mode of communication mapping rule

| Derivation Path: TS 24.588 Figure 5.3.1.54 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Length of V2X service identifier to default mode of communication mapping rule contents |  | ‘Set to the actual length of 'V2X service identifier to default mode of communication mapping rule contents' in bytes |  |  |
| V2X service identifiers |  | See Table 4.7D.2.2-14 |  |  |
| DMC |  | '00'B | Default mode of communication is set to unicast |  |
| Spare |  | ‘0000 00’B |  |  |

<End of modified section 2>

<Start of modified section 3>

4.8.3.3.3	Default settings of UICC and USIM for V2X

EFUST (USIM Service Table):

Same as clause 4.9.3.4 of TS 36.508 [2].

EFVST (V2X Service Table)

If service n°119 is "available" in the USIM Service Table, this file shall be present. This EF indicates the coding of the V2X management objects and which V2X services are available.

File size:	2 Bytes

Default values:	Bytes 1 to 2 (HEX):	01 02

Coding of the V2X management objects is according to 3GPP TS 24.588 [113].

Service n°2 V2X policy configuration data over PC5 is supported.

NOTE:	The default value for NR support of V2X services is different from that for LTE V2X in clause 4.9.3.4 of TS 36.508 [2].

EFV2XP_PC5 (V2X data policy over PC5)

If service n°2 is "available" in EFVST, this file shall be present. This EF contains V2X in 5GS UE policies over PC5. The format of the V2X in 5GS UE policies over PC5 are specified in 3GPP TS 24.588 [113].

The V2X in 5GS UE policies over PC5 contents:

Table 4.8.3.3.3-1: V2X data policy over PC5

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| V2X data policy over PC5 Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer | 'FF FF FF FF FF FF FF FF FF FF'H | M | X1 |
| Indicator bits | '1000 0000'B | M | 1 |
| Served by E-UTRA or served by NR Tag | '80' | M | 1 |
| Length | X2 | M | Note 2 |
| Served by E-UTRA or served by NR information | See Table 4.7D.2.2-2 | M | X2 |
| Not served by E-UTRA and not served by NR Tag | '81' | O | 1 |
| Length | X3 | O | Note 2 |
| Not served by E-UTRA and not served by NR information | See Table 4.7D.2.2-6 | O | X3 |
| V2X service identifier to Tx profiles mapping rules Tag | '82' | O | 1 |
| Length | X4 | O | Note 2 |
| V2X service identifier to Tx profiles mapping rules information | See Table 4.7D.2.2-12 |  | X4 |
| Privacy config Tag | '83' | O | 1 |
| Length | X5 | O | Note 2 |
| Privacy config information | See Table 4.7D.2.2-15 |  | X5 |
| V2X communication over PC5 in E-UTRA Tag | '84' | O | 1 |
| Length | X6 | O | Note 2 |
| V2X communication over PC5 in E-UTRA information | See Table 4.7D.2.2-19 | O | X6 |
| V2X communication over PC5 in NR Tag | '85 | O | 1 |
| Length | X7 | O | Note 2 |
| V2X communication over PC5 in NR Information | See Table 4.7D.2.2-31 | O | X7 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

<End of modified section 3>

<Start of modified section 4>

4.9.22	Test procedure for establishing unicast mode NR sidelink communication / Initiating UE side

4.9.22.1	Scope

The purpose of this procedure is to establish unicast mode sidelink communication.

4.9.22.2	Procedure description

4.9.22.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.22.2.2	Procedure sequence

Table 4.9.22.2.2-1: Procedure for establishing unicast mode sidelink communication (Initiating UE side)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | Void | - | - |
| - | EXCEPTION: Steps 1a1 to 1b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 1a1 | IF the UE is in state 1N-B or 4-A, the UE is configured by upper layer to establish unicast mode link. NOTE: This can be done by sending AT COMMAND +CCUTLE to close test loop function. | - | - |
| 1b1 | ELSE IF the UE is in state 3N-B, the SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP |
| 1b2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE |
| 2 | The UE sends a DIRECT LINK ESTABLISHMENT REQUEST message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST |
| 3 | The NR-SS-UE sends a DIRECT LINK SECURITY MODE COMMAND message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMMAND |
| 4 | The UE sends a DIRECT LINK SECURITY MODE COMPLETE message. | --> | PC5-S: DIRECT LINK SECURITY MODE COMPLETE |
| 5 | The NR-SS-UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT |
| 6 | The UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. NOTE: Unless specifically specified in the test case prose, the UE shall select the DRB parameters as specified in the pre-configured parameters. | --> | PC5-RRC: RRCReconfigurationSidelink |
| 7 | The NR-SS-UE sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5-RRC: RRCReconfigurationCompleteSidelink |
| 8 | Void | - | - |
| - | EXCEPTION: Steps 8a1 to 8b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 8a1 | IF the UE is in state 1N-B or 4-A, the SS sends AT COMMAND +CCUTLE to open test loop function | - | - |
| 8b1 | ELSE IF the UE is in state 3N-B, the SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP |
| 8b2 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE |

4.9.22.3	Specific message contents

All specific message contents shall be according subclause 4.6 and 4.7B with the exceptions below.

Table 4.9.22.3-1: +CCUTLE (Table 4.9.22.2.2-1, Step 1)

Derivation Path: Table 4.7B-1 with condition Close and Transmit

Table 4.9.22.3-2: CLOSE UE TEST LOOP (Table 4.9.22.2.2-1, Step 1b1)

| Derivation Path: 36.508 [2] Table 4.7A-3 with condition UE TEST LOOP MODE E(V2X Transmission) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode E LB setup |  |  |  |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 0 1 | ‘01’ indicates V2X UE triggered to transmit NR sidelink communication with single spatial layer. |  |

Table 4.9.22.3-3: DIRECT LINK ESTABLISHMENT REQUEST (Table 4.9.22.2.2-1, Step 2)

Derivation Path: Table 4.7D.1-7 with condition Tx

Table 4.9.22.3-4: DIRECT LINK SECURITY MODE COMMAND (Table 4.9.22.2.2-1, Step 3)

Derivation Path: Table 4.7D.1-18 with condition Rx

Table 4.9.22.3-5: DIRECT LINK SECURITY MODE COMPLETE (Table 4.9.22.2.2-1, Step 4)

Derivation Path: Table 4.7D.1-19 with condition Tx

Table 4.9.22.3-6: DIRECT LINK ESTABLISHMENT ACCEPT (Table 4.9.22.2.2-1, Step 5)

Derivation Path: Table 4.7D.1-8 with condition Rx

Table 4.9.22.3-7: RRCReconfigurationSidelink (Table 4.9.22.2.2-1, Step 6)

| Derivation Path: Table 4.6.1A-3 with condition TX and SL_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  | 1 entry |  |  |
| SL-PQFI-r16[1] |  | (1..63) | entry 1 |  |
| } |  |  |  |  |
| sl-MappedQoS-FlowsToReleaseList-r16 |  | Not present |  |  |
| sl-SDAP-Header-r16 |  | present |  |  |
| } |  |  |  |  |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  |  |
| sl-LogicalChannelIdentity-r16 |  | (4..32) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.9.22.3-8: RRCReconfigurationCompleteSidelink (Table 4.9.22.2.2-1, Step 7)

Derivation Path: Table 4.6.1A-4 with condition RX

Table 4.9.22.3-9: +CCUTLE (Table 4.9.22.2.2-1, Step 8a1)

Derivation Path: Table 4.7B-1 with condition Open

4.9.23	Test procedure for establishing unicast mode NR sidelink communication / Peer UE side

4.9.23.1	Scope

The purpose of this procedure is to establish unicast mode sidelink communication.

4.9.23.2	Procedure description

4.9.23.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.23.2.2	Procedure sequence

Table 4.9.23.2.2-1: Procedure for establishing unicast mode sidelink communication (Peer UE side)

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The NR-SS-UE sends a DIRECT LINK ESTABLISHMENT REQUEST message. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST |
| 2 | The UE sends a DIRECT LINK SECURITY MODE COMMAND message. | --> | PC5-S: DIRECT LINK SECURITY MODE COMMAND |
| 3 | The NR-SS-UE sends a DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMPLETE |
| 4 | The UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT |
| 5 | The NR-SS-UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink |
| 6 | The UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5-RRC: RRCReconfigurationCompleteSidelink |

4.9.23.3	Specific message contents

All specific message contents shall be according subclause 4.6 and 4.7B.

Table 4.9.23.3-1: DIRECT LINK ESTABLISHMENT REQUEST (Table 4.9.23.2.2-1, Step 1)

Derivation Path: Table 4.7D.1-7 with condition Rx

Table 4.9.23.3-2: DIRECT LINK SECURITY MODE COMMAND (Table 4.9.23.2.2-1, Step 2)

Derivation Path: Table 4.7D.1-18 with condition Tx

Table 4.9.23.3-3: DIRECT LINK SECURITY MODE COMPLETE (Table 4.9.23.2.2-1, Step 3)

Derivation Path: Table 4.7D.1-19 with condition Rx

Table 4.9.23.3-4: DIRECT LINK ESTABLISHMENT ACCEPT (Table 4.9.23.2.2-1, Step 4)

Derivation Path: Table 4.7D.1-8 with condition Tx

Table 4.9.23.3-5: RRCReconfigurationSidelink (Table 4.9.23.2.2-1, Step 5)

Derivation Path: Table 4.6.1A-3 with condition RX and SL_DRB

Table 4.9.23.3-5: RRCReconfigurationCompleteSidelink (Table 4.9.23.2.2-1, Step 6)

Derivation Path: Table 4.6.1A-4 with condition TX

<End of modified section 4>

<Start of modified section 5>

4.9.30	Test procedure for releasing unicast mode NR sidelink communication

4.9.30.1	Scope

The purpose of this procedure is to release unicast mode sidelink communication.

4.9.30.2	Procedure description

4.9.30.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A with a PC5 unicast mode sidelink communication established.

4.9.30.2.2	Procedure sequence

Table 4.9.30.2.2-1: Procedure for releasing unicast mode NR sidelink communication

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The NR-SS-UE sends a DIRECT LINK RELEASE REQUEST message. | <-- | PC5-S: DIRECT LINK RELEASE REQUEST |
| 2 | The UE sends a DIRECT LINK RELEASE ACCEPT message. | --> | PC5-S: DIRECT LINK RELEASE ACCEPT |

4.9.30.3	Specific message contents

Table 4.9.30.3-1: DIRECT LINK RELEASE REQUEST (Step 1, Table 4.9.30.2.2-1)

Derivation Path: Table 4.7D.1-11 with condition Rx

Table 4.9.30.3-2: DIRECT LINK RELEASE ACCEPT (Step 2, Table 4.9.30.2.2-1)

Derivation Path: Table 4.7D.1-12 with condition Tx

<End of modified section 5>

<Start of modified section 6>

4.11.2	GNSS Scenarios

The following GNSS scenarios shall be used.

-	Rinex navigation data: the required navigation data file(s) available in the GNSS orbital data sig zip file specified in TS 37.571-5 [49] Annex B are given in Table 4.11.2-1.

Table 4.11.2-1: Rinex navigation data files for NR sidelink testing

| GNSS supported by UE | Rinex navigation file(s) (1) |
| --- | --- |
| GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
| GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| BDS | Sig GNSS BDS 2020_9_17 Rinex.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigation data files are used |  |

-	UE location(s) and motion:

Latitude: the simulated latitude(s) are given in Table 4.11.2-3

Longitude: the simulated longitude(s) are given in Table 4.11.2-3

Height: the simulated height is 30m

Motion: the simulated motion(s) are given in Table 4.11.2-2

Table 4.11.2-2: UE location(s) and motion(s) for NR sidelink testing

| Scenario number and description | Step # | Action (Location details given in Table 4.11.2-3) | Notes |
| --- | --- | --- | --- |
| Scenario #1: static in Geographical area #1 Note 1 | 1 | Static at location #1 |  |
| Scenario #2: move from inside Geographical area #1 to outside Geographical area #1 Note 1 | 1 | Static at location #1 |  |
|  | 2 | Trigger from test case to move in a straight line at 15m/s from current location to next location | Simulation leaves Geographical area #1 after 905 m, 60s after the trigger. An additional 1s is added to allow for UE position accuracy of +/- 15m. An additional 10s is added to allow for UE position update. Total time 71s |
|  | 3 | Static at location #2 |  |
| Note 1: Geographical area #1 is defined in clause 4.7D.2.2-9 and is defined by three points with the following coordinates: Point 1: latitude: 35.753056, longitude: 139.689167 Point 2: latitude: 35.735278, longitude: 139.689167 Point 3: latitude: 35.744167, longitude: 139.709167 |  |  |  |

<End of modified section 6>