3GPP TSG-RAN5 Meeting #102	R5-240437

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4229 | rev |  | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS broadcast TC 14.1.2.1-Test Loop Mode C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. 38.331 already indicated how to perform Broadcast MRB establishment and Broadcast MRB release, but Broadcast MRB modification is based on UE implementation.It is possible when UE receive Broadcast MRB configuration(e.g move to a new cell), UE always release all MRB and then establish MRB. It is also possible that UE could not release MRB ,just perform delta configuration. [38.331 5.9.3.1] NOTE: How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation. [38.331 5.9.3.2] The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in. 2.According to 38.321, LCID+G-RNTI identified a logical channel for broadcast MRB.Therefore, same LCID with different G-RNTI can not be treated the same broadcast MRB. [38.331 6.2.1] NOTE 1: For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs. 3.36.509/38.509 request SS start test loop mode C after MRB establishment. However, the senario for UE releasing MRB and then establishing MRB after test loop mode C started is not specificed. [36.509 5.4.2] A prerequisite for UE test loop mode C is that at least one MTCH data radio bearer has been established between SS and UE. [38.509 5.3.2] Same as TS 36.509 [6], subclause 5.4.2 with the following exceptions: - where MTCH data radio bearer is replaced by MBS radio bearer; |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Update test procedure to Close loop mode C after every possible MRB establishment. 2.Update the power level table for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformat UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.2.1	MBS Broadcast/ Service Continuity/ Cell reselection/ frequency prioritization

14.1.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC IDLE state and is receiving MBS broadcast service via broadcast MRB and camped on a cell providing SIB21 indicating a MBS FSAI in mbs-FSAI-IntraFreq-r17 and the same MBS FSAI is also indicated for this MBS broadcast service in MBS User Service Description (USD)}

ensure that {

when { an intra-frequency neighbour cell providing the MBS service and an inter-band neighbour cell not providing the MBS service becomes better than the serving cell }

then { UE performs cell reselection to the intra-frequency cell even if the inter-frequency cell is better and continues MBS reception }

}

(2)

with { UE in NR RRC IDLE state and interested to receive MBS broadcast service and camped on a cell not providing SIB20 }

ensure that {

when { serving cell providing SIB21 indicating a MBS FSAI in mbs-FSAI-InterFreq-r17 and the same MBS FSAI is also indicated for this MBS broadcast service in MBS User Service Description (USD) }

then { UE performs cell reselection to the inter-frequency neighbour cell even if the serving cell is better and starts MBS reception }

}

(3)

with { UE in NR RRC INACTIVE state and is receiving MBS data via broadcast MRB and camped on a cell providing SIB20 and SIB21 indicating a MBS FSAI in mbs-FSAI-IntraFreq-r17 and the same MBS FSAI is also indicated for this MBS broadcast service in MBS User Service Description (USD) }

ensure that {

when { an intra-frequency neighbour cell providing the MBS service and an inter-band neighbour cell not providing the MBS service becomes better than the serving cell }

then { UE performs cell reselection to the intra-frequency cell even if the inter-frequency cell is better and continues MBS reception }

}

14.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.6.5.1; TS 38.304, clause 5.2.4.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.6.5.1]

In RRC_IDLE and RRC_INACTIVE, the UE applies the normal cell reselection rules with the following modifications:

-	the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 [10] are met;

-	when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session) or the UE is no longer interested in receiving the service(s), the UE no longer prioritises the frequency providing these MBS broadcast service(s).

[TS 38.304, clause 5.2.4.1]

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

1)	SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

2)	Either:

-	One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or

-	SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

-	SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as the SIB20 is provided by the cell on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

NOTE 0h:	Example scenarios in which such down-prioritisation may be needed include the cases where camping is not possible for the UE on the MBS broadcast frequency (e.g. the MBS broadcast frequency belongs to a PLMN different from UE's registered PLMN) while the UE can receive the MBS broadcast service when camped on another frequency than the MBS broadcast frequency or current frequency.

[TS 23.247, clause 6.5.4]

The MBS Frequency Selection Area (FSA) ID is used for broadcast MBS session to guide the frequency selection of the UE.

MBS FSA ID identifies a preconfigured area within, and in proximity to, which the cell(s) announces the MBS FSA ID and the associating frequency (details see TS 38.300 [9]).

14.1.2.1.3	Test description

14.1.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1), NR Cell 11 (TAI-2) and NR Cell 23 (TAI-2).

-	The SS configures the NR Cell 23 as the "Serving cell" and NR Cell 1 and NR Cell 11 as "Non-suitable "Off" cell".

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 23.

-	System information combination NR-25 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1 and NR cell 11.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 23(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.1.2.1.3.2	Test procedure sequence

Table 14.1.2.1.3.2-1 and 14.1.2.1.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1, NR Cell 11 and NR Cell 23 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and "T2" are applied at the point indicated in the Main behaviour description in Table 14.1.2.1.3.2-3.

Table 14.1.2.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | -85 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 23 and SrxlevNR Cell 1 > ThreshNR Cell 1, HighP |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -85 | -79 | The power level values are assigned to satisfy RNR Cell 1 < R NR Cell 11 < RNR Cell 23 and Srxlev NR Cell 1 > ThreshServing, LowP |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | -79 | The power level values are assigned to satisfy RNR Cell 11 < R NR Cell 1 < RNR Cell 23 and Srxlev NR Cell 11 > ThreshServing, LowP |

Table 14.1.2.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -100 | Off | -91 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 23 and SrxlevNR Cell 1 > ThreshNR Cell 1, HighP |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -100 | -91 | -82 | The power level values are assigned to satisfy RNR Cell 1 < R NR Cell 11 < RNR Cell 23 and Srxlev NR Cell 1 > ThreshServing, LowP |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -100 | -82 | The power level values are assigned to satisfy RNR Cell 11 R NR Cell 1 < RNR Cell 23 and Srxlev NR Cell 11 > ThreshServing, LowP |

Table 14.1.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts to broadcast SIB21 (according to System information combination NR-24) as defined in TS 38.508-1 [4] clause 4.4.3.1.2) on NR Cell 23 including mbs-FSAI-InterFreqList-r17 indicating MBS-FSAI-r17=1 for the frequency of NR Cell 1. | - | - | - | - |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification on NR Cell 23. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 4 | UE is made interested in receiving MBS service ID ‘000001’H associated with MBS FSA ID 1. (Note 1) | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message on NR Cell 1. | --> | NR RRC: RRCSetupRequest | 2 | P |
| 6-10 | Steps 2 to 6a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 1. | - | - | - | - |
| 11 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1 | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 18-20, the steps described in Table 14.1.2.1.3.2-4 may take place, depending on the UE implementation. | - | - | - | - |
| 12-20 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | EXCEPTION: Step 21 is repeated 5 times. | - | - | - | - |
| 21 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 22-29 | Void. | - | - | - | - |
| 30 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 31 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 32 | Check: Is the number of reported MBS Packets received on the MTCH in step 31 greater than zero? | - | - | 2 | P |
| 33 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 34 | The SS changes NR Cell 1, NR Cell 11 and NR Cell 23 levels according to the row "T1" in table Table 14.1.2.1.3.2-1/2. | - | - | - | - |
| 35 | The UE transmits an RRCSetupRequest message on NR Cell 11. | --> | NR RRC: RRCSetupRequest | 1 | P |
| 36-40 | Steps 2 to 6a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 41 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 11. | - | - | - | - |
| 41A-41H | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 11. | - | - | - | - |
| 41I | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 41J | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 41K-41L | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| 41M | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 42 is repeated 5 times. | - | - | - | - |
| 42 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| - | EXCEPTION: In parallel to the events described in steps 49-52, the steps described in Table 14.1.2.1.3.2-4 may takeplace, depending on the UE implementation. | - | - | - | - |
| 43-50 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 11. | - | - | - | - |
| 51 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 52 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 53 | Check: Is the number of reported MBS Packets received on the MTCH in step 52 greater than zero? | - | - | 1 | P |
| - | EXCEPTION: Steps 54a1-54a21 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if inactiveState is configured | - | - | - | - |
| 54a1 | IF pc_inactiveState THEN the The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 54a2 | The SS changes NR Cell 1, NR Cell 11 and NR Cell 23 levels according to the row "T2" in table Table 14.1.2.1.3.2-1/2. | - | - | - | - |
| 54a3 | The UE transmits an RRCSetupRequest message on NR Cell 1. | --> | NR RRC: RRCSetupRequest | 3 | P |
| 54a4-54a8 | Steps 2 to 6a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 1. | - | - | - | - |
| 54a9 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| 54a9A-54a9H | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 54a9I | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 54a9J | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 54a9K-54a9L | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| 54a9M | The SS transmits an RRCRelease message including suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 54a10 is repeated 5 times. | - | - | - | - |
| 54a10 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| - | EXCEPTION: In parallel to the events described in steps 54a17-54a20, the steps described in Table 14.1.2.1.3.2-4 may takeplace, depending on the UE implementation. | - | - | - | - |
| 54a11-54a18 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1. | - | - |  |  |
| 54a19 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 54a20 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 54a21 | Check: Is the number of reported MBS Packets received on the MTCH in step 54a20 greater than zero? | - | - | 3 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

Table 14.1.2.1.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE transmits an MBSInterestIndication message. | --> | MBSInterestIndication | - | - |

14.1.2.1.3.3	Specific message contents

Table 14.1.2.1.3.3-1: SIB1 of NR Cell 23 (preamble and all steps, Table 14.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 14.1.2.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| si-SchedulingInfo-v1700 | Not present |  | Preamble and NR Cell 23 |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { | 1 entry |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus-r17 | broadcasting |  |  |
| si-WindowPosition-r17 | 3 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |
| type1-r17 | sibType21 |  | Step1-64 and NR Cell 23 |
| } |  |  |  |
| valueTag-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.1.3.3-2: SIB1 of NR Cell 1 and NR Cell 11 (preamble and all steps, Table 14.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 14.1.2.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { | 2 entries |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus-r17 | broadcasting |  |  |
| si-WindowPosition-r17 | 3 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |
| type1-r17 | sibType20 |  |  |
| } |  |  |  |
| valueTag-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SchedulingInfo2-r17 [2] SEQUENCE { |  | entry 2 |  |
| si-BroadcastStatus-r17 | broadcasting |  |  |
| si-WindowPosition-r17 | 4 | entry number for si-SchedulingInfo in SIB1 +2 |  |
| si-Periodicity-r17 | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |
| type1-r17 | sibType21 |  |  |
| } |  |  |  |
| valueTag-r17 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.1.3.3-3: ServingCellConfigCommonSIB (Table 14.1.2.1.3.3-1 and Table 14.1.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.2.1.3.3-4 |  |
| } |  |  |  |

Table 14.1.2.1.3.3-4: DownlinkConfigCommonSIB (Table 14.1.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.2.1.3.3-5 |  |
| } |  |  |  |

Table 14.1.2.1.3.3-5: BWP-DownlinkCommon (Table 14.1.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditioni MBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.1.3.3-6: SIB21 of NR Cell 1 and NR Cell 11 and NR Cell 23 (all steps, Table 14.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 | Not present |  | NR Cell 23 |
| mbs-FSAI-IntraFreq-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  | NR Cell 1, NR Cell 11 |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| mbs-FSAI-InterFreqList-r17 | Not present |  | NR Cell 1, NR Cell 11 |
| mbs-FSAI-InterFreqList-r17 SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 { | 1 entry |  | NR Cell 23 |
| MBS-FSAI-InterFreq-r17[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq-r17 | ARFCN-ValueNR of NR Cell 1 |  |  |
| mbs-FSAI-List-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.1.3.3-7: ACTIVATE TEST MODE (preamble, Table 14.1.2.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.2.1.3.3-8: CLOSE UE TEST LOOP (step 20a1, 41K, 54a9K, Table 14.1.2.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Broadcast MRB

Table 14.1.2.1.3.3-9: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 30, step 51, step 53a19, Table 14.1.2.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.1.2.1.3.3-10: RRCRelease (step 54a1,step54a9M, Table 14.1.2.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

<End of modified section 1>