3GPP TSG-RAN5 Meeting #102	R5-240449

# Athens, Greece, 26th February – 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2893 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resubmission of Annex B Corrections to BWP RMCs and configurations for RedCap RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETSI MCC (Huawei, HiSilicon, Starpoint) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - DLBWP.1.3 RedCap/ULBWP.1.3 RedCap are designed as the RMC for RedCap-specific dedicated BWP, and the BW of DLBWP/ULBWP.1.3 is 52/51/66/32 RBs for SCS = 15/30/120/240kHz, which equals to CBW in RedCap test. As a result, in NCD-SSB based measurement TCs (16.6.1.1/2/9/10, 17.6.1.3, 17.6.2.1), DLBWP.1.3 contains both CD-SSB and NCD-SSB. Then a non-conformant RedCap UE which doesn’t support NCD-SSB can still pass the test since it can perform measurement on CD-SSB. To avoid this situation. We suggest change the BW of DLBWP/ULBWP.1.3 same the DLBWP/ULBWP.1.2, i.e. 25/25/32/16 RBs for SCS = 15/30/120/240kHz. - Re-establishment TCs 16.3.2.1.1/2/3/4 - Initial BWP RMC used in 16.3.2.4/5/6 are incorrect since RAN4 hasn’t define DLBWP.0.1 RedCap/ULBWP.0.1 RedCap in 38.133. Suggest align with Rel-15 re-establishment TCs, i.e using DLBWP.0.1/ULBWP.0.1. - DLBWP.1.1 RedCap/ULBWP.1.1 RedCap is used as dedicated BWP RMC in 16.3.2.1.1/2/3, which is not aligned with 16.3.2.1.4 (DLBWP.1.1/ULBWP.1.1), considering these TC are irrelevant to RedCap-specific BWP, we suggest use DLBWP.1.1/ULBWP.1.1, which simplifies TE implementation. - Redirection TC 16.3.2.3.1/2/3/4 - Similar changes to dedicated BWP RMC as re-establishment TCs - BFR TC 16.5.2.3/4 - Similar changes to dedicated BWP RMC as re-establishment TCs |  |  |  |  |  |  |  |  |
| Summary of change: |  | - BW of DLBWP/ULBWP.1.3 RedCap is updated. - BWP configurations in TCs mentioned are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-conformant UE may pass the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8A.1, A.8A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a resubmission of the missed changes in A.8A.1, A.8A.2 of R5-236736, CR# 2763 |  |  |  |  |  |  |  |  |

<Start of modified section >

## A.8A.1	Downlink BWP configurations

Table A.8A.1-1: Downlink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 RedCap |  |  |  | DLBWP.1.2 RedCap |  |  |  | DLBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 |
| NOTE 1: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3A. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## A.8A.2	Uplink BWP configurations

Table A.8A.2-1: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 RedCap |  |  |  | ULBWP.1.2 RedCap |  |  |  | ULBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 | 25 | 25 | 32 | 16 |
| NOTE 1: RBa is the same as RBa for DLBWP.1.1 RedCap. NOTE 2: RBb is the same as RBb for DLBWP.1.2 RedCap. NOTE 3: RBc is the same as RBc for DLBWP.1.3 RedCap. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section >