3GPP TSG-RAN5 Meeting #102	R5-240466

# Athens, Greece, 26 February – 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2647 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add general spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing general spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added inter-band configuration CA_n71A-n77A and CA_n71A-n78A into Table 6.5A.3.1.1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General spurious emissions for UE co-existence requirements for CA_n71A-n77A and CA_n71A-n78A will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905 ... CR 0860 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TPA for CA_n71A-n78A in CR 0860. TPA for CA_n71A-n77A already included in TR 38.905 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

6.5A.3.1.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions requirement with frequency range as indicated in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous and non-contiguous UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For inter-band CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier, then it does not apply.

For intra-band contiguous CA, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5A.3.1.0-1 from the edge of the aggregated channel bandwidth.

For intra-band non-contiguous CA, the spurious emission limits apply to frequency ranges that are more than FOOB in Table 6.5.3.1.3-1 away from the edges of each carrier in the gap and out of the gap.

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5A.3.1.1.5-1 for inter-band UL CA configuration and Table 6.5A.3.1.1.5-3 for intra-band contiguous UL CA and intra-band non-contiguous configuration.

Table 6.5A.3.1.1.5-1: General spurious emissions test requirements for inter-band UL CA

| Frequency Range |  | Maximum Level |  | Measurement Bandwidth |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test requirements for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |
| 135 MHz ≤ f ≤ 270 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1440 MHz ≤ f ≤ 1650 MHz 2055 MHz ≤ f ≤ 2250 MHz 3630 MHz ≤ f ≤ 3765 MHz 5340 MHz ≤ f ≤ 5550 MHz 5550 MHz ≤ f ≤ 5745 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |
| 90 MHz ≤ f ≤ 220 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1005 MHz ≤ f ≤ 1100 MHz 2800 MHz ≤ f ≤ 2895 MHz 2925 MHz ≤ f ≤ 3080 MHz 3680 MHz ≤ f ≤ 3810 MHz 4720 MHz ≤ f ≤ 4875 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |
| 40 MHz ≤ f ≤ 660 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1320 MHz ≤ f ≤ 1880 MHz 4620 MHz ≤ f ≤ 5780 MHz 7140 MHz ≤ f ≤ 7760 MHz 8520 MHz ≤ f ≤ 9580 MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n2A-n5A Configuration |  |  |  |  |  |  |  |
| 152 MHz ≤ f ≤ 262 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1001 MHz ≤ f ≤ 1086 MHz 2674 MHz ≤ f ≤ 2759 MHz 2851 MHz ≤ f ≤ 2996 MHz 3498 MHz ≤ f ≤ 3608 MHz 4524 MHz ≤ f ≤ 4669 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n14A Configuration |  |  |  |  |  |  |  |
| 254 MHz ≤ f ≤ 334 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1052 MHz ≤ f ≤ 1122 MHz 2638 MHz ≤ f ≤ 2708 MHz 2902 MHz ≤ f ≤ 3032 MHz 3426 MHz ≤ f ≤ 3506 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |
| 9 kHz ≤ f < 150 kHz |  | -36 dBm |  | 1 kHz |  |  |  |
| 150 kHz ≤ f < 30 MHz |  | -36 dBm |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 270 MHz |  | -36 dBm |  | 100 kHz |  |  |  |
| 1640 MHz ≤ f ≤ 1850 MHz 5190 MHz ≤ f ≤ 5610 MHz 7250 MHz ≤ f ≤ 7520 MHz 8950 MHz ≤ f ≤ 9310 MHz |  | -30 dBm |  | 1 MHz |  |  |  |
| Test requirements for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |
| 25 MHz ≤ f <30 MHz |  | -36 dBm |  | 10kHz |  |  |  |
| 30 MHz ≤ f ≤ 120 MHz 795 MHz ≤ f ≤ 905 MHz |  | -36 dBm |  | 100kHz |  |  |  |
| 2505 MHz ≤ f ≤ 2700 MHz 3470 MHz ≤ f ≤ 3615 MHz 4300 MHz ≤ f ≤ 4485 MHz |  | -30 dBm |  | 1MHz |  |  |  |
| Test requirements for CA_n3A-n28A Configuration |  |  |  |  |  |  |  |
| 214 MHz ≤ f ≤ 379 MHz 962 MHz ≤ f ≤ 1000 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1082 MHz 2413 MHz ≤ f ≤ 2533 MHz 2672 MHz ≤ f ≤ 2867 MHz 3116 MHz ≤ f ≤ 3281 MHz 4123 MHz ≤ f ≤ 4318 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1515 MHz ≤ f ≤ 2490 MHz 4815 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7770 MHz 8310 MHz ≤ f ≤ 10185 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |
| 270 MHz ≤ f ≤ 380 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1515MHz ≤ f ≤ 2090 MHz 4815 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7370 MHz 8310 MHz ≤ f ≤ 9385 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n48A Configuration |  |  |  |  |  |  |  |
| 1852 MHz ≤ f ≤ 2052 MHz 2701 MHz ≤ f ≤ 2876 MHz 4374 MHz ≤ f ≤ 4549 MHz 5198 MHz ≤ f ≤ 5398 MHz 6251 MHz ≤ f ≤ 6576 MHz 7924 MHz ≤ f ≤ 8249 MHz |  | -30 dBm |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n66A Configuration |  |  |  |  |  |  |  |
| 12 MHz ≤ f ≤ 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f ≤ 132 MHz 861 MHz ≤ f ≤ 956 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2534 MHz ≤ f ≤ 2736 MHz 3358 MHz ≤ f ≤ 3478 MHz 4244 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n30A Configuration |  |  |  |  |  |  |  |  |
| 709 MHz ≤ f ≤ 739 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1507 MHz ≤ f ≤ 1527 MHz 2093 MHz ≤ f ≤ 3113 MHz 3812 MHz ≤ f ≤ 3842 MHz 3881 MHz ≤ f ≤ 3911 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n66A Configuration |  |  |  |  |  |  |  |  |
| 114 MHz ≤ f ≤ 204 MHz 912 MHz ≤ f ≤ 992 MHz |  |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2498 MHz ≤ f ≤ 2578 MHz 2622 MHz ≤ f ≤ 2772 MHz 3286 MHz ≤ f ≤ 3376 MHz 4208 MHz ≤ f ≤ 4358 MHz |  |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n14A-n77A Configuration |  |  |  |  |  |  |  |  |
| 1704 MHz ≤ f ≤ 3412 MHz 4088 MHz ≤ f ≤ 5796 MHz 5808 MHz ≤ f ≤ 9198 MHz |  |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |
| 1576 MHz ≤ f ≤ 2136 MHz 2438 MHz ≤ f ≤ 2968 MHz 4132 MHz ≤ f ≤ 4662 MHz 4964 MHz ≤ f ≤ 5524 MHz 5738 MHz ≤ f ≤ 6768 MHz 7432 MHz ≤ f ≤ 8462 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n24A-n41A Configuration |  |  |  |  |  |  |  |
| 563 MHz ≤ f ≤ 825 MHz 835.5 MHz ≤ f ≤ 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1063.5 MHz 3331.5 MHz ≤ f ≤ 3753.5 MHz 4122.5 MHz ≤ f ≤ 4350.5 MHz 5749 MHz ≤ f ≤ 6011 MHz 6618.5 MHz ≤ f ≤ 7040.5 MHz |  | -25 dBm+TT |  | 1 MHz |  | NS-01 signalled for n41 |  |
|  |  | -30 dBm+TT |  | 1 MHz |  | NS-04 signalled for n41 |  |
| Test requirements for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |
| 229 MHz ≤ f ≤ 447 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1889.5 MHz ≤ f ≤ 2073.5 MHz 5176.5 MHz ≤ f ≤ 5360.5 MHz 5439.5 MHz ≤ f ≤ 5773.5 MHz 6803 MHz ≤ f ≤ 7021 MHz 8726.5 MHz ≤ f ≤ 9060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n24A-n77A Configuration |  |  |  |  |  |  |  |
| 21 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 947 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1639.5 MHz ≤ f ≤ 2573.5 MHz 4926.5 MHz ≤ f ≤ 7521 MHz 8226.5 MHz ≤ f ≤ 10060.5 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n25A-n66A Configuration. 70 MHz ≤ f ≤ 205 MHz; -36 dBm+TT; 100 kHz. 1505 MHz ≤ f ≤ 1710 MHz 1920 MHz ≤ f ≤ 2120 MHz 3560 MHz ≤ f ≤ 3695 MHz 5270 MHz ≤ f ≤ 5610 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n77A Configuration. 500 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 2350 MHz 4685 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8030 MHz 8450 MHz ≤ f ≤ 10315 MHz; -30 dBm+TT; 1 MHz. Test requirements for CA_n25A-n78A Configuration. 100 MHz ≤ f ≤ 530 MHz; -36 dBm+TT; 100 kHz. 1385 MHz ≤ f ≤ 1950 MHz 4685 MHz ≤ f ≤ 5750 MHz 7000 MHz ≤ f ≤ 7630 MHz 8450 MHz ≤ f ≤ 9515 MHz; -30 dBm+TT; 1 MHz. |
| Test requirements for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |
| 12 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f ≤ 152 MHz 861 MHz ≤ f ≤ 966 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2524 MHz ≤ f ≤ 2746 MHz 3338 MHz ≤ f ≤ 3478 MHz 4234 MHz ≤ f ≤ 4409 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |
| 3 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 82 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 846 MHz ≤ f ≤ 896 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 2509 MHz ≤ f ≤ 2606 MHz 3323 MHz ≤ f ≤ 3408 MHz 4204 MHz ≤ f ≤ 4269 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz |  | -25 dBm+TT |  | 1MHz |  |  |  |
| Test requirements for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 3497 MHz 4003 MHz ≤ f ≤ 5696 MHz 5852 MHz ≤ f ≤ 9148 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n28A-n79A Configuration |  |  |  |  |  |  |  |
| 2904 MHz ≤ f ≤ 3594 MHz 3652 MHz ≤ f ≤ 4297 MHz 5103 MHz ≤ f ≤ 5748 MHz 5806 MHz ≤ f ≤ 6496 MHz 8052 MHz ≤ f ≤ 9297 MHz 9503 MHz ≤ f ≤ 10748 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |
| 8 MHz ≤ f ≤30 MHz |  | -36 dBm+TT |  | 10kHz |  |  |  |
| 30 MHz ≤ f ≤980 MHz |  | -36 dBm+TT |  | 100kHz |  |  |  |
| 1710MHz ≤ f ≤ 2504MHz |  | -30 dBm+TT |  | 1MHz |  |  |  |
| 6110MHz ≤ f ≤ 7690MHz |  |  |  |  |  |  |  |
| 9392MHz ≤ f ≤ 10380MHz |  |  |  |  |  |  |  |
| 11296MHz ≤ f ≤ 12690MHz |  |  |  |  |  |  |  |
| Test requirements for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |
| 10 MHz ≤ f < 30 MHz |  | -36 dBm+TT |  | 10 kHz |  |  |  |
| 30 MHz ≤ f < 280 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1770 MHz ≤ f ≤ 1990 MHz 5260 MHz ≤ f ≤ 5690 MHz 6970 MHz ≤ f ≤ 7260 MHz 8810 MHz ≤ f ≤ 9180 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |
| 130 MHz ≤ f ≤ 310 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1840 MHz ≤ f ≤ 2005 MHz 5245 MHz ≤ f ≤ 5705 MHz 6940 MHz ≤ f ≤ 7120 MHz 8795 MHz ≤ f ≤ 9110 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n48A-n71A Configuration |  |  |  |  |  |  |  |
| 2154 MHz ≤ f ≤ 2374 MHz 2852 MHz ≤ f ≤ 3037 MHz 4213 MHz ≤ f ≤ 4398 MHz 4876 MHz ≤ f ≤ 5096 MHz 6402 MHz ≤ f ≤ 6737 MHz 7763 MHz ≤ f ≤ 8098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |
| 314 MHz ≤ f < 454 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1520 MHz ≤ f ≤ 2490 MHz 4820 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n66A-n78A Configuration |  |  |  |  |  |  |  |
| 260 MHz ≤ f ≤ 380 MHz |  | -36 dBm +TT |  | 100 kHz |  |  |  |
| 1520 MHz ≤ f ≤ 2090 MHz 4820 MHz ≤ f ≤ 5890 MHz 6720 MHz ≤ f ≤ 7360 MHz 8310 MHz ≤ f ≤ 9380 MHz |  | -30 dBm +TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |
| 299 MHz ≤ f < 384 MHz 997 MHz ≤ f < 1000 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |  |
| 1000 MHz ≤ f ≤ 1047 MHz 2358 MHz ≤ f ≤ 2408 MHz 2692 MHz ≤ f ≤ 2757 MHz 3021 MHz ≤ f ≤ 3106 MHz 4053 MHz ≤ f ≤ 4118 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 3537 MHz 3963 MHz ≤ f ≤ 5596 MHz 5902 MHz ≤ f ≤ 9098 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |
| Test requirements for CA_n71A-n78A Configuration |  |  |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 2474 MHz 2602 MHz ≤ f ≤ 3137 MHz 3963 MHz ≤ f ≤ 4498 MHz 4626 MHz ≤ f ≤ 5196 MHz 5902 MHz ≤ f ≤ 6937 MHz 7263 MHz ≤ f ≤ 8298 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |  |

Table 6.5A.3.1.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

-------- End of changes --------