3GPP TSG-RAN5 Meeting #102	R5-240479

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3039 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for remote Initial Access procedure under NR sidelink U2N Relay / Remote UE side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure for remote UE Initial Access procedure under NR sidelink U2N Relay / Remote UE side haven’t be clarified according to TS 23.304 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding it in the procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | '4.9.Y' shall be resolved according to TS38.508-1 CR 3038 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.X	Test procedure for remote UE Initial Access procedure under NR sidelink U2N Relay / Remote UE side

4.9.X.1	Scope

The purpose of this procedure is testing remote UE behaviour when remote UE of NR sidelink U2N Relay is performing Initial Access procedure.

4.9.X.2	Procedure description

Table 4.9.X.2-1: Procedure for remote UE Initial Access procedure under NR sidelink U2N Relay / Remote UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Same as table 4.5.7.2-1, steps 1a1-1A. | - | - | - | - |
| 2 | The UE initiated 5G ProSe U2N Relay Discovery procedure specified in subclause 4.9.Y. | - | - | - | - |
| - | EXCEPTION: Steps 3a1 and 3b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 3a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.39 is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.40 is performed to establish unicast mode ProSe Direct communication between the UE and the NR-SS-UE. | - | - | - | - |
| 4a1-5 | Same as table 4.5.7.2-1, steps 3a1-4. | - | - | - | - |
| 6 | The UE transmits an RRCSetupRequest message. | --> | MAC PDU (including NR RRC: RRCSetupRequest) | - | - |
| 7 | The NR-SS-UE transmits an RRCSetup message. | <-- | MAC PDU (including NR RRC: RRCSetup) | - | - |
| 8 | The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message. | --> | MAC PDU (including NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST) | - | - |
| 9 | The NR-SS-UE transmits a SecurityModeCommand message. | <-- | MAC PDU (including NR RRC: SecurityModeCommand) | - | - |
| 10 | The UE transmits a SecurityModeComplete message. | --> | MAC PDU (including NR RRC: SecurityModeComplete) | - | - |
| 11 | The NR-SS-UE transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message. | <-- | MAC PDU (including NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT) | - | - |
| 12 | The UE transmits a ULInformationTransfer message and a REGISTRATION COMPLETE message. | --> | MAC PDU (including NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE) | - | - |
| - | EXCEPTION: Step 13 describes behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE performs a specific action. | - | - | - | - |
| 13 | IF pc_noOf_PDUsSameConnection_Relay > 0 THEN the generic procedure for UE requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection_Relay. | - | - | - | - |
| - | EXCEPTION: Steps 14a1 depends on the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - | - | - |
| 14a1 | IF connected without release is not present THEN, the NR-SS-UE transmits an RRCRelease message. | <-- | MAC PDU (including NR RRC: RRCRelease) | - | - |

4.9.X.3	Specific message contents

All specific message contents shall be according subclause 4.6 with the exceptions below.

Table 4.9.X.3-1: Message RRCSetup (steps 7, Table 4.9.X.2-1)

Derivation path: Table 4.6.1-21 with condition L2RemoteUE

Table 4.9.X.3-2: Message RRCReconfiguration (steps 13, Table 4.9.X.2-1)

Derivation path: Table 4.6.1-13 with condition L2RemoteUE and RELAY

Table 4.9.X.3-3: Message RRCRelease (steps 14a1, Table 4.9.X.2-1)

Derivation path: Table 4.6.1-16 with condition L2RemoteUE

<End of Changed Section>