3GPP TSG-RAN5 Meeting #102	R5-240480

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3040 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of test procedure for Out_of_Coverage_with_Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test states of UE connect to the NR network through U2N sidelink relay is more complicated than a normal out of coverage state. It should not be placed in test states of 4.4A and generic procedures of 4.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing states Out_of_Coverage_with_Relay and related generic procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The states of Out_of_Coverage_with_Relay is not clearly indicating UE states. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4A.5, 4.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.4A.5	Mapping of test state IDs and test parameters to generic procedures, generic procedure parameters and specific message conditions

Depending on the test case preamble requested test state ID and test parameters the SS shall:

1> use the applicable generic procedure as specified in Table 4.4A.5-1 using the:

2> applicable generic procedure parameters as specified in Table 4.4A.5-1 and Table 4.4A.5-2; and

2> applicable message conditions as specified in Table 4.4A.5-2.

Table 4.4A.5-1: Test state ID mapping to generic procedures and Connectivity generic procedure parameter

| Test state ID |  |  |  |  |  |  |  | Generic Procedure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RRC state field |  | Connectivity field |  |  | Variant field |  |  | Name |  | Generic procedure parameter (Note 1) |  |  | Clause |
| 0 |  |  | - |  |  | A |  |  | SWITCHED_OFF |  |  |  | 4.5.5 |
| 0 |  |  | N |  |  | B |  |  | SWITCHED_OFF |  | Connectivity=NR |  | 4.5.5 |
| 0 |  |  | E |  |  | B |  |  | SWITCHED_OFF |  | Connectivity=E-UTRA |  | 4.5.5 |
| 0 | W |  |  | B |  |  | SWITCHED_OFF |  |  |  |  | Connectivity=WLAN | 4.5.5 |
| 1 |  | N |  |  | A |  |  | RRC_IDLE |  | Connectivity=NR |  |  | 4.5.2 |
| 1 |  | E |  |  | A |  |  | RRC_IDLE |  | Connectivity=E-UTRA |  |  | 4.5.2 |
| 1 | W |  |  | A |  |  | Ipsec_SA_Released |  |  |  |  | Connectivity=WLAN | 4.5.2 |
| 1 | N |  |  | B |  |  | RRC_IDLE_with_SL |  |  |  |  | Connectivity=NR, Sidelink = On | 4.5.2 |
| 2 |  | N |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=NR |  |  | 4.5.3 |
| 2 |  | E |  |  | A |  |  | RRC_INACTIVE |  | Connectivity=E-UTRA |  |  | 4.5.3 |
| 3 |  | N |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=NR |  |  | 4.5.4 |
| 3 |  | E |  |  | A |  |  | RRC_CONNECTED |  | Connectivity=E-UTRA |  |  | 4.5.4 |
| 3 | W |  |  | A |  |  | Ipsec_SA_Established |  |  |  |  | Connectivity=WLAN | 4.5.4 |
| 3 | N |  |  | B |  |  | RRC_CONNECTED_with_SL |  |  |  |  | Connectivity=NR, Sidelink = On | 4.5.4 |
| 4 | - |  |  | A |  |  | Out_of_Coverage |  |  |  |  | Sidelink = On | 4.5.7 |
| Note 1: In addition to the Connectivity parameter specified in this table the applicable additional generic procedure parameters and conditions as stated in Table 4.4A.5-2 shall be used |  |  |  |  |  |  |  |  |  |  |  |  |  |

<skip unchanged sections>

4.5.8	Void