3GPP TSG-RAN5 Meeting #102	R5-240483

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3043 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 5G ProSe information elements of UE policies for 5G ProSe remote |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.7C.1.5 5G ProSe information elements of UE policies for 5G ProSe remote UE was FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated cl. 4.7C.1.5 5G ProSe information elements of UE policies for 5G ProSe remote. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink relay will be not applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7C.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.7C.1.5	5G ProSe information elements of UE policies for 5G ProSe remote UE

–	ProSeP Info = {UE policies for 5G ProSe remote UE}

Table 4.7C.1.5-1: ProSeP Info = {UE policies for 5G ProSe remote UE}

| Derivation Path: TS 24.555[59] Figure 5.6.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ProSeP info type | ‘0100’B | UE policies for 5G ProSe remote UE |  |
| PAI | ‘0’B |  |  |
| NSII | ‘0’B |  |  |
| Spare | ‘00’B |  |  |
| Length of ProSeP info contents | Set to the actual length of ProSeP info contents |  |  |
| Validity timer | 'FF FF FF FF FF'O | 5 octets, Expiration UTC time of validity of the UE policies, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Served by NG-RAN | See Table 4.7C.1.5-2 |  |  |
| Not served by NG-RAN | See Table 4.7C.1.5-5 |  |  |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information | See Table 4.7C.1.5-13 |  |  |
| User info ID for discovery | ‘000001’O |  |  |
| RSC info list | See Table 4.7C.1.5-14 |  |  |
| Privacy timer | 'FF FF FF FF FF'O |  |  |
| N3IWF selection information for 5G ProSe layer-3 remote UE | Not Present |  |  |
| 5G PKMF address information | Not Present |  |  |

–	Served by NG-RAN

Table 4.7C.1.5-2: Served by NG-RAN

| Derivation Path: TS 24.555[59] Figure 5.6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of served by NG-RAN contents | Set to the actual length of 'NG-RAN contents' in bytes |  |  |
| L3RI | ‘0’B |  |  |
| Spare | ‘0000000’B |  |  |
| Authorized PLMN list for layer-2 relay UE | See Table 4.7C.1.5-3 |  |  |

–	Authorized PLMN list

Table 4.7C.1.5-3: Authorized PLMN list

| Derivation Path: TS 24.555[59] Figure 5.6.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of authorized PLMN list contents | Set to the actual length of 'authorized PLMN list contents ' in bytes |  |  |
| Authorized PLMN 1 | See Table 4.7C.1.5-4 |  |  |

–	PLMN ID

Table 4.7C.1.5-4: PLMN ID

| Derivation Path: TS 24.555[59] Figure 5.6.2.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MCC digit 1 | Set according to parameter given in test case |  |  |
| MCC digit 2 | Set according to parameter given in test case |  |  |
| MCC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 3 | Set according to parameter given in test case |  |  |
| MNC digit 1 | Set according to parameter given in test case |  |  |
| MNC digit 2 | Set according to parameter given in test case |  |  |

–	Not served by NG-RAN

Table 4.7C.1.5-5: Not served by NG-RAN

| Derivation Path: TS 24.555[59] Figure 5.6.2.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of not served by NG-RAN contents | Set to the actual length of 'not served by NG-RAN contents' in bytes |  |  |
| NR radio parameters per geographical area list for UE-to-network relay discovery | See Table 4.7C.1.5-6 |  |  |
| NR radio parameters per geographical area list for UE-to-network relay communication | See Table 4.7C.1.5-7 |  |  |
| Default PC5 DRX configuration for UE-to-network relay discovery | See Table 4.7C.1.5-12 |  |  |

–	NR radio parameters per geographical area list for UE-to-network relay discovery

Table 4.7C.1.5-6: NR radio parameters per geographical area list for UE-to-network relay discovery

| Derivation Path: TS 24.555[59] Figure 5.6.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR radio parameters per geographical area list for UE-to-network relay discovery contents | Set to the actual length of 'NR radio parameters per geographical area list for UE-to-network relay discovery contents' in bytes |  |  |
| Radio parameters per geographical area info 1 | See Table 4.7C.1.5-8 |  |  |

–	NR radio parameters per geographical area list for UE-to-network relay communication

Table 4.7C.1.5-7: NR radio parameters per geographical area list for UE-to-network relay communication

| Derivation Path: TS 24.555[59] Figure 5.6.2.7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR radio parameters per geographical area list for UE-to-network relay communication contents | Set to the actual length of 'NR radio parameters per geographical area list for UE-to-network relay communication contents' in bytes |  |  |
| Radio parameters per geographical area info 1 | See Table 4.7C.1.5-8 |  |  |

–	Radio parameters per geographical area info

Table 4.7C.1.5-8: Radio parameters per geographical area info

| Derivation TS 24.555[59] Figure 5.6.2.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters per geographical area contents | Set to the actual length of 'radio parameters per geographical area contents' in bytes |  |  |
| Geographical area | See Table 4.7C.1.5-9 |  |  |
| Radio parameters | See Table 4.7C.1.5-11 |  |  |
| Spare | ‘0000000’B |  |  |
| MI | '1'B | Operator managed |  |

–	Geographical area

Table 4.7C.1.5-9: Geographical area

| Derivation Path: TS 24.555[59] Figure 5.6.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of geographical area contents | Set to the actual length of 'geographical area contents' in bytes |  |  |
| Coordinate 1 | See Table 4.7C.1.5-10 |  |  |
| Coordinate 2 | See Table 4.7C.1.5-10 |  |  |
| Coordinate 3 | See Table 4.7C.1.5-10 |  |  |

–	Coordinate area

Table 4.7C.1.5-10: Coordinate area

| Derivation Path: TS 24.555[59] Figure 5.6.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Latitude | 35.753056 |  | Coordinate 1 |
|  | 35.735278 |  | Coordinate 2 |
|  | 35.744167 |  | Coordinate 3 |
| Longitude | 139.689167 |  | Coordinate 1 |
|  | 139.689167 |  | Coordinate 2 |
|  | 139.709167 |  | Coordinate 3 |

–	Radio parameters

Table 4.7C.1.5-11: Radio parameters

| Derivation Path: TS 24.555[59] Figure 5.6.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of radio parameters contents | Set to the actual length of 'radio parameters contents' in bytes |  |  |
| Radio parameters contents | See Table 4.10.1-1: SL-PreconfigurationNR |  |  |

–	Default PC5 DRX configuration for UE-to-network relay discovery

Table 4.7C.1.5-12: Default PC5 DRX configuration for UE-to-network relay discovery

| Derivation Path: TS 24.555[59] Figure 5.6.2.11a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of default PC5 DRX configuration for UE-to-network relay discovery contents | Set to the actual length of 'default PC5 DRX configuration for UE-to-network relay discovery contents' in bytes |  |  |
| Default PC5 DRX configuration for UE-to-network relay discovery contents | See Table 4.6.6-38 |  |  |

–	Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

Table 4.7C.1.5-13: Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information

| Derivation Path: TS 24.555[59] Figure 5.6.2.11b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information contents | Set to the actual length of 'default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information contents' in bytes |  |  |
| Default destination layer-2 ID 1 | ‘000040’O |  |  |

–	RSC info list

Table 4.7C.1.5-14: RSC info list

| Derivation Path: TS 24.555[59] Figure 5.6.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC info list contents | Set to the actual length of 'RSC info list contents' in bytes |  |  |
| RSC info 1 | See Table 4.7C.1.5-15 |  |  |

–	RSC info

Table 4.7C.1.5-15: RSC info

| Derivation Path: TS 24.555[59] Figure 5.6.2.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC info contents | Set to the actual length of 'RSC info contents' in bytes |  |  |
| RSC list | See Table 4.7C.1.5-16 |  |  |
| Security related parameters for discovery | See Table 4.7C.1.5-17 |  |  |
| LI | '10'B | Layer 2 |  |
| NSI | '0'B |  |  |
| CPSI | '0'B | security procedure over control plane is not used |  |
| TDI | '0'B | Traffic descriptor field is not included |  |
| Spare | '000'B |  |  |
| NR-PC5 UE-to-network relay security policies | See Table 4.7C.1.5-20 |  |  |
| PDU session parameters of layer-3 relay | Not Present |  |  |
| Traffic descriptor | Not Present |  |  |

–	RSC list

Table 4.7C.1.5-16: RSC list

| Derivation Path: TS 24.555[59] Figure 5.6.2.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of RSC list contents | Set to the actual length of 'RSC list contents' in bytes |  |  |
| RSC 1 | '000010'O |  |  |

–	Security related parameters for discovery

Table 4.7C.1.5-17: Security related parameters for discovery

| Derivation Path: TS 24.555[59] Figure 5.6.2.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Security related parameters validity timer | 'FF FF FF FF FF'O | 5 octets, Expiration UTC time of validity, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds) |  |
| Code-sending security parameters | See Table 4.7C.1.5-18 |  |  |
| Code-receiving security parameters | See Table 4.7C.1.5-19 |  |  |

–	Code-sending security parameters

Table 4.7C.1.5-18: Code-sending security parameters

| Derivation Path: TS 24.555[59] Figure 5.6.2.15a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDUSK | '0'B | DUSK field is not included |  |
| PDUIK | '0'B | DUIK field is not included |  |
| PDUCK | '0'B | DUCK and encrypted bitmask fields are not included |  |
| Spare | ‘00000’B |  |  |
| DUSK | Not Present |  |  |
| DUIK | Not Present |  |  |
| DUCK | Not Present |  |  |
| Encrypted bitmask | Not Present |  |  |

–	Code-receiving security parameters

Table 4.7C.1.5-19: Code-receiving security parameters

| Derivation Path: TS 24.555[59] Figure 5.6.2.15b |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDUSK | '0'B | DUSK field is not included |  |
| PDUIK | '0'B | DUIK field is not included |  |
| PDUCK | '0'B | DUCK and encrypted bitmask fields are not included |  |
| Spare | ‘00000’B |  |  |
| DUSK | Not Present |  |  |
| DUIK | Not Present |  |  |
| DUCK | Not Present |  |  |
| Encrypted bitmask | Not Present |  |  |

–	NR-PC5 UE-to-network relay security policies

Table 4.7C.1.5-20: NR-PC5 UE-to-network relay security policies

| Derivation Path: TS 24.555[59] Figure 5.4.2.34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of NR-PC5 unicast security policies contents | Set to the actual length of 'NR-PC5 unicast security policies contents' in bytes |  |  |
| NR-PC5 unicast security policy 1 | See Table 4.7C.1.3-35 |  |  |

–	PDU session parameters for layer-3 relay UE

Table 4.7C.1.5-21: PDU session parameters for layer-3 relay UE

| Derivation Path: TS 24.555[59] Figure 5.6.2.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of PDU session parameters for layer-3 relay UE contents | Set to the actual length of 'PDU session parameters for layer-3 relay UE contents' in bytes |  |  |
| PDU session type | '001'B |  | IPv4 |
|  | '010'B |  | IPv6 |
|  | '011'B |  | IPv4v6 |
| PDNN | '0'B |  |  |
| PSNSSAI | '0'B |  |  |
| PSSCM | '0'B |  |  |
| PATP | '0'B |  |  |
| Spare | '0'B |  |  |
| DNN | Not Present |  |  |
| S-NSSAI | Not Present |  |  |
| SSC mode | Not Present |  |  |
| Access type preference | Not Present |  |  |
| Spare | Not Present |  |  |

–	Traffic descriptor

Table 4.7C.1.5-22: Traffic descriptor

| Derivation Path: TS 24.555[59] Figure 5.6.2.16a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Length of traffic descriptor contents | Set to the actual length of 'traffic descriptor contents' in bytes |  |  |
| Traffic descriptor | Coded according to figure 5.2.2 and table 5.2.1 in clause 5.2 of 3GPP TS 24.526 |  |  |

<End of Changed Section>