3GPP TSG-RAN5 Meeting #102	R5-240495

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3051 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add IEs CandidateTCI-State and CandidateTCI-UL-State |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add and align the IEs CandidateTCI-State and CandidateTCI-UL-State with core specification updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IEs CandidateTCI-State and CandidateTCI-UL-State have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be fully applicable and aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	CandidateBeamRS

Table 4.6.3-15A: CandidateBeamRS

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateBeamRS-r16 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CandidateTCI-State

Table 4.6.3-15B: CandidateTCI-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateTCI-State-r18 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

#### –	CandidateTCI-UL-State

Table 4.6.3-15C: CandidateTCI-UL-State

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CandidateTCI-UL-State-r18 ::= SEQUENCE { |  |  |  |
| FFS |  |  |  |
| } |  |  |  |

<End of modified section>