3GPP TSG-RAN5 Meeting #102	R5-240515

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.010-1 | CR | 5169 | rev | - | Current version: | 13.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to GSM testcase 44.2.5.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2024-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of ciphering algorithms GEA1 and GEA2 have been prohibited from all releases of 3GPP core specifications published in December 2023. 3GPP SA requested RAN5 to enable verification of non-support of GEA1 and GEA2 in all releases (R5-240018). RAN5 test specifications need to be updated to align with the latest version of core specifications. It was also agreed to update testcase 44.2.5.2.5 to include checking of non support of GEA2. The Prose for 44.2.5.2.5 needs to be updated accordingly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the prose of 44.2.5.2.5 to check non support of GEA2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase Prose will not be aligned with core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 44.2.5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 44.2.5.2.5	Ciphering mode / Non support of GEA1 / Non support of GEA2

44.2.5.2.5.1	Conformance requirement

It is mandatory for GEA3 and non encrypted mode (i.e. GEA0) to be implemented in mobile stations. GEA4 may be implemented in the mobile stations.

NOTE 1:	Mobile stations are not allowed to implement GEA1 or GEA2 from Release 97 onwards.

Reference(s):

3GPP TS 43.020 Annex D.4.9

44.2.5.2.5.2	Test Purpose

To verify that MS does not apply GEA1 nor GEA2 ciphering algorithm.

44.2.5.2.5.3	Method of Test

Initial Conditions

System Simulator:

One cell operating in network operation mode II.

Mobile Station:

MS has a valid IMSI. MS is Idle Updated.

Specific PICS statements:

-	MS operation mode B (TSPC_operation_mode_B).

-	MS operation mode C (TSPC_operation_mode_C).

-	Switch off on button (TSPC_Feat_OnOff).

-	Automatic GPRS attach procedure at switch on or power on (TSPC_AddInfo_on_auto_GPRS_AP).

PIXIT statements:

-

Test procedure

MS sends ATTACH REQUEST. The SS checks that GPRS Encryption Algorithm GEA/1 and GEA/2 bits are 0.

The SS sends GMM CIPHERING AND AUTHENTICATION REQUEST with Cipher algorithm GEA1 or GEA2. MS sends GMM STATUS message with Cause Value #95.

Maximum duration of test

5 minutes.

Expected sequence

The sequence is performed for execution counter K=1 when Cipher algorithm = GEA1, for K=2 when Cipher algorithm = GEA2

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The MS is set in MS operation mode B or C (see PICS). |
| 2 | MS |  | The MS is powered up or switched on and initiates an attach (see PICS). |
| 3 | MS -> SS | ATTACH REQUEST | GPRS Encryption Algorithm GEA/1= 0 and GEA/2 = 0 Attach type = 'GPRS attach' Mobile identity = IMSI Message not ciphered |
| 4 | SS -> MS | GMM AUTHENTICATION AND CIPHERING REQUEST | Request authentication. For K=1, Cipher algorithm = GEA1 For K=2, Cipher algorithm = GEA2 Ciphering On Set GPRS-CKSN-1 RAND Message not ciphered |
| 5 | MS -> SS | GMM STATUS | Cause Value #95 Message not ciphered |
| 6 | SS -> MS | ATTACH REJECT | GMM cause = ''Network failure' Message not ciphered |

Specific message contents

None.