3GPP TSG-RAN5 Meeting #102	R5-240545

# Athens, Greece, 26th February– 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4252 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NE-DC RRC test case 8.2.2.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Note in Table 8.2.2.2.3.3.2-1 about EXCEPTION before Step 4D refers to the wrong step 4B instead of step 4D. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update reference in EXCEPTION to step 4D in Table 8.2.2.2.3.3.2-1 instead of 4B in the same table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong reference will remain in Table 8.2.2.2.3.3.2-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.2.2.3	Split SRB Establishment and Release / NE-DC

8.2.2.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NE-DC }

ensure that {

when { UE receives an RRCReconfiguration message on SRB1 to configure split SRB1 and split SRB2 with PDCP duplication activated }

then { UE configures the split SRBs and sends an RRCConnectionReconfigurationComplete message on split SRB1 with the PDCP PDU duplicated on the MCG and SCG path }

}

(2)

with { UE in RRC_CONNECTED state with NE-DC and split SRB configured }

ensure that {

when { UE receives an RRCReconfiguration message on split SRB1 over the SCG path to release split SRB1 and split SRB2 }

then { UE releases split SRB1 and split SRB2 and sends an RRCConnectionReconfigurationComplete message on SRB1 over the MCG path}

}

(3)

with { UE in RRC_CONNECTED state in NE-DC mode with split SRB1 and split SRB2 configured with PDCP duplication }

ensure that {

when { UE receives a CounterCheck on split SRB1 over the SCG path }

then { UE replies with CounterCheckResponse on split SRB1 duplicated on the MCG and SCG path}

}

(4)

with { UE in RRC_CONNECTED state in NE-DC mode with split SRB1 and split SRB2 configured with PDCP duplication }

ensure that {

when { UE receives an IDENTITY REQUEST on split SRB2 over the SCG path }

then { UE replies with IDENTITY RESPONSE on split SRB2 duplicated on the MCG and SCG path}

8.2.2.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.3.5.3 and TS 36.331: clauses 5.3.5.3, 5.3.10.1a and 5.3.10.17. Unless and otherwise stated these are Rel-15 requirements

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

…

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

…

1>	set the content of the RRCReconfigurationComplete message as follows:…

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the UE is configured with NE-DC:

2>	if the received RRCConnectionReconfiguration message was included in an NR RRCResume message:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCResumeComplete as specified in TS 38.331 [82], clause 5.3.13.4;

2>	else:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCReconfigurationComplete as specified in TS 38.331 [82], clause 5.3.5.3;

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

[TS 36.331, clause 5.3.10.1a]

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModListSCG that is not part of the current UE E-UTRA SCG configuration (i.e. SCG RLC bearer establishment):

2>	apply the specified configuration defined in 9.1.2 for the corresponding SRB;

2>	establish an (SCG) RLC entity in accordance with the received rlc-Config;

2>	establish a (SCG) DCCH logical channel in accordance with the received logicalChannelConfig and with the logical channel identity set in accordance with 9.1.2;

2>	if the UE is configured with DC:

3>	associate the established SCG RLC bearer and DCCH logical channel with the E-UTRA PDCP entity with the same value of srb-Identity within the current UE configuration;

3>	configure the E-UTRA PDCP entity to activate duplication with t-Reordering set to infinity;

2> else (i.e. the UE is configured with NE-DC):

3>	associate the SCG RLC bearer and DCCH logical channel with the NR PDCP entity, i.e. as configured by NR see TS 38.331 [82], identified with the same srb-Identity within the current UE configuration;

1>	for each srb-Identity value included in the srb-ToAddModListSCG that is part of the current UE SCG configuration (SCG RLC bearer reconfiguration):

2>	re-establish the SCG RLC entity, if reestablishRLC is included;

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the DCCH logical channel in accordance with the received logicalChannelConfig;

[TS 36.331, clause 5.3.10.17]

The UE shall:

1>	for each srb-Identity value included in srb-ToReleaseList or in srb-ToReleaseListSCG that is part of the current UE configuration:

2>	if the SRB configuration does not include an E-UTRA PDCP entity (release the SCG RLC bearer configuration):

3>	re-establish the RLC entity as specified in TS 36.322 [7] for this SRB;

3>	configure the E-UTRA PDCP entity to deactivate duplication;

2>	release the RLC entity or entities;

2>	release the DCCH logical channel;

2>	if srb-Identity value is set to 4, release the PDCP entity;

8.2.2.2.3.3	Test description

8.2.2.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and E-UTRA Cell 1 is the PSCell.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NE-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4.

8.2.2.2.3.3.2	Test procedure sequence

Table 8.2.2.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS is configured not to send RLC ACK on split SRB1. | - | - | - | - |
| 1 | SS transmits RRCReconfiguration message containing Eutra RRCConnectionReconfiguration message to configure split SRB1 and split SRB2. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| - | EXCEPTION: In parallel to the event described in step 2 the step specified in Table 8.2.2.2.3.3.2-2 is taking place. | - | - | - | - |
| 2 | Check: Does the UE send a RRCReconfigurationComplete message containing Eutra RRCConnectionReconfigurationComplete message on split SRB1 duplicated over the SCG path? Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 1 | P |
| 2A | After steps in table 8.2.2.2.3.3.2-2 are completed, the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged. | - | - | - | - |
| 2B | The SS is configured not to send RLC ACK on split SRB1. | - | - | - | - |
| 3 | SS transmits CounterCheck message for E-UTRA capabilities on split SRB1 over the SCG path. | <-- | CounterCheck | - | - |
| - | EXCEPTION: In parallel to the event described in step 4 the steps specified in Table 8.2.2.2.3.3.2-3 is taking place. | - | - | - | - |
| 4 | Check: Does the UE send CounterCheckResponse message on split SRB1 duplicated over the SCG path? Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | CounterCheckResponse | 3 | P |
| 4A | After steps in table 8.2.2.2.3.3.2-3 are completed, the SS is configured to send RLC ACK on split SRB1 and the last message received in step 4 is acknowledged. | - | - | - | - |
| 4B | The SS is configured not to send RLC ACK on split SRB2. | - | - | - | - |
| 4C | The SS sends DLInformationTransfer message (carrying IDENTITY REQUEST message) on split SRB2 over the SCG path. | <-- | DLInformationTransfer (IDENTITY REQUEST) | - | - |
| - | EXCEPTION: In parallel to the event described in step 4D the step specified in Table 8.2.2.2.3.3.2-4 is taking place. | - | - | - | - |
| 4D | Check: Does the UE send the ULInformationTransfer message (carrying IDENTITY RESPONSE message) on split SRB2 duplicated over the SCG path? Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | ULInformationTransfer (IDENTITY RESPONSE) | 4 | P |
| 4E | After steps in table 8.2.2.2.3.3.2-4 are completed, the SS is configured to send RLC ACK on split SRB2 and the last message received in step 4D is acknowledged. | - | - | - | - |
| 5 | SS transmits RRCReconfiguration message containing Eutra RRCConnectionReconfiguration message on split SRB1 to release split SRB1 and split SRB2 over the SCG path. | <-- | RRCReconfiguration (RRCConnectionReconfiguration) | - | - |
| 6 | Check: Does the UE send RRCReconfigurationComplete message containing Eutra RRCConnectionReconfigurationComplete message on SRB1 over the MCG path? Note: The SS makes sure that the UE does not send any message on the SCG path. The SS sends the RLC ACK for this message. | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | 2 | P |

Table 8.2.2.2.3.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message on SRB1 over the MCG path/NR Cell 1 Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | RRCReconfigurationComplete (RRCConnectionReconfigurationComplete) | - | - |

Table 8.2.2.2.3.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an CounterCheckResponse message on SRB1 over the MCG path/NR Cell 1 Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | CounterCheckResponse | - | - |

Table 8.2.2.2.3.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an DLInformationTransfer (IDENTITY RESPONSE) message on SRB2 over the MCG path/NR Cell 1 Note: The UE RLC might re-transmit this message several times (no more than maxRetxThreshold) until RLC ACK is sent by the SS. | --> | DLInformationTransfer (IDENTITY RESPONSE) | - | - |

8.2.2.2.3.3.3	Specific message contents

Table 8.2.2.2.3.3.3-1: RRCReconfiguration (step 1 Table 8.2.2.2.3.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | Table 8.2.2.2.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| eutra-SCG | RRCConnectionReconfiguration | Table 8.2.2.2.3.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-2: RadioBearerConfig (Table 8.2.2.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition SRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb-ToAddModList SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod { | 2 entries |  |  |
| SRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| SRB-Identity | SRB-Identity with condition SRB1 |  |  |
| pdcp-Config | PDCP-Config-SRB1 | Table 8.2.2.2.3.3.3-3 |  |
| } |  |  |  |
| SRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| SRB-Identity | SRB-Identity with condition SRB2 |  |  |
| reestablishPDCP | Not present |  |  |
| discardOnPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config-SRB2 | Table 8.2.2.2.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-3: PDCP-Config-SRB1 (Table 8.2.2.2.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split_SRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC | Not present |  | Step 5 |
| moreThanOneRLC SEQUENCE { |  |  | Step 1 |
| primaryPath SEQUENCE { |  |  |  |
| logicalChannel | 1 |  |  |
| } |  |  |  |
| pdcp-Duplication | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-4: PDCP-Config-SRB2 (Table 8.2.2.2.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split_SRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC | Not present |  | Step 5 |
| moreThanOneRLC SEQUENCE { |  |  | Step 1 |
| primaryPath SEQUENCE { |  |  |  |
| logicalChannel | 2 |  |  |
| } |  |  |  |
| pdcp-Duplication | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-5: RRCConnectionReconfiguration (Table 8.2.2.2.3.3.3-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition NE-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-Configuration-r12 | SCG-Configuration-r12-NE-DC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.4.3.3.3-6: SCG-Configuration-r12-NE-DC (Table 8.2.2.2.3.3.3-2)

| Derivation Path: TS 36.508 [7], Table 4.6.3-19G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| scg-Configuration-r12 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| scg-ConfigPartMCG-r12 | Not present |  |  |
| scg-ConfigPartSCG-r12 SEQUENCE { |  |  |  |
| radioResourceConfigDedicatedSCG-r12 SEQUENCE { |  |  |  |
| drb-ToAddModListSCG-r12 | Not present |  |  |
| srb-ToAddModListSCG-r15 SEQUENCE (SIZE (1..maxSRB)) OF SEQUENCE { | 2 entries |  |  |
| SRB-ToAddMod[1] | SRB-ToAddMod-DEFAULT with condition SRB1 | entry 1 |  |
| SRB-ToAddMod[2] | SRB-ToAddMod-DEFAULT with condition SRB2 | entry 2 |  |
| } |  |  |  |
| mac-MainConfigSCG-r12 | MAC-MainConfig-RBC using condition DRX_L |  |  |
| rlf-TimersAndConstantsSCG-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-7: RRCReconfigurationComplete (Table 8.2.2.2.3.3.2-1, step 2 and 6, Table 8.2.2.2.3.3.2-2, step 1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-14 with condition NE-DC

Table 8.2.2.2.3.3.3-8: CounterCheck (Table 8.2.2.2.3.3.2-1, Step 3)

| Derivation Path: TS 38.508-1[4] Table 4.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheck SEQUENCE { |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info { | 2 entries |  |  |
| DRB-CountMSB-Info[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity of DRBn | DRBn is the MCG DRB established in preamble |  |
| countMSB-Uplink | 0 |  |  |
| countMSB-Downlink | 0 |  |  |
| } |  |  |  |
| DRB-CountMSB-Info[2] SEQUENCE { |  | entry 2 |  |
| drb-Identity | DRB-Identity of DRBj | DRBj is the SCG DRB established in preamble |  |
| countMSB-Uplink | 0 |  |  |
| countMSB-Downlink | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.2.2.3.3.3-9: CounterCheckResponse (Table 8.2.2.2.3.3.2-1, Step 4, Table 8.2.2.2.3.3.2-3, Step 1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { | 2 entries |  |  |
| DRB-CountInfo[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity of DRBn | DRBn is the MCG DRB established in preamble |  |
| count-Uplink | Not checked |  |  |
| count-Downlink | Not checked |  |  |
| } |  |  |  |
| DRB-CountInfo[2] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity of DRBj | DRBj is the SCG DRB established in preamble |  |
| count-Uplink | Not checked |  |  |
| count-Downlink | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |