3GPP TSG-RAN5 Meeting #102	R5-240551

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3676 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of GEA2 from TS 34.123-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of ciphering algorithms GEA1 and GEA2 have been prohibited from all releases of 3GPP core specifications published in December 2023. RAN5 test specifications need to be updated to align with the latest version of 3GPP core specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the GPRS ciphering algorithm PIXIT px_GPRS_CipheringAlgorithm, the possibility to select GEA2 has been removed. NOTE: The possibility to select GEA1 had already been removed some time ago. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 34.123-3 will not be aligned with 3GPP TSG decision. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## B.1.1	BasicM test suite parameter declarations

The following parameters are common to all ATSs.

Table B.1.1: BasicM PIXIT

| Parameter name | Description | Type | Default value | Supported value |
| --- | --- | --- | --- | --- |
| px_AuthAMF | Authentication Management Field (16 bits). The value shall be different from '1111 1111 1111 1111'B (AMFresynch). | B16 | See note 2 |  |
| px_AuthK | Authentication Key (128 bits) | B128 | '01011110010010101011001101011000100100010011011101011101001010101110111010000001001011100110011111000011000010011010011000101001'B |  |
| px_AuthN | Value of n to initialize tcv_Auth_n (length of extended response) min 31, max 127 (3GPP TS 34.108 [3], clause 8.1.2) | INTEGER | 127 |  |
| px_AuthRAND | Random Challenge (128 bits) | BITSTRING | '01010101…01'B |  |
| px_GPRS_CipheringAlgorithm | Cipher algorithm | GPRS_CipheringAlgorithm_Type | '011'B | Allowed values gea0 ’000’B, gea3 ’011’B, gea4 ’100’B |
| px_CipheringOnOff | Security mode - TRUE if ciphering is applicable | BOOLEAN | TRUE |  |
| px_CN_DomainTested | CN domain to be tested. This parameter is used in test cases that handle both PS and CS domains. | CN_DomainIdentity | ps_domain |  |
| px_DL_MaxCC_TB_bits | Maximum sum of number of bits of all convolutionally coded transport blocks being received at an arbitrary time instant. | MaxNoBits | b163840 |  |
| px_DL_MaxCCTrCH | Maximum number of Simultaneous CCTrCH for downlink | MaxSimultaneousCCTrCH_Count | 8 |  |
| px_DL_MaxTB_bits | Maximum sum of number of bits of all transport blocks being received at an arbitrary time instant. | MaxNoBits | b163840 |  |
| px_DL_MaxTF | Maximum number of TF for downlink | MaxNumberOfTF | tf1024 |  |
| px_DL_MaxTFS | Maximum number of TFC in the TFCS for downlink | MaxNumberOfTFC_DL | tfc1024 |  |
| px_DL_MaxTrCHs | Maximum number of simultaneous transport channels for downlink. | MaxSimultaneousTransChsDL | e32 |  |
| px_DL_MaxTTI_TB | Maximum total number of transport blocks received within TTIs that end within the same 10 ms interval. | MaxTransportBlocksDL | tb512 |  |
| px_FRESH | Value for FRESH | Fresh | See note 1 |  |
| px_FDD_OperationBand | Applicable for FDD The operation band under test as defined in 34.108 clause 5.1.1. | INTEGER | 1, see note 3 | px_UARFCN_D_Mid, px_UARFCN_D_Low and px_UARFCN_D_High shall take the values according to the value of px_FDD_OperationBand. |
| px_GuardTimerExtension | Guard timer extension value (in seconds). A reasonable value shall be used. | INTEGER | 0 |  |
| px_IMSI_Def | Default IMSI value | HEXSTRING | '001010123456063'H |  |
| px_JapanMCC | Japan MCC to be used for Band VI | HEXSTRING | '442'H |  |
| px_PriScrmCode | Applicable for FDD Primary scrambling code | PrimaryScramblingCode | 100 |  |
| px_MaxAM_EntityNumberRLC_Cap | Maximum AM Entity Number for RLC. | MaximumAM_EntityNumberRLC_Cap | am30 |  |
| px_MaxNoDPDCH_BitsTransmitted | Part of UL_PhysChCapabilityFDD | MaxNoDPDCH_BitsTransmitted | b57600 |  |
| px_MaxNoDPCH_PDSCH_Codes | Part of DL_PhysChCapabilityFDD. INTEGER (1..8). | INTEGER | 8 |  |
| px_MaxNoPhysChBitsReceived | Part of DL_PhysChCapabilityFDD. | MaxNoPhysChBitsReceived | b76800 |  |
| px_MaxRLC_WindowSize | Maximum RLC window size. | MaximumRLC_WindowSize | mws4095 |  |
| px_MS_ClsmkESIND | default Early Sending Indication | B1 | '0'B |  |
| px_MS_ClsmkRevLvl | default Revision Level | B2 | '10'B |  |
| px_PTMSI_Def | default PTMSI | OCTETSTRING | 'C2345678'O |  |
| px_PTMSI_SigDef | default PTMSI signature (3 octets, 3GPP TS 24.008 [9], clause 10.5.5.8). | OCTETSTRING | 'AB1234'O |  |
| px_RAT | Applicable for FDD This parameter is used to specify which radio access technology is being used for the current test execution. Valid values: fdd and tdd | RatType | fdd |  |
| px_RRC_CS_ServTested | CS service to be tested for RRC test cases. | RRC_ServTested | Speech |  |
| px_RRC_PS_ServTested | PS service to be tested for RRC test cases. | RRC_ServTested | ps_Interactive |  |
| px_SRNC_Id | SRNC Id | SRNC_Identity | '0000 0000 0001'B |  |
| px_SRNTI | S RNTI | S_RNTI | '0000 0000 0000 0000 0001'B |  |
| px_TCellA | TCell value for cell A or 1. Except for the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 256 |  |
| px_TCellB | TCell value for cell B or 2. Except for the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 512 |  |
| px_TCellC | TCell value for cell C or 3. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 1536 |  |
| px_TCellD | TCell value for cell D or 4. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 321 |  |
| px_TCellE | TCell value for cell E or 5. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 833 |  |
| px_TCellF | TCell value for cell F or 6. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 6577 |  |
| px_TCellG | TCell value for cell G or 7. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 7253 |  |
| px_TCellH | TCell value for cell H or 8. Except the first created cell, the value 0 applied in ts_SS_CellCfg. | Tcell | 4351 |  |
| px_TimerDequeuePCO | Additional time for dequeueing PCO | INTEGER | 5000 (ms) |  |
| px_TMSI_Def | Default TMSI | OCTETSTRING | '12345678'O |  |
| px_TotalRLC_AM_BufferSize | Total RLC AM buffer size for R99. The values are according to TotalRLC-AM-BufferSize defined in 3GPP TS 25.331. | TotalRLC_AM_BufferSize | NA |  |
| px_UARFCN_D_Mid | Applicable for FDD Mid Range downlink UARFCN value | INTEGER | 10700 |  |
| px_UARFCN_D_Low | Applicable for FDD Low Range downlink UARFCN value | INTEGER | 10563 |  |
| px_UARFCN_D_High | Applicable for FDD High Range downlink UARFCN value | INTEGER | 10837 |  |
| px_UE_OpModeDef | Default UE operation mode (either opModeA or opModeC). (For most UEs this corresponds class-A or class-C, and can not be changed by the user) | UE_OperationMode | opModeA |  |
| px_UE_PositioningNetworkAssistedGPS_Sup | UE positioning capability: supports the network assisted GPS | NetworkAssistedGPS_Supported | networkBased |  |
| px_UE_PowerClass | UE_PowerClass value of the band under test (px_FDD_OperationBand or px_TDD_OperationBand). | UE_PowerClass | 1 |  |
| px_UL_MaxCC_TB_bits | Maximum sum of number of bits of all convolutionally coded transport blocks being transmitted at an arbitrary time instant. | MaxNoBits | b163840 |  |
| px_UL_MaxTB_bits | Maximum sum of number of bits of all transport blocks being transmitted at an arbitrary time instant. | MaxNoBits | b163840 |  |
| px_UL_MaxTF | Maximum number of TF for uplink. | MaxNumberOfTF | tf1024 |  |
| px_UL_MaxTFS | Maximum number of TFC in the TFCS for uplink. | MaxNumberOfTFC_DL | tfc1024 |  |
| px_UL_MaxTrCHs | Maximum number of simultaneous transport channels for uplink. | MaxSimultaneousTransChsUL | e32 |  |
| px_UL_MaxTTI_TB | Maximum total number of transport blocks transmitted within TTIs that start at the same time. | MaxTransportBlocksUL | tb512 |  |
| px_UL_ScramblingCode | Applicable for FDD UL scrambling code value to be used by UE. | UL_ScramblingCode | 0 |  |
| px_TDD_OperationBand | Applicable for TDD The operation band under test as defined in 34.108 clause 5.1.2. | INTEGER | 1 | px_UARFCN_Mid, px_UARFCN_Low and px_UARFCN_High shall take the values according to the value of px_TDD_OperationBand. |
| px_UARFCN_Mid | Applicable for TDD Mid Range UARFCN value | INTEGER | 9550 |  |
| px_UARFCN_Low | Applicable for TDD Low Range UARFCN value | INTEGER | 9504 |  |
| px_UARFCN_High | Applicable for TDD High Range UARFCN value | INTEGER | 9596 |  |
| px_UARFCN_Mid_S1 | Applicable for TDD Mid Range UARFCN value | INTEGER | 9545 |  |
| px_UARFCN_Mid_S2 | Applicable for TDD Mid Range UARFCN value | INTEGER | 9555 |  |
| px_UARFCN_High_S1 | Applicable for TDD High Range UARFCN value | INTEGER | 9591 |  |
| px_UARFCN_High_S2 | Applicable for TDD High Range UARFCN value | INTEGER | 9586 |  |
| NOTE 1: No default value can be proposed (Manufacturer defined value). NOTE 2: No default value can be proposed, because not enough information is available in 3GPP TS 34.109 [4], clause 8.1.2. NOTE 3: This value shall be set in synchronization with the values that are being set for the 3 other pixits with: px_UARFCN_D_High, px_UARFCN_D_Mid, px_UARFCN_D_Low. |  |  |  |  |