3GPP TSG-RAN5 Meeting #102	R5-240561

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0336 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of clause 5.5.2.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Depending on the implementation the UE may start service authorisation either at SIP registration or using the SIP PUBLISH for MCX service settings. For clarification a new condition SERVICE_AUTH has been added to Table 5.5.1-1 (TS 36.579-1 CR 0334) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use of SERVICE_AUTH instead of CONFIG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0334 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.13	SIP REGISTER

This message is sent by the UE.

Table 5.5.2.13-1: SIP REGISTER

| Derivation Path: TS 24.229 [16] clause A.2.1.4.12, A.2.2.4.12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] |  |
| Method | "REGISTER" |  |  |  |
| Request-URI | SIP URI of the home domain name (px_MCX_SIP_HomeDomain_A) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the home domain name of the SIP core (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.2 (e.g. when there is a USIM only) |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Route | Not present |  | RFC 3261 [22] |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" | UE uses UDP for registration |  | UDP |
|  | "SIP/2.0/TCP | UE uses TCP for registration |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN |  |  |  |
| port | any value if present |  |  | SIP_REGISTER_INITIAL |
|  | any value if present |  |  | TCP |
|  | protected server port of the UE when using UDP |  |  | UDP |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | same value as in the initial REGISTER |  |  |  |
|  | Default public user id (px_MCX_SIP_PublicUserId_A_1) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the default public user id (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.4B (e.g. when there is a USIM only) |  | SIP_REGISTER_INITIAL |
| port | not present |  |  |  |
| tag | any value |  |  |  |
| To |  |  |  |  |
| addr-spec | same value as in From-header |  |  |  |
| tag | Not present |  |  |  |
| Contact |  |  | RFC 3261 [22] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| port | any value if present |  |  | SIP_REGISTER_INITIAL |
|  | protected server port of the UE |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
| feature-param | g.3gpp.mcdata.sds | SDS is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_SDS |
| feature-param | g.3gpp.mcdata.fd | FD is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_FD |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  | MCPTT |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata " | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | SDS is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_SDS |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" | FD is supported | TS 24.282 [87] clause 7.2.1 | MCDATA AND pc_MCData_FD |
| feature-param | "audio" |  |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| feature-param | “expires=600000” if present |  |  |  |
| Expires | Present if no expires parameter in Contact header |  | RFC 3261 [22] RFC 3903 [43] |  |
| value | "600000" |  |  |  |
| Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Supported |  |  | RFC 3261 [22] RFC 6442 [62] RFC 4488 [36] |  |
| option-tag | "path" |  |  |  |
| option-tag | "timer" |  |  |  |
| Cseq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  | SIP_REGISTER_INITIAL |
|  | value sent by the UE in previous REGISTER incremented by one |  |  |  |
| method | "REGISTER" |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any value |  |  |  |
| Security-Client |  |  | RFC 7315 [52] |  |
| mechanism-name | "ipsec-3gpp" |  |  |  |
| algorithm | "hmac-sha-1-96" |  |  |  |
| protocol | "esp" (if present) |  |  |  |
| mode | "trans" (if present) |  |  |  |
| encrypt-algorithm | "des-ede3-cbc" or "aes-cbc" |  |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |  |
| port-c | protected client port |  |  |  |
| port-s | protected server port |  |  |  |
| Security-Verify | Not present |  | RFC 3329 [53] | SIP_REGISTER_INITIAL |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security Server header sent by SS |  |  |  |
| Authorization |  |  | RFC 2617 [72], RFC 3310 [96] | SIP_REGISTER_INITIAL |
| username | Private user id (px_MCX_SIP_PrivateUserId_A) if available at the UE or derived from the IMSI otherwise | Depending on the UE configuration the UE may know the private public user id (e.g. when there is an ISIM) or the UE needs to derive it from the IMSI as according to 23.003 [69] clause 13.3 (e.g. when there is a USIM only) |  |  |
| realm | same home domain name as used in Request-URI |  |  |  |
| nonce | "" | Empty string |  |  |
| digest-uri | same SIP-URI as used as Request-URI |  |  |  |
| opaque | any value if present |  |  |  |
| qop | any value if present |  |  |  |
| cnonce | any value if present |  |  |  |
| nc | any value if present |  |  |  |
| algorithm | any value if present |  |  |  |
| response | "" | Empty string |  |  |
| Authorization |  |  | RFC 2617 [72], RFC 3310 [96] |  |
| username | same value as for condition SIP_REGISTER_INITIAL |  |  |  |
| realm | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |  |
| nonce | same value as in WWW-Authenticate header sent by SS |  |  |  |
| digest-uri | same SIP-URI as used as Request-URI |  |  |  |
| opaque | same value as sent by the server in “401 Unauthorized for REGISTER” |  |  |  |
| qop | "auth" |  |  |  |
| cnonce | any value | value assigned by UE affecting the response calculation |  |  |
| nc | nonce-count value | counter to indicate how many times the UE has sent the same value of nonce within successive REGISTERs, initial value shall be 1 |  |  |
| algorithm | “AKAv1-MD5” |  |  |  |
| response | Digest response | calculated by the client according to RFC 2617 |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-specs | Access network technology and, if applicable, the cell ID |  |  |  |
| Content-Type |  |  | RFC 5621 [58] | SERVICE_AUTH |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| value | any value | length of the message body |  |  |
| Message-body |  |  | RFC 3261 [22] | SERVICE_AUTH |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 | MCPTT |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | MIKEY |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/mikey" |  | RFC 3830 [24] |  |
| MIME-part-body | MIKEY message as described in Table 5.5.9.1-1 | MIKEY message, containing the CSK | TS 33.180 [94] |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| SIP_REGISTER_INITIAL | Initial unprotected REGISTER |
| For further conditions see table 5.5.1-1 |  |