3GPP TSG-RAN5 Meeting #102	R5-240567

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0352 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MO call release of calls using a pre-established session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The MMI commands to trigger MO call release keeping the pre-established session are not consistent and not precise. - Even though the first-to-answer call in test case 6.2.22 uses a pre-established session, the call release at step 13 is as for no pre-established session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Wording of MMI commands to trigger MO call release keeping the pre-established session aligned to the title of the procedure being used (36.579-1 5.3A.4: “MCPTT CO call release keeping the pre-established session”) - Step 13 of test case 6.2.22 changed to MO call release keeping the pre-established session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.5.3.2, 6.1.1.17.3.2, 6.1.2.2.3.2, 6.1.2.14.3.2, 6.2.9.3.2, 6.2.10.3.2, 6.2.18.3.2, 6.2.22.3.2, 6.2.26.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

6.1.1.5.3.2	Test procedure sequence

Table 6.1.1.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a group call using the pre-established session with automatic commencement mode and implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a group call with automatic commencement mode? | - | - | 1 | P |
| 2A | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| 3-6 | Void | - | - | - | - |
| 7 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1 to release the call? | - | - | 2 | P |
| 9 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.1.1.17.3.2	Test procedure sequence

Table 6.1.1.17.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a broadcast group call using the pre-established session with automatic commencement mode and implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to establish a broadcast group call with automatic commencement mode? | - | - | 1 | P |
| 3-5 | Void | - | - | - | - |
| 6 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1? | - | - | 2 | P |
| 8 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.1.2.2.3.2	Test procedure sequence

Table 6.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a chat group call using the pre-established session with implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to establish a chat group call with implicit floor control? | - | - | 1 | P |
| 3-5 | Void | - | - | - | - |
| 6 | The SS sends Floor Granted. | <-- | Floor Granted | - | - |
| 7 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 1 | P |
| 8 | The procedure 'MCPTT CT call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.5.3-1 is performed. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Make the UE (MCPTT client) request the establishment of an emergency chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 10A | E-UTRA/EPC signalling according to TS 36.579-1 [2] clause 5.4.3 'MCX CO communication in E-UTRA' takes place | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) send a SIP REFER request with <emergency_ind> set true? | --> | SIP REFER | 2 | P |
| 12 | The SS sends a SIP 403 (Forbidden). | <-- | SIP 403 (Forbidden) | - | - |
| 12A | The SS waits 2 seconds before it releases the RRC connection. | - | - | - | - |
| 13 | Check: Does the UE (MCPTT client) notify the user that the emergency group call is not allowed? (NOTE 1) | - | - | 2 | P |
| 14 | Make the UE (MCPTT client) request the establishment of an emergency chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 14A-15A | Check: Does the UE (MCPTT client) correctly perform steps 1a1 to 3 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to initiate establishment of an emergeny chat group call? | - | - | 2 | P |
| 16 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CT session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3.4.3-1 starting with step 2 to establish the emergency chat group call? | - | - | 2 | P |
| 17-17A | Void | - | - | - | - |
| 18 | Check: Does the UE (MCPTT client) notify the user that the emergency group call has been established? (NOTE 1) | - | - | 2 | P |
| 19 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 20 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1. | - | - | - | - |
| 21 | Void | - | - | - | - |
| 22 | Make the UE (MCPTT client) request the establishment of an imminent peril chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 22A | E-UTRA/EPC signalling according to TS 36.579-1 [2] clause 5.4.3 'MCX CO communication in E-UTRA' takes place | - | - | - | - |
| 23 | Check: Does the UE (MCPTT client) send a SIP REFER request with <imminentperil_ind> set to true? | --> | SIP REFER | 3 | P |
| 24 | The SS sends a SIP 403 (Forbidden). | <-- | SIP 403 (Forbidden) | - | - |
| 25 | Check: Does the UE (MCPTT client) notify the user that the imminent peril group call is not allowed? (NOTE 1) | - | - | 3 | P |
| 25A | The SS waits 2 seconds before it releases the RRC connection | - | - | - | - |
| 26 | Make the UE (MCPTT client) request the establishment of an imminent peril chat group call using the pre-established session. (NOTE 1). | - | - | - | - |
| 26A-27A | Check: Does the UE (MCPTT client) correctly perform steps 1a1 to 3 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to initiate establishment of an imminent peril chat group call? | - | - | 3 | P |
| 28 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CT session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3.4.3-1 starting with step 2 to establish the imminent peril chat group call? | - | - | 3 | P |
| 29- 29A | Void | - | - | - | - |
| 30 | Check: Does the UE (MCPTT client) notify the user that the imminent peril group call has been established? (NOTE 1) | - | - | 3 | P |
| 31 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 32 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1? | - | - | 4 | P |
| 33-34 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.1.2.14.3.2	Test procedure sequence

Table 6.1.2.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a chat group call using the pre-established and the active functional alias, with automatic commencement mode and implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to establish a chat group call with implicit floor control using an active functional alias? | - | - | 1 | P |
| 3 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| 4 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 1 | P |
| 5 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1 | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.2.9.3.2	Test procedure sequence

Table 6.2.9.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a private call to user B using the pre-established session with automatic commencement mode and without floor control. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish an MCPTT private call without floor control? | - | - | 1 | P |
| 3-5 | Void | - | - | - | - |
| 5A | Make the UE (MCPTT client) request the floor. (NOTE 1). | - | - | - | - |
| 5B | Check: Does the UE (MCPTT client) send a Floor Request message in the next 5 sec? | --> | Floor Request | 1 | F |
| 6 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1? | - | - | 2 | P |
| 8-9 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The media plane control messages related to call setup/release over a pre-established session are sent over the channel used for media plane control. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.2.10.3.2	Test procedure sequence

Table 6.2.10.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CT Call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.8.3-1? | - | - | 1 | P |
| 2 | Void | - | - | - | - |
| 3 | Check: Does the UE (MCPTT client) notify the user that the call has been established? (NOTE 1) | - | - | 1 | P |
| 4 | Make the UE (MCPTT client) request the floor. (NOTE 1). | - | - | - | - |
| 4A | Check: Does the UE (MCPTT client) send a Floor Request message in the next 5 sec? | --> | Floor Request | 1 | F |
| 5 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CT call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.5.3-1? | - | - | 2 | P |
| 6 | Void | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) notify the user that the call has been released? (NOTE 1): | - | - | 2 | P |
| 8 | Void | - | - | - | - |
| 9 | Make the UE (MCPTT client) request the establishment of a private call to user B using the pre-established session with automatic commencement mode and without floor control. (NOTE 1). NOTE: This is to verify that although the Client has released the call it has not terminated the pre-established session. | - | - | - | - |
| 10 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a private call without floor control? | - | - | 2 | P |
| 11-12 | Void | - | - | - | - |
| 13 | Wait for 5 sec. | - | - | - | - |
| 14 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 15 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1 | - | - | - | - |
| 16 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The media plane control messages related to call setup/release over a pre-established session are sent over the channel used for media plane control. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.2.18.3.2	Test procedure sequence

Table 6.2.18.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a remotely initiated ambient listening call to user B using the pre-established session. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a remotely initiated ambient listening call? | - | - | 1 | P |
| 3 | The SS (MCPTT server) sends a Floor Taken message with the Permission to Request the Floor field set to '0'. | <-- | Floor Taken | - | - |
| 4 | Check: Does the UE (MCPTT client) notify the user that the remotely initiated ambient listening call has been established? (NOTE 1) | - | - | 1 | P |
| 5 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1? | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.2.22.3.2	Test procedure sequence

Table 6.2.22.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of an MCPTT first-to-answer call with targeted users B and C using the pre-established session. (NOTE 1) | - | - | - | - |
| 2a1-4 | Check: Does the UE (MCPTT client) correctly perform steps 1a1-3 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1? NOTE: The UE sends a SIP REFER which is responded with a SIP 200 OK. | - | - | 1 | P |
| 5-6 | Void | - | - | - | - |
| 7 | The SS (MCPTT server) sends a SIP re-INVITE containing the a=key-mgmt attribute. NOTE: The SS simulates the case when the first-to-answer Client sent a SIP 200 (OK) response containing an SDP answer including an a=key-mgmt "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 8a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place when the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 8a1 | The UE (MCPTT client) sends a SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| 9 | Check: Does the UE (MCPTT client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | 3 | P |
| 10 | The SS (MCPTT server) sends a SIP ACK. | <-- | SIP ACK | - | - |
| 10A-10B | Check: Does the UE (MCPTT client) correctly perform steps 4-5 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1? NOTE: The UE acknowledges the Connect sent by the SS. | - | - | 2 | P |
| 11 | Check: Does the UE (MCPTT client) notify the user that the call has been established? (NOTE 1) | - | - | 3 | P |
| 12 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 13 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1 | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The media plane control messages related to call setup/release over a pre-established session are sent over the channel used for media plane control. |  |  |  |  |  |

<End of modification>

<Start of modification>

6.2.26.3.2	Test procedure sequence

Table 6.2.26.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2-3A | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed requesting the establishment of a remotely initiated private call. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5-5C | Check: Does the UE (MCPTT client) correctly perform steps 2-5 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a remotely initiated private call? | - | - | 1 | P |
| 6 | Check: Does, in the next 5 sec, the UE (MCPTT client) send a SIP MESSAGE confirming the progress of the MCPTT remotely initiated private call? | --> | SIP MESSAGE | 1 | F |
| 6A | Check: Does the UE (MCPTT client) notify the user that a remotely initiated private call has been established? (NOTE 1) | - | - | 1 | F |
| 7 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 8 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1. | - | - | - | - |
| 9-11 | Steps 1a1-3 of procedure 'MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 are performed requesting the establishment of a remotely initiated private call. | - | - | - | - |
| 12-12C | Check: Does the UE (MCPTT client) correctly perform steps 2-5 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a remotely initiated private call? | - | - | 2 | P |
| 13 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX SIP MESSAGE CO' as described in TS 36.579-1 [2] Table 5.3.32.3-1 confirming the progress of the MCPTT remotely initiated private call? | - | - | 2 | P |
| 14 | Void | - | - | - | - |
| 15 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 16 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

Table 6.2.26.3.2-2: Void

<End of modification>