3GPP TSG-RAN5 Meeting #102	R5-240575

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0360 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of testcase 6.2.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 24.379 clause 11.1.6.2.1.1 (Client originating procedures for On-demand ambient listening call) "if this is a remotely initiated ambient listening call," the MCPTT client "shall comply with the conditions for an implicit request to grant the floor to the terminating MCPTT client as specified in clause 6.4". According to 24.379 clause 6.4 an implicit request to grant the floor to the terminating MCPTT client requires the "mc_implicit_request" 'fmtp' attribute to be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - mc_implicit_request added to SDP offer (Table 6.2.14.3.3-3) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.14	On-network / Private Call / Ambient listening call / Remotely initiated Ambient listening call / Remotely initiated ambient listening call release / Success / Client Originated (CO) / Server initiated ambient call release

6.2.14.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate Remotely initiated ambient listening }

ensure that {

when { UE (MCPTT User) requests the establishment of a remotely initiated ambient listening call }

then { UE (MCPTT Client) sends a SIP INVITE message (end-to-end security context provided) requesting the establishment of a remotely initiated ambient listening call, and, after indication from the MCPTT Server that the call was established notifies the MCPTT user, and does not do any floor request }

}

(2)

with { UE (MCPTT Client) having initiated a remotely initiated ambient listening call, and, having been notified by the server that the MCPTT server is capable of receiving a SIP BYE from an MCPTT client to release an ambient-listening call }

ensure that {

when { UE (MCPTT User) wants to release the MCPTT remotely initiated ambient listening call }

then { UE (MCPTT Client) sends a SIP BYE request and after receiving a SIP 200 (OK) terminates the call }

}

(3)

with { UE (MCPTT Client) having initiated a remotely initiated ambient listening call }

ensure that {

when { UE (MCPTT client) receives a SIP BYE message including a <release-reason> element set to "administrator-action" }

then { UE (MCPTT Client) sends a SIP 200 (OK) response and terminates the call }

}

6.2.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 11.1.6.2.1.1, 11.1.6.2.1.3, 11.1.6.2.1.4, 6.2.1, 6.4, F.1.3. Unless otherwise stated these are Rel-14 requirements.

[TS 24.379, clause 11.1.6.2.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT ambient listening call that has been authorised successfully by the requesting MCPTT client, the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to a public service identity of the participating MCPTT function serving the MCPTT user;

2)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

3)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref contain with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

7)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "ambient-listening";

8)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <ambient-listening-type> element set to a value of:

...

b)	"remote-init", if the MCPTT user has requested a remotely initiated ambient listening call;

9)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCPTT ID of the targeted MCPTT user, according to rules and procedures of IETF RFC 5366 [20];

NOTE 1:	the targeted MCPTT user is the listened-to MCPTT user in the case of a remotely initiated ambient listening call or the listening MCPTT user in the case of a locally initiated listening call.

10)	if an end-to-end security context needs to be established then:

a)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [78];

b)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [78];

c)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [78];

d)	shall encrypt the PCK to a UID associated to the MCPTT client using the MCPTT ID and KMS URI of the invited user and a time related parameter as described in 3GPP TS 33.180 [78];

e)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [78];

f)	shall add the MCPTT ID of the originating MCPTT to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [78]; and

g)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCPTT user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [78];

11)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.1 and with a media stream of the offered media-floor control entity;

...

13)	if this is a remotely initiated ambient listening call, shall comply with the conditions for an implicit request to grant the floor to the terminating MCPTT client as specified in subclause 6.4;

14) shall include in the SIP INVITE request a Priv-Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [18]; and

15)	shall send the SIP INVITE request towards the participating MCPTT function according to 3GPP TS 24.229 [4].

...

Upon receiving a SIP 200 (OK) response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	if this is a remotely initiated ambient listening call, shall notify the user that the call has been successfully established;

[TS 24.379, clause 11.1.6.2.1.3]

Upon receiving a request from an MCPTT user to release an MCPTT ambient listening call:

The MCPTT client:

...

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

3)	shall generate a SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4] and IETF RFC 6086 [64]; and

4)	shall send the SIP BYE request within the dialog of the MCPTT ambient call session as specified in 3GPP TS 24.229 [4].

Upon receipt of the SIP 200 (OK) response to the SIP BYE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

[TS 24.379, clause 11.1.6.2.1.4]

Upon receipt of a SIP BYE request in the dialog of an ambient listening session, the MCPTT client:

1)	shall comply with the procedures of subclause 6.2.6;

[TS 24.379, clause 6.2.1]

The SDP offer shall contain only one SDP media-level section for MCPTT speech according to 3GPP TS 24.229 [4] and, if floor control shall be used during the session, shall contain one SDP media-level section for a media-floor control entity according to 3GPP TS 24.380 [5].

When composing an SDP offer according to 3GPP TS 24.229 [4] the MCPTT client:

...

2)	shall include an "m=audio" media-level section for the MCPTT media stream consisting of:

...

c)	if the SDP offer is for an ambient listening call:

i)	if this is a remotely initiated ambient listening call, include an "a=recvonly" attribute; or

[TS 24.379, clause 6.4]

An initial SIP INVITE request fulfilling the following criteria shall be regarded by the MCPTT server as an implicit request to grant the floor to the terminating MCPTT client when the originating MCPTT client:

1)	initiates a remotely initiated MCPTT ambient listening call; and

2)	includes the "mc_implicit_request" 'fmtp' attribute in the associated UDP stream for the floor control in the SDP offer/answer as specified in 3GPP TS 24.380 [5] clause 12.

[TS 24.379, clause F.1.3]

If the <mcpttinfo> contains the <mcptt-Params> element then:

...

2)	the <session-type> can be included with:

...

e)	a value of "ambient-listening" to indicate the MCPTT client wants to make an ambient listening call;

...

19)	the <anyExt> can be included with the following elements not declared in the XML schema:

a)	an <ambient-listening-type> of type "xs:string":

i)	set to a value of "remote-init" when the listening MCPTT user of an ambient listening call initiates the call; or

6.2.14.3	Test description

6.2.14.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlining "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCPTT UE registration' as specified in TS 36.579-1 [2] clause 5.4.2.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	The MCPTT user is authorized to initiate remotely initiated ambient listening: the <allow-request-remote-initiated-ambient-listening> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true"

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.2.14.3.2	Test procedure sequence

Table 6.2.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a remotely initiated ambient listening call to user B. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3A.1.3-1 to establish a remotely initiated ambient listening call without implicit floor control according to option a of NOTE 1 in TS 36.579.1 [2] Table 5.3A.1.3-1? | - | - | 1 | P |
| 3 | Void | - | - | - | - |
| 4 | The SS (MCPTT server) sends a Floor Taken message with the Permission to Request the Floor field set to '0'. | <-- | Floor Taken | - | - |
| 5 | Check: Does the UE (MCPTT client) notify the user that the remotely initiated ambient listening call has been established? (NOTE 1) | - | - | 1 | P |
| 6 | Make the UE (MCPTT client) request the floor. (NOTE 1) | - | - | - | - |
| 6A | Check: Does the UE (MCPTT client) send a Floor Request message in the next 5 sec? | --> | Floor Request | 1 | F |
| 7 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 36.579-1 [2] Table 5.3.10.3-1? | - | - | 2 | P |
| 9-10 | Void | - | - | - | - |
| 11 | Make the UE (MCPTT client) request the establishment of a remotely initiated ambient listening call to user B. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3A.1.3-1 to establish a remotely initiated ambient listening call without implicit floor control according to option a of NOTE 1 in TS 36.579.1 [2] Table 5.3A.1.3-1? | - | - | 1 | P |
| 13 | Void | - | - | - | - |
| 14 | The SS (MCPTT server) sends a Floor Taken message with the Permission to Request the Floor field set to '0'. | <-- | Floor Taken | - | - |
| 15 | Void | - | - | - | - |
| 16 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CT call release' as described in TS 36.579-1 [2] Table 5.3.12.3-1? | - | - | 3 | P |
| 17-18 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.2.14.3.3	Specific message contents

Table 6.2.14.3.3-1: SIP INVITE from the UE (Steps 2, 12, Table 6.2.14.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  |  |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-param | “require” |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.14.3.3-3 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.14.3.3-2 |  |  |  |

Table 6.2.14.3.3-2: MCPTT-Info in SIP INVITE (Table 6.2.14.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition PRIVATE-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| session-type | "ambient-listening" |  |  |  |
| anyExt |  |  |  |  |
| ambient-listening-type | "remote-init" |  |  |  |

Table 6.2.14.3.3-3: SDP Message in SIP INVITE (Table 6.2.14.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition PRIVATE-CALL, INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media description[1] |  | Media description for audio |  |  |
| media attribute |  | a=line attribute=recvonly |  |  |
| recvonly |  |  |  |  |

Table 6.2.14.3.3-4: SIP 200 (OK) from the SS (Steps 2, 12, Table 6.2.14.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Feature-Caps |  |  |  |  |
| fcap-name | "g.3gpp.mcptt.ambient-listening-call-release" | Indicates that the MCPTT server is capable of receiving a SIP BYE from an MCPTT client to release an ambient-listening call |  |  |
| Message-body |  |  |  |  |
| SDP message | SDP message as described in Table 6.2.14.3.3-4A |  |  |  |

Table 6.2.14.3.3-4A: SDP Message in SIP 200 OK (Table 6.2.14.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media description[1] |  | Media description for audio |  |  |
| media attribute |  | a=line attribute=sendonly |  |  |
| sendonly |  |  |  |  |

Table 6.2.14.3.3-5: Floor Taken (Steps 4, 14, Table 6.2.14.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.6.7-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "0" | The receiver is NOT permitted to request floor |  |

Table 6.2.14.3.3-6: Void

Table 6.2.14.3.3-7: SIP BYE from the SS (Step 16, Table 6.2.14.3.2-1;
step 1, TS 36.579-1 [2] Table 5.3.12.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1, condition MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "multipart/mixed" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.14.3.3-8 |  |  |  |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  |  |  |
| MIME-part-body | Signatures for XML MIME bodies as described in TS 36.579-1 [2] Table 5.5.13.1-2 |  |  |  |

Table 6.2.14.3.3-8: MCPTT-Info in SIP BYE (Table 6.2.14.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| anyExt |  |  |  |  |
| release-reason | "administrator-action" |  |  |  |