3GPP TSG-RAN5 Meeting #102	R5-240586

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5284 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT RLC TC 22.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TC 23.3.2.3 step 24A some text has been dropped erroneously. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TC 23.3.2.3 step 24A the missing text was re-instated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The prose will remain erroneous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 22.3.2.3	NB-IoT / AM RLC / In sequence delivery of upper layers PDUs/ Different numbers of length indicators

##### 22.3.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives duplicate AMD PDUs }

then { UE discards the duplicate AMD PDUs }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU with a SN gap }

then { UE sends STATUS PDU to request retransmissions of PDUs in the SN gap }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives PDUs within a SN gap }

then { RLC reassembles and reorders the AMD PDUs and deliver them to the upper layer in sequence }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU or an AMD PDU segment with no LI field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU or an AMD PDU segment more than one LI field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

##### 22.3.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.322, clause 4.2.1.3.3. Unless otherwise stated these are Rel-13 requirements.

[TS 36.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives RLC data PDUs, it shall:

-	detect whether or not the RLC data PDUs have been received in duplication, and discard duplicated RLC data PDUs;

-	reorder the RLC data PDUs if they are received out of sequence;

-	detect the loss of RLC data PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the reordered RLC data PDUs and deliver the RLC SDUs to upper layer in sequence.

At the time of RLC re-establishment, the receiving side of an AM RLC entity shall:

-	if possible, reassemble RLC SDUs from the RLC data PDUs that are received out of sequence and deliver them to upper layer;

-	discard any remaining RLC data PDUs that could not be reassembled into RLC SDUs;

-	initialize relevant state variables and stop relevant timers.

[TS 36.322, clause 6.2.2.5]

Length: 11 bits for RLC UM, 11 bits or 15 bits for RLC AM. The length of the LI field for RLC AM is configured by upper layers.

The LI field indicates the length in bytes of the corresponding Data field element present in the RLC data PDU delivered/received by an UM or an AM RLC entity. The first LI present in the RLC data PDU header corresponds to the first Data field element present in the Data field of the RLC data PDU, the second LI present in the RLC data PDU header corresponds to the second Data field element present in the Data field of the RLC data PDU, and so on. The value 0 is reserved.

##### 22.3.2.3.3	Test description

22.3.2.3.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

UE:

-	None.

Preamble:

-	The UE is in state 2B-NB according to [18] the exceptions listed in table 22.3.2.3.3.1-1 with test loop mode G closed.

Table 22.3.2.3.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit-r13 | ms4000 |
| enableStatusReportSN-Gap-r13 | true |

22.3.2.3.3.2	Test procedure sequence

If the start RLC UL and DL sequence numbers to be used at start of test body are non zero, but X and Y respectively due to transmission/reception of RLC PDU’s in preamble on bearer to be used, then any sequence number  ‘n’  in test procedure maps as :

- UL SQN n maps to SQN X+n MOD 1024 &

- DL SQN n maps  to SQN Y+n MOD 1024

Table 22.3.2.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS does not respond to PRACH preambles transmitted by UE for Uplink transmission, but instead allocates the UL C-RNTI grant on NPDCCH when specified in the test sequence. Size of each RLC UL SDU #1 - #3 is PRBS of 256 bits (32 octets), size of each RLC UL SDU #4 - #16 is PRBS of 80 bits (10 octets). The size of the RLC SDUs used in downlink is L. | - | - | - | - |
| 1 | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#1 without LI field. | <-- | AMD PDU#1 (SN=0) | - | - |
| 2 | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#1 without LI field. | <-- | AMD PDU#1 (SN=0) | - | - |
| 2A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 2 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) | <-- | (UL Grant) | - | - |
| 3 | Check: Does the UE transmit RLC UL SDU#1? | --> | (RLC UL SDU#1) | 1,4 | P |
| 4 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=1) | - | - |
| 5 | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#2, and the 1st part of RLC DL SDU#3. | <-- | AMD PDU#2 (SN=1) | - | - |
| 5A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 5 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) | <-- | (UL Grant) | - | - |
| 6 | Check: Does the UE transmit RLC UL SDU#2 with the poll bit set? | --> | (RLC UL SDU#2) | 1 | P |
| 7 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=2) | - | - |
| 8 | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#2, and the 1st part of RLC DL SDU#3. | <-- | AMD PDU#2 (SN=1) | - | - |
| 8A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 8 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) | <-- | (UL Grant) | - | - |
| 9 | Check: Does the UE transmit RLC UL SDU#2? | --> | (RLC UL SDU#2) | 1 | F |
| 10 | The SS transmits an AMD PDU to the UE. This PDU contains the 2nd part of RLC DL SDU#3. | <-- | AMD PDU#3 (SN=2) | - | - |
| 10A | In the search space of the 3rd NPDCCH period bundle after the transmission at step 10 the SS schedules an UL grant of size 296 bits. (Note 1, Note 7) | <-- | (UL Grant) | - | - |
| 11 | Check: Does the UE transmit RLC UL SDU#3? | --> | (RLC UL SDU#3) | 1 | P |
| 12 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=3) | - | - |
| 13 | The SS transmits an AMD PDU to the UE. This PDU contains the last part of RLC DL SDU#6. | <-- | AMD PDU#6 (SN=5) | - | - |
| 14 | The SS transmits an AMD PDU to the UE. This PDU contains the 2nd part of RLC DL SDU#5, and the 1st part of RLC DL SDU#6. | <-- | AMD PDU#5 (SN=4) | - | - |
| 15 | The SS does not allocate any uplink grant. | - | - | - | - |
| 16 | The SS transmits an AMD PDU to the UE. This PDU carries RLC DL SDU#4 and the 1st part of RLC DL SDU#5. | <-- | AMD PDU#4 (SN=3) | - | - |
| 17 | In the search space of the 3rd NPDCCH period bundle after step 16 the SS schedules an UL grant of size 504 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#4, RLC DL SDU#5 and RLC DL SDU#6 and report RLC Status PDU. (Note 2, Note 7) | <-- | (UL grant) | - | - |
| 18 | Check: Does the UE transmit a STATUS PDU with ACK_SN=6 and an AMD PDU containing RLC UL SDU#4, RLC UL SDU#5 and RLC UL SDU#6 in its data field? | --> | STATUS PDU and AMD PDU (RLC UL SDU#4, RLC UL SDU#5, RLC UL SDU#6) | 3 | P |
| 19 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=4) | - | - |
| 20 | The SS transmits an AMD RLC PDU to the UE. This PDU contains the last part of RLC DL SDU#9. | <-- | AMD PDU#9 (SN=8, P=1) | - | - |
| 20A | In the search space of the 3rd NPDCCH period bundle after step 20 the SS schedules an UL grant of size 88 bits. (Note 3, Note 7) | <-- | (UL Grant) | - | - |
| 21 | Check: Does the UE transmit a STATUS PDU NACK_SN/E1/E2 fields set correctly to inform SS of missing PDUs #7, #8, (ACK_SN=9, NACK_SN= 6, NACK_SN= 7)? | --> | STATUS PDU | 2 | P |
| 22 | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#8, and the 1st part of RLC DL SDU#9. | <-- | AMD PDU#8 (SN=7) | - | - |
| 23 | The SS does not allocate any uplink grant. | - | - | - | - |
| 24 | The SS transmits an AMD PDU to the UE. This PDU carries SDU#7. (Note 7) | <-- | AMD PDU#7 (SN=6) | - | - |
| 24A | In the search space of the 3rd NPDCCH period bundle after step 24 the SS schedules an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#7, RLC DL SDU#8 and RLC DL SDU#9 and report RLC Status PDU. (Note 4) | <-- | (UL grants) | - | - |
| 25 | Check: Does the UE transmit a STATUS PDU with ACK_SN=9 and an AMD PDU containing RLC UL SDU#7, RLC UL SDU#8 and RLC UL SDU#9 in its data field? | --> | STATUS PDU and AMD PDU (RLC UL SDU#7, RLC UL SDU#8, RLC UL SDU#9) | 3 | P |
| 26 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=5) | - | - |
| 27 | The SS transmits an AMD PDU to the UE. This PDU contains RLC DL SDU#10, RLC DL SDU#11 and RLC DL SDU#12 with two LI fields. | <-- | AMD PDU#10 (SN=9) | - | - |
| 28 | In the search space of the 3rd NPDCCH period bundle after step 27 the SS schedules an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#10, RLC DL SDU#11 and RLC DL SDU#12. (Note 5, Note 7) | <-- | (UL grant) | - | - |
| 29 | Check: Does the UE transmit an AMD PDU containing RLC UL SDU#10, RLC UL SDU#11 and RLC UL SDU#12 in its data field? | --> | AMD PDU (RLC UL SDU#10, RLC UL SDU#11, RLC UL SDU#12) | 5 | P |
| 30 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=6) | - | - |
| 31 | The t-PollRetransmit-r13 timer for AMD PDU#11 expires and SS assumes that the transmission of AMD PDU#11 containing RLC DL SDU#13, RLC DL SDU#14, RLC DL SDU#15 and RLC DL SDU#16 is failed and considers AMD PDU#11 for re-transmission. | - | - | - | - |
| 32 | The SS transmits an AMD PDU segment to the UE. This PDU segment contains RLC DL SDU#13 without LI field. | <-- | AMD PDU segment (SM=10) | - | - |
| 33 | In the search space of the 3rd NPDCCH period bundle after step 32 the SS schedules an uplink grant of size 208 bits allowing the UE to transmit 1 RLC SDU. (Note 6, Note 7) | <-- | (UL grant) | - | - |
| 34 | Check: Does the UE transmit a STATUS PDU with NACK_SN=10, SOStart, SOEnd and ACK_SN=11 and an AMD PDU containing RLC UL SDU#13 in its data field? | --> | AMD PDU (RLC UL SDU#13) | 4 | P |
| 35 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=7) | - | - |
| 36 | The SS transmits AMD PDU segment to the UE. This PDU segment contains SDU#14, SDU#15 and SDU#16 with two LI fields. | <-- | AMD PDU segment | - | - |
| 37 | In the search space of the 3rd NPDCCH period bundle after step 32 the SS schedules an UL grant of size 328 bits sufficient for the UE to loopback the PRBS parts of RLC DL SDU#14, RLC DL SDU#15 and RLC DL SDU#16. (Note 5, Note 7) | <-- | (UL grant) | - | - |
| 38 | Check: Does the UE transmit an AMD PDU containing RLC UL SDU#14, RLC UL SDU#15 and RLC UL SDU#16 in its data field? | --> | AMD PDU (RLC UL SDU#14, RLC UL SDU#15, RLC UL SDU#16) | 5 | P |
| 39 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=8) | - | - |
| NOTE 1: UL grant of 296 bits (ITBS=9, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Short BSR subheader + 1 byte MAC PDU subheader, 1 byte Short BSR + 34 bytes MAC SDU(16-bit RLC AMD PDU header + 256 bit UL RLC SDU)). NOTE 2: UL grant of 504 bits (ITBS=10, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report and a AMD PDU (a 4 byte Status Report with ACK_SN and 1 NACK_SN (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + (12-bit NACK_SN/E1/E2) + 5-bit Padding) or ACK_SN and 2 NACK_SN (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2*(12-bit NACK_SN/E1/E2) + 1-bit Padding) + a 35 bytes RLC AMD PDU(5 bytes RLC AMD PDU header + 3*10 bytes UL RLC SDU) + 6 bytes MAC PDU subheader + 1 byte Short BSR + n bytes MAC Padding). NOTE 3: UL grant of 88 bits (ITBS=6, =0, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report with ACK_SN and 2 NACK_SNs (4 bytes MAC PDU subheader + 1 byte Short BSR + 5 bytes RLC Status PDU (4-bit D/C/CPT + 10-bit ACK_SN + 1-bit E1 + 2 x (12-bit NACK_SN/E1/E2) + 1-bit Padding) + 1 byte MAC Padding ). NOTE 4: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a Status Report and a AMD PDU. NOTE 5: UL grant of 328 bits (ITBS=10, =1, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit a AMD PDU (4 bytes MAC PDU subheader + a 35 bytes RLC AMD PDU(5 bytes RLC AMP PDU header + 3*10 bytes UL RLC SDU) + 1 byte Short BSR + 1 byte MAC Padding). NOTE 6: UL grant of 208 bits (ITBS=4, =2, see TS 36.213 Table 16.5.1.2-2) is chosen to allow the UE to transmit one PDU at a time. (MAC subheader: 1 byte Padding subheader + 1 byte Short BSR subheader + 2 bytes MAC PDU subheader + 1 byte MAC PDU subheader, 1 byte Short BSR + 12 bytes MAC SDU(2 bytes RLC AMD PDU header + 10 bytes bit UL RLC SDU) + 8 bytes MAC SDU (8 bytes of RLC STATUS PDU with NACK_SN and SOstart+SOend fields). NOTE 7: NPDCCH period bundle is specified in TS 36.523-3 [20] clause 7A.14.2. |  |  |  |  |  |

22.3.2.3.3.3	Specific message contents

None.

<End of modified section>