# Title:	Test Tolerances for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

Source:	Qualcomm, Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT).

The test case in question, from 38.533, is:

- 6.2.1,		NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)

- 16.2.1.2,	NR SA FR1 CG-SDT for 2Rx UE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.12.0 Annex A.

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

A.6.2.1.1	Test purpose and Environment

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. The test includes two sub-tests, Sub-test#1 for testing valid TA where UE can initiat CG-SDT transmission, and Sub-test#2 for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 is passed. For each sub-test, UE is configured with CG-SDT configurations when entering RRC Inactive state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3 and T4 repectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6 repectively. There is one cell, which is the active NR cell in FR1. Figure A.6.2.1.1-1 shows the variation of the RSRP over the duration of Sub-test#1, and Figure A.6.2.1.1-2 shows the variation of the RSRP over the duration of Sub-test#2.

In Sub-test#1:

Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC connected mode.

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2. TE must be W2 before TF.

Test equipment triggers UL data arrival at UE lower layer at time point TF. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

After time point TG, RRC status is changed from RRC INACTIVE to RRC CONNECTED.

In Sub-test#2:

Prior to the time point TA, the UE shall pass Sub-test#1 and have entered RRC connected mode. Otherwise, Sub-test#2 shall not be executed.

From time point TA to time point TD, RSRP is set to P2.

At time point TC, which is W1 after time point TB, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P2 to P0.

Test equipment triggers UL data arrival at UE lower layer at time point TF. TF is 3360ms after TD. After time point TF, test equipment observes whether UE transmits with CG-SDT no later than TG which is W3 after TF.

W1 equals to 640ms and W2 equals to 640ms based on requirements in clause 5.5.3. W3 is 860ms.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT Sub-test#1

Figure A.6.2.1.1-2: RSRP variation model for CG-SDT Sub-test#2

A.6.2.1.2	Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.7 |  |
| T1 | s | 1,2,3 | 0.4 |  |
| T2 | s | 1,2,3 | 1.28 |  |
| T3 | s | 1,2,3 | 2.72 |  |
| T4 | s | 1,2,3 | 1.5 |  |
| T5 | s | 1,2,3 | 1.68 |  |
| T6 | s | 1,2,3 | 4.22 |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | 14 |  |
| cg-SDT-RSRP-ThresholdSSB | dBm | 1,2,3 | -110 |  |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | infinity |  |
| CG-SDT resource period | ms |  | 320 | configuredGrantConfig Peridocity at 38.331 Config 1,2 : sym320x14, config 3 : sym640x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
|  | 1,2,3 | dBm/15kHz | -100 |  |  |  |  |  |
|  | 1,2 | dBm/SSB SCS | -100 |  |  |  |  |  |
|  | 3 | dBm/SSB SCS | -97 |  |  |  |  |  |
|  | 1,2,3 | dB | 0 | 8 | 0 | 18 | 18 | 0 |
| SS RSRP | 1,2,3 | dBm/SSB SCS | -100 | -92 | -100 | -82 | -82 | -100 |
| Io | 1,2 | dBm/9.36MHz | -69.04 | -63.41 | -69.04 | -53.98 | -53.98 | -69.04 |
|  | 3 | dBm/38.16MHz | -62.93 | -57.31 | -62.93 | -47.88 | -47.88 | -62.93 |
|  | 1,2,3 | dB | 0 | 8 | 0 | 18 | 18 | 0 |

A.6.2.1.3	Test requirements

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit PUSCH at CG-SDT resource within 860ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit PUSCH at CG-SDT resources after TF until the end of the test at time point TG.

The rate of correct events observed during repeated tests shall be at least 90%.

# 3. Discussion

The test consists in two subtests, the first one comprising of 4 time periods (T1, T2, T3 and T4), and the second one comprising of 2 time periods (T5, and T6). Each of the two subtests performs a different check:

- Subtest 1: checks that the UE correctly initiates CG-SDT operation when the SS-RSRP is above threshold (positive test)

- Subtest 2: checks that the UE correctly does not initiate CG-SDT operation when the SS-RSRP is below threshold (negative test)

The execution of subtest 2 is conditioned to the verdict on subtest 1: subtest 2 is only performed if subtest 1 passed.

From a Test Tolerance analysis perspective, each subtest can be analyzed independently – when the UE is performing one of the two checks described above, only the test conditions and parameters for that particular subtest apply. It is helpful to show the SS-RSRP variation as well as the criteria the UE needs to satisfy –or not satisfy, respectively– on each of the subtests:

Figure 3-1: SS-RSRP variation for subtest 1

Figure 3-2: SS-RSRP variation for subtest 2

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters need to be copied from Tables A.6.2.1.2-1, A.6.2.1.2-2 and A.6.2.1.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The signalled parameters cg-SDT-RSRP-ChangeThreshold and cg-SDT-RSRP-ThresholdSSB are also copied.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In subtest 1, during T4, the UE shall measure the SS-RSRP level of SSB#0 and compare it with threshold cg-SDT-RSRP-ThresholdSSB. Furthermore, the UE shall measure the SS-RSRP level of SSB#0 and compare it with the stored SS-RSRP level of SSB#0 during T2, when the CG-SDT configuration was received, and with the signaled thresholds. One of the criteria to be satisfied is that the SS-RSRP variation between these two time instances is less than cg-SDT-RSRP-ChangeThreshold. Therefore, the following derived parameter is calculated:

- |RSRP(T4) – RSRP(T2)|

Which needs to be less than cg-SDT-RSRP-ChangeThreshold in order to be satisfied.

Similarly, in subtest 2, during T6, the UE shall measure the SS-RSRP level of SSB#0 and compare it with threshold cg-SDT-RSRP-ThresholdSSB, furthermore, the UE shall measure the SS-RSRP level of SSB#0 and compare it with the stored SS-RSRP level of SSB#0 during T5, when the CG-SDT configuration was received, and with the signaled thresholds. One of the criteria to be satisfied is that the SS-RSRP variation between these two time instances is greater than cg-SDT-RSRP-ChangeThreshold. Therefore, the following derived parameter is calculated:

- |RSRP(T6) – RSRP(T5)|

Which needs to be greater than cg-SDT-RSRP-ChangeThreshold in order to be not satisfied.

c) Uncertainties

The SS provides 1 NR Cell without fading to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE SS-RSRP measurement accuracy need to be considered. Since UE SS-RSRP measurements are compared with a signaled threshold, any potential inaccuracy could affect the test verdict. The comparasion consists of two aspects:

- First, the UE shall compare the RSRP in T4 & T6 with a absolute RSRP threshlold cg-SDT-RSRP-ThresholdSSB. So intra-frequency RSRP absolute accuracy is needed to be considered;

- Second, the UE shall compare the relative RSRP |RSRP(T4)- RSRP(T2)| & |RSRP(T6)- RSRP(T5)| with differential RSRP threshold. So intra-frequency RSRP relative accuracy is needed to be considered.

However, the only accuracy requirements defined for idle/inactive state in 38.133 are accuracy requirements for EMR measurements (clause 10.1.2B.1) which can’t be referred here. The reasons are:

- UE may not support EMR measurements since it is an optional capability. Furthermore, UE which only supports single carrier operation (i.e. RedCap UEs) doesn’t support EMR measurements for sure.

- There is also no relative accuracy requirements for EMR measurments.

We propose to use the intra-frequency SS-RSRP measurement accuracy requirements for connected state specified in 38.133 clause 10.1.2 as the reference. The reasons are:

- According to 38.133 clause 5.5.3, the time UE having completed RSRP1 (T1’) can be earlier than the time UE receiving RRCRelease message with SDT configuration (T1), which implies than RSRP1 can be obtained when the UE is in connected state and complies with the accuracy requirements in the connected state.

Table 5.5.3-1 Valid measurement for FR1

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

- Techinally, the measurement performed in idle/inactive state and the measurement performed in connected state shall have similar accuracy requirements.

- It’s mandatory for UE to support intra-frequency SS-RSRP measurement in connected state. Furthermore, both intra-frequency SS-RSRP measurement absolute accuracy and relative accuracy are well defined.

10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

10.1.2.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.2.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2 | 3 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

To apply accuracy requirements, the conditions in TS 38.133 clause B.2.2 need to be satisfied:

B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The above side conditions can be consolidated into the following criteria, taking band group NR_FDD_FR1_H since it is the most restrictive:

- SSB Es/Iot ≥ -3dB

- SSB_RP ≥ -123.5dBm/15kHz

- Io ≥ -117.5dBm/15kHz

- Io ≤ -50dBm/Ch BW

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the SS-RSRP, so the sensitivity factor is 1.000 during all time intervals. Similarly, an error of 1dB in the Es/Noc would cause 1dB error in the SS-RSRP, so the sensitive factor is also 1.000 during all time intervals.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle. Uncertainties are calculated separately for each subtest and for each time interval.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE reporting accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE reporting accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

Configurations 1 and 2

Signaled parameter cg-SDT-RSRP-ChangeThreshold and cg-SDT-RSRP-ThresholdSSB remains unchanged.

Subtest 1

No offset is needed.

Subtest 2

No offset is needed.

Configuration 3

Signaled parameter cg-SDT-RSRP-ThresholdSSB is decreased by 1dB to ensure RSRP in T6 ≥ cg-SDT-RSRP-ThresholdSSB when considering all uncertainties for both subtests.

Subtest 1

Freq 1 Noc is decreased by 3.7dB to ensure Io in T4 are below -50dBm/Ch BW.

More details about the other requirements are presented in the analysis spreadsheet.

Subtest 2

Freq 1 Noc is decreased by 3.7dB to ensure Io in T5 are below -50dBm/Ch BW.

More details about the other requirements are presented in the analysis spreadsheet.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

The values used to offset each parameter have been found empirically by observing the minimum/maximum parameter values, for each requirement specified in step g).

g) Check controlled parameters Min/Max

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.2.1 contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Io requirement, section g) and associated Source/Reference

# 6. Treatment of other similar test cases

The RedCap CG-SDT test case 16.2.1.2 have the same power settings and the same test requirements as the SA test analysed in this document. For these test cases:

- Test parameters for config 1/2/4 are identical with that of 6.2.1 config 1/2 with the exception that HD-FDD duplex mode is used in config 4. Then the same test tolerances for 6.2.1 config 1/2 apply considering duplex mode bringing no any additional measurement uncertainties.

- Test parameters for config 3 are identical with that of 6.2.1 config 3 with the exception that the BW is only 20MHz (51 RBs). As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 6.2.1 config 3:

  - Channel bandwidth and RBs in Channel bandwidth, section a) and associated comment

From the analysis one can observe that the only difference is that Io value is 10*(log106/51) = 3.18dB lower. Then the test tolerance to be added is changed to

  - Freq 1 Noc is decreased by 0.5dB to ensure maximum Io <= -50dBm/Ch BW