3GPP TSG-RAN5 Meeting #102	R5-240734

# Athens, Greece, Feb 26th – Mar 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0657 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT analysis for RedCap RRM test case 16.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To provide TT analysis for RedCap RRM TC 16.2.1.2. - Existing TT analysis for Rel-17 SDT TC 6.2.1 needs to be updated in following aspects: - It’s inappropariate to use RSRP absolute accuracy for EMR in TT analysis for SDT test. - First, EMR is an optional capability. It’s doesn’t make sense to use EMR accuracy requirements for a UE don’t support EMR. For an example, RedCap UEs may support CG-SDT but certainly don’t support EMR since EMR is for CA/DC operation. - UE needs to compare the differential RSRP ¦RSRP(T4)- RSRP(T2)¦ & ¦RSRP(T6)- RSRP(T5)¦ with RSRP threshold. So intra-frequency RSRP relative accuracy, rather than absolute accuracy, should be considered here. However, there is no relative accuracy requirements for EMR. So we suggest use intra-frequency SS-RSRP measurement accuracy requirements for connected state instead. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TT analysis for RedCap RRM TC 16.2.1.2 is added to the attachement of TS 38.903. - Existing TT analysis for Rel-17 SDT TC 6.2.1 is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Remove the following .zip file from the attachements of TS 38.903

“38.533 6.2.1 TT.zip”

Add the following .zip file to the attachements of TS 38.903:

“38.533 6.2.1+16.2.1.2 TT.zip”

<End of modified section 1>