# Title:	Test Tolerances for Random access tests in FR1

# Source:	Rohde & Schwarz, Huawei, Hisilicon

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for 2-step Contention Based Random Access Test.

The test cases in question, from TS 38.533, are:

- 4.3.2.2.3, 2-step Contention based random access test in FR1 for PSCell in EN-DC

- 4.3.2.2.4, 2-step Non-contention based random access test in FR1 for PSCell in EN-DC

- 6.3.2.2.3, 2-step Contention based random access test in FR1 for NR standalone

- 6.3.2.2.4, 2-step Non-Contention based random access test in FR1 for NR standalone

- 16.3.2.2.6, NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE

- 16.3.2.2.8, NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v16.12.0 Annex A.

A standalone test case is chosen, to show only the NR-related parameters.

<< Extract from TS 38.133 >>

##### A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone

A.6.3.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.3.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.3.1-2.

Table A.6.3.2.2.3.1-1: Supported test configurations for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.3.1-2: General test parameters for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  | Config 1 |  | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 1 | As defined in A.3.20.2.1. |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.3.2.2.3.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA preamble transmission shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.1.2 the System Simulator shall transmit a MsgB containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit an ACK if the MsgB with a successRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble and if the Contention Resolution is successful.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB(s) contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.1.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

### A.3.20.2	MsgA configurations in FR1

#### A.3.20.2.1	FR1 MsgA configuration 1

FR1 MsgA configuration 1 in this clause provides the typical MsgA configuration for SSB-based contention based random access for 2-step RA type in FR1.

Table A.3.20.2.1-1: Parameters for FR1 MsgA configuration 1

| Field | Value | Comment |
| --- | --- | --- |
| msgA-prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| msgA-RO-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| msgA-MCS | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 |  |
| PUSCH allocation length | 14 |  |
| mappingTypeMsgA-PUSCH | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Whether to apply delta MCS |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

# 3. Discussion

The test consists of three subtests with one cell configured; the difference between the Subtests is related to the UE behaviour scenarios for the random access procedure

The UE shall calculate MsgA transmission power according to the MsgA power formula, and apply this power level at the first preamble or additional preambles. The transmit timing of all MsgA transmissions shall be within the accuracy specified in TS 38.133. Overall there are five types of requirement:

- Choosing the correct SSB index according to levels and msgA-RSRP-ThresholdSSB – see section 4

- Absolute power of first MsgA preamble – see section 5

- Absolute power of first MsgA PUSCH transmission – see section 5A

- Relative power of subsequent four MsgA preambles – see section 6

- Timing of each MsgA preamble – see section 7

The setting of test parameters/threshold levels and calculation of the absolute power of the first MsgA preamble is included in the accompanying spreadsheet analysis.

# 4. Calculation of Test Tolerances

The power configuration for A.6.3.2.2.3 is identical to the 4-step RACH test case A.6.3.2.2.1. The same TT analysis performed for RACH_1 (see Section 8 of TR 38.903) is applicable. There is no need to repeat the TT analysis here.

# 5. Calculation of Test Requirements for first MsgA preamble Absolute power

The configuration of the relevant parameters (preamble received target power, accuracy limits, etc.) are identical to the A.6.3.2.2.1 4-step RACH test case. The same TT analysis performed for RACH_1 (see Section 8 of TR 38.903) is applicable. There is no need to repeat the TT analysis here.

# 5A. Calculation of Test Requirements for first MsgA PUSCH Absolute power

The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1.

### 6.3.4	Power control

#### 6.3.4.1	General

The requirements on power control accuracy apply under normal conditions.

#### 6.3.4.2	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error.

The minimum requirement specified in Table 6.3.4.2-1 apply in the power range bounded by the minimum output power as specified in clause 6.3.1 and the maximum output power as specified in clause 6.2.1.

Table 6.3.4.2-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

The UE absolute power tolerance includes any effect of RSRP inaccuracy, so the UE RSRP reporting accuracy is not used in the calculation.

When the test system uncertainties and the UE Absolute power tolerance are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE Absolute power tolerance. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

The total uncertainty for this requirement is (±8.0 dB±9.0 dB ±2.1dB) = ± 19.1 dB, note that in the UE calculation of MsgA PUSCH power is directly dependent on RSRP, so any uncertainty in the test system downlink signal level will directly affect the MsgA PUSCH power.

# 6. Calculation of Test Requirements for subsequent PRACH preambles Relative power

The UE Relative power tolerance is given in TS 38.101-1:

#### 6.3.4.3	Relative power tolerance

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is larger than 20ms.

The minimum requirements specified in Table 6.3.4.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 6.3.1 and the measured PUMAX as defined in sub-clause 6.2.1.

Table 6.3.4.3-1: Relative power tolerance

| Power step P (Up or down) (dB) | All combinations of PUSCH and PUCCH transitions (dB) | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames (dB) | PRACH (dB) |
| --- | --- | --- | --- |
| ΔP < 2 | ± 2.0 (NOTE) | ± 2.5 | ± 2.0 |
| 2 ≤ ΔP < 3 | ± 2.5 | ± 3.5 | ± 2.5 |
| 3 ≤ ΔP < 4 | ± 3.0 | ± 4.5 | ± 3.0 |
| 4 ≤ ΔP ≤ 10 | ± 3.5 | ± 5.5 | ± 3.5 |
| 10 ≤ ΔP < 15 | ± 4.0 | ± 7.0 | ± 4.0 |
| 15 ≤ ΔP | ± 5.0 | ± 8.0 | ± 5.0 |
| NOTE: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods: for a power step ΔP ≤ 1 dB, the relative power tolerance for transmission is ± 0.7 dB. |  |  |  |

For MsgA transmission, the power is calculated based on section 5.1.2 of TS 38.321:

1>	select the value of DELTA_PREAMBLE according to clause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to msgA-PreambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

The value of the DELTA_PREAMBLE:

## 7.3	DELTA_PREAMBLE values

The DELTA_PREAMBLE preamble format based power offset values are presented in Tables 7.3-1 and 7.3-2.

Table 7.3-1: DELTA_PREAMBLE values for long preamble formats.

| Preamble Format | DELTA_PREAMBLE values |
| --- | --- |
| 0 | 0 dB |
| 1 | -3 dB |
| 2 | -6 dB |
| 3 | 0 dB |

Table 7.3-2: DELTA_PREAMBLE values for short preamble formats.

| Preamble Format | DELTA_PREAMBLE values (dB) |
| --- | --- |
| A1 | 8 + 3 × μ |
| A2 | 5 + 3 × μ |
| A3 | 3 + 3 × μ |
| B1 | 8 + 3 × μ |
| B2 | 5 + 3 × μ |
| B3 | 3 + 3 × μ |
| B4 | 3 × μ |
| C0 | 11 + 3 × μ |
| C2 | 5 + 3 × μ |

where μ is the sub-carrier spacing configuration determined by msg1-SubcarrierSpacing or msgA-SubcarrierSpacing, as specified in TS 38.331 [5] and Table 4.2-1 in TS 38.211 [8], and the preamble formats are given by prach-ConfigurationIndex or msgA-PRACH-ConfigurationIndex, as specified in TS 38.331 [5] and Tables 6.3.3.2-2 to 6.3.3.2-4 in TS 38.211 [8].

From TS 38.211, Preamble format is A1 (TS 38.211 Table 6.3.3.2-2 for msgA-prachConfigurationIndex 102), therefore deltaPreamble = 8 + 3xμ = 8dB for 15 kHz, 11dB for 30kHz.

FR1 MsgA configuration 1 specifies msgA-powerRampingStep as 2dB, so the UE PRACH step tolerance is ±2.5dB. The test system uncertainty value for a relative power measurement on the uplink is ±0.7 dB.

The test system uncertainties is added arithmetically to the UE Relative power tolerance. If the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

Normal conditions = (±2.5 dB ±0.7dB) = ± 3.2 dB

# 7. Calculation of Test Requirements for Transmit Timing

The UE Timing requirement is given in TS 38.133:

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Uplink SCS is assumed to be the same as the DL SCS. Tc is defined in TS 38.211:

## 4.1	General

Throughout this specification, unless otherwise noted, the size of various fields in the time domain is expressed in time units  where  Hz and . The constant  where ,  and .

Check limits:

Tightest requirement for FR1 in NR is 7*64*Tc = 448*Tc, and the measurement uncertainty has been agreed for 4.3.2.2.1 as 112*Tc.

Formula: Upper limit + TT, Lower limit – TT:

In Configuration 1, selecting 15kHz SCS: ± 12*64*Tc ± 112*Tc gives ± 880*Tc

In Configuration 2, selecting 30kHz SCS: ± 8*64*Tc ± 112*Tc gives ± 624*Tc

# 8. Treatment of other configurations within the same test case

TS 38.533 test case 6.3.2.2.3 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- Noc level, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Cell 1 SSB_RP index 0 minimum requirement, section g) and associated comment

- Freq 1 Io minimum requirement, section g) and associated comment

# 9. Treatment of related test cases

38.533 also contains a similar SA non-contention based test case in clause 6.3.2.2.4, which has the same signal levels and the same signalled parameters related to level, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

38.533 also contains similar EN-DC test cases in clauses 4.3.2.2.3 and 4.3.2.2.4, which have the same NR signal levels and the same signalled parameters related to level, so we propose to apply the same level uncertainties and Test Tolerances. There is no need to provide a tab on the spreadsheet as all the numeric values are identical.

TS 38.533 also contains similar test cases 16.3.2.2.6/16.3.2.2.8 for 2 Rx RedCap UEs in clause 16.3.2. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 2 Rx share the same requirements according to 38.133. For these 2 Rx RedCap test cases:

- Test parameters for config 1/4 are identical with that of 6.3.2.2.3 config 1 with the exception that HD-FDD duplex mode is used in config 4. Then the same test tolerances for 6.3.2.2.3. config 1 apply considering duplex mode bringing no any additional measurement uncertainties.

- Test parameters for config 2 are identical with that of 6.3.2.2.1 config 1 with the exception that TDD duplex mode is used in config 2. As a result, the DELTA_PREAMBLE value shall be 5dB rather 8dB according to 38.321 cl.7.3. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 6.3.2.2.3 config 1:

  - DELTA_PREAMBLE, section a) and associated comment.

From the analysis one can observe that the only difference is that the test requirement of the MsgA preamble power shall become:

PPRACH = Preceived,target (-120) +Δpreamble (5) + PSSB  (-5) – PL-RS RSRP (-95) = -25 [dBm]

The same test tolerances for 6.3.2.2.3 config 1 apply.

- Test parameters for config 3 are identical with that of 6.3.2.2.3 config 2 with the exception that the BW is only 20MHz (51 RBs). As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 6.3.2.2.3 config 2:

  - Channel bandwidth and RBs in Channel bandwidth, section a) and associated comment

From the analysis one can observe that the only difference is that Io value is 10*(log106/51) = 3.18dB lower. Then the same test tolerances for 6.3.2.2.3 config 3 apply considering that minimum Io requirement is satisfied with a margin much larger than 3.18dB.