# Title:	Test Tolerances for 1Rx RedCap inter-frequency SS-SINR relative accuracy tests in FR1

Source:			Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for 1Rx RedCap inter-frequency SS-SINR relative measurement accuracy tests for FR1.

The test cases in question, from TS 38.533, are:

- 16.7.3.3.2, NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v17.12.0 Annex A.

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE

A.16.7.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1A.12.1.

A.16.7.3.3.2	Test parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.16.7.3.3.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.16.7.3.3.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.16.7.3.3.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.16.7.3.3.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 | freq2 |  | freq1 |  |  | freq2 |  | freq1 |  | freq2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 4 |  |  | HD-FDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 |  | - |  | 0 | - |  | 0 |  | - |  |  |
| TRS configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  |  | - |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  |  | - |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,4 |  | ms | - |  | 3 |  | - | 3 |  | - |  | 3 |  |  |
|  |  | Config 2,3 |  | s | - |  | 3 |  | - | 3 |  | - |  | 3 |  |  |
| SMTC configuration |  | Config 1,4 |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4 |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.1 RedCap FR1 |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 |  | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,3,4 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | -119.5 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -116.5 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
| Note2 | Config 1,2,4 |  |  | dBm/SCS | -88 |  |  | -108.5 |  |  |  |  | Same as Noc for 15kHz |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  |  | -105.5 |  |  |  |  | -116.5 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  |  | 20 |  | -4.0 |  |  | -4.0 |
|  |  |  |  | dB | -1.75 |  |  | 20 |  |  |  |  | -4.0 |  |  |  |
| SS-RSRP Note3 | Config 1,2,4 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  |  | -88.5 |  |  |  |  | -123.5 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -123 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -122.5 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -122 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -121.5 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -121 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -120.5 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  |  | -85.5 |  |  |  |  | -120.5 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -119.5 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
| SS-SINRNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  |  | 20 |  |  |  |  | -4.0 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  |  | -60.5 |  |  |  |  | -90.09 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -89.59 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -89.09 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -88.59 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -88.09 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -87.59 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -87.09 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -86.59 |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 18.36MHz | -54.87 |  |  | -57.57 |  |  |  |  | -87.97 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -87.47 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -86.97 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -86.47 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -85.97 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -85.47 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -84.97 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -84.47 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x1 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.16.7.3.3.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1A.12.1.1 and 10.1A.12.1.2.

# 3. Discussion

The test case has 3 subtests: Test 1, Test 2 and Test 3. The subtests are designed to test the UE at three points in the level range over which the SS-SINR accuracy requirement applies:

- Test 1 applies at the highest total power and interference, where the reporting accuracy is +/-4.5dB, Io just below -50dBm and SSB Es/Iot >-3dB

- Test 2 applies at a very high SNR, where the reporting accuracy is +/-4.5dB, Io below -50dBm and SSB Es/Iot  > -3dB

- Test 3 applies at the lowest power where the reporting accuracy is +/-5dB, Io just above minimum requirement and SSB Es/Iot >-4dB.

The UE measures and reports the Relative SS-SINR of Cell 2.

There are 4 test configurations according to Table A.16.7.3.3.2-1. Test Configurations 1,2,4 have identical power settings, so they will be analysed together. Test Configuration 3 has different power settings and it will be analysed in a separate tab in the accompanying excel sheet.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.4.7.3.2.2-2 and A.10.1.14.1.2-1 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. Signalled parameters including the threshold are also copied. For this test, the key parameters are:

- Noc at frequency 1

- Noc at frequency 2

- Cell 1 Es / Noc

- Cell 2 Es / Noc

All other parameters such as SS-RSRP, SS-SINR, Es/Iot, etc. are device and not independently settable by the test system.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

The derived parameter SS-SINR is calculated according to TS 38.215.

c) Uncertainties

The Test system provides 2 cells on 2 frequencies, with AWGN. We propose to control the following parameters:

- AWGN absolute power on carrier frequency 1, Noc uncertainty: ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- AWGN absolute power on carrier frequency 2, Noc uncertainty: ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 ±0.3 dB averaged over BWConfig

- Ratio of cell 1 signal / AWGN, Ês1 / Noc1 ±0.3 dB for PRBs #RBJ-RBJ+19

- Ratio of cell 2 signal / AWGN, Ês2 / Noc2 ±0.3 dB averaged over BWConfig

- Ratio of cell 2 signal / AWGN, Ês2 / Noc2 ±0.3 dB for PRBs #RBJ-RBJ+19

The uncertainties appear in section c) of the accompanying spreadsheet.

In this test the UE measures the SS-SINR of Cell 2 over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19.

In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

Two sets of parameters are therefore given. The set averaged over the configured bandwidth have similar values to those already used for other tests. The set averaged over PRBs # RBJ-RBJ+19 have wider values, but constraining the deviation enough not to widen the SS-SINR reporting range too much.

We note also that the outcome of this test is a range of allowed SS-SINR values reported by the UE. The UE reporting accuracy is therefore taken into account when determining the test limits, although it is not an uncertainty of the test system itself.

This choice forms a minimum set (separately for PRBs #RBJ-RBJ+19, and for “averaged over BWConfig“), so the superposition principle can be applied if necessary.

The absolute level of Noc directly affects the UE’s SS-SINR measurement, and is specified for frequency bands up to 6GHz.

The UE measures the SS-SINR and reports it. The relative measurement accuracy is given in TS 38.133:

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1

The accuracy requirements in clause 10.1.14.1.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.12.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1A.12.1.2-1: SS-SINR Inter frequency relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 5 | 5 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The relative accuracy requirement refers to SS-SINR side conditions in TS 38.133 Annex B.2.16:

B.2.16	Conditions for NR inter-frequency measurements

This clause defines the following conditions for RedCap NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.16-1 and Table B.2.16-2 for 1 Rx and 2 Rx RedCap for FR1 NR cells.

The conditions are defined in Table B.2.16-3 for FR2 NR cells.

Table B.2.16-1: Conditions for inter-frequency measurements in FR1 for 1 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_RC_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_RC_FR1_C | -124 | -121 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -123 | -120 |  |
|  | NR_FDD_RC_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_G | -122 | -119 |  |
|  | NR_FDD_RC_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The side conditions to satisfy are:

-	Cell 1 and Cell 2 SSB Ês/Iot ≥ -3dB for sub-test 1,2 and SSB Ês/Iot ≥ -4dB for sub-test 3.

-	Cell 1 and Cell 2 SSB_RP ≥ -125+ΔBG_offset dBm/15kHz;

-	Freq 1 Io and Freq 2 Io ≥ -121+ΔBG_offset dBm/15kHz;

-	Freq 1 Io and Freq 2 Io < -50dBm/Ch BW.

d) Controlled parameters critical to verdict

In many RRM test cases, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-SINR reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each cell they are the SSB Es/Iot range, the SSB_RP range, and the Io power range, over which the UE meets the specified SS-SINR reporting accuracy. The 11 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

The uncertainty in UE reporting accuracy is also listed in a separate row. The range of reported results is affected by both the Test system stimulus setting uncertainty and the UE reporting accuracy.

In general all the values are listed for Test 1, Test 2 and all the variants of Test 3.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc1 on frequency 1 would cause 1dB error in the Cell 1 RSRP, so the sensitivity factor is 1.000 for all tests. However the same error of 1dB in the absolute AWGN level Noc1 would cause no change to Cell 2 Es/Iot, because Cell 2 is on a different frequency, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Io which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

For example 1:

- In Test 1, the effect of Cell 1 Es/Noc uncertainty on Frequency 1 Io  is x 0.401

These factors can also be derived intuitively, by looking at the pie chart for Test 1 in section 3. For example, Cell 1 forms 40.1% of the total Io. A change in the power of Cell 1 Es/Noc alone is diluted in the overall interference.

For example 2:

- In Test 1, the effect of Cell 2 Es/Noc uncertainty on Cell 2 RSRQ is 0.599

It can be derived intuitively by looking at the pie chart for Test 1 in section 3. For example , Cell 2 SS-SINR = Cell 2 SSB Es/Iot*. The sensitivity factor of Cell 2 Es/Noc to cell 2 Es is 1, Cell 2 Es/Noc to Io is 0.401, thus Cell2 Es/Noc to Cell2 SS-SINR is (1-0.401) = 0.599.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903. Uncertainties are calculated separately for Tests 1, 2 and all the variants of Test 3.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE SS-SINR reporting accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE SS-SINR reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

In all test configurations, for Test 3, we observe that the Cell 2 SSB Es/Iot is very close to the lower limit of -4dB, and that the test equipment uncertainties could take the UE outside the allowed range for the side condition in B.2.3. The Cell 2 Es/Noc is therefore increased in order to achieve Cell 2 SSB Es/Iot > -4dB. The same is true for Cell 1 SSB Es/Iot, so the Cell 1 Es/Noc has been increased as well, even though this may not affect directly the purpose of the test.

The rest of the parameters are within the allowed ranges

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

The Cell 1 Es/Noc has been increased by 0.35 dB to ensure that Cell 1 SSB Es/Iot stays within range. The Cell 2 Es/Noc has been increased by 0.35 dB to ensure that Cell 2 SSB Es/Iot stays within range.

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as at least some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (SSB Es/Iot range, SSB_RP power range, Io power range and SS-SINR range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

For this test, the verdict is based on the SS-SINR values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-SINR reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SS-SINR values that a conformant UE could report.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 16.7.3.3.2 also contains configurations with different subcarrier spacing. This additional configuration has been treated in a separate tab on the spreadsheet. The corresponding offsets for Test Configuration 3 have already been detailed in section e and f of this document.

From the analysis one can observe that same test tolerances also apply to 16.7.3.3.2 config 3 sub-test 3.